3GPP TSG RAN WG1 #109-e		R1-2203359

e-Meeting, May 9th – 20th, 2022

Title: 	Maintenance on Enhancements to IAB

Source: 	ZTE, Sanechips

Agenda item:	8.10

Document for:	Discussion and decision

Introduction

This contribution discusses some remaining issue and provides some text proposals on enhancements to the IAB resource multiplexing in Rel-17 IAB.

Discussions

Remaining issue about H/S/NA configuration

The following working assumption has been made in RAN1#106bis-e on H/S/NA configuration for DU cell.

Working Assumption(RAN1#106bis-e) If both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, one of the following is selected: - Alt. 1: An IAB node applies the frequency domain H/S/NA only if the IAB node is currently operating in a non-TDM multiplexing mode in the slot, otherwise the Rel-16 time domain H/S/NA configuration is applied.

The above working assumption was captured in 38.213[2].

If an indication of hard, soft, or unavailable type is provided for an RB set in a symbol of a slot, the IAB-DU applies the configuration of hard, soft, or unavailable type provided by Rel-17 frequency-domain IAB-DU-Resource-Configuration-H/S/NA-Config [16, TS 38.473] when the IAB-node uses simultaneous transmission and reception in the slot.

However, although the above working assumption has been in discussion for many times, it has not been confirmed because there may be inconsistent understanding (i.e. conflict) between IAB nodes or between IAB node and its donor CU when whether or not an IAB node can operate under a given non-TDM multiplexing mode is left to IAB implementation in Rel-17 according to the following agreement in RAN1#107-e. For example, it is difficult to make a consistent understanding on the selected H/S/NA configuration on the IAB-node DU among the IAB-node, its parent node(s), and its donor CU. This may cause the parent node to be unable to make appropriate decisions on the available resources of IAB-MT. In addition, semi-static H/S/NA configurations would be coordinated for the purpose of CLI and/or resource multiplexing in DC. It is impossible/difficult to make the peer node/neighbour node know the dynamic selected HSNA configuration at the IAB node.

Agreement(RAN1#107-e) Whether or not an IAB node can operate under a given non-TDM multiplexing mode (i.e. multiplexing info in 38.473) is left to IAB implementation in Rel-17

Dynamic H/S/NA configuration selection between Rel-17 frequency domain H/S/NA configuration and Rel-16 time domain H/S/NA configuration may lead to inconsistent understanding of H/S/NA types applied on an IAB-node DU among the IAB-node(s), its parent node(s), and its donor CU.

Dynamic H/S/NA configuration selection between Rel-17 frequency domain H/S/NA configuration and Rel-16 time domain H/S/NA configuration will not play a good role in resource coordination in case of DC, and CLI management.

For H/S/NA configuration, one conclusion made in RAN1#108-e meeting [1] is as follows:

Conclusion Defer discussion (contribution driven) about potential specification impact for conflict resolution between parent and child IAB nodes in case the Rel-17 H/S/NA configuration is provided but the IAB node needs to instead operate according to the Rel-16 H/S/NA configuration (based on its implementation as agreed previously in RAN1#107-e) in the slot until RAN#109-e.

Firstly, it was agreed in RAN1#107-e that whether or not an IAB node can operate under a given non-TDM multiplexing mode (i.e. multiplexing info in 38.473) is left to IAB implementation in Rel-17 as shown above, but there is no conclusion that whether Rel-17 H/S/NA configuration or Rel-16 H/S/NA configuration is applied is left to IAB implementation.

Secondly, for conflict resolution, FL summary in the last meeting RAN1#108-e provides the following 4 alternatives:

- Alt. 1: TDM fallback / FDM resume indication from child node (e.g. MAC CE)

- Alt. 2: An IAB node does not expect conflicting Rel-16 and Rel-17 H/S/NA configurations for a given RB set

- Alt. 3: A prioritization rule for TDM H/S/NA vs. FDM H/S/NA configurations for a given RB set is specified

- Alt. 4: No additional specification needed

For Alt. 1, since whether or not an IAB node can operate under a given non-TDM multiplexing mode is left to IAB implementation, i.e. IAB-DU cell switches dynamically between non-TDM and TDM multiplexing mode, it is difficult to solve the conflict. In addition, Alt.1 will result in additional signalling design for RAN2/3.

For Alt. 2 and Alt. 3, the solution is not clear. For Alt. 4, there is no additional specification needed if “when the IAB-node uses simultaneous transmission and reception in the slot” in TS38.213[2] is deleted.

Therefore, we suggest that Rel-17 frequency domain H/S/NA configuration, if provided, always overrides Rel-16 time domain H/S/NA configuration.

If both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, Rel-17 frequency domain H/S/NA configuration is applied.

Adopt the following Text proposal on H/S/NA configuration in TS38.213:

If an indication of hard, soft, or unavailable type is provided for an RB set in a symbol of a slot, the IAB-DU applies the configuration of hard, soft, or unavailable type provided by Rel-17 frequency-domain IAB-DU-Resource-Configuration-H/S/NA-Config [16, TS 38.473] when the IAB-node uses simultaneous transmission and reception in the slot.

For H/S/NA configuration, another conclusion made in RAN1#108-e meeting [1] is as follow:

Conclusion Defer discussion (contribution driven) about potential specification impact if the configured RB sets-of an IAB-DU HSNA resource configuration do not cover the entire carrier bandwidth until RAN1#109-e.

Since Rel-16 HSNA resource configuration for an IAB-DU cell is always provided for the whole carrier, Rel-16 HSNA resource configuration can apply to the RBs which does not belong to any RB set if the configured RB sets of an IAB-DU HSNA resource configuration do not cover the entire carrier bandwidth.

If the configured RB sets of an IAB-DU HSNA resource configuration do not cover the entire carrier bandwidth, Rel-16 HSNA resource configuration can be adopted by the RBs which does not belong to any RB set.

Remaining issue about DCI format 2_5

For DCI format 2_5, two conclusion made in RAN1#108-e meeting [1] are as follows:

Conclusion Defer discussion (contribution driven) about extending the maximum payload size of DCI format 2_5 to increase the number of IAB-DU cells that can be provided with availability information for soft resources until RAN1#109-e. Conclusion Defer discussion (contribution driven) of potential issues/solutions for multiple DCI Format 2_5 indications with overlapping durations until RAN1#109-e.

In our view, these two issues are optimization issues, and their benefit is less obvious. Therefore, we propose:

For the maximum payload size of DCI format 2_5, Rel-16 mechanism is reused.

For multiple DCI Format 2_5 indications with overlapping durations, Rel-16 mechanism is reused.

Text proposal about the description of resourceAvailabilty in TS38.213

In RAN1#108-e [1], the following agreement is made. However, highlighted yellow text “If an RB set group is not provided, only one resourceAvailablity-Rel17 is included in AvailabilityCombination-Rel17 to indicate availability of soft resources in one or multiple slots for all RB sets of a DU cell” has not been captured in TS38.213[2] as shown in Table 1.

Agreement Enhance the RRC signaling for the configuration of DCI Format 2_5 to include the configuration of availability indication for soft resources in multiple RB set groups by introducing the following new RRC parameters: - AvailabilityCombination-Rel17 to include multiple resourceAvailaibity-Rel17 indications, where each resourceAvailaibity-Rel17 indicates the availability of soft resources in one or multiple slots for each configured RB set group in sequence. - The RB set groups are configured for all availabilityCombinationId(s) with the following parameters and are applied over each slot: - Number of RB set groups. - FFS: max value - Number of RB sets for each group. - FFS: max value - If an RB set group is not provided, only one resourceAvailablity-Rel17 is included in AvailabilityCombination-Rel17 to indicate availability of soft resources in one or multiple slots for all RB sets of a DU cell.

Table 1. availability indication in TS38.213

- a set of availability combinations by availabilityCombinations, where each availability combination in the set of availability combinations includes - resourceAvailability indicating availability of soft symbols in one or more slots for the IAB-DU cell, or one or multiple resourceAvailability with each resourceAvailablity indicating availability of soft resources in one or more slots for one RB set group where one RB set group includes one or multiple RB sets, and

Therefore, we propose to capture above agreement in TS38.213:

Adopt the following Text proposal on resourceAvailabilty in TS38.213:

- a set of availability combinations by availabilityCombinations, where each availability combination in the set of availability combinations includes - resourceAvailability indicating availability of soft symbols in one or more slots for the IAB-DU cell, or one resourceAvailability indicating availability of soft resource of the all RB sets in one or more slots for the IAB-DU cell, or one or multiple resourceAvailability with each resourceAvailablity indicating availability of soft resources in one or more slots for one RB set group where one RB set group includes one or multiple RB sets, and

Text proposal about the description of restricted beam in TS38.213

TS38.213 [2] provides the description of restricted beam for an IAB-DU cell, as shown in Table 3. We can observe from the highlighted yellow text that restricted beam is only applied to non-TDM multiplexing mode. However, we have no agreements about this. In addition, there are some benefits for CLI if restricted beam is also applied to TDM multiplexing mode. E.g. parent node can provide restricted beam for IAB-DU cell if the reception of the parent node is interfered by the transmission from IAB-DU cell.

Table 3. the description of restricted beam in TS38.213

The IAB-node can be provided by the parent node a set of RS resource indexes that indicate quasi co-location properties of an IAB-DU cell where simultaneous transmission/reception from the IAB-MT and transmission from the IAB-DU cells is restricted by Child IAB-DU Restricted Beam Indication MAC CE as described in [11, TS 38.321]. The IAB-DU does not transmit on a cell if the IAB node is operating in a non-TDM multiplexing mode using an indicated RS resource index on a symbol or RB set configured as soft in an IAB-DU cell - when it is not indicated as available by resourceAvailability

There is no agreement that restricted beam is only applied to non-TDM multiplexing mode.

There are some benefits for CLI if restricted beam is also applied to TDM multiplexing mode.

Adopt the following Text proposal on restricted beam in TS38.213:

The IAB-node can be provided by the parent node a set of RS resource indexes that indicate quasi co-location properties of an IAB-DU cell where simultaneous transmission/reception from the IAB-MT and transmission from the IAB-DU cells is restricted by Child IAB-DU Restricted Beam Indication MAC CE as described in [11, TS 38.321]. The IAB-DU does not transmit on a cell if the IAB node is operating in a non-TDM multiplexing mode using an indicated RS resource index on a symbol or RB set configured as soft in an IAB-DU cell - when it is not indicated as available by resourceAvailability

Conclusion

According to the discussion above, we provide the following proposals:

Observation 1:	Dynamic H/S/NA configuration selection between Rel-17 frequency domain H/S/NA configuration and Rel-16 time domain H/S/NA configuration may lead to inconsistent understanding of H/S/NA types applied on an IAB-node DU among the IAB-node(s), its parent node(s), and its donor CU.

Observation 2:	Dynamic H/S/NA configuration selection between Rel-17 frequency domain H/S/NA configuration and Rel-16 time domain H/S/NA configuration will not play a good role in resource coordination in case of DC, and CLI management.

Observation 3:	There is no agreement that restricted beam is only applied to non-TDM multiplexing mode.

Observation 4:	There are some benefits for CLI if restricted beam is also applied to TDM multiplexing mode.

Proposal 1: If both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, Rel-17 frequency domain H/S/NA configuration is applied.

Proposal 2: Adopt the following Text proposal on H/S/NA configuration in TS38.213:

Proposal 3: If the configured RB sets of an IAB-DU HSNA resource configuration do not cover the entire carrier bandwidth, Rel-16 HSNA resource configuration can be adopted by the RBs which does not belong to any RB set.

Proposal 4: For the maximum payload size of DCI format 2_5, Rel-16 mechanism is reused.

Proposal 5: For multiple DCI Format 2_5 indications with overlapping durations, Rel-16 mechanism is reused.

Proposal 6: Adopt the following Text proposal on resourceAvailabilty in TS38.213:

-	a set of availability combinations by availabilityCombinations, where each availability combination in the set of availability combinations includes

-	resourceAvailability indicating availability of soft symbols in one or more slots for the IAB-DU cell, or one resourceAvailability indicating availability of soft resource of the all RB sets in one or more slots for the IAB-DU cell, or one or multiple resourceAvailability with each resourceAvailablity indicating availability of soft resources in one or more slots for one RB set group where one RB set group includes one or multiple RB sets, and

Proposal 7: Adopt the following Text proposal on restricted beam in TS38.213:

The IAB-node can be provided by the parent node a set of RS resource indexes that indicate quasi co-location properties of an IAB-DU cell where simultaneous transmission/reception from the IAB-MT and transmission from the IAB-DU cells is restricted by Child IAB-DU Restricted Beam Indication MAC CE as described in [11, TS 38.321]. The IAB-DU does not transmit on a cell if the IAB node is operating in a non-TDM multiplexing mode using an indicated RS resource index on a symbol or RB set configured as soft in an IAB-DU cell

-	when it is not indicated as available by resourceAvailability

References

Chair's Notes RAN1#108-e

TS38.213 V17.1.0 2022.03