3GPP TSG RAN WG1 #109-e		R1-2203360

e-Meeting, May 8th – May 20th ,2022

Title: 	Maintenance on resource allocation for power saving

Source: 	ZTE, Sanechips

Agenda item:	8.11.1

Document for:	Discussion and decision

Introduction

The following agreements were achieved in RAN1#108-e.

# Agreement

The lower bound of M value for CPS in the case of periodic transmission (contiguousSensingWindowPeriodic) for both resource (re)selection and re-evaluation / pre-emption checking is a non-zero value (lower bound for M is 5)

Note: CATT indicated that they do not agree to the technical benefits of this agreement

# Agreement

When a UE is triggered to perform re-evaluation and pre-emption checking for aperiodic transmission (Prsvp_TX=0) in slot n and the minimum M slots for CPS cannot be guaranteed,

- UE senses in all available slots starting from the resource (re)selection trigger slot of the same TB to Tproc,0SL+Tproc,1SL slots earlier than tyiSL.

  - The UE re-evaluation and pre-emption checking is based on all available sensing results after n-T0

# Conclusion

# The existing Step 5 and 5a are applicable for UE configured for partial sensing by its higher layer.

# Agreement

In Step 6 c) of TS38.214 Section 8.1.4, when UE is configured with partial sensing by its higher layer, adopt the following changes:

- t'n'SL=tyiSL-Tproc,1SL if slot tyiSL-Tproc,1SL belongs to the set t'0SL,t'1SL,...,t'T'max-1SL, otherwise, slot t'n'SL is the first slot after slot tyiSL-Tproc,1SL belonging to the set t'0SL,t'1SL,...,t'T'max-1SL.

- Option D: Tscal=tyLSL-(tyiSL-Tproc,1SL) converted to milliseconds, where slot tyLSL is the last slot of the Y or Y’ candidate slots.

- Slot tyiSL is the first slot of the selected/remaining set of Y or Y’ candidate slots.

This paper elaborates our consideration on Rel-17 SL maintenance.

Discussions

Consideration on the update for Q value in sensing procedure with additional sensing occasion enabled

Figure. 1

The Q value in step 6c) of sensing procedure was heatedly discussed during RAN1#108-e. The motivation for introducing the update of Q lies in the situation where only the additional sensing occasion (in gray) can be detected (Prsvp_RX'<n'-m≤2Prsvp_RX') as slot m. As shown in Figure.1, to loop through the Q occasions with the detection of the most recent occasion, q needs to start from 2 and ends at Q=TscalPrsvp_RX+1 regardless of whether Tscal is larger or smaller than Prsvp_RX

For the case with the most recent occasion (n'-m≤Prsvp_RX') detected including the case when both most recent (in blank) and additional sensing occasion, the Q value together with the text from Rel-16 can be re-used regardless of whether Tscal is larger or smaller than Prsvp_RX

When neither the most recent nor the additional sensing occasion is detected, Q may take either 1 or 2 and Rel-16 spec. is preferred unchanged for simplicity.

Adopt the following TP to TS 38.214 to update the Q value in the sensing procedure

- if UE is configured with partial sensing, Q=TscalPrsvp_RX  if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX', Q=TscalPrsvp_RX+1 if Prsvp_RX'<n'-m≤2Prsvp_RX' where t'n'SL = t'yiSL-Tproc,1SLif slot t'yiSL-Tproc,1SL belongs to the set t'0SL,t'1SL,...,t'T'max-1SL, otherwise slot t'n'SL is the first slot after slot t'yiSL-Tproc,1SL  belonging to the set t'0SL,t'1SL,...,t'T'max-1SL; otherwise Q=1.

Conclusion

- Adopt the following TP to TS 38.214 to update the Q value in the sensing procedure

- if UE is configured with partial sensing, Q=TscalPrsvp_RX  if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX', Q=TscalPrsvp_RX+1 if Prsvp_RX'<n'-m≤2Prsvp_RX' where t'n'SL = t'yiSL-Tproc,1SLif slot t'yiSL-Tproc,1SL belongs to the set t'0SL,t'1SL,...,t'T'max-1SL, otherwise slot t'n'SL is the first slot after slot t'yiSL-Tproc,1SL  belonging to the set t'0SL,t'1SL,...,t'T'max-1SL; otherwise Q=1.

References

- “RAN1 Chair’s Notes”, 3GPP TSG RAN WG1 #108-e, e-Meeting, Feb 21st – Mar. 3rd, 2022.