3GPP TSG RAN WG1 #109-e		R1-2203364

e-Meeting, May 8th – May 20th ,2022

Title: 	On UE features for NR sidelink enhancement

Source: 	ZTE, Sanechips

Agenda item:	8.16.11

Document for:	Discussion and decision

Introduction

During RAN1#108-e, the following was achieved.

Agreement

- For the synchronization sources for NR sidelink transmission for FGs 32-4/4a,

  - Add a new FG 32-4b for synchronization sources for NR sidelink transmission as follows

    - eNB parts are included as another FG, FFS detail

| 32. NR_SL_enh | 32-4b | Synchronization sources for NR sidelink transmission | 1) UE supports GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. 2) UE can transmit NR sidelink based on the synchronization to an gNB 3) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb if the UE supports Components 1 and 2 4) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true if the UE supports Components 1 and 2. 5) UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3 or 32-4 or 32-4a 6) UE supports SyncRef UE as the synchronization reference and can receive S-SSB in NR sidelink if it supports 15-1. |  | Yes | No |  | Per band | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 1 is only required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Components 2/3/4 are not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- FG 32-5a is split into one FG for Tx capability and another for Rx capability as follows

  - FFS whether/how the FGs are split further

| 32. NR_SL_enh | 32-5a-1 | Transmitting Inter-UE coordination scheme 1 in NR sidelink mode 2 | 1) UE can transmit and receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit and receive an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set]. FFS whether/how to split FG 32-5a-1 into multiple FGs | [TBD] | [Yes] | [Yes] | UE does not support transmitting inter-UE coordination scheme 1 in NR sidelink mode 2. UE does not receive an explicit request for inter-UE coordination information | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-5a-2 | Receiving Inter-UE coordination scheme 1 in NR sidelink mode 2 | 1) UE can receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can receive transmit an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set]. FFS whether/how to split FG 32-5a-2 into multiple FGs | [TBD] | [Yes] | [Yes] | UE does not support receiving inter-UE coordination scheme 1 in NR sidelink mode 2. UE does not transmit an explicit request for inter-UE coordination information | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. |

Agreement

- Add FG 32-6: Reception of Scheme 1 inter-UE coordination information over 2nd SCI

| 32. NR_SL_enh | 32-6 | Reception of Scheme 1 inter-UE coordination information over 2nd SCI | 1) UE can receive Scheme 1 inter-UE coordination transmission over 2nd SCI that is used in addition to the MAC-CE carrying the same inter-UE coordination information in the same transmission. FFS whether to separate capabilities for preferred/non-preferred resource set and explicit request |  | No | NoYes | UE is not required to decode SCI 2-C for inter-UE coordination information | Per FS | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- Add FG 32-7: Determination of expected conflict in Scheme 2 based on RSRP difference

| 32. NR_SL_enh | 32-7 | Determination of expected conflict in Scheme 2 based on RSRP difference | 1) UE can determine a conflict for overlapping resource reservation between UE-B and another UE based on RSRP difference of the two reservations | 32-5b-1 | Yes | Yes |  | Per band | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- FG 32-5a-1 is not split into FGs for preferred resource set and another FGs for non-preferred resource set

- FG 32-5a-2 is split into FGs for preferred resource set and another FGs for non-preferred resource set

  - At least one of the split FG 32-5a-2 is added as a prerequisite FG for FG 32-5a-1, FFS which one or both

Agreement

- For the synchronization sources for NR sidelink transmission for FGs 32-4/4a, add another FG 32-4c for eNB type synchronization sources for NR sidelink transmission with FG 32-4b as prerequisite

| 32. NR_SL_enh | 32-4c | eNB type synchronization sources for NR sidelink transmission | 1) UE can transmit NR sidelink based on the synchronization to an eNB. 2) If UE supports component 1 in FG 32-4b, UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 3) If UE supports component 1 in FG 32-4b, UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. | 32-4b | Yes | No |  | Per band | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- Type of FG 32-4b is per band

Agreement

- If UE reports more than one FGs of 15-3, 32-4 and 32-4a, the reported value B in each FG is the total number of SL processes and the same among those FGs.

Agreement

- The column of “Need for the gNB to know if the feature is supported” for FGs 32-4 and 32-4a are “Yes”

Agreement

- The column of “Applicable to the capability signalling exchange between UEs (Sidelink WI only)” for FGs 32-4 and 32-4a are “No”

Agreement

- PSFCH Tx capability is jointly reported for inter-UE coordination and HARQ-ACK reporting as a component in FG 32-5b-1

  - Value range for M is {4, 8, 16}

  - Add following note in FG 32-5b-1

    - If UE reports more than one FGs of 15-11 and 32-5b-1, the reported value M in each FG is the total number and the same among those FGs.

- PSFCH Rx capability is jointly reported for inter-UE coordination and HARQ-ACK reporting as a component in FG 32-5b-2

  - Value range for N is {5, 15, 25, 32, 35, 45, 50, 64}

  - Add following note in FG 32-5b-2

    - If UE reports more than one FGs of 15-11 and 32-5b-2, the reported value N in each FG is the total number and the same among those FGs.

Agreement

- The column of “Need for the gNB to know if the feature is supported” for FGs 32-5a-1, 32-5a-2, 32-5b-1 and 32-5b-2 are “Yes”

Agreement

- The column of “Applicable to the capability signalling exchange between UEs (Sidelink WI only)” for FGs 32-5a-1, 32-5a-2, 32-5b-1 and 32-5b-2 are “Yes”

Agreement

- Define Rel-17 LTE UE feature list for NR sidelink enhancement based on NR Rel-17 FGs

Agreement

- FG 32-6 is split to two FGs as follows

| 32. NR_SL_enh | 32-6-1 | Reception of Scheme 1 inter-UE coordination information over 2nd SCI | 1) UE can receive Scheme 1 inter-UE coordination transmission over 2nd SCI that is used in addition to the MAC-CE carrying the same inter-UE coordination information in the same transmission. FFS whether to separate capabilities for preferred/non-preferred resource set and explicit request | FFS | No Yes | Yes | UE is not required to decode SCI 2-C and the associated PSSCH for inter-UE coordination information | Per FS [Per band] | [N.A]. | [N.A]. | [N.A.] | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-6-2 | Reception of Scheme 1 explicit request over 2nd SCI | 1) UE can receive an explicit request for inter-UE coordination information of both preferred resource set and non-preferred resource set over 2nd SCI that is used in addition to the MAC-CE carrying the explicit request in the same transmission | FFS | Yes | Yes | UE is not required to decode SCI 2-C and the associated PSSCH for explicit request | [Per band] | [N.A.] | [N.A.] | [N.A.] | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |

Agreement

- The column of “Need for the gNB to know if the feature is supported” for FG 32-7 is “No”

Agreement

- Type of FG 32-x is supported as follows

  - FG 32-4: Per FS

  - FG 32-4a: Per band

  - FG 32-4c: Per band

  - FG 32-5a-2: Per band

  - FG 32-5a-3: Per band

  - FG 32-5b-2: Per band

  - FG 32-6-1: Per band

This paper discusses UE features for NR sidelink enhancement taking into account the above as well as [1].

Discussions

Prerequisites

The prerequisite FG/components for 32-4, 32-5a need to be discussed for this meeting.

For 32-4 transmitting NR sidelink mode 2 with partial sensing, given the necessary components for mode 2 Tx have already been captured, the prerequisites should include the basic components related to the functionality of NR sidelink such as 15-1 receiving NR sidelink and 15-4 Synchronization for NR sidelink.

For 32-5a IUC,15-1 and 15-4 should be the prerequisite of IUC reception 32-5a-2/32-5a-3.

For 32-5a-1, in addition to at least one of 32-5a-2 and 32-5a-3, either 32-4b/32-4c or 15-3 needs to be supported as prerequisite for the transmission of IUC information.

The prerequisites to 32-4, 32-5a are captured respectively as below

The prerequisites to 32-4 are 15-1 and 15-4

The prerequisites to 32-5a-2/32-5a-3 are 15-1 and 15-4

The prerequisites to 32-5a-1 are at least one of 32-5a-2 and 32-5a-3, 32-4b/32-4c or 15-3

gNB/eNB/UE knowledge of UE capability

The gNB/eNB/UE knowledge of the capability is mostly settled except for that for the two sub-FGs for 32-2. It's suggested to directly confirm that gNB needs to know the reception capability while the knowledge is not needed from UE perspective.

Confirm that gNB needs to know the 32-2a and 32-2b is supported and UE does not need such knowledge.

Feature Type

The feature types for the following FGs remain unsettled

- FG 32-2a reception of PSFCH

- FG 32-2b reception of S-SSB

- 32-5a-1 Transmitting inter-UE coordination scheme 1 in NR sidelink mode 2

- 32-5b-1 Transmitting inter-UE coordination scheme 2 in NR sidelink mode 2

- 32-6-2 Reception of Scheme 1 explicit request over 2nd SCI

- 32-7 Determination of expected conflict in Scheme 2 based on RSRP difference

The first two feature types can be made per band according to Rel-16 relevant reception feature types.

For the dual features 32-5a-1 and 32-6-2, 32-5b-1 and 32-7, the feature types had better be set the same. Given previous discussions, the reason for making them as per FS instead of per band or per band combination is to make the feature more compatible with the use case with the need to operate in different bands from different band combinations. In the meantime, the Rel-16 relevant Tx feature types are mostly per band, it's proposed thus to consider the below compromised proposal having scheme 1 related Tx per band while scheme 2 per FS or vice versa similar to the handling to the pair of 32-4 and 32-4a.

Adopt the following feature type definition for each FG

FG 32-2a reception of PSFCH is of per band

FG 32-2b reception of S-SSB is of per band

32-5a-1 Transmitting inter-UE coordination scheme 1 in NR sidelink mode 2 is of per band

32-5b-1 Transmitting inter-UE coordination scheme 2 in NR sidelink mode 2 is of per FS

32-6-2 Reception of Scheme 1 explicit request over 2nd SCI is of per band

32-7 Determination of expected conflict in Scheme 2 based on RSRP difference is of per FS

Note on random selection in exceptional resource pool

In 32-4/32-4a, the note on performing SL reception according to Rel-16 random selection in this FG, without the note, it's assumed the UE can support random selection according to Rel-17 configuration or triggering in all the RPs, thus the additional implication shall be UE can support random selection in the exceptional resource pool and thus perform SL communication with Rel-16 SL UE therein. It's suggested to keep the note.

The note in 32-4a 'Note: Random selection according to Rel-16 in the exceptional pool is supported.' is kept.

LTE feature group

Regarding the LTE feature group, as per previous RAN1 agreement that LTE FGs should be set based on those from NR as well as the latest RAN agreement on the split of PSFCH and S-SSB reception FGs, it's proposed to introduce the alignment below, i.e. splitting of the FG 4-2 into two sub-FGs, S-SSB reception and PSFCH reception.

The FG4-2 should be split into two sub-FGs

One sub-FG for support of only S-SSB reception

Another sub-FG for support of PSFCH reception with pre-requisite of S-SSB reception

| 4. NR_SL_enh | 4-2a | Receiving NR sidelink of PSFCH | 1) UE can receive NR PSFCH | 4-2b | Yes | [No] |  | [Per band] | N.A. | N.A. |  | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4-2b | Receiving NR sidelink of S-SSB | 1) UE can receive NR S-SSB |  | Yes | [No] |  | [Per band] | N.A. | N.A. |  | Optional with capability signalling. |

Conclusion

This paper concludes with the following including a proposal on the full UE feature list.

Proposal 1:	The prerequisites to 32-4, 32-5a are captured respectively as below

•	The prerequisites to 32-4 are 15-1 and 15-4

•	The prerequisites to 32-5a-2/32-5a-3 are 15-1 and 15-4

•	The prerequisites to 32-5a-1 are at least one of 32-5a-2 and 32-5a-3, 32-4b/32-4c or 15-3

Proposal 2:	Confirm that gNB needs to know the 32-2a and 32-2b is supported and UE does not need such knowledge.

Proposal 3:	Adopt the following feature type definition for each FG

•	FG 32-2a reception of PSFCH is of per band

•	FG 32-2b reception of S-SSB is of per band

•	32-5a-1 Transmitting inter-UE coordination scheme 1 in NR sidelink mode 2 is of per band

•	32-5b-1 Transmitting inter-UE coordination scheme 2 in NR sidelink mode 2 is of per FS

•	32-6-2 Reception of Scheme 1 explicit request over 2nd SCI is of per band

•	32-7 Determination of expected conflict in Scheme 2 based on RSRP difference is of per FS

Proposal 4:	The note in 32-4a 'Note: Random selection according to Rel-16 in the exceptional pool is supported.' is kept.

Proposal 5:	The FG4-2 should be split into two sub-FGs

•	One sub-FG for support of only S-SSB reception

•	Another sub-FG for support of PSFCH reception with pre-requisite of S-SSB reception

| 4. NR_SL_enh | 4-2a | Receiving NR sidelink of PSFCH | 1) UE can receive NR PSFCH | 4-2b | Yes | [No] |  | [Per band] | N.A. | N.A. |  | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4-2b | Receiving NR sidelink of S-SSB | 1) UE can receive NR S-SSB |  | Yes | [No] |  | [Per band] | N.A. | N.A. |  | Optional with capability signalling. |

Consider the change to the features as below

| c | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-2a | Receiving NR sidelink of PSFCH | 1) UE can receive NR PSFCH | 32-2b | Yes | No |  | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
|  | 32-2b | Receiving NR sidelink of S-SSB | 1) UE can receive NR S-SSH |  | Yes | No |  | Per band |  |  |  |  |  |
| 32. NR_SL_enh | 32-4 | Transmitting NR sidelink mode 2 with partial sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can perform periodic-based partial sensing and resource allocation operation. 5) UE can perform contiguous partial sensing and resource allocation operation. 6) UE can transmit using the subcarrier spacing and CP length defined for a given band in RAN4 8) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 10) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 11) DL pathloss based open loop power control when mode 2 is configured by NR Uu | 15-1,15-4 | Yes | No | UE does not support transmission according to the partial sensing and resource allocation | Per FS | N.A. | N.A. | N.A. | Note: Random selection [according to Rel-16] in the exceptional pool is supported. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Candidate values for B are {8,16} If UE reports more than one FGs of 15-3, 32-4 and 32-4a, the reported value B in each FG is the total number of SL processes and the same among those FGs. Note: Component 6 is not required to be signalled in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Component-6 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-6 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} Component-6 candidate value set for CP length: {NCP,NCP and ECP} (ECP only applies to SCS of 60 kHz) Note: Component 10 is only required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 11 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| 32. NR_SL_enh | 32-4a | Transmitting NR sidelink mode 2 with random resource selection | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with random resource selection configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can transmit using the subcarrier spacing and CP length defined for a given band in RAN4 5) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 6) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 7) DL pathloss based open loop power control when mode 2 is configured by NR Uu | [TBD] | Yes | No | UE does not support transmission according to the random resource selection and resource allocation | Per band | N.A. | N.A. | N.A. | [Note: Random selection according to Rel-16 in the exceptional pool is supported.] Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Candidate values for B are {8,16} If UE reports more than one FGs of 15-3, 32-4 and 32-4a, the reported value B in each FG is the total number of SL processes and the same among those FGs. Note: Component 4 is not required to be signalled in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Component-4 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-4 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} Component-4 candidate value set for CP length: {NCP,NCP and ECP} (ECP only applies to SCS of 60 kHz) Note: Component 6 is only required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 7 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| 32. NR_SL_enh | 32-4b | Synchronization sources for NR sidelink transmission | 1) UE supports GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. 2) UE can transmit NR sidelink based on the synchronization to an gNB 3) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb if the UE supports Components 1 and 2 4) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true if the UE supports Components 1 and 2. 5) UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3 or 32-4 or 32-4a [6) UE supports SyncRef UE as the synchronization reference and can receive S-SSB in NR sidelink if it supports 15-1.] |  | Yes | No |  | Per band | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 1 is only required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Components 2/3/4 are not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| 32. NR_SL_enh | 32-4c | eNB type synchronization sources for NR sidelink transmission | 1) UE can transmit NR sidelink based on the synchronization to an eNB. 2) If UE supports component 1 in FG 32-4b, UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 3) If UE supports component 1 in FG 32-4b, UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. | 32-4b | Yes | No |  | Per band | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| 32. NR_SL_enh | 32-5a-1 | Transmitting Inter-UE coordination scheme 1 in NR sidelink mode 2 | 1) UE can transmit inter-UE coordination information of preferred resource set/non-preferred resource set in NR sidelink mode 2. 2) UE can receive an explicit request for inter-UE coordination information of both preferred resource set and non-preferred resource set. | [At least one of 32-5a-2 and 32-5a-3] 32-4b/32-4c or 15-3 | Yes | Yes | UE does not support transmitting inter-UE coordination scheme 1 in NR sidelink mode 2. [UE does not receive an explicit request for inter-UE coordination information] | Per band | [N.A.] | [N.A.] | [N.A.] | FFS | Optional with capability signalling. |
| 32. NR_SL_enh | 32-5a-2 | Receiving Inter-UE coordination information of preferred resource set in NR sidelink mode 2 | 1) UE can receive inter-UE coordination information of preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit an explicit request for inter-UE coordination information of preferred resource set only. | [TBD] | Yes | Yes | UE does not support receiving inter-UE coordination of preferred resource set in NR sidelink mode 2. [UE does not transmit an explicit request for inter-UE coordination information of preferred resource set only] | Per band | N.A. | N.A. | N.A. | FFS | Optional with capability signalling. |
| 32. NR_SL_enh | 32-5a-3 | Receiving Inter-UE coordination information of non-preferred resource set in NR sidelink mode 2 | 1) UE can receive inter-UE coordination information of non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit an explicit request for inter-UE coordination information of non-preferred resource set only. | [TBD] | Yes | Yes | UE does not support receiving inter-UE coordination of non-preferred resource set in NR sidelink mode 2. [UE does not transmit an explicit request for inter-UE coordination information of non-preferred resource set only] | Per band | N.A. | N.A. | N.A. | FFS | Optional with capability signalling. |
| 32. NR_SL_enh | 32-5b-1 | Transmitting Inter-UE coordination scheme 2 in NR sidelink mode 2 | 1) UE can transmit inter-UE coordination information of presence of expected/potential resource conflict in NR sidelink mode 2. 2) UE can transmit up to M PSFCH(s) resources in a slot | 32-5b-2, TBD | Yes | Yes | UE does not support transmitting inter-UE coordination scheme 2 in NR sidelink mode 2. | Per FS | [N.A.] | [N.A.] | [N.A.] | [Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1] Candidate values for M are {4, 8, 16} If UE reports more than one FGs of 15-11 and 32-5b-1, the reported value M in each FG is the total number and the same among those FGs. | Optional with capability signalling. |
| 32. NR_SL_enh | 32-5b-2 | Receiving Inter-UE coordination scheme 2 in NR sidelink mode 2 | 1) UE can receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. 2) UE can receive up to N PSFCH(s) resources in a slot. | [TBD] | Yes | Yes | UE does not support receiving inter-UE coordination scheme 2 in NR sidelink mode 2. | Per band | N.A. | N.A. | N.A. | [Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1] Candidate values for N are {5, 15, 25, 32, 35, 45, 50, 64} If UE reports more than one FGs of 15-11 and 32-5b-2, the reported value N in each FG is the total number and the same among those FGs. | Optional with capability signalling. |
| 32. NR_SL_enh | 32-6-1 | Reception of Scheme 1 inter-UE coordination information over 2nd SCI | 1) UE can receive Scheme 1 inter-UE coordination transmission over 2nd SCI that is used in addition to the MAC-CE carrying the same inter-UE coordination information in the same transmission. | FFS | Yes | Yes | UE is not required to decode SCI 2-C and the associated PSSCH | Per band | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |
| 32. NR_SL_enh | 32-6-2 | Reception of Scheme 1 explicit request over 2nd SCI | 1) UE can receive an explicit request for inter-UE coordination information of both preferred resource set and non-preferred resource set over 2nd SCI that is used in addition to the MAC-CE carrying the explicit request in the same transmission | FFS | Yes | Yes | UE is not required to decode SCI 2-C and the associated PSSCH | Per band | [N.A.] | [N.A.] | [N.A.] | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |
| 32. NR_SL_enh | 32-7 | Determination of expected conflict in Scheme 2 based on RSRP difference | 1) UE can determine a conflict for overlapping resource reservation between UE-B and another UE based on RSRP difference of the two reservations | 32-5b-1 | No | Yes |  | Per FS | [N.A.] | [N.A.] | [N.A.] | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |

References

R1-2202929, Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e including remaining RAN1 issues, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#108-e