3GPP TSG RAN WG1 #109-e		R1-2203366

e-Meeting, May 9th – 20th, 2022

Title: 	Discussion on physical layer structures and procedures for SL-U

Source: 	ZTE, Sanechips

Agenda item:	9.4.1.2

Document for:	Discussion and decision

Introduction

In RAN#94, the WID of NR SL evolution is approved, and one of the topics is to study and specify support of sidelink on unlicensed spectrum [1] as below:

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

In some regions where OCB and PSD requirement is not needed, the legacy NR SL design can be reused, while in the regions where OCB and PSD requirement should be guaranteed, some enhancements on SL channels and signals may be considered. In this contribution, the involved issues about physical channel structures and procedures are discussed.

Discussions

  - 2.1 General consideration on physical structure design

In RLAN specification of ETSI, it is defined that equipment may have simultaneous transmissions on more than one operating channel with a Nominal Channel Bandwidth of 20 MHz, and for the regions where OCB and PSD are required, the occupied channel bandwidth shall be between 80 % and 100 % of the Nominal Channel Bandwidth [2]. Following the rules, for SL-U operation, the bandwidth of 20MHz is proposed to be a channel on which channel access procedure is performed. When UE operates within a carrier/BWP larger than 20 MHz, multiple channels access procedure similar as NR-U is supported..

For SL-U operation:

Bandwidth of 20MHz is proposed to be a channel on which channel access procedure is performed.

Multiple channels access procedure similar as NR-U is supported when UE operates on a carrier/BWP with larger than 20 MHz bandwidth .

In NR-U, to meet the OCB requirement in FR1, IRB and RB set are defined and an IRB consists of a set of discrete RBs in the BWP. For IRB in NR-U, only 15kHz SCS and 30kHz SCS were specified. However in sidelink, 60kHz SCS is already supported in FR1 for band 46 and band 96, so if OCB is not required, 60kHz SCS can be considered for SL-U so that SL UE can have more time domain opportunity to access the channel. The definition of RB set and IRB can be reused for SL-U when OCB is required. Besides, UE can be also provided with N (N>=1) RB sets, and each RB set is a set of RBs within an approximately 20 MHz segment of the carrier/BWP.

When OCB is required, IRB and RBset defined in NR-U are supported in SL-U, and 15/30kHz SCS can be considered.

When OCB is not required, legacy physical channel design for NR sidelink can be reused in SL-U, and 15/30/60kHz SCS can be considered.

  - 2.2 SL BWP and SL resource pool

For SL communication, the configuration for SL carrier, SL BWP and SL resource pools should be supported. Based on the configuration, UE can transmit and receive SL physical channels and signals within a resource pool. For SL carrier and BWP configuration on unlicensed spectrum, legacy definition in NR SL can be reused. For SL resource pool configuration on unlicensed spectrum, it is better to define SL resource pool based on RB sets if OCB is required.

For SL-U carrier and BWP configuration, legacy definition in NR SL can be reused.

For SL-U resource pool configuration, it should correspond to an integer number of RB sets if OCB is required.

FFS: the SL-U resource pool configuration if OCB is not required.

In frequency domain, legacy NR SL resource pool consists of several continuous subchannels which contain continuous RBs according to the configuration with subchannel being the minimum transmission unit in frequency domain. The field of FRIV in SCI is used to indicate the PSSCH frequency size and location. The design based on subchannel can also be considered in SL-U, but the difference is that if IRB is used, subchannel in SL-U may consist of a set of discrete RBs within one or more RB sets to meet the OCB and PSD requirement. In the following two figures, two kinds of subchannel structure are depicted.

Figure 1 legacy NR SL subchannel with continous RBs

Figure 2 SL subchannel with discrete RBs

In NR-U, between RB sets in a carrier, a guard band can be configured to reduce interference. For SL-U, when UE operates on a wideband larger than one RB set, whether to support the RBs in guard band for transmission/reception needs to be discussed. If it is supported as in NR-U, in case there is difference in terms of the RB set occupation for the initial transmission and retransmission of a TB, the guard band overhead needs to be accounted for in the TB size determination procedure similar to the PSFCH overhead situation specified in Rel-16.

In time domain, for legacy NR SL, a resource pool consists of a set of logical slots in a SFN period by a bitmap. Especially for a TDD carrier, the time resources for sidelink should exclude non-UL slots, SSB slots, slots due to not enough symbols for SL transmission and reserved slots. When it comes to unlicensed spectrum for SL-U, and if gNB just works on licensed band, then the unlicensed band for SL-U is similar with ITS band for SL UE in 3GPP system, in this case all slots on unlicensed band can be available.

Besides SL-U, if there is NR-U operation as well as non-3GPP system (Wi-Fi, Bluetooth, etc), the time resource determination of SL-U resource pool should be considered and the legacy NR SL logical slot generation mechanism can be reused. At least non-UL slot, slot due to not enough symbols for SL transmission are excluded from the time resource of SL-U resource pool. For the SL SSB slot, whether it can be counted in the slots of resource pool can be further studied.

Moreover, following Rel-16 SL bit mapping method, if the bit for mapping is not all ‘1’, the slots in a resource pool are probably nonadjacent, this is not benefit for maintaining the COT. To maximize the gain of COT sharing, all physical slots should be assumed available similar as ITS band, and the bit for mapping is proposed to set as all ‘1’.

For SL-U resource pool, support that:

In frequency domain: a resource pool includes an integer number of continuous subchanels, and each subchannel consists of a set of discrete/continuous RBs

    - FFS the usage of RBs in guard band(s) when UE operates on a wideband larger than one RB sets

In time domain: bit mapping mechanism in R16 SL can be reused, and it is proposed to set all of the bit for mapping to ‘1’.

    - At least non-UL slot, slot due to not enough symbols for SL transmission are excluded from the time resource of SL-U resource pool when gNB works on unlicensed band, FFS SL SSB.

  - 2.3 SL channel and signals

Slot structure

Legacy NR SL slot contains an AGC symbol at first position, a gap symbol at last position, and the remaining symbols are used as PSCCH/PSSCH when PSFCH resources are not configured. Besides, additional one gap symbol, one AGC symbol and one PSFCH symbol exist after PSCCH/PSSCH symbol in the slot when PSFCH resources are configured, as shown in figure 3.

(a)SL slot without PSFCH                         (b)SL slot with PSFCH

Figure 3 SL slot structures

AGC and Tx/Rx switch problem still exist in SL-U transmission/reception, so the slot structure of legacy NR SL should be the baseline.

Legacy NR SL slot structures should be the baseline for SL-U

If legacy SL slot structure is considered, there are some symbols not conveying any signal both between and within slots, and the idle time is usually larger than 16us. This will make it difficult to continuously occupy the channel on unlicensed band, to avoid this situation, UE can transmit some occupancy signal to maintain the channel, so that the gap between two adjacent transmission is small enough to guarantee the COT sharing for channel continuously occupied.

Occupancy signal can be transmitted to maintain the COT , FFS the occupancy signal

SL SSB

SSB in NR SL occupies 11RBs in frequency domain and 13 symbols (for normal CP) in time domain within SL slot, which does not meet the OCB requirement. To solve this problem, three options can be considered:

- Option1: Repeating the SSB resource in frequency so that the bandwidth of SSB can be approximately 80%*20M=16M within a RBset, as shown in figure 4

Figure 4

- Option2: The 11RBs of SSB is located as a comb manner in frequency domain so that the total bandwidth of SSB can be approximately 16M within a RBset, as shown in figure 5

Figure 5

- Option3：Mapping SSB to more RBs in frequency domain and less symbols in time domain, and a slot may contain multiple SSB occasions, as shown in figure 6

Figure 6

Comparing with option 1 and option 2, option 3 may need more specification work to design a new SL SSB, in contrast, it has an advantage that more transmission occasions of S-SSB are valid in a slot, and fewer time resource is necessary in contrast to slot based SSB transmission.

For SL SSB, three options below can be considered to meet the OCB requirement:

Option1: Repeating the SSB resource in frequency

Option2: RBs of SSB is located as a comb manner in frequency domain

Option3: Mapping SSB to more RBs in frequency domain and less symbols in time domain

In NR R16, SSB resource is configured with a periodicity of 160ms, and two or three set of resources are configured within 160ms periodicity, based on the (pre)configuration and synchronization priority rules, UE can complete the synchronization procedure. This mechanism can be used as the starting point of SL-U. Considering on unlicensed band the SSB signal may fail to access the channel at some of SSB occasions, this will affect the synchronization performance, to alleviate the influence of LBT failure, more transmission occasions can be considered for each set of SSB resources in the synchronization period.

R16 SL synchronization procedure can be reused for SL-U, and more transmission occasions are considered for each set of SSB resources within the synchronization period

PSSCH/PSCCH

In NR SL, PSSCH is transmitted in the SL resource pool on one subchannel or multiple continuous subchannels, and PSCCH uses a number of RBs starting from the lowest RB index in PSSCH frequency domain region, and a subchannel can be configured at most 25 continuous RBs, which can not meet the OCB requirement. For SL-U, if OCB is required, in frequency domain, the IRBs for PSSCH are distributed in N(N>=1) RBsets and M (M>=1)interlaces, and PSCCH occupies the RBs of PSSCH with the starting RB in the lowest RB set index and interlace index, the number of RBs of PSCCH are determined according to the resource pool configuration.

If OCB is required, the IRBs for PSSCH are distributed in N (N>=1) RBsets and M (M>=1)interlaces, and PSCCH occupies the RBs of PSSCH with the starting RB in the lowest RB set index and interlace index.

PSFCH

In Rel-16, a PSFCH just occupies one RB and two duplicated symbols. For unlicensed band, if OCB is required, the frequency of PSFCH can be expanded to more RBs using a IRB within a RB set. The interlaced transmission of PUCCH format 0 in NR-U can be taken as a reference.

Besides, for each of PSCCH/PSSCH, there will be one or one set of certain PSFCH resource(s) at set time and frequency location, this can be called one to one mapping rule between PSCCH/PSSCH and PSFCH, but there is a similar problem as SSB transmission, PSFCH may also fail to access the channel on unlicensed band if only one transmission occasion is configured. To increase the robustness of SL HARQ feedback, one to many mapping rules between PSCCH/PSSCH and PSFCH resource should be considered, then UE can choose any one PSFCH transmission occasion successfully accessed by LBT to transmit the feedback information.

If OCB is required, support the frequency-domain granularity of a PSFCH resource as an IRB within a RB set

Support more than one PSFCH transmission occasions mapping to a PSCCH/PSSCH on unlicensed band

PTRS, DMRS and CSI-RS

Considering SL PTRS is supported for FR2, but SL-U in this release targets FR1, SL PTRS is not considered in SL-U.

For SL DMRS and CSI-RS, they are conveyed within PSCCH/PSSCH, considering these signals will not be transmitted standalone, if OCB is required, it can be guaranteed along with transmission of PSCCH/PSSCH, so the legacy NR SL reference signals design can be reused for SL-U.

Legacy NR SL DMRS and CSI-RS design can be reused for SL-U

  - 2.4 Resource selection mechanism for SL-U

In NR Rel-16/17, SL UE can obtain its physical layer resources either by grant from gNB called mode 1, or by UE itself called mode 2. For mode1 resource selection, gNB can be responsible for maintaining the channel through reasonable schedule similar with NR-U, while for mode 2 resource selection, if COT sharing is supported, legacy method is hard to maintain the channel within COT initiated by one UE, because the resources of the UEs within the COT are selected randomly in respective resource selection windows without any coordination., so  continuous transmission between UEs within the COT may not be guaranteed. In this case, enhancement for resource selection within a COT can be considered, which can make sure that gap between the transmissions within COT are small enough, as shown in Figure 7.

Figure 7 Enhanced mode 2 resource selection method

Legacy Rel-16 mode 1 resource selection mechanism can be supported as baseline for SL-U

FFS enhanced mode 2 resource selection method to guarantee continuous transmissions within a COT

  - 2.5 Power control for SL-U

Open loop power control are supported in Rel-16 SL to alleviate system interference. In Rel-18, the same requirement is still valid if considering NR-U would coexist with SL-U, so the of power control principle in Rel-16 SL can be reused for SL-U. Moreover, in some regions, transmit power are additionally limited by Max Mean EIRP and power/EIRP density (X dBm/MHz) [4], so SL-U UE need to take the Max Mean EIRP density into consideration additionally.

Rel-16 power control principle can be reused for SL-U, and Max Mean EIRP and power density limit should be considered additionally if necessary.

  - 2.6 CSI measurement and feedback

Rel-16 SL support wide-band CQI/RI feedback according to CSI-RS in unicast scenario, and MAC CE is used to convey the CSI feedback considering the aperiodic case. For SL-U, following legacy resource allocation/selection mechanism and considering channel access procedure, transmissions cannot be guaranteed periodic and the channel interference is more complex, e.g., the channel occupied by a UE may be larger than a LBT channel, so sub-band CSI feedback can be considered for SL-U. Based on above analysis, Rel-16 aperiodic CSI measurement and feedback mechanism can be the start point for SL-U.

Rel-16 CSI measurement and feedback mechanism can be the start point for SL-U

FFS sub-band CSI feedback

Conclusion

According to the discussion above, we provide the following observations and proposals on physical layer structure for sidelink on FR1 unlicensed spectrum:

Proposal 1:	For SL-U operation:

•	Bandwidth of 20MHz is proposed to be a channel on which channel access procedure is performed.

•	Multiple channels access procedure similar as NR-U is supported when UE operates on a carrier/BWP with larger than 20 MHz bandwidth .

Proposal 2:	When OCB is required, IRB and RBset defined in NR-U are supported in SL-U, and 15/30kHz SCS can be considered.

Proposal 3:	When OCB is not required, legacy physical channel design for NR sidelink can be reused in SL-U, and 15/30/60kHz SCS can be considered.

Proposal 4:	For SL-U carrier and BWP configuration, legacy definition in NR SL can be reused.

Proposal 5:	For SL-U resource pool configuration, it should correspond to an integer number of RB sets if OCB is required.

•	FFS: the SL-U resource pool configuration if OCB is not required.

Proposal 6:	For SL-U resource pool, support that:

•	In frequency domain: a resource pool includes an integer number of continuous subchanels, and each subchannel consists of a set of discrete/continuous RBs

i.	FFS the usage of RBs in guard band(s) when UE operates on a wideband larger than one RB sets

•	In time domain: bit mapping mechanism in R16 SL can be reused, and it is proposed to set all of the bit for mapping to ‘1’.

ii.	At least non-UL slot, slot due to not enough symbols for SL transmission are excluded from the time resource of SL-U resource pool when gNB works on unlicensed band, FFS SL SSB.

Proposal 7:	Legacy NR SL slot structures should be the baseline for SL-U

Proposal 8:	Occupancy signal can be transmitted to maintain the COT , FFS the occupancy signal

Proposal 9:	For SL SSB, three options below can be considered to meet the OCB requirement:

•	Option1: Repeating the SSB resource in frequency

•	Option2: RBs of SSB is located as a comb manner in frequency domain

•	Option3: Mapping SSB to more RBs in frequency domain and less symbols in time domain

Proposal 10:	R16 SL synchronization procedure can be reused for SL-U, and more transmission occasions are considered for each set of SSB resources within the synchronization period

Proposal 11:	If OCB is required, the IRBs for PSSCH are distributed in N (N>=1) RBsets and M (M>=1)interlaces, and PSCCH occupies the RBs of PSSCH with the starting RB in the lowest RB set index and interlace index.

Proposal 12:	If OCB is required, support the frequency-domain granularity of a PSFCH resource as an IRB within a RB set

Proposal 13:	Support more than one PSFCH transmission occasions mapping to a PSCCH/PSSCH on unlicensed band

Proposal 14:	Legacy NR SL DMRS and CSI-RS design can be reused for SL-U

Proposal 15:	Legacy Rel-16 mode 1 resource selection mechanism can be supported as baseline for SL-U

•	FFS enhanced mode 2 resource selection method to guarantee continuous transmissions within a COT

Proposal 16:	Rel-16 power control principle can be reused for SL-U, and Max Mean EIRP and power density limit should be considered additionally if necessary.

Proposal 17:	Rel-16 CSI measurement and feedback mechanism can be the start point for SL-U

•	FFS sub-band CSI feedback

References

RP-220300 New WID on NR sidelink evolution

ETSI EN 301 893, Harmonized European Standard, "Broadband Radio Access Networks (BRAN); 5 GHz high performance RLAN"

TS 38.101-1 User Equipment (UE) radio transmission and reception;Part 1: Range 1 Standalone

TS 38.889 Study on NR-based access to unlicensed spectrum