3GPP TSG RAN WG1 #109-e			R1-2203369

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.2.1

Source:	InterDigital, Inc.

Title:	Remaining issues for initial access operation in 52.6-71GHz

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss the remaining issues of initial access including discovery burst transmission window (DBTW) and Q parameter indication.

# Discussions

## Discovery Burst Transmission Window

In RAN1 #107bis-e, following LS replies from RAN2 and RAN4 to RAN1 were provided.

RAN2 reply to LS on using the spare bit in signalling of the Q parameter:

- A1: RAN2 does not agree to using the spare bit in MIB for the signalling of FR2-2 QCL assumptions for SSB. Respond to RAN1 LS accordingly. - A2: The legacy MIB is used for FR2-2 (i.e. we do not define new MIB for FR2-2). - A3: Up to RAN1 how to resolve QCL configuration (no suggestions from RAN2). This need not be included in LS to RAN1.

RAN4 agreement on channelization in FR2-2 ‎[4]:

Agreement: Consider the different channelization for licensed band(s) and unlicensed band(s) - Fixed sync raster for unlicensed bands - Fixed scheme should not be constrained by IEEE channel raster - Send LS to RAN1 to make sure that RAN1 accommodates the solution already now for both fixed and floating sync raster. - For the contiguous carrier aggregation, the channel spacing of adjacent channels should be multiple of the larger SCS, i.e., 960KHz, used by two channels/CCs - Floating sync raster for licensed bands - Stick to the agreement last meeting for 3 x 17.28MHz as the minimum granularity. - Refer to gap between adjacent GSCN values is not smaller than 3 x 17.28MHz - FFS: Unlicensed bands tries to use the sub-set of sync raster for licensed bands

In RAN1#108-e [5], the following working assumption was made:

Working assumption - Use 1 bit for Q in MIB - SubcarrierSpacingCommon field will be used to convey value of {32, 64} for operation with shared spectrum channel access Note that this is revising the working assumption made in RAN1#107-e on “use 2 bits for Q, {SubcarrierSpacingCommon, spare bit in MIB}”

In beyond 52.6GHz, the same divisions of the spectrum have been allocated to different license regimes in different neighboring countries. Thus, the licensed and unlicensed bands may overlap, which is different from the NR-U in the 5/6GHz band.

In operation with shared spectrum channel access, a UE needs to determine the license regime prior to decoding MIB or as part of the MIB. One solution that was discussed in previous meetings was based on the different sync raster sets to inform the UE ahead of detecting the SSB block.

In RAN1#106bis-e, it was agreed to support the following agreement on indication of the license regime.

Agreement: - Indication of licensed and unlicensed operation is not explicitly indicated in MIB or PBCH payload. - FFS: Whether or not to indicate licensed regime by different synchronization raster entries.

The abovementioned RAN4 agreement is on having different channelization for licensed and unlicensed bands. As such, fixed sync raster is considered for unlicensed bands while unlicensed bands could potentially use the sub-set of sync raster for licensed bands. Considering this agreement, the indication of license regime cannot be accomplished based on sync raster as similar sync raster may be used for both licensed and unlicensed.

Considering the working assumption, there is only one single bit left for indicating the Q parameter that would be indicating only two values. Considering 64 candidate SSB positions, the two value Q parameters could be limiting. Also, in RAN1 #107bis-e, there were discussions on considering the “reserved” status, among the Q parameter values, to indicate the license regime. Considering the single bit indication for Q parameter that is not going to be applicable anymore.

It is observed that despite the need for multiple indications, there is a shortage of available payload bits. There were some proposals on using the LSB of kSSB as a means of indication, considering the ongoing RAN4 channel and sync raster design. However, relying on kSSB would make the Q-parameter indication dependent on RAN4 design and constraining. Therefore, we propose to use other parts of SSB block for indicating the required operating parameters.

Observation 1: Using a single bit indication for the Q parameter is not sufficient for indication of QCL relation for up to 64 candidate SSB positions in beyond 52.6GHz.

### Indication of Q parameter based on the PBCH DMRS index

In NR Rel. 16, DMRS in PBCH is transmitted by using a pseudo-random sequence, wherein a seed for the pseudo-random sequence cinit is defined as 
cinit=211iSSB+1NIDcell/4+1+26iSSB+1+NIDcell mod 4

where, iSSB=iSSB for Lmax>4 for FR2-2, and iSSB is three LSB bits of the candidate SS block index.

We are proposing to consider iSSB=iSSB+rnQ, where the parameters r and nQ can be considered as indexes that can be used for indicating operating parameters.

The parameter nQ can be interpreted as a license regime indicator. For example, nQ=0 implies operation without shared spectrum channel access and nQ=1 indicates operation with shared spectrum channel access.

The value of parameter r is r=2m, m≥3, where different values of r can be used as a basis to indicate different modes of operation or operating parameters. In this way, the detection of r=8 can be interpreted as a value for indication of Q parameter.

During the initial access, when a UE detects an SS block, the UE decodes PBCH DMRS and respective value for iSSB. So, for example if 0≤iSSB<7, this implies licensed operation and that nQ=0. As another example, if 8≤iSSB<15, this implies unlicensed operation and that nQ=1 and r=8, and so on.

Proposal 1. For SSB blocks in the frequency beyond 52.6GHz, use the index associated to the DMRS in PBCH (iSSB) as a means of indication for Q parameter in addition to the license regime.

# Summary

In this contribution, we discussed the remaining issues on initial access aspects. From the discussions, we made following observations and proposals:

Observation 1: Using a single bit indication for the Q parameter is not sufficient for indication of QCL relation for up to 64 candidate SSB positions in beyond 52.6GHz.

Proposal 1. For SSB blocks in the frequency beyond 52.6GHz, use the index associated to the DMRS in PBCH (iSSB) as a means of indication for Q parameter in addition to the license regime.

# References

- RP-211584, “Revised WID: Extending current NR operation to 71 GHz”, Qualcomm, Intel,

- RAN1 chairman’s notes, RAN1#107-e, November 2021,

- RAN1 chairman’s notes, RAN1#107bis-e, January 2022,

- R4-2202365, RAN4 #101-bis-e, WF on general aspects and system parameters of FR2-2, Intel Corporation, January 2022,

- RAN1 chairman’s notes, RAN1#108-e, February-March 2022.