3GPP TSG RAN WG1 #109-e			R1-2203371

e-Meeting, May 9th – May 20th, 2022

Agenda Item:	8.2.3

Source:	InterDigital, Inc.

Title:		Remaining issues for PDSCH/PUSCH enhancements to supporting 52.6-71 GHz band in NR

Document for:	Discussion and Decision

# Introduction

In RAN1 #108-e [1], RAN1 continued the discussion on enhancements for PDSCH and PUSCH to support NR operation in 52.6 GHz to 71 GHz band. Based on the discussion, the agreements and the conclusions given in the Annex were reached on PDSCH/PUSCH enhancements alongside relevant TPs [1]. In this contribution, we discuss potential enhancements for remaining PDSCH/PUSCH related aspects in 52.6 – 71GHz and associated standard impact.

# Discussions

## Clarification of Whether Scheduled Implies Configured or Valid SLIVs

A design problem that was extensively discussed in RAN1#108-e [1] is whether to consider valid SLIV or configured SLIVs for determining if out-of-order (OOO) scheduling is taking place.  Other design problem remains undecided is whether valid or configured SLIVs scheduled by the first DCI should be used for the definition of corresponding timeline requirements with NN-K1 indication. That is, when a first DCI schedules multiple PDSCHs and provides an inapplicable value of k1 in its PDSCH-to-HARQ_feedback timing indicator field to multiplex the corresponding HARQ-ACK information in a PUCCH or PUSCH in a slot indicated by the PDSCH-to-HARQ_feedback timing indicator field in a second DCI.

For both problems, the majority view was to consider valid SLIVs to avoid unnecessary scheduling restrictions that may create if configured SLIVs are used. The opponent companies preferred configured SLIVs for OOO scheduling determination and timeline requirements in NN-K1 as operation based on configured SLIVs is robust and can be considered as the default behavior. While understanding both pros and cons of the two choices, we prefer to consider valid SLIVs for both OOO and NN-K1 cases due to scheduling flexibility it can provide.

Proposal 1: Out-of-order scheduling determination should be based on valid SLIVs.

Proposal 2: When a first DCI schedules multiple PDSCHs and provides an inapplicable value of k1 in its PDSCH-to-HARQ_feedback timing indicator filed to multiplex the corresponding HARQ-ACK information in a PUCCH or PUSCH in a slot indicated by the PDSCH-to-HARQ_feedback timing indicator filed in a second DCI, only the valid SLIVs scheduled by the first DCI are considered for definition of the corresponding timeline requirements.

## DMRS Bundling Across Multiple PUSCHs

In RAN1#107-bis-e [2], whether to support DMRS bundling for multi-PUSCH scheduling by a single DCI in FR2-2 was discussed. Subsequently, it was agreed that the DMRS bundling across multiple PUSCHs scheduled by a single DCI is not supported for NR operation in FR2-2 in Rel-17. It was also agreed to further discuss the applicability of DMRS bundling FG developed in CovEnh WI for the same TB to FR2-2 with 120 kHz SCS.

When the applicability of DMRS bundling FG developed in CovEnh WI for FR2-2 with 120 kHz SCS is considered in the previous discussions, two key differences of the two cases were highlighted. First, the CovEnh WI only considers contiguous scheduling of PUSCHs while multi-PUSCH scheduling in FR2-2 can have slot level gaps between scheduled PUSCHs. Second, in CovEnh WI, repetition of the same TB is considered while in multi-PUSCH scheduled by a single DCI in FR2-2, each slot corresponds to different TB. Having said that, it is worth to note that TB repetition across multiple contiguous slots can be supported in FR2-2 by using the existing features. In such a situation, DMRS bundling FG developed in CovEnh WI could be used to improve the channels estimation performance without requiring any further specifications. Therefore, the use of this FG developed in CovEnh WI should not be prohibited in FR2-2. Based on the discussion, we make the following proposal.

Proposal 3: The use of DMRS bundling FG developed in CovEnh WI for the same TB should not be prohibited in FR2-2 with 120 kHz SCS.

# Summary

In this contribution, we discussed the issues on PDSCH/PUSCH enhancements of NR in 52.6 – 71 GHz. From the discussions, we made following observation and proposals:

Proposal 1: Out-of-order scheduling determination should be based on valid SLIVs.

Proposal 2: When a first DCI schedules multiple PDSCHs and provides an inapplicable value of k1 in its PDSCH-to-HARQ_feedback timing indicator filed to multiplex the corresponding HARQ-ACK information in a PUCCH or PUSCH in a slot indicated by the PDSCH-to-HARQ_feedback timing indicator filed in a second DCI, only the valid SLIVs scheduled by the first DCI are considered for definition of the corresponding timeline requirements.

Proposal 3: The use of DMRS bundling FG developed in CovEnh WI for the same TB should not be prohibited in FR2-2 with 120 kHz SCS.

# References

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #108-e, February-March 2022,

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #107bis-e, January 2022.

# Annex

Agreement Support the following values for cg-minDFI-Delay - SCS 120 kHz: 7, m*14, - SCS 480 kHz: 7*4, m*14*4, - SCS 960 kHz: 7*8, m*14*8, where m = {1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32} Agreement - The case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV, is considered as out-of-order scheduling and is not expected by UE. - This applies also when one of two DCIs is single-PDSCH (or single-PUSCH) scheduling DCI, including the case that one DCI schedules multi-slot PDSCH (or PUSCH repetition type A or B). - Note: This doesn’t apply when each of two DCIs schedules multi-slot PDSCH (or PUSCH repetition type A or B) as in Rel-15/Rel-16 - Note: This doesn’t apply when each of the two DCIs schedules single PDSCH (or single PUSCH) as in Rel-15/Rel-16 - Note: It is separately discussed whether the scheduled SLIV is based on configured SLIV or valid SLIV. Working assumption The TP#1 below for TS 38.213 section 9.1.3.1 is endorsed. - TP#1 (Construction of bundling group based on “configured” SLIVs) If a UE is provided numberOfHARQ-BundlingGroups and is not provided harq-ACK-SpatialBundlingPUCCH for a serving cell c, the UE generates HARQ-ACK information over transport block groups (TBGs) for PDSCH receptions where, for a maximum number of NPDSCHmax PDSCH receptions scheduled by a DCI format on the serving cell, a maximum number of TBGs NHARQ-ACK,cTBG,max is provided by numberOfHARQ-BundlingGroups. If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions on the serving cell c, the UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for first TBs and, if applicable, generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for second TBs in the NPDSCH,c PDSCH receptions as described in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c. For a TBG with at least one PDSCH not overlapping with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, if provided, the UE assumes that a PDSCH in the TBG is correctly received if the PDSCH overlaps with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated. For a TBG that includes only PDSCH(s) overlapping with UL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, if provided, NACK is generated for the TBG.. Agreement - When re-transmission of DL SPS is indicated by DCI format 1_1, the PDCCH indicates a TDRA row index including only one SLIV. - When re-transmission of UL CG is indicated by DCI format 0_1, the PDCCH indicates a TDRA row index including only one SLIV. Agreements - Text Proposal TP#B (for 38.213, Clause 9.1.2.1) in section 4.2 of R1-2202679 is endorsed. - Text Proposal TP#C (for 38.214, Clause 6.1) in section 4.3 of R1-2202679 is endorsed. - Text Proposal TP#E (for 38.213, Clause 10.3) in section 4.5 of R1-2202679 is endorsed. - Text Proposal TP#H (for 38.214, Clauses 5.1 and 6.1) in section 4.8 of R1-2202679 is endorsed. Conclusion For a DCI that can schedule multiple PUSCHs, it is clarified that the following M is counted based on scheduled PUSCHs indicated by the TDRA information field. - CSI-request: When the DCI schedules M PUSCHs, the PUSCH that carries the aperiodic CSI feedback is M-th scheduled PUSCH for M <= 2, or (M-1)-th scheduled PUSCH for M > 2. Note: Aperiodic CSI feedback will be dropped if M-th scheduled PUSCH for M <= 2 (or (M-1)-th scheduled PUSCH for M > 2) is collided with semi-static DL or SSB symbols. Agreement The following TP for TS 38.214 Clause 6.1.2.1 is endorsed -------------------------------------------Start of TP#F1 for TS 38.214 Clause 6.1.2.1----------------------------------------------- 6.1.2.1 Resource allocation in time domain If a UE is configured with pusch-TimeDomainAllocationListForMultiPUSCH-r17 in which one or more rows contain multiple SLIVs for PUSCH on a UL BWP of a serving cell, the UE does not apply pusch-AggregationFactor, if configured, to DCI format 0_1 on the UL BWP of the serving cell and the UE does not expect to be configured with numberOfRepetitions in pusch-TimeDomainAllocationListForMultiPUSCH-r17. ------------------------------------------------------------End of TP#F1---------------------------------------------------------------- Agreement The following TP for TS 38.214 Clause 5.1.3.2 is endorsed -------------------------------------------Start of TP#J1 for TS 38.214 Clause 5.1.3.2 ----------------------------------------------- 5.1.3.2 Transport block size determination In case the higher layer parameter maxNrofCodeWordsScheduledByDCI indicates that two codeword transmission is enabled, then one of the two transport blocks is disabled by DCI format 1_1 if IMCS = 26 and if rvid = 1 for the corresponding transport block. When the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17], either the first or the second transport block of all scheduled PDSCHs is disabled by the DCI format 1_1 if IMCS = 26 and if rvid = 2 for the corresponding transport block of all scheduled PDSCHs. If both transport blocks are enabled, transport block 1 and 2 are mapped to codeword 0 and 1 respectively. If only one transport block is enabled, then the enabled transport block is always mapped to the first codeword. ------------------------------------------------------------End of TP#J1---------------------------------------------------------------- Agreement For multi-PDSCH scheduling via a single DCI with 'tdmSchemeA' for single DCI based multi-TRP mechanism, - If at least one of the repetitions of the PDSCH collides with semi-static UL symbols, the corresponding PDSCH (i.e., both repetitions) is considered as invalid. - Note: No specification impact on Type-1 HARQ-ACK codebook construction is expected, as a consequence of this agreement. - Note: This is not applied for the case when the multi-PDSCH DCI schedules only a single PDSCH. Agreement When a DCI format indicates TCI state update without scheduling PDSCH reception, the PDCCH indicates a TDRA row index including only one SLIV. Agreement For generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs, if OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, considering at least the followings. - For a serving cell c configured with numberOfHARQ-BundlingGroups with NHARQ-ACKTBG,max=1, Nm,creceived (in nHARQ-ACK,TB formula) corresponding to the DCI in PDCCH monitoring occasion m is given by 2 if harq-ACK-SpatialBundlingPUCCH is not provided and the DCI schedules two codewords, or given by 1 otherwise. - If the UE is configured with numberOfHARQ-BundlingGroups with NHARQ-ACKTBG,max>1 for a serving cell or not configured with numberOfHARQ-BundlingGroups but configured with PDSCH-TimeDomainResourceAllocationListForMultiPDSCH for a serving cell, nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,multi. - For a serving cell c configured with numberOfHARQ-BundlingGroups with NHARQ-ACKTBG,max>1, Nm,creceived (in nHARQ-ACK,multi formula) corresponding to the DCI in PDCCH monitoring occasion m is given by 2*X (X= the number of TBGs, each including at least one valid PDSCH which are constructed based on the PDSCH(s) scheduled by the DCI) if harq-ACK-SpatialBundlingPUCCH is not provided and the DCI schedules two codewords, or given by X otherwise. - For a serving cell c not configured with numberOfHARQ-BundlingGroups but configured with PDSCH-TimeDomainResourceAllocationListForMultiPDSCH, Nm,creceived (in nHARQ-ACK,multi formula) corresponding to the DCI in PDCCH monitoring occasion m is given by 2*Y (Y= the number of valid PDSCH receptions scheduled by the DCI) if harq-ACK-SpatialBundlingPUCCH is not provided and the DCI schedules two codewords, or given by Y otherwise. - Note: “valid PDSCH” implies a PDSCH not overlapping with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, if provided