3GPP TSG RAN WG1 #109-e		                                    R1-2203372

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.2.4

Source:	InterDigital, Inc.

Title:	Remaining issues for channel access mechanisms

Document for:	Discussion and Decision

# Introduction

Channel access mechanisms for operation between 52.6GHz and 71GHz was discussed during the SI [1][2]. During RAN1 #104-e – RAN1 #108-e, multiple agreements were reached with respect to channel access. They are provided herein when relevant.

In this contribution we address open issues on channel access and discuss our preferences for channel access for operation up to 71GHz.

# Discussion

## COT with multiple beams

The following agreement was reached during RAN1 #108-e:

Agreement

Support gNB as the initiating device to resume transmission with a Cat 2 LBT if there is gap longer than Y us from the previous transmission from initiating device or responding device

- Note this is motivated by regions where LBT is required before each transmission (say Japan)

- Y is left for initiating device implementation and should comply with local regulation but no less than 8us

The requirement of the type of LBT (or no LBT at all) prior to a transmission on a beam (e.g. after beam switching) can depend on the size of the gap between two transmissions. For two transmissions from an initiating device, a maximum gap beyond which Cat 2 LBT is used, should also be specified. The gap should be determined as the time from a last transmission on a beam “covering” the beam to be used in a subsequent transmission.

Proposal 1: A UE determines whether to use Cat 2 LBT based on the gap duration defined as the time between an upcoming transmission and a preceding transmission on at least the same beam pair.

During RAN1 #108-e, the following agreement was reached:

Agreement

When independent per-beam LBT sensing is performed at gNB, a transmission is allowed to occur on a beam if the corresponding LBT procedure has been successful before the channel occupancy start time.

- Note: For multi-beam transmission, channel occupancy start time corresponding to all Tx beams is aligned.

- FFS: When independent per-beam LBT sensing is performed at UE.

If all transmissions are dropped by a UE when at least one per-beam LBT fails, the benefit of independent per-beam LBT is lost. The likelihood that interference in a first beam would be present in a second beam is determined as a function of the beamwidth and direction of the beams. Dropping all transmissions due to a single failed per-beam LBT means that even though a beam suffers little or no interference, transmission on it would depend on the channel access performance (i.e. interference) on the set of all scheduled beams, regardless of orthogonality of the beams. This is essentially equivalent to quasi-omni-directional LBT and it would lead to inefficient use of spatial resources and unnecessarily delayed transmissions. Moreover, it would greatly reduce channel access diversity.

For SDM, enabling the UE to transmit on a subset of beams where LBT is successful does not require further specification. For TDM, a gap may be created if a first transmission in a burst was scheduled for a beam for which per-beam LBT failed. In such a case, the UE may determine whether to perform LBT (e.g. CAT 2 LBT) as per Proposal 1.

Some companies have argued that this issue is similar to transmissions spanning multiple LBT subbands as discussed in Rel-16 NR-U. For the case of a transmission spanning multiple LBT subbands, the issue was that the resource allocation could be affected by the set of acquired subbands. Therefore a UE would require quick processing time to determine the appropriate resource allocation to use as a function of the acquired set of subbands, which was deemed unfeasible. In the case of multi-beam operation, a TB is confined to a single beam. Therefore, transmitting on a subset of beams does not add any complexity compared to the case where a UE transmits on a single beam and fails to acquire the beam. The UE simply drops the transmission of a TB on the beam(s) where LBT is not successful.

Proposal 2: When independent per-beam LBT sensing is performed at the UE, a transmission is allowed to occur on a beam if the corresponding LBT procedure for that beam has been successful before the channel occupancy start time.

COT-SI

During RAN1 107b-e and 108-e, it was discussed whether to introduce beam specific COT-SI (COT duration) delivery in DCI 2_0 and whether this applies to SFI and SSGS as well.

Beam-specific COT-SI is required for appropriate COT sharing when beam-specific LBT is used. If beams are not indicated by the COT-SI, a UE will not know whether sharing a COT is possible for transmission on a specific beam. For cross-COT scheduling and for CG transmissions sharing a COT, a UE may not know whether the COT within which the transmission is scheduled has been initiated with successful LBT on the appropriate beam. Given that search-spaces may be associated with specific beams, it is also natural that beam-specific search-space group switching be triggered by the COT-SI.

Proposal 3: Introduce beam specific COT-SI delivery in DCI 2_0 applicable to COT duration and SSGS.

# Conclusion

The contribution discusses channel access for unlicensed bands up to 71GHz. We provide the following proposals:

Proposal 1: A UE determines whether to use Cat 2 LBT based on the gap duration defined as the time between an upcoming transmission and a preceding transmission on at least the same beam pair.

Proposal 2: When independent per-beam LBT sensing is performed at the UE, a transmission is allowed to occur on a beam if the corresponding LBT procedure for that beam has been successful before the channel occupancy start time.

Proposal 3: Introduce beam specific COT-SI delivery in DCI 2_0 applicable to COT duration and SSGS.

# References

RP-193259, “Study on supporting NR from 52.6GHz to 71GHz”

3GPP TR 38.808, “Study on supporting NR from 52.6 GHz to 71 GHz”, v1.0.0

RP-210862, “Extending current NR operation to 71GHz”

R1-2200803, “Feature Lead Summary for channel access of 52.6GHz to 71GHz”, Qualcomm Inc.