3GPP TSG RAN WG1 #109-e			R1-2203374

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.2.3.1

Source:	InterDigital, Inc.

Title:	Discussion for evaluation on AI/ML for beam management

Document for:	Discussion and Decision

# Introduction

In RAN#94-e [1], the study item for AI/ML has been approved for NR Air Interface. The study will focus on the following use cases.

Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project.

For the agreed use cases, the following objectives have been agreed.

- Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition

In this contribution, we discuss evaluation methodology, KPI and target scenarios for AI/ML beam management and provide preliminary evaluation results for AI/ML aided beam prediction.

# Discussions

## Evaluation methodology

For AI/ML, need of specification enhancements should be well justified with appropriate baseline and KPIs with proper target scenarios. In addition, complex specification enhancements should be avoided unless benefits are justified. In this section, we provide our views on baseline, KPIs and Target scenarios for AI/ML aided beam management.

### Baseline

To correctly evaluate benefits from AI/ML algorithms for beam management, a correct baseline should be established. In RAN#93-e [2] and RAN#94-e [3], the following levels of inter-node coordination and information exchange were provided during the discussion.

0a) No collaboration framework: AI/ML algorithms purely implementation based and not requiring air-interface changes. 0b) No collaboration framework with modified Air-Interface catering to efficient implementation-based AI/ML algorithms. 1) Inter-node assistance to improve the respective nodes’ AI/ML algorithms. This would apply to UEs getting assistance from gNBs (for training, adaptation, etc.) and vice-versa. This level does not require model exchange between network nodes. 2) Joint ML operation between UEs and gNBs. This level requires AI/ML model instruction or exchange between network nodes.

Legacy beam management with Rel-17 without AI/ML algorithms is not an appropriate baseline as implementation-based AI/ML operation is available for UE and gNB implementations. Having said that, level 0a) in the above could be an appropriate baseline to accurately evaluate the benefits of AI/ML with specification enhancements.

Observation 1: Legacy beam management with Rel-17 without AI/ML algorithms is not an appropriate baseline as implementation-based AI/ML operation is available for UE and gNB implementations.

Proposal 1: ‘No collaboration framework: AI/ML algorithms purely implementation based and not requiring air-interface changes’ could be an appropriate baseline to accurately evaluate the benefits of AI/ML with specification enhancements.

### KPIs

For KPIs, the following parameters should be considered to accurately evaluate benefits for AI/ML beam aided management.

- Difference between estimated qualities and actual qualities

  - Difference between estimated qualities and actual qualities could be a good option to evaluate benefits of AI/ML aided beam prediction. For the quality, L1-RSRP can be considered as a mandatory value, but L1-SINR can be considered as optional as the current NR specification supports L1-SINR as well as L1-RSRP.

- Optimal beam selection accuracy (%)

  - Comparison of optimal beam selection accuracy of a baseline and an enhanced feature is a simple and effective method to evaluate benefits of AI/ML aided beam management.

- System performance based on the selected optimal beams

  - Comparison of system performances (e.g., throughput) with a baseline and an enhanced feature is a best method as the system performances summarizes all the benefits from AI/ML aided beam management such as RS overhead, impact of optimal beam selection, UE reporting latency and other possible benefits.

Proposal 2: The following parameters should be considered to accurately evaluate benefits of AI/ML aided beam management.

- Difference between estimated qualities and actual qualities

- Optimal beam selection accuracy (%)

- System performance based on the selected optimal beams

### Target scenarios

As proposed in our companion contribution [4], the following scenarios should be considered for AI/ML aided beam prediction.

- Simple specification extension of UE reporting which enables AI/ML beam prediction

  - Possible simple specification extension could be investigated. For example, introduction of new CRI/SSBRI reporting mechanism (instead of reporting up to 4 best beams) or increasing maximum number of CRIs/SSBRIs.

- Partial beam measurement

  - AI/ML based beam prediction with partial beam measurement could be investigated. For example, estimation of 64 beams based on measurements of 16 or 32 beams can be considered. For partial beam measurement, both UE side and gNB side determination should be considered.

- Beam prediction in time domain

  - UE information on time domain could be investigated. For example, AI/ML based beam prediction with UE reporting with associated time slot information can be considered.

- Association between beams with different beam widths

  - Association between beams with different beam widths could be investigated. For example, robust estimation of whole spatial characteristics by utilizing wide beams and accurate beam identification by utilizing narrow beams can be considered.

- Handling of UE Rx beams

  - Acquisition of UE Rx beam information for gNB could be investigated for AI/ML model training as well as AI/ML aided beam prediction.

Proposal 3: Consider the following AI/ML aided beam prediction scenarios for evaluations.

- Simple specification extension of UE reporting which enables AI/ML beam prediction

- Partial beam measurement

- Beam prediction in time domain

- Association between beams with different beam widths

- Handling of UE Rx beams

# Preliminary evaluation results

In this section, we provide preliminary evaluation results of beam prediction based on partial measurements with 24 beams to be estimated and 12 beams to be received for each sector. Table 1 shows Root Mean Square Error (RMSE) of RSRP values between actual values and estimated values.

|  | RMSE (dB) |
| --- | --- |
| RMSE | 1.78 |

Table 1: Root Mean Square Error (RMSE) between AI estimated RSRPs and actual RSRPs

In addition, Figure 1 provides accuracy of AI based optimal beam selection and baseline (beam selection based on 12 beams) with the following definition:Optimal: An optimal beam with a highest RSRP value based on actual RSRP values of the whole 24 beams

- AIML-based: A beam with a highest RSRP value based on estimated RSRP values by AI/ML of the whole 24 beams

- Baseline: A beam with a highest RSRP value based on measured RSRP values of the 12 beams.

In this evaluation, we introduced error margin to observe meaningful accuracy of AI based optimal beam selection. Based on the error margin, the optimal beam selected by AI was counted as misdetection only when the selected beam is different with the optimal beam and RSRP loss from the selected beam > the error margin. In Figure 1, AI aided beam selection achieves more than 90% selection accuracy when error margin is larger than 1dB by consuming 50% of the measurement overhead for the exhaustive measurement and shows 25% better accuracy when the error margin is low.

Figure 1: Accuracy of AI based optimal beam selection with error margin

Observation 2: AI aided beam selection achieves more than 90% selection accuracy when error margin is larger than 1dB by consuming 50% of the measurement overhead for the exhaustive measurement.

Observation 3: AI aided beam selection achieves 25% selection accuracy than baseline.

Proposal 4: Further study benefits of AI/ML aided beam prediction.

# Summary

In this contribution, we discuss evaluation methodology, KPI and target scenarios for AI/ML beam management and provide preliminary evaluation results for AI/ML aided beam prediction. From the discussions, we made the following observations and proposals:

Observation 1: Legacy beam management with Rel-17 without AI/ML algorithms is not an appropriate baseline as implementation-based AI/ML operation is available for UE and gNB implementations.

Observation 2: AI aided beam selection achieves more than 90% selection accuracy when error margin is larger than 1dB by consuming 50% of the measurement overhead for the exhaustive measurement.

Observation 3: AI aided beam selection achieves 25% selection accuracy than baseline.

Proposal 1: ‘No collaboration framework: AI/ML algorithms purely implementation based and not requiring air-interface changes’ could be an appropriate baseline to accurately evaluate the benefits of AI/ML with specification enhancements.

Proposal 2: The following parameters should be considered to accurately evaluate benefits of AI/ML aided beam management.

- Difference between estimated qualities and actual qualities

- Optimal beam selection accuracy (%)

- System performance based on the selected optimal beams

Proposal 3: Consider the following AI/ML aided beam prediction scenarios for evaluations.

- Simple specification extension of UE reporting which enables AI/ML beam prediction

- Partial beam measurement

- Beam prediction in time domain

- Association between beams with different beam widths

- Handling of UE Rx beams

Proposal 4: Further study benefits of AI/ML aided beam prediction.

# References

- RP-213599, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”,

- RP-211662, “Moderator's summary for discussion [RAN93e-R18Prep-12] AI/ML”,

- RP-212668, “Moderator's summary for discussion [RAN94e-R18Prep-08] AI/ML for Air Interface”

- R1-2203375, “Discussion for other aspects on AI/ML for beam management”.

# Appendix

## Evaluation assumptions

| Parameter | Values |
| --- | --- |
| Carrier frequency, GHz | 28 GHz |
| Bandwidth, MHz | 400MHz |
| Subcarrier spacing, kHz | 30kHz |
| Channel model | InF-DH |
| Hall size | 120(L) x 60(W) m, D – 20 m |
| BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. |
| Room height | 10 m |
| Number of floors | 1 |
| Clutter parameters: {density , height , size } | InF-DH - {20%, 2m, 10m} |
| UE Model Parameters |  |
| UE noise figure, dB | 9 dB |
| UE antenna configuration | Panel model 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (2, 4, 2, 1, 1) |
| UE antenna radiation pattern | Omni, 0dBi |
| Network synchronization | Fully synchronized |
| UE/gNB RX and TX timing error | 0 ns |
| UE horizontal drop procedure | Uniformly distributed over the horizontal evaluation area within convex hull of horizontal BS deployment |
| UE antenna height | 1.5 m |
| gNB Model Parameters |  |
| Total gNB TX power, dBm | 23 dBm |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 |
| gNB antenna radiation pattern | 3-sector (120 degrees apart from each other) |
| gNB antenna height | 8 m |

## AI/ML parameters

| Data Description |  |
| --- | --- |
| Complete Dataset Description (RSRP_Actual) | Each sample includes 72 RSRP measurements for beams (24 RSRP measurements per sector) from one TRP to one UE in addition to the locations of the TRP and the UE |
| Input Dataset Description (RSRP_Input) | Each sample includes 36 RSRP measurements for beams (12 RSRP measurements per sector) from one TRP to one UE in addition to the locations of the TRP and the UE |
| Output Dataset Description (RSRP_Estimate) | Estimating the missing 36 RSRP measurements in the input dataset per sample by learning from the complete dataset |
| Beams Description | 24 beams per sector per TRP. Beams are generated from the combination of beams at 8 azimuth and 3 elevations angels |
| Number of Samples | 1800 |
| Size of Training (Testing) Set | 80% (20%) of the dataset |
| Validation | 5-fold cross-validation |
| ML Model Parameters |  |
| Model Description | A regression model to estimate each of the missing 36 RSRP measurements given the available 36 measurements, UE location, and TRP location |
| Model Parameters | Linear regression model with bias, linear terms, and pure quadratic terms |
| Evaluation | - Actual Best Beam per sample = argmax(RSRP_Actual) - Estimated Best Beam per sample = argmax(RSRP_Estimate) - RSRP_Loss = ¦RSRP_Actual(Actual Best Beam) – RSRP_Actual(Estimated Best Beam)¦ - When (RSRP_Loss > Error Margin), this is counted as best beam misdectection - Accuracy = (Number of Samples - Number of Best Beam Misdetections) / Number of Samples |