3GPP TSG RAN WG1 #109-e			R1-2203375

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.2.3.2

Source:	InterDigital, Inc.

Title:	Discussion for other aspects on AI/ML for beam management

Document for:	Discussion and Decision

# Introduction

In RAN#94-e [1], the study item for AI/ML has been approved for NR Air Interface. The study will focus on the following use cases.

Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project.

For the agreed use cases, the following objectives have been agreed.

- Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition

In this contribution, we discuss potential issues and associated standard impacts to support AI/ML for beam management in NR air interface.

# Discussions

In Rel-15, DL beam management procedures for P1 (initial beam selection), P2 (beam refinement for gNB Tx beam) and P3 (beam refinement for UE Rx beam) were defined. Although actual beam selection for beam management is up to gNB implementation, Rel-15 beam management procedures are basically designed for exhaustive searching of gNB transmission beams and/or UE reception beams. As analog beams cannot be multiplexed in time domain, the exhaustive searching requires enormous amount of resource overheads as well as large latency. In Rel-16/17, non-coherent JT based on multi-TRP, unified TCI states and other specification enhancements were introduced, however, the basic principle of exhaustive searching for beam measurement did not change. Especially, the overhead and the latency issues are more serious for FR2-2 as utilization of narrower beams than FR2-1 is expected to compensate the increased pathloss. Given the situation, the following specification enhancements should be considered for Rel-18 AI/ML beam management.

Simple specification extension of UE reporting which enables AI/ML beam prediction

To enable AI/ML beam prediction based on gNB implementation, simple extension of the current UE reporting should be considered. As the current NR specification, which supports UE reporting with up to 4 best CRIs/SSBRIs with L1-RSRP or L1-SINR, can be very limited for gNB estimation, some specification enhancements such as increasing number of possible best CRIs/SSBRIs, introduction of absolute RSRPs for other CRIs/SSBRIs than the best and other possible extensions can be considered.

Observation 1: The current NR specification supporting UE reporting with up to 4 best CRIs/SSBRIs with L1-RSRP or L1-SINR can be very limited for gNB estimation.

Proposal 1: Study benefits of simple specification extension of UE reporting such as increasing number of possible best CRIs/SSBRIs, introduction of absolute RSRPs for other CRIs/SSBRIs than the best and other possible extensions and other possible extensions.

Partial beam measurement

To reduce required RS overheads and latency for beam management, UE measurement and reporting based on partial beam measurement should be considered. For example, UE can measure a subset of all possible beams and qualities of other beams than the measured beams can be estimated by using AI/ML. For the partial beam measurement, both of the following two AI/ML beam prediction methods should be considered.

- UE side beam prediction

  - For partial beam measurement, UE side beam prediction should be supported as the UE has better information on UE position, UE direction and UE reception beam. For example, the UE can recommend best beams based on the UE prediction by measuring partial beams.

- gNB side beam prediction

  - As well as UE side beam prediction, gNB side beam prediction should be supported as the gNB has better computing power, power consumption and information exchange between multiple TRPs.

Figure 1. Example of partial beam measurement

Observation 2: Partial beam measurement which allows beam prediction by measuring only a subset of beams could be beneficial for reducing RS overheads and reporting latency.

Proposal 2: Study benefits of partial beam measurement for specification enhancement to reduce RS overheads and UE reporting latency.

Observation 3: For partial beam measurement, both UE side beam prediction and gNB side beam prediction have benefits.

Proposal 3: Study benefits of both UE side beam prediction and gNB side beam prediction for partial beam measurement.

Beam prediction in time domain

In Rel-15 beam management, if measurement restriction for channel and interference is not configured, how to handle multiple measurements in multiple transmission occasions are totally up to UE implementation. If measurement restriction for channel and interference is configured, UE should consider only latest measurement for UE reporting. The measurement restriction in the current specification is not to consider time domain characteristics, but to efficiently utilize RS transmissions for multiple analog/digital beams. However, for gNB which predicts beams by using AI/ML, time domain characteristics of beam measurements are essential as well as spatial domain characteristics of beam measurements. Especially, such time domain information will be crucial for certain predictable scenarios such as highway or HST. Given that, specification enhancements such as UE reporting with associated time slot information should be considered.

Observation 4: The current NR specification supports measurement restriction to limit UE measurement, however, measurement restriction is to efficiently utilize RS transmissions for multiple beams not to consider time domain characteristics of beam measurement.

Observation 5: For gNB which predicts beams by using AI/ML, time domain characteristics of beam measurements are essential as well as spatial domain characteristics.

Proposal 4: Study benefits of specification enhancements such as UE reporting with associated time slot domain information.

Association between beams with different beam widths

Although wide beamwidth for SSB and narrow beamwidth for CSI-RS were considered for the initial design of NR from Rel-15, association between beams with different beam widths was not considered in efficient ways. For AI/ML based beam prediction, such association could be utilized to achieve better prediction accuracy. For example, robust estimation/identification of whole spatial characteristics could be done by utilizing wide beams and accurate beam identification could be done based on the acquired wide beam information by utilizing narrow beams.

Observation 6: The current NR specification does not consider association between beams with different beam widths.

Observation 7: Utilizing association between beams with different beam widths can provide benefits for prediction accuracy e.g., robust estimation/identification of whole spatial characteristics with wide beams and accurate beam identification with narrow beams.

Proposal 5: Study benefits of specification enhancements on association between beams with different beam widths.

Figure 2. Example of association between beams with different beam widths

Handling of UE Rx beams

For Rel-15 beam management, actual mapping between DL Tx beam and UE Rx beam is totally based on UE implementation and there’s no way to identify actual UE beam information for a DL Tx beam by gNB. The implementation-based UE Rx beam selection works for Rel-15 as the gNB needs to know only a beam index for actual transmission. However, for AI/ML based beam prediction, UE Rx beam information such as beam identity and beam direction is crucial to accurately predict beam qualities.

Observation 8: For Rel-15 beam management, actual mapping between DL Tx beam and UE Rx beam is totally based on UE implementation.

Observation 9: The implementation-based UE Rx beam selection works for Rel-15, however, UE Rx beam information is crucial to accurately predict beam qualities for AI/ML based beam prediction.

Proposal 6: Study benefits of specification enhancements on acquiring UE Rx beam information.

# Summary

In this contribution, we discuss potential issues and associated standard impacts to support AI/ML for beam management. From the discussions, we made the following observations and proposals:

Observation 1: The current NR specification supporting UE reporting with up to 4 best CRIs/SSBRIs with L1-RSRP or L1-SINR can be very limited for gNB estimation.

Observation 2: Partial beam measurement which allows beam prediction by measuring only a subset of beams could be beneficial for reducing RS overheads and reporting latency.

Observation 3: For partial beam measurement, both UE side beam prediction and gNB side beam prediction have benefits.

Observation 4: The current NR specification supports measurement restriction to limit UE measurement, however, measurement restriction is to efficiently utilize RS transmissions for multiple beams not to consider time domain characteristics of beam measurement.

Observation 5: For gNB which predicts beams by using AI/ML, time domain characteristics of beam measurements are essential as well as spatial domain characteristics.

Observation 6: The current NR specification does not consider association between beams with different beam widths.

Observation 7: Utilizing association between beams with different beam widths can provide benefits for prediction accuracy e.g., robust estimation/identification of whole spatial characteristics with wide beams and accurate beam identification with narrow beams.

Observation 8: For Rel-15 beam management, actual mapping between DL Tx beam and UE Rx beam is totally based on UE implementation.

Observation 9: The implementation-based UE Rx beam selection works for Rel-15, however, UE Rx beam information is crucial to accurately predict beam qualities for AI/ML based beam prediction.

Proposal 1: Study benefits of simple specification extension of UE reporting such as increasing number of possible best CRIs/SSBRIs, introduction of absolute RSRPs for other CRIs/SSBRIs than the best and other possible extensions and other possible extensions.

Proposal 2: Study benefits of partial beam measurement for specification enhancement to reduce RS overheads and UE reporting latency.

Proposal 3: Study benefits of both UE side beam prediction and gNB side beam prediction for partial beam measurement.

Proposal 4: Study benefits of specification enhancements such as UE reporting with associated time slot domain information.

Proposal 5: Study benefits of specification enhancements on association between beams with different beam widths.

Proposal 6: Study benefits of specification enhancements on acquiring UE Rx beam information.

# References

- RP-213599, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”.