3GPP TSG RAN WG1 #109-e			R1-2203376

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.8.1

Source:	InterDigital, Inc.

Title:	Discussions on side control information to enable NR network-controlled repeaters

Document for:	Discussion and Decision

# Introduction

In RAN#94-e [1], new study item for investigating NR network-controlled repeaters was agreed, where the objectives identified for the study item are as follows:

The study on NR network-controlled repeaters is to focus on the following scenarios and assumptions: - Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands, while during the study FR2 deployments may be prioritized for both outdoor and O2I scenarios. - For only single hop stationary network-controlled repeaters - Network-controlled repeaters are transparent to UEs - Network-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously NOTE1: Cost efficiency is a key consideration point for network-controlled repeaters. Study and identify which side control information below is necessary for network-controlled repeaters including assumption of max transmission power [RAN1] - Beamforming information - Timing information to align transmission / reception boundaries of network-controlled repeater - Information on UL-DL TDD configuration - ON-OFF information for efficient interference management and improved energy efficiency - Power control information for efficient interference management (as the 2nd priority) Study and identify L1/L2 signaling (including its configuration) to carry the side control information [RAN1] Study the following aspects of network-controlled repeater management - Identification and authorization of network-controlled repeaters [RAN2, RAN3] NOTE2: Coordination with SA3 may be needed.

In this contribution, we discuss the side control information required to enable NR network-controlled repeaters.

# Discussions

Network-controlled repeaters can be considered as repeater (relay) nodes that can be configured via side control information to perform further advanced/smart operations. Some alternatives for the side-control information have been proposed in this SID objectives. We believe that the features that certainly differentiate the network-controlled repeaters from legacy RF repeaters are the beamforming capabilities and dynamic Tx/Rx switching capabilities for TDD. In this section the beamforming and full duplexing mode aspects are considered.

Proposal 1 Consider prioritizing the study on supporting side-control information for beam management and TDD aspects in network-controlled repeater design.

Fig. 1, Illustration on network-controlled repeater’s beam sweeping

## Beam Management Issues

Beam management based on beam sweeping has proven to be an effective procedure for finding the best beam between the UE and gNB. Considering the network-controlled repeaters with beamforming capabilities, the beam sweeping needs to be performed between the repeaters and UEs, see Fig.1.

As such, the gNB can share beamforming pattern information for beam sweeping procedure. However, one main issue with beam sweeping at the repeater is the need for the gNB to transmit RS per beam direction for the repeater to forward them toward the UEs. This could add extra overhead and consume time resources at gNB in case of repeater with large number of the beams.

Observation 1. The legacy beam sweeping procedure between network-controlled repeater and UEs can cause overhead and consume time resources at the gNB, specifically in case of network-controlled repeaters with large number of beams.

The gNB can associate different beams as part of beam sweeping procedure, e.g., with different beamwidth. Therefore, by sharing information on associations between the beams, the repeater can perform beam sweeping over one or more beams that are associated with each other.

As such, the gNB can ask the repeater to perform beam sweeping only for the beams that are associated with a certain direction. This could result in much lower overhead at the gNB.

Observation 2. The overhead at gNB, caused by beam sweeping between network-controlled repeater and UEs, can be reduced by sharing information on associations between the beams .

Proposal 2. Essential enhancements should be considered for beam sweeping in operation with network-controlled repeaters.

## Full Duplex Issues

Considering network-controlled repeaters to operate in full duplexing modes can be beneficial in taking advantage of scheduling flexibility. For example, in NR time division duplex (TDD), transmission pattern can be configured per slot, where the patterns include uplink (UL), downlink (DL) and flexible (F). While the UL and DL slots include symbols dedicated for UL and DL, respectively, the transmission pattern for the symbols in flexible slots can be configured based on Slot Format. As such, flexible scheduling can be used to allocate UL and DL capacity more dynamically and improve system spectrum efficiency.

Observation 3. Considering network-controlled repeaters to operate in full duplexing modes can be beneficial in taking advantage of scheduling flexibility in TDD schemes.

The transmission pattern’s flexibility can potentially cause repeater-to-repeater cross-link interference (CLI). For example, UL-to-DL interference may cause a repeater-to-repeater CLI when a repeater is having DL transmission from gNB at the same time as another repeater with UL transmission to gNB, on fully overlapping or partially overlapping frequency resources. Therefore, it may be necessary to consider side-control information in network-controlled repeaters with full duplex properties to measure and report potential interferences.

Proposal 3. Consider supporting means of CLI measurement and reporting at the network-controlled repeaters that support full duplexing modes.

# Summary

In this contribution, we discussed the side control information required to enable NR network-controlled repeaters. As follows, the observations and proposals are provided:

Observation 1. The legacy beam sweeping procedure between network-controlled repeater and UEs can cause overhead and consume time resources at the gNB, specifically in case of network-controlled repeaters with large number of beams.

Observation 2. The overhead at gNB, caused by beam sweeping between network-controlled repeater and UEs, can be reduced by sharing information on associations between the beams.

Observation 3. Considering network-controlled repeaters to operate in full duplexing modes can be beneficial in taking advantage of scheduling flexibility in TDD schemes.

Proposal 1. Consider prioritizing the study on supporting side-control information for beam management and TDD aspects in network-controlled repeater design.

Proposal 2. Essential enhancements should be considered for beam sweeping in operation with network-controlled repeaters.

Proposal 3. Consider supporting means of CLI measurement and reporting at the network-controlled repeaters that support full duplexing modes.

# References

- RP-213700, “New SI: Study on NR Network-controlled Repeaters”, ZTE Corporation.