3GPP TSG RAN WG1 #109-e														     R1-2203379

e-Meeting, May 9th – 20th, 2022

Agenda Item:		9.1.1.2

Source:				InterDigital, Inc.

Title:						Discussion on Multiple TA for multi-TRP

Document for:		Discussion and Decision

# Background

RAN plenary #94e approved the WID in [1] for Rel-18 MIMO enhancements. As described in WID, one of the goals in Objective 7 is to study and specify the operation related to multi-DCI multi-TRP deployment allowing for two timing advance TAs loops. In this context, for the case of simultaneous UL transmissions, the operation is limited to the description of Objective 6 in WID.

We are listing below for convenience the objectives 6 and 7 as they are part of the following discussion:

- Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC. - Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

# Discussion

In this contribution, we share our analysis and perspective on potential impacts of the enhancements on the specifications.

# Multi-TRP deployment scenarios analysis

In Rel-17, for both intra and inter-cell, multi-TRP operations were limited to within a CP reception. That means, no major UE timing adjustments in downlink were expected. For convenience we are listing here a table with the CP and symbol length for different SCS options:

| Parameter / Numerology (µ) | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| Subcarrier Spacing (KHz) | 15 | 30 | 60 | 120 | 240 |
| OFDM Symbol Duration (us) | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 |
| Cyclic Prefix Duration (us) | 4.69 | 2.34 | 1.17 | 0.57 | 0.29 |
| OFDM Symbol including CP (us) | 71.35 | 35.68 | 17.84 | 8.92 | 4.46 |

Table 1. Symbol and CP length for different numerologies

As we can see in Table 1, for 120KHz and 240KHz SCS, since CP size is quite small, employing two independent TA values in FR2 is required for supporting a viable multi-TRP operation.

Currently, for non-collocated transmission points, RAN4 specifications (36.133, 38.133) defines some time restrictions in terms of Maximum Receive Time Difference (MRTD) and Maximum Transmit Time Difference (MTTD). These limits are imposed due to the different UE capabilities in terms of baseband processing and/or network synchronization cases.

For example, for NR-DC synchronous case or NR inter-band CA within FR2, the maximum receive time difference is 8us. This corresponds roughly a bit less that a full symbol for 120KHz or two symbols for 240KHz numerologies.

Observation 1:  For NR-DC synchronous case or NR inter-band CA within FR2 the maximum receive time difference defined in 38.133 specification is 8us.

Observation 2: The baseband and time processing may impose limits of the Maximum Receive Time Difference and Maximum Transmit Time Difference for the multi-TRP scenario with 2 TAs scenario.

Even for synchronous operation in FR2, the cell synchronization requirement remains at 3us. Furthermore, for MIMO transmission, a 260ns uncertainty error may be observed between transmitter branches. Hence, all these uncertainties are considered in the definition of the current MTTD and MRTD timing requirements.

Therefore, in operation with more than one TA value, definition, and enforcement of MRTD and MTTD may become even more complicated. A similar approach should be a baseline to be considered for a multi-TRP scenario with multiple TA values, however the impact of UE processing/capability for enforcing time alignment should be studied in both RAN1 and RAN4 WGs.

Defining the deployment conditions/scenarios can be a first step into this study considering the applicable frequency range. Considering all uncertainties and the non-collocated TRPs scenario, it needs to be checked whether an 8us receive time difference would be sufficient. However, we suggest that this decision to be taken rather early due to its impact and RAN4 shall be involved in the process.

Proposal 1: Consider, for both RAN1 and RAN4, studying the multi-TRP deployment scenarios in terms of Maximum Receive Time Difference and Maximum Transmit Time Difference for this study.

# Specification impact analysis

For further discussion, the Primary TRP (pTRP) denotes the one that broadcast SSBs and Additional TRP (aTRP) denotes the one that has only CSI-RS for tracking and measurements.

The timing advance commands are sent by network over MAC CE. Thus, for a UE to receive a TA command, it must decode a PDSCH that contains the related MAC CE for TA command. In current operation, the MAC entity is related to a specific cell, while in our case for this study where two TA values are received, it needs to be determined where and how to apply a TA command.

As the TRPs are not collocated, the UE may move along within the TRPs combined coverage, and even it may be possible to receive two TA commands with opposite timing application. Thus, one or two TA commands are possible scenarios.

Observation 3: The UE may receive one TA for a specific TRP or two simultaneous TA commands for both TRPs even with opposite timing action.

Since MAC entity is serving both TRP in the intra-cell case, the TA command target has to be correctly identified.

Observation 4: The MAC entity is serving both TRP in the intra-cell case, and thus the TA command target must be correctly identified.

The WID allows for simultaneous transmission over two panels PUSCH+PUSCH and PUCCH+PUCCH combinations. When different TAs are allowed, depending on the Maximum Transmit Time Difference that may be defined we can envision the overlapping and non-overlapping parts of the UL transmissions. Currently the 38.213 specifies that the first symbol of the slot where the TA is applied will get shorter. Since in our scenario, the UE may receive two simultaneous TA commands for the TRPs, we see the necessity to investigate the scenario with two simultaneous UL transmissions and two TAs to get a clear understanding of the procedure at least on where and how to apply the two TAs.

Observation 5: The scenario involving two simultaneous UL transmissions and two TAs requires attention to get a clear understanding of the TA application procedure from the physical layer standpoint.

Nonetheless, the initial timing of an aTRP that is to be added to the UE configuration is important. Thus, after adding a secondary TRP, the UE and the network have to establish the correct timing, and this may require an initial RACH procedure. Upon transmission of a RACH, and the following RAR message, the UE will be in sync with aTRP for UL transmission. Furthermore, the network may ask for a RACH procedure using a PDCCH order. Or a beam failure that would require a PRACH procedure may occur as well. Such a RACH-related procedures for establishing the two TA loops needs to be investigated to get a clear understanding of the procedures. We can also envision a PRACH procedure that may have to overlap with other UL transmissions and this situation may need to be studied and related issues to be resolved.

Proposal 2: RAN1 to study related procedures for determination of where and how to apply a TA command when two TA values are received.

# Conclusions

This contribution discussed the issues on establishing two TA loops for intra-cell M-TRP operation with the two TAs loops. Based on the presented discussion, we make the following observations and proposals:

Observation 1:  For NR-DC synchronous case or NR inter-band CA within FR2 the maximum receive time difference defined in 38.133 specification is 8us.

Observation 2: The baseband and time processing may impose limits of the Maximum Receive Time Difference and Maximum Transmit Time Difference for the multi-TRP scenario with 2 TAs scenario.

Observation 3: The UE may receive one TA for a specific TRP or two simultaneous TA commands for both TRPs even with opposite timing action.

Observation 4: The MAC entity is serving both TRP in the intra-cell case, and thus the TA command target must be correctly identified.

Observation 5: The scenario involving two simultaneous UL transmissions and two TAs requires attention to get a clear understanding of the TA application procedure from the physical layer standpoint.

Proposal 1: Consider, for both RAN1 and RAN4, studying the multi-TRP deployment scenarios in terms of Maximum Receive Time Difference and Maximum Transmit Time Difference for this study.

Proposal 2: RAN1 to study related procedures for determination of where and how to apply a TA command when two TA values are received.

# References

- RP-213598, “New WID: MIMO evolution for Downlink and Uplink”, Samsung, 3GPP RAN Meeting #94e, December 6-17, 2021

- 3GPP 38.133, NR; Requirements for support of radio resource management, Rel-17, 2021