3GPP TSG RAN WG1 #109-e			       R1-2203380

e-Meeting, May 9th – 20th, 2022

Agenda Item:		9.1.2

Source:				InterDigital, Inc.

Title:						Aspects of CSI Enhancements

Document for:		Discussion and Decision

# Introduction

The Rel-18 MIMO WID [1] proposes to study enhancements to the CSI measurement and reporting. The WID scope includes the following objectives on CSI enhancements:

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - UE reporting of time-domain channel properties measured via CSI-RS for tracking […] - Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off - […] - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

The WID has outlined two specific deployment scenarios for study: High Doppler, and Coherent Joint Transmission (C-JT). In this contribution, we provide our views on Type-II codebook enhancements for these two scenarios.

# Background

For Rel-18, the WID requires that the focus of CSI enhancements should be on the Type-II codebook. NR Rel-17 supports two types of codebooks, i.e., Type-I and Type-II codebooks. Type-I codebook is mainly designed for single-user multiple-input-multiple-output (SU-MIMO), whereas the motivation behind Type-II codebook is multi-user (MU-MIMO) operation. The structure of both codebooks, i.e., Type-I and Type-II codebooks in matrix form is as W=W1W2. For both codebooks, the wideband channel state information (CSI) is contained in W1, whereas the subband CSI is captured in W2. Type I and Type-II codebooks have a history that dates back to the introduction of two-dimensional antenna arrays in LTE.

Type-II codebook provides significant performance improvements as compared to Type-I codebook. There are two variants of the Type-II codebook that are supported in Rel-17. The first variant enables the UE to report multiple spatial basis vectors and linear combination coefficients from a set of quantized codebooks to approximate the channel matrix and can support up to two-layer transmissions. The second variant, an enhanced version of the Type-II codebook named Enhanced Type-II codebook, was introduced in Rel. 16. In Enhanced Type-II codebook, the maximum number of supported layers is increased to four. To prevent excessive growth of UCI payload size, a frequency domain compression of W2 information is introduced.

However, the feedback overhead of Type-II codebooks remains significant and approximately linearly increasing with the number of subbands. The performance of Type-II CSI is still far from a MIMO operation with ideal CSI. As such, there is an ample room for improvement for Type-II based CSI measurement and reporting to achieve a better trade-off between performance and feedback overhead.

# High Doppler Enhancements

Rel-18 High Doppler scenario

The first objective of Rel-18 MIMO work aims to enhance CSI reporting for high/medium UE velocities. Per WID, Rel-18 aims at exploiting time-domain correlation / Doppler-domain information to assist downlink precoding in frequency range 1 (FR1). More specifically, Rel-18 intends to refine the existing Rel-17 codebook without any modification to the spatial and frequency domain basis and to study UE reporting of CSI for possible enhancements.

For a UE with high velocity, the properties of a wireless channel changes at a fast rate. The degree of the change in properties of the wireless channel depend on the UE velocity and on the carrier frequency. Particularly, a change in the frequency is observed at the receiver due to the relative motion between the transmitter and a receiver. Apart from the observed change in the carrier frequency, relative motions between a transmitter and a receiver causes spectral broadening. The effect of spectral broadening can loosely be defined as the range of frequencies over which a received Doppler spectrum essentially remains non-zero, and the phenomenon is widely known as the Doppler spread. Doppler spread is quantified by the time-rate of change of the wireless channel. Explicitly, the coherence time of the wireless channel is the inverse of the Doppler spread. The Doppler spread directly depends on two parameters, i.e., the operating frequency and the UE velocity. Therefore, the motivation of using FR1 for high/medium UE velocities becomes clear.

Definition of high/medium velocity

In general, a MIMO system can be a closed- or an open-loop system. The deciding factor in favour of selection of closed-loop versus an open-loop operation lies in whether the achieved gain by introducing CSI feedback overhead is justified. For example, if a UE is traveling at a very high velocity, there may not be a point in using a closed-loop MIMO operation, as the CSI feedback rapidly becomes outdated. On the other hand, if by tolerating some additional CSI overhead, the performance can be improved, then a closed-loop MIMO operation will be beneficial. Unfortunately, the wording “high/medium” in the Rel-18 objective is not clear. Such ambiguity in expressing a target maximum velocity is not going to help for developing an efficient and robust solution. In our view, an explicit value of UE velocity/Doppler shift/Doppler spread is needed to better understand the requirements behind the objective set for Rel-18, such that a proper solution can be devised.

Observation 1: The description of the objective is not clear, i.e., the wording high/medium UE velocities is ambiguous and a hurdle in the development of an efficient solution.

Proposal 1: Specify a UE velocity/Doppler or a range of UE velocities/Dopplers to set the ground for an effective and robust design.

The performance of Type-II codebook(s) entirely depends on the amount of the required feedback overhead. If the feedback overhead is reduced, several elements of the CSI report can be quantized with a higher resolution for reporting, which results in an enhanced performance. The Enhanced Type-II codebook uses the same principles for achieving performance gain over Type-II codebook. In our view, compression in other domains can also be applied for performance enhancements.

Time Domain Compression / Doppler Domain Information

The feedback overhead of Type-II codebook(s) is directly proportional with the number of subbands. The Enhanced Type-II codebook applies frequency domain compression on W2 to make it sparse in nature leading to feedback overhead reduction. The employed compression exploits the correlation between channels of different subbands. Such approach can be also used in time domain where time-domain correlation between adjacent subbands can be used to further reduce the feedback overhead. In the event of a UE with high velocity, the adjacent subbands in time-domain may experience a nearly similar trend of change in channel characteristic.

Observation 2: Time domain compression has the potential to further reduce the feedback overhead of Type-II codebooks.

Proposal 2: Study time domain compression for reducing the feedback overhead of Type-II codebooks.

In high mobility environments, the channel coherence time becomes smaller. Hence, the reported CSI by the UE becomes quickly outdated as the wireless channel changes at a fast rate. Instead of exploiting the channel coherence time, long term channel statistics can be used to assist DL precoding. It is well-known that path loss and the shadowing effects remains the same for longer durations as compared to factors responsible for quantifying coherence time. The duration over which the channel large-scale parameters, i.e., path loss and shadowing remains constant is known as a channel stationarity time, which is much longer than the coherence time. In a scenario with high UE velocity, DL precoding can be mainly assisted by the information contained in W1, however some information in W2 may still be needed.

Observation 3: In a scenario with high UE velocity, DL precoding can be mainly assisted by the information contained in W1, however only some information in W2 may be needed.

Proposal 3: RAN 1 to study elimination of low impact elements in W2.

To support implementation of an efficient time domain compression, some enhancements for CSI mechanism can be envisioned. At a high level, two separate CSI processes can be considered where the first process would be to support the second process. For example, the first process would be to evaluate stationarity of the wireless channel and derive sufficient information required for time domain compression and/or prediction. Then, the second CSI process would be to perform the actual CSI measurements for the MIMO channel and support CSI reporting for prediction. Therefore, one can expect that CSI-RS transmission for the first process would be at a higher rate than that of the second process. Furthermore, it becomes necessary that the existing priority reporting levels for CSI reporting to be tailored for feedback reduction. In other words, certain elements of the compressed CSI report can be prioritized for reporting, while other elements may be dropped or reported at a much slower rate.

Observation 4: To enable implementation of an efficient time domain compression and CSI prediction, support of two separate CSI processes is required.

Proposal 4: Consider support of two separate CSI processes where,

  - the first process would be to evaluate stationarity of the wireless channel and derive sufficient information required for time domain compression and/or prediction.

  - and, the second CSI process would be to perform the actual CSI measurements for the MIMO channel and support CSI reporting for prediction.

Feedback Overhead Reduction Methods

The existing Rel. 17 methods for reporting several elements of the CSI are complex and resource hungry. Particularly, the reporting of subband CSI elements contained in W2 requires comprehensive feedback overhead. CSI elements within W2 can be divided into two categories, i.e., subband amplitude coefficients and phase combining coefficients. In Enhanced Type-II codebook, each subband amplitude coefficient is quantized using three bits and each subband phase coefficient is quantized using four bits. The matrix W2 also contains information for reporting indices of the dominant non-zero coefficients pertaining to each beam and each discrete Fourier transform (DFT) vector.

Rel-17 uses a bitmap for reporting the indices of dominant subband amplitude coefficients. Similarly, two bitmaps are used to report the quantized subband amplitude coefficients and quantized phase combining coefficients. The Type-II codebook feedback can be reduced by adjusting the number of non-zero subband amplitude coefficients by using different combinations of frequency domain parameters for quantifying the number of non-zero coefficients, which was well studied by RAN1 during the development phase of Enhanced Type-II codebook. The feedback overhead can also be reduced by using smaller quantization bits for reporting subband and wideband amplitude coefficients and phase coefficients. However, for the case of high speed UEs, there is an opportunity to revisit these aspects and consider new methods for reporting indices of the dominant coefficients, subband amplitude coefficients, and phase combining coefficients. Therefore, in our view, it would be important to study alternative reporting methods for further reducing the feedback overhead while considering their implementation and operational complexity to keep up with the high reporting demands of high UE velocity scenarios.

Observation 5: The existing reporting methods for reporting the wideband and subband coefficients is an expensive process in terms of feedback overhead.

Proposal 5: Study alternative reporting methods with reduced feedback overhead for reporting the indices of non-zero coefficients, subband amplitude coefficients, phase coefficients and wideband amplitude coefficients.

# C-JT Enhancements

Rel-17 CSI enhancements for mTRP

Rel-17 recognized the need to introduce enhancements to the CSI measurement and reporting procedure to support mTRP. Hence, an agenda item was tasked with studying CSI for Non-Coherent Joint Transmission (NC-JT). The objective was to enhance the CSI reporting framework in FR1 and FR2 to enable more dynamic channel/interference hypotheses in a DL deployment with multi-TRP and multi-panel UEs.

Prior to Rel-17, a different CSI reporting setting was required for each measurement hypothesis. A UE would be configured with different combinations of Channel Measurement Resources (CMRs) and Interference Measurement Resources (IMRs) for either single TRP or NC-JT assumption. This increased the configuration complexity and could only support NC-JT measurement hypothesis through multiple CSI reports. Rel-17 enables a single CSI report setting to be configured with multiple measurement hypothesis such that a UE may report in a single CSI report CSIs for more than one measurement hypothesis.

As such, two CMR groups are introduced, where each is associated to a TRP.  Thus, groups 1 and 2 are configured with K1 and K2 resources, respectively where K=K1+K2 CMRs is defined by UE capability. Resources from group 1 can be paired with resources from group 2 which signifies that these resources may be used for NC-JT measurement assumptions. Up to N such pairs are RRC configurable as part of the CSI reporting setting.

Prior to Rel-17, the CSI-RS Resource Indicator (CRI) was used to identify a single CSI-RS resource. With Rel-17, the CRI can now also identify CSI-RS pairs for CJT. Therefore, a UE can generate a CSI report with CRIs that either identify one CSI-RS (single TRP hypothesis), or CSI-RS pairs (NC-JT hypothesis).

In addition, two different CSI reporting modes are now configurable. In reporting Option 1, a CSI reporting setting can be configured to report X CSIs assuming sTRP measurement hypothesis, and 1 CSI assuming NC-JT measurement hypothesis. The value X is configurable and can take any value from the set {0,1,2}. The UE can report a complete set of CSIs for multiple hypotheses which gives more flexibility to the network for scheduling the UE between sTRP or mTRP. However, this may require high CSI reporting overhead for the UCI to fit all CSIs. Instead of reporting multiple hypothesis, in reporting Option 2, a UE reports the best CSI between NCJT and sTRP. The CRI maps back to either a sTRP or mTRP hypothesis, so the network can determine which type is included in the CSI report.

Rel-18 C-JT mTRP scenario

Although the Rel-17 enhancements provided some tools for mTRP CSI reporting, it remains focused on the NC-JT scenario. For Rel-18, based on the fourth objective, the scope of the work is extended to cover the C-JT scenario as shown in Figure 1.

Figure 1 C-JT deployment scenario

The WID focuses on a deployment where a UE is expected to measure the channel from up to 4 TRPs in an FR1 deployment. Each TRP-UE link is served by a different spatial filter that provides maximum received signal strength at the UE. Amongst the measurement set, a subset of TRPs can be scheduled in C-JT mode towards one UE. C-JT is a transmission mode where a precoder is jointly calculated for more than one spatially separated TRP such that the signal coherently combines at the receiver. For example, TRP1 and TRP2 may apply one precoder of N=N1+N2 antenna ports, where N1 and N2 are the number of antenna ports of TRP1 and TRP2, respectively.

Since the antenna ports are not co-located, tight coordination is required between TRPs to optimize the effectiveness of C-JT. This type of scenario thus requires accurate information about the fading channel to achieve spatial coherency across transmit antenna ports. Since FDD is the focus and high accuracy is important to C-JT, it has been determined that Rel-18 should focus on refining the Type-II codebook to support the C-JT hypothesis.

Reporting overhead of Type-II CSI for mTRP

One major challenge of using the Type-II CSI is the high reporting overhead. It is a powerful tool where a UE can indicate multiple spatial basis vectors, frequency domain basis vectors, and associated combining coefficients. It gives the network a much more accurate approximation of the fading channel than was possible with Type I codebook. Basis vectors and combining coefficients are quantized into several codebooks which can go up to subband frequency granularity. This generates many indices that a UE needs to report for a single UE-TRP link. A straightforward application of Type-II CSI to the C-JT case would require an N-fold increase in reporting indices just for sTRP reporting, and even more when considering all possible TRP pairs for C-JT. The considered solutions should therefore try to provide complete information for precoders without overloading the UCI. The resources required for the UCI to carry multiple CSI hypothesis combinations are limited, and their utility should be maximized so as to not occupy too many resources for control signaling.

Observation 6: A straightforward application of Type-II CSI for up to 4 TRP C-JT CSI requires many indices for all possible C-JT pairs and sTRP measurement hypothesis.

Proposal 6: Consider solutions that minimize the reporting overhead of Type-II CSI when C-JT precoders are determined.

Precoder co-phasing factor

Another distinction between NC-JT and C-JT is that NC-JT precoders are calculated separately per TRP and do not consider the coherency at the UE reception. Rel-17 Type-II precoders are sufficient in NC-JT for the UE to determine per TRP CSI and report them back to the network. However, for C-JT, a subset of antenna ports from the precoder are sent from TRP1, and another subset for TRP2. Due to the spatial separation, the signals arrive at the UE with different phase shifts which need to be corrected. There are several ways of capturing the co-phasing factor between TRPs while reusing as much as possible the established Type-II precoding framework. Additionally, the co-phasing factor depends on which selection of TRPs are scheduled for C-JT. Therefore, there are reporting overhead issues that need to be considered as well when discussing solutions on how the co-phasing information is indicated.

Observation 7: Co-phasing information between physically separated antenna ports is part of the C-JT precoder design.

Proposal 7: Study how to capture the co-phasing information between C-JT TRPs using the Type-II CSI precoding framework.

Multiple hypothesis reporting

Based on Rel-17 enhancements, a UE is configurable with sTRP measurement hypothesis or NC-JT measurement hypothesis. C-JT adds another measurement hypothesis that a UE needs to consider. The network can configure the CMRs and IMRs for the UE to measure any possible hypothesis. However, the issue is at the UE side with the more involved CSI calculation complexity. The amount of computation required by a UE increases significantly if configured with all 3 hypotheses due to the number of TRPs, pairs of TRPs, and CSI type (e.g., sTRP, NC-JT, or C-JT). This will have an impact on the CSI processing time, and it also increases the UCI payload because more than one hypothesis can be reported. For example, not all hypotheses may be relevant at every CSI reporting opportunity. It should be specified which hypothesis does the network expect in a CSI report, and how a UE determines which hypothesis to report for any given CSI report. This issue is compounded by the complexity of determining a complete set of indices for Type-II CSI reporting. Therefore, Rel-18 solutions should envisage ways to handle the variable TRP numbers, and different measurement hypothesis types to avoid straining the UE and UCI resources.

Observation 8: UE needs to consider C-JT measurement hypothesis in addition to sTRP and NC-JT which increases the CSI computational complexity.

Proposal 8: Study CSI reporting enhancements to handle multi-hypothesis reporting.

# Conclusion

In this contribution, we provided our views regarding CSI enhancements for Rel-18 MIMO. Based on the presented discussion, we make the following observations and proposals:

Observation 1: The description of the objective is not clear, i.e., the wording high/medium UE velocities is ambiguous and a hurdle in the development of an efficient solution.

Observation 2: Time domain compression has the potential to further reduce the feedback overhead of Type-II codebooks.

Observation 3: In a scenario with high UE velocity, DL precoding can be mainly assisted by the information contained in W1, however some information in W2 may still be needed.

Observation 4: To enable implementation of an efficient time domain compression and CSI prediction, support of two separate CSI processes is required.

Observation 5: The existing reporting methods for reporting the wideband and subband coefficients is an expensive process in terms of feedback overhead.

Observation 6: A straightforward application of Type-II CSI for up to 4 TRP C-JT CSI requires many indices for all possible C-JT pairs and sTRP measurement hypothesis.

Observation 7: Co-phasing information between physically separated antenna ports is part of the C-JT precoder design.

Observation 8: UE needs to consider C-JT measurement hypothesis in addition to sTRP and NC-JT which increases the CSI computational complexity.

Proposal 1: Specify a UE velocity/Doppler or a range of UE velocities/Dopplers to set the ground for an effective and robust design.

Proposal 2: Study time domain compression for reducing the feedback overhead of Type-II codebooks.

Proposal 3: RAN 1 to study elimination of low impact elements in W2.

Proposal 4: Consider support of two separate CSI processes where,

  - the first process would be to evaluate stationarity of the wireless channel and derive sufficient information required for time domain compression and/or prediction.

  - and, the second CSI process would be to perform the actual CSI measurements for the MIMO channel and support CSI reporting for prediction.

Proposal 5: Study alternative reporting methods with reduced feedback overhead for reporting the indices of non-zero coefficients, subband amplitude and phase coefficients and wideband amplitude coefficients.

Proposal 6: Consider solutions that minimize the reporting overhead of Type-II CSI when C-JT precoders are determined.

Proposal 7: Study how to capture the co-phasing information between C-JT TRPs using the Type-II CSI precoding framework.

Proposal 8: Study CSI reporting enhancements to handle multi-hypothesis reporting.

# References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung, RAN Meeting #94e, December 2021