3GPP TSG RAN WG1 Meeting #109-e											        R1-2203381

e-Meeting, May 9th – 20th, 2022

Agenda Item:		9.1.3.1

Source:				InterDigital, Inc.

Title:						High Capacity DMRS

Document for:		Discussion and Decision

# Background

In RAN plenary #94, the WID for Rel-18 MIMO enhancements was finalized [1]. According to the WID, DMRS enhancements for both uplink and downlink are among the considered improvements for Rel-18 MIMO.

Per chairman’s guidance, besides the objective of increasing orthogonal DMRS ports for uplink and downlink MU-MIMO transmissions as stated in Objective 3, this agenda item is also tasked to support DMRS design for 8 TX UL SU-MIMO operation as described in Objective 5.

Objective 3: Study, and if justified, specify larger number of orthogonal DMRS ports for downlink and uplink MU-MIMO (without increasing the DM-RS overhead), only for CP-OFDM, - Striving for a common design between DL and UL DMRS - Up to 24 orthogonal DM-RS ports, where for each applicable DMRS type, the maximum number of orthogonal ports is doubled for both single- and double-symbol DMRS Objective 5: Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

# Discussion

In this contribution, we first provide an overview of the current DMRS in NR, and then we share our perspectives on potential solutions for DMRS capacity enhancements.

# NR DMRS Design

In NR Rel-17, the DMRS configuration that defines placement of reference signals in time and frequency is designed to address a variety of scenarios with the aim of maintaining high design flexibility and forward compatibility, while addressing implementation complexity and constraints from the receiver perspective. Moreover, the placement pattern of the pilot is a vital part of the design of an efficient MIMO communication system because it impacts the number of orthogonal antenna ports supported for SU/MU-MIMO transmission.

At a high level, DMRS mapping in NR is principally defined by two main configurations of mapping types, i.e., Type A or B where each can be further configured as Type 1 or 2. Configuration types A/B define time domain mapping of a DMRS configuration, while types 1/2 specify frequency density of the DMRS resources.

For mapping type A, the position, l0 of the first DMRS symbol is placed on the third or fourth OFDM symbol, whereas for mapping type B, l0 relative to the start of the scheduled PDSCH or PUSCH resources. Also, for mapping type A, the duration, ld is between the first OFDM symbol of the slot and the last OFDM symbol of the scheduled PDSCH (or PUSCH) resource while for mapping type B, the ld is the number of OFDM symbols of scheduled PDSCH (or PUSCH) resource allocation. When it comes to increasing the number of DMRS ports, DMRS mapping type may not be a relevant aspect of the discussion, as it primarily defines the range of PDSCH and DMRS mappings within a slot. On the other hand, enhancements for configuration Types 1/2 would be the main outcome of an effort for increasing the number of DMRS ports.

Figure 1 shows some examples of two DMRS configuration types for a CP-OFDM waveform that are referred to as DMRS configuration Type 1 and DMRS configuration type 2. Both configurations support single-symbol and a double-symbol DMRS mappings. To use the available resources efficiently, orthogonal cover codes (OCC) are used to support multi-port operation of DMRSs. For example, in the shown single-symbol configuration, two antenna ports using the same resource elements are orthogonalized by using a length 2-OCC, whereas for a double-symbol configuration, four antenna ports are orthogonalized by using a length 4-OCC.

The DMRS configuration types specify the density of the DMRS in the frequency domain and determines the maximum number of orthogonal antenna ports that can be supported in per physical resource block. Therefore, the main difference between the configuration types is that DMRS Type 1 has a denser frequency domain occupancy of the pilots in comparison to DMRS Type 2, thus making it more resilient to frequency domain channel variations. On the other hand, type 2 can have more orthogonal antenna ports (although there is a trade-off of reduced DMRS density). Thus, DMRS Type 1 can support up to 4 orthogonal ports a single-symbol and up to 8 orthogonal ports for a double-symbol configuration, whereas DMRS Type 2 can support up to 6 orthogonal ports and up to 12 orthogonal ports for a single-symbol and double-symbol configuration respectively – thus, making it more suitable for MU-MIMO.

Single-symbol DMRS, length 2-OCC                	Double symbol DMRS. Length 4-OCC

(a) – DMRS configuration type 1

Single-symbol DMRS, length 2-OCC                    Double symbol DMRS. Length 4-OCC

(b) – DMRS configuration type 2

Figure 1 Exemplary cases of DMRS configurations for a CP-OFDM waveform

# Design Restrictions and Guidelines

Based on the above description and general objectives defined in WID, we can consider a basic set of guidelines for the enhancements required for increasing the number of DMRS ports. Also, such guidelines help to focus the scope of the work and have a faster progress during the phase of the work item.

Waveform

The use case scenarios considered in both Objectives 3 and 5 are all intended for CP-OFDM waveform. As also implied in Objective 3, UL MU-MIMO is only relevant for when UEs are experiencing a relatively high SNR, therefore the de facto mode of operation for uplink MU-MIMO is always based on CP-OFDM. On the other hand, since Objective 5 is to enable 8 Tx UL operation to support 4 and more layers per UE operation, again the utilized waveform will be also based on CP-OFDM, as DFT-s-OFDM does not support multi-layer transmission.

Observation 1:  The use case scenarios considered in both Objectives 3 and 5 are all intended for CP-OFDM waveform.

Proposal 1: RAN1 enhancements for increasing the number of DMRS ports are only to support CP-OFDM waveforms.

Low Doppler

The use case scenarios considered in both Objectives 3 and 5 are all intended for almost static channel. In Objective 3, MU-MIMO is only relevant for very low mobility conditions when UEs can be paired with some consistency. Therefore, the rate of variations of the channel over time for MU-MIMO can be assumed minimal. In case of high rate of change in channel, SU-MIMO will be a better mode of transmission. Furthermore, since targeted UEs in Objective 5 are mainly have zero or very low mobility, again an almost static channel can be considered.

Observation 2:  The use case scenarios considered in both Objectives 3 and 5 are all intended for almost static channel.

Proposal 2: RAN1 evaluation of proposed enhancements should be only for channels with low Doppler.

Sparse Mapping

If there is no restriction on the overhead, it is easy to increase the number ports to almost any arbitrary number. However, as briefly mentioned in the Objective 3, the proposed solutions should require no increase in the DM-RS overhead. Therefore, some trade-offs in frequency and time features of DMRS need to be made to be able to support a higher number of DMRS ports. For example, lower density transmission of DMRS in frequency and time domains can be utilized.

Observation 3:  If there is no restriction on the overhead, it is easy to increase the number ports to almost any arbitrary number.

Proposal 3: RAN1 studies lower density transmission of DMRS in frequency and time domains.

Depending on the use case scenarios (for example, low doppler environments) and the DCI scheduling assignment (regarding whether more orthogonal DMRS ports are required), a UE may support up to 24 orthogonal DMRS ports as considered in Objective 3 based one of the following DMRS patterns as shown in Figure 2.

Single-symbol DMRS, length 2-OCC                    Double-symbol DMRS, length 4-OCC

(a) - DMRS pattern 1 for a CP-OFDM waveform

Single-symbol DMRS, length 2-OCC                    Double-symbol DMRS, length 4-OCC

(b) - DMRS pattern 2 for a CP-OFDM waveform

Figure 2 – Exemplary cases of proposed DMRS patterns for a CP-OFDM waveform

Unlike DMRS type 1 and 2 configurations in NR Rel.17, it can be observed that the density of the DMRS pilots have been reduced - with an added advantage of increased number of orthogonal DMRS ports. Thus, the DMRS patterns (1 and 2) use OCC length-2 and OCC length-4 for the single-symbol and double symbol DMRS configurations, respectively to support multi-port operation. Even though, the DMRS patterns in Figure 3 yield more orthogonal ports, the need to support new CDM groups within same RB should be considered.

Self-contained OCC

By increasing the number of DMRS ports, it may be needed to support longer OCC lengths, for example length 8. Even if, OCC lengths do not increase in length, grouping of DMRS ports within a slot will need to be rearranged such that more CDM groups for supporting more ports can be defined. As a design rule, it is important to assure that new CDM groups do not cross slot boundaries. In other words, every scheduled RB should be self-contained by having all the required CDM groups within the same RB.

Observation 4:  Grouping of DMRS ports within a slot will need to be rearranged such that more CDM groups for supporting more DMRS ports can be defined.

Proposal 4: Every scheduled RB should be self-contained by having all the required CDM groups within the same RB.

The DMRS patterns in Figure 3 can be considered to obtain more orthogonal DMRS ports. The DMRS patterns 3 and 4 both use OCC length-4 (for single-symbol DRMS) and OCC length-8 (for double symbol DMRS) for multi-port operation. Also, it is easy to observe that the placements of the DMRS pilots are within a slot and the required CDM groupings are within the scheduled RB. In addition, by using longer length OCCs, the DMRS patterns 3 and 4 can support up to 24 orthogonal DMRS ports.

Single-symbol DMRS, length 4-OCC                                  Double-symbol DMRS, length 8-OCC

(a) - DMRS pattern 3 for a CP-OFDM waveform

Single-symbol DMRS, length 4-OCC                                  Double-symbol DMRS, length 8-OCC

(b) - DMRS pattern 4 for a CP-OFDM waveform

Figure 3 – Exemplary cases of DMRS patterns with longer OCC length for a CP-OFDM waveform

# Conclusions

In this contribution we provided an overview of DMRS design in NR and provided some basic guidelines for increasing the number of DMRS ports. Based on the presented discussion, we make the following observations and proposals:

Observation 1:  The use case scenarios considered in both Objectives 3 and 5 are all intended for CP_OFDM waveform.

Observation 2:  The use case scenarios considered in both Objectives 3 and 5 are all intended for almost static channel.

Observation 3:  If there is no restriction on the overhead, it is easy to increase the number ports to almost any arbitrary number.

Observation 4:  Grouping of DMRS ports within a slot will need to be rearranged such that more CDM groups for supporting more DMRS ports can be defined.

Proposal 1: RAN1 enhancements for increasing the number of DMRS ports are only to support CP-OFDM waveforms.

Proposal 2: RAN1 evaluation of proposed enhancements should be only for channels with low Doppler.

Proposal 3: RAN1 studies lower density transmission of DMRS in frequency and time domains.

Proposal 4: Every scheduled RB should be self-contained by having all the required CDM groups within the same RB.

# References

- RP-213598, “New WI: MIMO evolution for Downlink and Uplink”, Samsung, 3GPP RAN Meeting #94e, December 6-17, 2021