3GPP TSG RAN WG1 #109-e			     R1-2203382

e-Meeting, May 9th – 20th 2022

Agenda Item:		9.1.3.2

Source:				InterDigital, Inc.

Title:						Enhanced SRS Operation

Document for:		Discussion and Decision

# Introduction

The Rel-18 MIMO WID [1] proposes several enhancements to uplink reference signals. The WID scope includes the following objectives on SRS enhancements:

4. Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: […] - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32 […] 5. Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

# Background

Prior to NR, 3GPP specifications were mainly targeting consumer devices such as smartphones which have a relatively similar form factor across the market. However, as wireless communication evolved, more and more devices of all different shapes and sizes, and with a wide range of service requirements are making use of 3GPP standards. One objective of the NR specification is to address this challenge by coming up with solutions that target a wide range of devices with multiple applications so that the specification can be future-proof. Therefore, UE devices are expected to be capable of supporting more advanced and varied antenna structures.

The WID targets two scenarios for SRS enhancements. First, the Rel-18 WID aims at studying enhancements to use the SRS for channel estimation in the case of Coherent Joint Transmission (C-JT). A second objective of the WID is to enhance the SRS to enable transmission with more antenna ports (up to 8TX), and more layers (up to 4). In this contribution, we provide our views on these two objectives.

# C-JT Enhancements for TDD

Rel-18 C-JT scenario

In an sTRP scenario, SRS are used in TDD deployments to facilitate channel estimation by making use of channel reciprocity. Since the same carrier is used on DL and UL, the network can estimate the UL channel using an SRS, and the DL channel is assumed to be the same. The specification provides an SRS usage set to antennaSwitching for this purpose where the SRS transmitted on the UL is used for DL CSI estimation.

The Rel-18 WID focuses on a deployment such as Figure 1 where a UE is expected to measure the channel for up to 4 TRPs in an FR1 deployment. Each TRP-UE link is served by a different spatial filter that provides maximum received signal strength at the UE. Amongst the measurement set, a subset of TRPs can be scheduled in C-JT mode towards one UE. C-JT is a transmission mode where a precoder is jointly calculated for more than one spatially separated TRP such that the signal coherently combines at the receiver. For example, TRP1 and TRP2 may apply one precoder of N=N1+N2 antenna ports, where N1 and N2 are the number of antenna ports of TRP1 and TRP2, respectively.

Figure 1 Inter-TRP cross-SRS interference in channel estimation for C-JT

Based on the UL channel estimates calculated using the SRS, the network determines the DL precoder for the UE. In the C-JT case, TRP1 and TRP2 receive channel estimates of each UE-TRP link, and then the network determines the DL precoder for joint TRP1 and TRP2 transmission towards the UE. It is crucial that a C-JT precoder is calculated based on accurate channel estimate because the signal transmitted from non-co-located antenna ports needs to be coherently received at the UE. The network relies on the SRS to estimate the co-phasing factor between TRPs that factors into the C-JT precoding calculation. This figure highlights the issue if a straightforward application of existing SRS design is reused for mTRP. UE2 is sending an SRS resource towards TRP2 which generates interference at TRP2. The channel estimate for the UE1-TRP2 link is thus degraded which compromises the C-JT precoder computation. In the following, we highlight some areas of investigation that should be considered for the C-JT TDD scenario.

SRS interference management for mTRP

One constraint given by the WID is to reuse the existing SRS design as much as possible with the same root sequences, no additional resources occupied, and with existing comb structures. The SRS design of NR provides many flexible configurations of parameters such as root sequences, OCC grouping, time and frequency shifts, combs, etc… that allow the network to assign orthogonal or low cross-correlation parameters for different antenna ports or UEs. The added challenge is to now consider mTRP. More SRS resources are required to be configured per UE to enable it to perform measurements for all serving TRPs. It’s expected that the interference in the UL will increase because more SRS transmissions are active at a time which will be harmful to the overall UL channel estimates. It should be studied how to coordinate the configuration of different SRS parameters across multiple UEs to maximize the orthogonality between them. If orthogonality is not possible, then the low cross-correlation should be the second priority.

Observation 1: mTRP channel estimation requires more SRS transmissions by UEs which increases the total interference in the network.

Proposal 1: Study how to coordinate SRS configuration across multiple UEs to maintain orthogonality or low cross-correlation.

SRS resource selection for mTRP

Since Rel-16, the PDSCH can be sent through mTRP by configuring the PDSCH with different repetition modes (e.g., TDM, FDM, SFN). In this configuration, the UE expects to receive two TCIs in the grant that are mapped to the PDSCH repetitions. The transmission can be dynamically switched to sTRP by indicating the same TCI in both fields in the grant thus mapping the same TRP to all repetitions. The scheduling choice between sTRP and mTRP (and the choice of TRPs for C-JT) depends on the CSI reported by the UE for sTRP and mTRP assumptions. Other network considerations (e.g. resource utilization on different TRPs, interference in service area) also affect this decisions. Therefore, a UE may be expected to support multiple SRS transmissions for channel estimation with many configurations of spatial filters, and power control parameters to cover all TRPs.

However, only a subset of SRS resources may be relevant to the network for channel estimation. For example, a network may be able to schedule multiple UEs in C-JT or sTRP; however, a network may prefer at another time to reduce the number of UEs considered for C-JT due to, e.g., high resource utilization. Then, it would be beneficial to enable the network to select triggered SRS resources for sTRP or mTRP.

Existing SRS resource configuration assumes transmission to a sTRP which means spatial filters are associated to only one TRP. Depending on the UL target TRP, a UE may use a resource with a different spatial filter because the orientation of the UE with respect to the TRP affects the optimal beam direction. The existing mechanisms for updating the spatial filters for an SRS resource rely on RRC reconfiguration or MAC-CE activation. Similarly for power control, an SRS resource set can be configured with multiple candidate pathloss references, but only one is active at a time targeting a sTRP mode of operation. Mapping one SRS resource per triggering command may create issues in configuration overhead to handle all TRPs.

Therefore, there is room to further optimize the SRS resource triggering such that SRS resources are triggered flexibly for either sTRP or mTRP operation with different spatial filters and power control parameters.

Observation 2: Dynamic switching between sTRP and C-JT mode of operation requires flexible SRS configurations for both cases.

Proposal 2: Evaluate how to enhance the SRS triggering procedure to support the numerous SRS configurations for mTRP.

# 8TX Enhancements

Rel-18 8TX scenario

Figure 2 UL transmit process chain

Figure 2 shows an exemplary transmit processing chain in a UE. The process starts with first applying forward error correction coding and CRC attachment. This process may produce at least one codeword at the time. The coded bit sequence is then scrambled and modulated according to the configured modulation format, e.g., QPSK, 16QAM, etc. In a UE with multiple transmitter antennas, the sequence of modulated symbols is mapped to a number of parallel layers. The produced layers are then precoded according to the condition of the wireless channel and mapped to their corresponding antenna ports. Afterwards, proper waveforms are generated and transmitted over the air according to a set power. For each transmission, in parallel to the above process, some reference signal is generated, and mapped along with the modulated data symbols.

According to the Rel-17 NR specification, a UE can support an uplink transmission with the following capabilities:

- up to 4 layer transmission

- 4 transmission ports

- 4 PAs

- Single codeword transmission

- DFT-s-OFDM and CP-OFDM

Layer mapping and dual codeword support

Rel-17 UL is defined only for single codeword transmission. This contrasts with the DL where mapping for up to two codewords is defined. If the number of layers is less than or equal to 4, a DL transmission maps one codeword to 4 layers, and 2 codewords map over more than 4 layers.

In Rel-18 MIMO, to enhance coverage, reliability, and throughput, for certain categories of UEs, some enhancements to SRS are considered to enable up to 8 antenna transmission, and more than 4 layers. Since up to 8TX is considered for Rel-18, the number of supported layers is also increased. For higher number of layers, dual codeword may be used. In UL, the situation may be different since in AI 9.1.4.1 2 codewords are being considered with the 4 layers case. The codeword-to-layer mapping for UL should be defined with consideration of the different UL transmission modes being considered in Rel-18.

Observation 3: Codeword-to-layer mapping for UL may differ to the DL due to different UL transmission modes.

Proposal 3: Study impacts on the UL processing chain of supporting higher number of layers.

Similarly	to the layers, the dual codeword operation is a new feature of Rel-18 UL that bears consideration. Several fields of the scheduling grant are indicated per codeword such as MCS, NDI, RV, UL-SCH, priority indication, HARQ process number, DAI, CBGTI. For dual codeword support, two sets of these parameters are required at the UE. Solutions in Rel-18 should consider the signaling aspects for dual codewords, and how a UE receives indication of these parameters per codeword.

Observation 4: Dual codeword operation is a new scheduling mode for UL.

Proposal 4: Study the enhancements to UL resource grants when dual codeword is supported.

8TX power considerations

In NR Rel-16, two modes of full power operation are introduced to support full power transmission for partially coherent and non-coherent UEs. In Mode 1, the procedure is relatively straight forward. gNB only indicate TPMIs that have all non-zero elements. In Mode 2, according to the UE structure, UE reports the subset of precoders that can be used by gNB to support full power operation.

In UEs with large number of antennas, RF and antenna structure can be more complex and architecturally may be very different from one to another. As seen in the DL when 8TX or more antennas are considered, the array elements can be arranged in multiple configurations at the UE. To enhance the spatial directionality of the spatial filtering, the antenna elements can be arranged in two-dimensional arrays which allows for antenna ports to be virtualized in multiple configurations. Additionally, more the antenna elements can be distributed across more than one antenna panel and similarly for the antenna ports.

Moreover, the UEs for the considered use cases with 8TX operation have higher power rating than PC3, and a wider range of PA ratings. For example:

- FR2 for FWA: 										EIRPmin = 40 to 55 dBm per band

- FR2 for VEH (PC2): 								EIRPmin = 29 to 43 dBm per band

- FR2 for HP-non-handheld/industrial (PC4): 	EIRPmin=34 to 43 dBm per band

Therefore, there are more cases to consider depending on the number of antennas used at the UE, and how antenna ports are allocated across single or multi-panels. Since the UE can dynamically switch between different antenna structures and combinations, Rel-18 should consider solutions to achieve power allocation and indication for varied UL TX antennas.

Observation 5: 8TX UEs support more varied antenna array architectures with multiple panels.

Proposal 5: Evaluate solutions with different power control requirements for different antenna array architectures.

# Conclusion

In this contribution, we discussed the Rel-18 WID enhancements targeting SRS for TDD C-JT, and for 8 TX. We make the following observations and proposals:

Observation 1: mTRP channel estimation requires more SRS transmissions by UEs which increases the total interference in the network.

Observation 2: Dynamic switching between sTRP and C-JT mode of operation requires flexible SRS configurations for both cases.

Observation 3: Codeword-to-layer mapping for UL may differ to the DL due to different UL transmission modes.

Observation 4: Dual codeword operation is a new scheduling mode for UL.

Observation 5: 8TX UEs support more varied antenna array architectures with multiple panels.

Proposal 1: Study how to coordinate SRS configuration across multiple UEs to maintain orthogonality or low cross-correlation.

Proposal 2: Evaluate how to enhance the SRS triggering procedure to support the numerous SRS configurations for mTRP.

Proposal 3: Study impacts on the UL processing chain of supporting higher number of layers.

Proposal 4: Study the enhancements to UL resource grants when dual codeword is supported.

Proposal 5: Evaluate solutions with different power control requirements for different antenna array architectures.

# References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung, RAN Meeting #94e, December 2021