3GPP TSG RAN WG1 #109-e			       R1-2203383

e-Meeting, May 9th – 20th 2022

Agenda Item:		9.1.4.1

Source:				InterDigital, Inc.

Title:						Precoding for Uplink Multi-Panel

Document for:		Discussion and Decision

# Introduction

Rel-18 proposes further enhancements to the MIMO feature [1]. The WID scope includes Objective 6 on simultaneous multi-panel UL transmission in mTRP:

- Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

# Background

Higher carrier frequency bands such as FR2 enable deployments to deliver high throughput transmissions by using higher bandwidths. Devices operating in these bands can support a high number of antennas in a compact form factor due to the smaller minimum inter-antenna spacing required compared to FR1. These antenna elements are organized into antenna panels which can be placed at different locations on devices and facing different directions to provide maximum directional coverage. Precoding and beamforming enable a UE to target more than one TRP with satisfactory signal quality despite blockage and varying UE orientation.

In Rel-17, only TDM’d mode of operation of UE multi-panels for mTRP was considered in the uplink. Rel-18 aims to study another mode of operation where a UE may use more than one panel to simultaneously transmit on the uplink. In this contribution, we provide our views on the simultaneous multi-panel (SMP) UL mode of operation.

# UL Enhancements for Simultaneous Multi-Panel

Rel-17 enhancements for mTRP and multi-panel

Rel-17 provided several enhancements to support single DCI (sDCI) uplink multi-TRP case. Rel-17 PUSCH and PUCCH can be scheduled through a grant with repetitions towards different TRPs by mapping different SRIs to the repetitions.

For multi-TRP PUSCH repetition, two SRS resource sets are configured corresponding to two different TRPs. A UE receives a DCI that contains two SRIs belonging to two different sets. The TDRA indicates the number of repetitions and is preconfigured with a mapping (e.g. cyclical or sequential) or SRIs to the repetitions. A UE may then determine the spatial filter to use per repetition. Similarly, for codebook-based precoding, a UE can determine the precoders per repetition based on two TPMIs indicated in the DCI. A further enhancement was added to the DCI to switch the ordering of the mapping to repetitions, and to dynamically switch between sTRP and mTRP. Two bits indicate whether the PUSCH is transmitted to TRP1 only, TRP2 only, mTRP starting with TRP1, or mTRP starting with TRP2.

For multi-TRP PUCCH, a UE receives a PUCCH configuration with repetitions, and two SRIs which are similarly preconfigured with a mapping (e.g. cyclical or sequential).

In either case of PUSCH or PUCCH, Since the repetitions target different TRPs with different pathloss, separate power control parameters are required for each TRP (e.g. alpha, P0, TPC). Rel-17 added two sets of power control parameters and a UE adjusts its transmission power depending on the targeted TRP per repetition.

With respect to panel operation, Rel-17 [2] introduced a feature to facilitate panel activation and selection in multi-panel UEs:

On Rel.17 enhancements to facilitate UE-initiated panel activation and selection, support the UE reporting a list of UE capability value [sets] - Each UE capability value [set] comprises the max supported number of SRS ports - Any two capability values [sets] are different - Whether the UE capability value [set] can be common across all BWPs/CCs in same band or BC can be discussed in UE feature session On Rel.17 enhancements to facilitate UE-initiated panel activation and selection, UE can report one index of UE capability value [set] for each reported CRI/SSBRI in one beam reporting.

A UE can therefore report more than one UE capability value sets, where each set indicates the maximum number of supported SRS ports. For each CRI/SSBRI in a CSI report for beam reporting, a UE reports the index of the associated capability value set. The network can therefore determine one or more active panels and their corresponding downlink reference signal used for measurement (e.g. CSI-RS or SSB). However, in all the enhancements discussed so far for Rel-17, the network can only schedule the UE to transmit on one panel at a time.

Rel-18 scenario

In Rel-18, we consider the mode of operation as illustrated in Figure 1. A UE is equipped with two or more panels of antenna elements. Each panel has separate control on power, amplitude and phase shifts so that different spatial filters and precoders can be generated at the same time. Two TRPs are deployed in the network and a device transmits simultaneously on Tx Beam 1 and Tx Beam 2 towards TRP1 and TRP2, respectively. A wide variety of devices are considered with different form factors for CPE, FWA, vehicular, or industrial applications.

Figure 1 Simultaneous multi-panel UL to mTRP

The objective of the WID is to use the simultaneous transmission for higher UL reliability and throughput. There are also clear benefits in terms of latency since a UE does not wait to do TDM transmissions in two separate time instances. In terms of reliability, both transmissions can be repetitions of a same transmission. As such, the network may receive at least one copy of transmission in case of loss of the second copy due to blockage or deep fades. Alternatively, the network may soft-combine the copies for increased decoding performance. Moreover, more repetitions can be sent in a shorter amount of time compared to Rel-17. In terms of throughput, the WID considers that up to two codewords can be scheduled in the UL which was previously not possible. Each panel may therefore for example transmit one codeword to achieve higher throughput.

Layer and antenna port mapping

Another key difference with Rel-17 is the relaxed requirement on the layer mapping. However, for Rel-18, the total number of layers across TRPs is limited to 4, but different variations of layers per panels are possible. For example, a UE may transmit to TRP1 and TRP2 with 1 and 2 layers, respectively. In Rel-17, the number of layers is the same for all TRPs. This impacted the design of the mTRP DCI since the dual SRI and TPMI fields were conditioned on the assumption that both transmissions use the same number of layers.

Similarly, the number of antenna ports (i.e. for DMRS) is constrained to a single antenna port indication in the sDCI. This limitation was not a problem for Rel-17 repetitions, but for Rel-18 SMP, we should consider a wider range of combinations. This applies to sDCI, and the newly considered mDCI case since the SMP scenario allows for coordinated or independent scheduling between antenna panels.

Therefore, beam/precoding indication (SRI and TPMI) with different layers and antenna ports per panel is one aspect to be enhanced for the SMP scenario.

Observation 1: Rel-17 mTRP enhancements for the UL only considered the case where the same number of layers and antenna ports is assumed for transmissions for both TRPs.

Proposal 1: Study enhancements for beam/precoding indication with different number of layers and antenna ports per TRP.

Panel and TRP selection

Rel-17 introduced some capability indication for multi-panel UEs to facilitate dynamic panel selection and activation. For Rel-18, there are other issues to consider for selecting pairs of panels. Depending on the form factor, a UE can be equipped with more than two panels where some may be inactive. Maximizing the benefits of multi-panel UEs depends on the UE’s ability to activate/deactivate different panel pairs for SMP based on the UE’s preference, and on the network’s indication.

Firstly, the optimal pair between a panel and a TRP needs to be selected, however that may be updated over time. For example, panel 1 may be facing towards TRP1 in one time instance, but it may be facing towards TRP2 in a second time instance due to device movement/rotation. Therefore, we can expect that the pairing between a panel and a TRP may change over time, and the pairing needs to be clearly understood by UE and network.

Secondly, the optimal pair of panels at any given time may change also due to changing channel conditions between a UE and TRPs. For example, for a UE with 3 panels, panels 1 and 2 may be initially the optimal pair. However later, due to UE movement, panel 2 and panel 3 may become the optimal pair. Therefore, solutions should consider proper indication and activation of optimal panels to gain the most benefits from SMP.

Finally, different selections depend as well on the target use case. SMP may not be the optimal mode of operation for every transmission. As stated in the objective, reliability or throughput are considered for enhancements, and SMP may be used to achieve a target level of performance. However, SMP may not always be possible at the UE (e.g., due to power limitations) or single panel may be better than SMP (e.g. due to P-MPR).

Observation 2: Optimal panel pairs for mTRP dynamically change over time.

Proposal 2: Study enhanced panel selection and pairing schemes with different TRPs for SMP.

Inter-panel interference

The simultaneous transmission of multiple beams/precoders is the source of additional interference. Resource scheduling can somewhat mitigate the interference if the resource allocation is orthogonal between panels. However, this severely restricts the scheduling decisions to a subset of RBs which could limit the throughput gains.  Rel-18 scenario should therefore be assumed for all overlapping cases, from non to full resource overlapping between panels.

The panel placement can also somewhat minimize the interference. For example, if one panel is located on the front of the device pointing outwards, and another panel is located on the back of the device pointing outwards, then the resulting main lobe of the spatial filters are likely pointing in opposite directions. Some inter-panel interference is thus mitigated; however, sidelobes can also contribute to interference even with opposite panel directions. In addition, the placement of panels is up to UE manufacturer’s choice and there is no guarantee that two panels selected for SMP are located such that no inter-panel interference affects transmission. The WID considers all types of device form-factor with arbitrary panel placement or architecture so SMP cannot rely on the assumption of independence between panels. Therefore, solutions for SMP should consider inter-panel interference, and specify enhancements that will minimize its impact.

Observation 3: Simultaneous transmission of different precoders/beams from multi-panel generates inter-panel interference due to overlapping resource allocation.

Proposal 3: Study solutions for mitigating inter-panel interference.

# Conclusions

In this contribution, we discussed the outcomes of Rel-17 mTRP UL enhancements and considered the new scenario for Rel-18 simultaneous multi-panel UL transmission for mTRP. We make the following observations and proposals:

Observation 1: Rel-17 mTRP enhancements for the UL only considered the case where the same number of layers is assumed for transmissions for both TRPs.

Observation 2: Optimal panel pairs for mTRP dynamically change over time.

Observation 3: Simultaneous transmission from multi-panel generates inter-panel interference.

Proposal 1: Study enhancements for beam/precoding indication with different number of layers per TRP.

Proposal 2: Study enhanced panel selection and pairing schemes with different TRPs for SMP.

Proposal 3: Study solutions for mitigating inter-panel interference.

# References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung, RAN Meeting #94e, December 2021

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #107-e, November 2021