3GPP TSG RAN WG1 Meeting #109-e	R1-2203387

e-Meeting,

Agenda Item:	8.16.14

Source:	MediaTek Inc.

Title:	UE features for IoT over NTN

Document for:	Discussion and Decision

- Introduction

In RAN#108-e, the RAN1 UE features are provided in [1] for Rel-17 IoT NTN. In this contribution, we provide our perspectives on the update RAN1 UE features.

- Discussion

In RAN1 #107-e, UL Segmented transmission was discussed, and the following agreements were made for NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC:

For NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC: Agreement UE pre-compensation per segment of NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC is applied from one segment to the next segment by using one or more of the following methods if supported by UE implementation 1. UE may drop / Insert samples / Puncture OFDM symbols 2. UE may blank subframes / slots where UE skip a slot or a subframe The total transmission time is not changed UE autonomously Drop / insert samples / Puncture OFDM symbols or Blank subframes / slots where UE drops a subframe / slot The method used for the UE pre-compensation is known to the eNB by a single UE capability - UE Blank subframes / slots where UE skip a slot or a subframe (slot is based on Sub Carrier Spacing) FFS Details of method(s) to drop / insert samples, blanking subframes / slots (slot is based on Sub Carrier Spacing)

In case the UE can support several methods to drop durations of uplink transmission between segments, it seems highly desirable that the UE always drops the shortest duration of uplink transmission to minimize the SNR loss at eNB receiver. The shortest duration should be as close to the timing adjustment corresponding to the TA calculated by the UE based on its GNSS location, the ephemeris, and common TA parameters if signalled. Note that in case the UE implementation can only support dropping of 2 slots, then it can only report 2 slots for its UE capability; if UE can drop one symbol, then this is sufficient to accommodate the maxim timing adjustment for the longest UL transmission segment, and there would be no need to drop one slot or 2 slots which would un-necessarily degrade SNR; if UE can optimally drops samples between segments then the there is no need for reporting of UE capability. The method used for the UE pre-compensation is known to the eNB by a single UE capability.

Proposal 1: add new text underlined for component 2-1a " 1. Support UE applies segmented UL transmission according to duration configuration by the network based on a single UE capability".

Proposal 2: Send LS to RAN2 to specify a single UE capability will be specified by RAN2 in TS 36.306

Note: When a single capability is signaled: UE drops one of the following durations of uplink transmission between segments (indicated by the capability):

- 1 slot (applicable to eMTC)

- 1 subframe (applicable to eMTC)

- 1 slot (applicable to NB-IoT)

- 2 slots (applicable to NB-IoT)

- 1 symbol (applicable to both eMTC and NB-IoT)

When capability is NOT signalled: UE follows legacy behavior at slot boundaries due to TA adjustment

# Conclusion

In this contribution, we discuss the Rel-17 UE features for NB-IoT and eMTC over NTN. The following proposals are presented:

Proposal 1: add new text underlined for component 2-1a " 1. Support UE applies segmented UL transmission according to duration configuration by the network based on a single UE capability".

Proposal 2: Send LS to RAN2 to specify a single UE capability will be specified by RAN2 in TS 36.306

Note: When a single capability is signaled: UE drops one of the following durations of uplink transmission between segments (indicated by the capability):

- 1 slot (applicable to eMTC)

- 1 subframe (applicable to eMTC)

- 1 slot (applicable to NB-IoT)

- 2 slots (applicable to NB-IoT)

- 1 symbol (applicable to both eMTC and NB-IoT)

When capability is NOT signalled: UE follows legacy behavior at slot boundaries due to TA adjustment

# References

- R1-2202856, “Summary of UE features for IoT over NTN.”

# Appendix A

Rel-17 UE features for LTE NB-IoT eMTC over NTN.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | [Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs)] | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. LTE_NBIOT_eMTC_NTN | 2-1 | Basic IoT over NTN support for eMTC | 1. UE derives its position based on its GNSS measurements [1-1. UE reports the information associated with the validity duration of GNSS] 2. UE receives serving satellite ephemeris in either state vector format or orbital element format 4. UE calculates UE specific TA in RRC_IDLE and RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris 6. UE calculates common TA in RRC_IDLE and RRC_CONNECTED according to the parameters provided by the network (UE considers common TA as 0 if the parameter is not provided) 7. For TA update in RRC_CONNECTED state, UE uses a combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops 8. In RRC_IDLE and RRC_CONNECTED state, UE calculates frequency pre-compensation to counter shift the Doppler experienced on the service link [in DL] [and] [in UL] 10. UE supports a validity timer of UL synchronization that is configured by the network 13. UE applies cell specific K_offset in timing relationship enhancements 15. UE estimates UE-gNB RTT and delays the starts of ra-ResponseWindow by UE-gNB RTT 16. Delay the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated 17. In RRC_IDLE state and RRC_CONNECTED state, UE pre-compensates the calculated frequency offset and TA in uplink transmissions 18. UE receives cell-specific K_offset/K_mac in system information |  | Yes | N/A | Release 17 eMTC UE cannot communicate via satellite | [Per UE/per band] | No | No |  | Optional with capability signalling For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported Note: This UE feature group is applicable only for IoT-NTN cell for communication via satellite, for any other cell this feature is not supported |
| 2. LTE_NBIOT_eMTC_NTN | 2-1a | Segmented UL transmission duration configuration for eMTC | 1. Support UE applies segmented UL transmission according to duration configuration by the network | 2-1 | Yes | N/A | [Release 17 eMTC UE cannot access communicate via [NTN/satellite] GEO and NGSO NTNs] | [Per UE/per band] | No | No | [For UEs support communicate via GEO and NGSO NTNs NSGO scenarios, it must indicate this FG is supported.] | Optional with capability signalling [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| 2. LTE_NBIOT_eMTC_NTN | 2-1b | Basic IoT over NTN support for NB-IoT | 1. UE derives its position based on its GNSS measurements [1-1. UE reports the information associated with the validity duration of GNSS] 2. UE receives serving satellite ephemeris in either state vector format or orbital element format 4. UE calculates UE specific TA in RRC_IDLE and RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris 6. UE calculates common TA in RRC_IDLE and RRC_CONNECTED according to the parameters provided by the network (UE considers common TA as 0 if the parameter is not provided) 7. For TA update in RRC_CONNECTED state, UE uses a combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops 8. In RRC_IDLE and RRC_CONNECTED state, UE calculates frequency pre-compensation to counter shift the Doppler experienced on the service link 10. UE supports a validity timer of UL synchronization that is configured by the network 13. UE applies cell specific K_offset in timing relationship enhancements 15. UE estimates UE-gNB RTT and delays the starts of ra-ResponseWindow by UE-gNB RTT 16. Delay the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated 17. In RRC_IDLE state and RRC_CONNECTED state, UE pre-compensates the calculated frequency offset and TA in uplink transmissions 18. UE receives cell-specific K_offset/K_mac in system information |  | Yes | N/A | Release 17 NB-IoT UE cannot communicate via satellite | [Per UE/per band] | No | No |  | Optional with capability signalling For UEs supporting NB-IoT NTN, it must indicate this FG is supported Note: This UE feature group is applicable only for IoT-NTN cell for communication via satellite, for any other cell this feature is not supported |
| 2. LTE_NBIOT_eMTC_NTN | 2-1c | Segmented UL transmission for NB-IoT | 1. UE applies segmented UL transmission according to duration configuration by the network | 2-1b | Yes | N/A | Release 17 NB-IoT UE cannot communicate via NGSO NTNs | [Per UE/per band] | No | No | For UEs supporting communicate via NGSO NTNs, it must indicate this FG is supported. | Optional with capability signalling Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported |
| 2. LTE_NBIOT_eMTC_NTN | 2-2 | Enhancing timing relationships using a time offset for eMTC | UE receives and applies UE specific K_offset, K_mac in timing relationship enhancements | 2-1 , 2-3 | Yes | N/A | eMTC UE does not know the offset to apply for UL transmission | [Per UE/per band] | No | No | The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. | Optional with capability signalling |
| 2. LTE_NBIOT_eMTC_NTN | 2-2a | Enhancing timing relationships using a time offset for NB-IoT | UE receives and applies UE specific K_offset, K_mac in timing relationship enhancements | 2-1b, 2-3a | Yes | N/A | NB-IoT UE does not know the offset to apply for UL transmission | [Per UE/per band] | No | No | The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. | Optional with capability signalling |
| 2. LTE_NBIOT_eMTC_NTN | 2-3 | TA pre-compensation reporting for eMTC | Support reporting of information about the UE specific TA pre-compensation | 2-1 | Yes | No | UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD | [Per UE/per band] | No | No | UE-specific TA reporting is supported in IoT-NTN | Optional with capability signalling |
| 2. LTE_NBIOT_eMTC_NTN | 2-3a | TA pre-compensation reporting for NB-IoT | Support reporting of information about the UE specific TA pre-compensation | 2-1b | Yes | No | UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD | [Per UE/per band] | No | No | UE-specific TA reporting is supported in IoT-NTN | Optional with capability signalling |