3GPP TSG RAN WG1 Meeting #109-e  	R1-2203388

e-Meeting, May 9th – 20th, 2022

Agenda Item: 8.14

Source: Moderator (MediaTek)

Title: Maintenance on NB-IoT/eMTC to support NTN: time and frequency

synchronization

Document for: Discussion and Decision

# Introduction

At the RAN#92 meeting, a new Work Item was approved for IoT Non Terrestrial Network (NTN) [1].  In this meeting, company views on UL synchronization for IoT NTN are summarized and observations/proposals on identified issues are made. Observations and proposals in Company’s TDoc contributions are listed in the Appendix.

The issues in contributions submitted to RAN1#109e for Re-17 IoT NTN for time and frequency synchronization aspects were summarized by the moderator in [2]. An initial assessment on each of the maintenance issues was provided based on the following classification:

- High priority (H): high-priority item (essential, pending issues, broken spec components) and proposed editorial changes that either enhance the clarity of the specs or correct mistakes

- Non-essential (N): all other purposes such as spec optimization and low priority issues

- Editorial (E): editorial issues that will be handled as editorial CRs (to be communicated to the editors/chairs)

Identified remaining IoT NTN-specific topics are discussed in Sections 2, 3, and 4. Common issues to NR NTN and IoT NTN are discussed in Section 5.

# [ACTIVE] UL Segmented Transmission

In RAN1#107-e, there are a few open issues (highlighted in yellow) related to segmented uplink transmission.

Agreement

UE pre-compensation per segment of NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC is applied from one segment to the next segment by using one or more of the following methods if supported by UE implementation

- UE may drop / Insert samples / Puncture OFDM symbols

- UE may blank subframes / slots where UE skip a slot or a subframe

The total transmission time is not changed

UE autonomously Drop / insert samples / Puncture OFDM symbols or Blank subframes / slots where UE drops a subframe / slot

The method used for the UE pre-compensation is known to the eNB by a single UE capability

- UE Blank subframes / slots where UE skip a slot or a subframe (slot is based on Sub Carrier Spacing)

FFS Details of method(s) to drop / insert samples, blanking subframes / slots (slot is based on Sub Carrier Spacing)

Agreement

For NB-IoT, UE pre-compensation per segment of NPRACH is applied from one segment to the next segment by using one or more of the following methods if supported by UE implementation

- UE may drop / Insert samples

- UE may blank subframe / repetition unit where UE drops a subframe / repetition unit

The total transmission time is not changed

FFS Details of method(s) to drop / insert samples / blank subframe / repetition unit

FFS Specification impact

Agreement

For eMTC, UE pre-compensation per segment of PRACH is applied from one segment to the next segment by drop / insert samples in Guard Period of PRACH preamble.

- The total transmission time is not changed

- FFS Details of method(s) to drop / insert samples

In the specifications UE is not allowed to adjust timing advance in the duration of repetitions as specified in TS 36.133 V16.8.0, Clause 7.20.2.

- [3GPP TS 36.133 V16.8.0, Section 7.20.2] When a repetition period is configured on the uplink for which R>1, the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission as defined above.

In the specifications, sample adjustment for TA is at slot boundaries, as specified inTS 36.213, Section 16.1.2

- When the UE's uplink NPUSCH transmissions in NB-IoT uplink slot n and NB-IoT uplink slot n+1 are overlapped due to the timing adjustment, the UE shall complete transmission of NB-IoT uplink slot n and not transmit the overlapped part of NB-IoT uplink slot n+1.

## Company views

The issues in contributions submitted to RAN1#109e for Re-17 IoT NTN for time and frequency synchronization aspects related to UL segmented transmission were summarized by the moderator in [2], and copied below in Table XX.

Table XX – Issues on time/frequency sync for UL segmented transmissioncommon to IoT NTN and IoT NTN

| Issue# | Issue | References | FL initial assessment | Company inputs (if any) |
| --- | --- | --- | --- | --- |
| 1-1 | [description of the issue] Capture RAN1#107e agreement on UE behaviour and Single UE capability for UL segmented transmission in specifications FL Note: Can be discussed in 8.16.14 UE features. RAN2 captures single UE capability (TS 36.306). RAN1 capture UE behaviour (TS 36.211) | R1-2203232, 3386,3632,3722, 3839, 4997 | H | Nokia, NSB: As mentioned in contributions, most companies think this should be specified in RAN1. The detail of UE capability should be firstly discussed and agreed in 8.14 before forward to 8.16.14. The detail of how to apply segment pre-compensation should be captured in spec and should be discussed in 8.14. We think Issue #1-1 should be with high priority. [ZTE]: We are supportive of capturing UE behaviour in TS 36.211 as FL drafted in RAN1#108e. And the UE capability should also be discussed in 8.16.14. [Thales]: Agree with FL initial assessment [SONY]: We think this issue should be high priority. There was a lot of discussion on this in RAN1#108e so it doesn’t seem to just be an editorial issue. How the gaps are created needs to be discussed (e.g. in section 5.3.4 of TS36.211). We also think that is a necessary to discuss how pre-compensation is applied within a segment. [Lockheed Martin] Agree with the FL. [Qualcomm]: High priority. Needs to be captured in the specifications. [Spreadtrum]: High priority. The detail of how to apply segment pre-compensation should be captured in TS 36.211 and discussed in 8.14. [Xiaomi]: We are supportive of capturing UE behavior in RAN1 spec. [Huawei, HiSilicon]: support to capture the UE behavior in (TS 36.211). [Mediatek]: Single UE capability can be discussed in AI 8.16.14 (and captured in RAN2 spec in TS 36.306). We are fine with RAN1 capture UE behaviour in TS 36.211. |

Issue 1-1: Capture RAN1#107e agreement on UE behaviour and Single UE capability for UL segmented transmission in specifications (H)

- Single UE capability can be discussed in AI 8.16.14 UE features for IoT NTN, RAN2 can capture single UE capability in RAN2 spec in TS 36.306.

- Capture UE behaviour in RAN1 spec in TS 36.211 can be discussed in AI 8.14 IoT NTN

## FIRST ROUND Discussion

ZTE, MediaTek, Spreadtrum, Ericsson proposed to support the TPs on TS 36.211 as in FL summary in RAN1#108-e. Ericsson similarly proposed to support TPs with minor revisions.

SONY proposed further clarification to specifiy how symbol-length gaps for segmented pre-compensation are handled. This specification can follow the form of the text in TS38.211 section 5.3.4:

“For BL/CE UEs, if one or more SC-FDMA symbol(s) are left empty due to guard period for narrowband or wideband retuning or due to segmented pre-compensation, the affected SC-FDMA symbol(s) shall be counted in the PUSCH mapping but not used for transmission of the PUSCH.”

Qualcomm proposed versions of TP#1 and TP#2 with simplicifcation with one subframe dropped for eMTC and one slot dropped for NB-IoT. For NB-IoT, the portion of a postponement due to NPRACH which coincides with a gap is counted as part of the gap.

Nokia proposed versions of TP#1 ad TP#2 with adding “If Nsegmentprecompensation  is not configured by network, after a transmission duration of Nsegmentprecompensation   time units, UE shall not transmit the overlapped part of next segment. “.

Apple proposed in uplink transmissions,

- if UE is to drop samples between two successive segments, it drops the tail samples of the earlier segment.

- if UE is to insert samples between two successive segments, it repeats the last sample of the earlier segment.

- if UE is to blank subframe/slot/repetition unit between two successive segments, it blanks the last subframe/slot/repetition unit of the earlier segment.

In RAN1#108-e, the single UE capability was discussed with moderator proposing a definition below:

The single UE capability that governs UE behavior w.r.t gaps between segments for PUSCH, PUCCH and NPUSCH, when the UE performs segmented pre-compensation, is as follows:

When a single capability is signaled: UE drops one of the following durations of uplink transmission between segments (indicated by the capability):

- 1 slot (applicable to eMTC)

- 1 subframe (applicable to eMTC)

- 1 slot (applicable to NB-IoT)

- 2 slots (applicable to NB-IoT)

- 1 symbol (applicable to both eMTC and NB-IoT)

When capability is NOT signalled: UE follows legacy behavior at slot boundaries due to TA adjustment

If agreed, the single UE capability clarifies that UE follows legacy behavior at slot boundaries due to TA adjustment. Further, it is sufficient from RAN1 viewpoint to capture in RAN1 spec TS 36.211 that the transmission gap of Ngapprecompensationtime units is according to the single UE capability, and the quantity Nsegmentprecompensation is provided by higher layers. As mentioned by SONY, the gap insertion method is handled in the same way as other capability / configuration signalling exchanges:

•	The UE signals which gap insertion methods it is capable of

•	The eNB configures the UE with a gap insertion method

It is up to RAN2 how to specify this signalling exchange. Further, as mentioned by ZTE, RAN1 can inform RAN4 the RAN1 required updates on TS 36.133 to support segment pre-compensation, i.e., allow UE to adjust UL timing during an ongoing repetition period at least in NTN.

FL recommendation: As recommended by moderator in preparation phase in Moderator summary in R1-2205110,  the single UE capability can be discussed in AI 8.16.14 and captured in RAN2 spec TS 36.306.

Based on the above simplified TP#1 and TP#2 are proposed below.

First Round proposal – 2.2

- Support the TP#1 Section 5.3.4, TS 36.211 for eMTC  and TP#2 Section 10.1.3.6, TS 36.211 for NB-IoT on UE behaviour for UL segmented transmssion

<TP1, Section 5.3.4, TS 36.211>

For BL/CE UEs, if one or more SC-FDMA symbol(s) are left empty due to guard period for narrowband or wideband retuning, or due to segmented pre-compensation, the affected SC-FDMA symbol(s) shall be counted in the PUSCH mapping but not used for transmission of the PUSCH.

For BL/CE UEs communicating over NTN, for PUSCH transmission, for frame structure type 1, after a transmission duration of Nsegmentprecompensation time units (which may include subframes that are not BL/CE UL subframes), a transmission gap of Ngapprecompensationtime units shall be counted for the PUSCH resource mapping but not used for transmission of the PUSCH, according to the single UE capability ue-CE-NeedSegmentedPrecompensationGaps, as specified in 3GPP TS 36.331. The quantity Nsegmentprecompensation is provided by higher layers.

<END TP1>

<TP2, Section 10.1.3.6, TS 36.211>

For NPUSCH, if one or more SC-FDMA symbol(s) are left empty due to segmented pre-compensation, the affected SC-FDMA symbol(s) shall be counted in the NPUSCH mapping but not used for transmission of the NPUSCH.

For a UE communicating over NTN, after transmissions and/or postponements due to NPRACH of Nsegmentprecompensation time units, for frame structure type 1, a transmission gap of Ngapprecompensation time units shall be counted for the NPUSCH resource mapping but not used for transmission of the NPUSCH according to the UE capability ue-NBIOT-NeedSegmentedPrecompensationGaps , as specified in 3GPP TS 36.331. The quantity Nsegmentprecompensation is provided by higher layers. The portion of a postponement due to NPRACH which coincides with a gap is counted as part of the gap.

<END TP2>

| Companies | Comments |
| --- | --- |
| Nokia, NSB | We think the following should be the way forward. 1), we propose the UE capability should be aligned with 3GPP working way, i.e. UE to report and network to decide and configure which UE-supported method to be used. 2), it should be clear that we have agreement in RAN1 107-e meeting “UE pre-compensation per segment of NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC is applied from one segment to the next segment by using one or more of the following methods if supported by UE implementation”, that the UE may support one or more method, which should be covered by single UE reported capability. 3) TR 36.763 assumes and the release 17 WI defines cellular IoT features up to Rel-16 are supported in release 17 IoT NTN. This means the legacy behavior on dropping will always be supported as in Section 16.1.2, TS 36.213. While, the reported UE capability for IoT NTN is for additional supported method besides the legacy behavior. 4) The UE may only support the legacy behavior, or UE may support also one or more of the following methods. So it would be clear that to define both the case A) if higher layer configures the gap size and B) if higher layer does not configure the gap size, i.e. using legacy dropping method. |
| ZTE | We support the TPs drafted by the moderator. Moreover, in legacy NB-IoT and eMTC, TA update during ongoing repetition is not allowed. Therefore, it should be specified that different TA pre-compensation values can be applied for different segments. Otherwise, segmented pre-compensation cannot work. We need to discuss how to capture this in RAN1 spec or discuss how to send LS to RAN4. |
| Lenovo | Since there are several transmission gap duration options (e.g., 1ms, 2ms, 1 symbol), it is better to consider the transmission gap (e.g., 1ms) is included in the transmission duration (e.g.,256ms), which makes the segments duration constant for different reported gap duration options. If after a transmission duration (e.g.,256ms), a transmission gap shall be counted for the NPUSCH resource mapping…(e.g., 1ms, 2ms), so the segment duration/period (256+1ms, 256+2ms) may be variable based on the UE capability. So we propose the TP updated as: For BL/CE UEs communicating over NTN, for PUSCH transmission, for frame structure type 1, after every a transmission duration of Nsegmentprecompensation time units (which may include subframes that are not BL/CE UL subframes), a transmission gap of Ngapprecompensationtime units shall be counted for the PUSCH resource mapping but not used for transmission of the PUSCH in the end of each transmission duration. |
| Apple | We are fine with the proposal. |
| Ericsson | Support the TPs |
| Lockheed Martin | TPs are OK |
| Qualcomm | TPs miss specifying the duration of the gap (based on the capability). The gap is one subframe for eMTC and one slot for NB-IoT. So, the following last line/clause should be added to the TP: the quantity Ngapprecompensation is one slot (for NB-IoT; replace slot by subframe for eMTC). We are OK to also add “or one symbol, as indicated by the capability” The group’s understanding of the capability should be clear: “dropping samples” is “legacy behavior”. Note that (see pasted standards text below) sample dropping at the boundary (segments will ALWAYS be at slot boundaries, by definition) due to TA mismatch is already supported: Section 16.1.2 (NB-IoT, current specs): “When the UE's uplink NPUSCH transmissions in NB-IoT uplink slot n and NB-IoT uplink slot n+1 are overlapped due to the timing adjustment, the UE shall complete transmission of NB-IoT uplink slot n and not transmit the overlapped part of NB-IoT uplink slot n+1.” So, the capability definition should be: - UE indicates capabilityThis UE NEEDS a slot/subframe(/symbol) gap - UE does NOT indicate capabilityThis UE doesn’t need any special treatment, and will adjust things between segments by sample dropping, based on current text (see above). [Note to vendors that DO need this gap: please realize that any “network ratification” of this capability risks your UE not being allowed to access an NTN cell if it needs a one subframe/slot gap.] |
| Spreadtrum | Support the TPs |
| Nordic | The duration of the precompensation gap should be specified, otherwise support the TPs |
| SONY | The eMTC proposal looks good. For the NB-IoT proposal, is it intentional that we are talking about NPRACH compensation gaps in text NPUSCH? For a UE communicating over NTN, after transmissions and/or postponements due to NPRACH of Nsegmentprecompensation time units, for frame structure type 1, a transmission gap of Ngapprecompensation time units shall be counted for the NPUSCH resource mapping but not used for transmission of the NPUSCH according to the UE capability ue-NBIOT-NeedSegmentedPrecompensationGaps , as specified in 3GPP TS 36.331. The quantity Nsegmentprecompensation is provided by higher layers. The portion of a postponement due to NPRACH which coincides with a gap is counted as part of the gap. We agree with the point from ZTE: “Moreover, in legacy NB-IoT and eMTC, TA update during ongoing repetition is not allowed. Therefore, it should be specified that different TA pre-compensation values can be applied for different segments. Otherwise, segmented pre-compensation cannot work. We need to discuss how to capture this in RAN1 spec or discuss how to send LS to RAN4.” The main issue about segmented UL transmissions is that the TA is not changed during a segment, but can change between segments. This is the prime thing that needs specifying, not what happens in the gap between segments. |

## SECOND ROUND Discussion

For this Issue#1, for TPs proposed in First Round proposal – 2.2

- ZTE, Apple, Ericsson, Lockheed Martin, Spectrum, Nordic, SONY, MediaTek support TP

- Lenovo support TP with with the following last line/clause added to the TP “in the end of each transmission duration”

- Qualcomm support TP with the following last line/clause added to the TP: the quantity Ngapprecompensation is one slot (for NB-IoT; replace slot by subframe for eMTC). We are OK to also add “or one symbol, as indicated by the capability”

On Lenovo, ZTE, SONY  comments, it is discussed in 8.16.14 UE features for IoT NTN to add text to component 2.1a and 2.1c   “UE applies segmented UL transmission according to duration configuration by the network based on a single UE capability, different values  (e.g., TA) for pre-compensation may be used per segment”.

On Qualcomm comment, the TPs mentioned “a transmission gap of Ngapprecompensation time units shall be counted for the NPUSCH resource mapping but not used for transmission of the NPUSCH according to the UE capability ue-NBIOT-NeedSegmentedPrecompensationGaps”. The UE capability ue-NBIOT-NeedSegmentedPrecompensationGaps can specify the duration of the of Ngapprecompensation .

- For NB-IoT, several companies already indicated in RAN1#107e, RAN1#108e that 1 ms = 2 slots is the duration of uplink transmission that needs to be dropped. It seems simpler in the RAN1 TPs to only refer to  ue-NBIOT-NeedSegmentedPrecompensationGaps, which will include all the possibilities for the duration of uplink transmission that needs to be dropped. A text of the single UE capability discussed in RAN1#108-e is included in Section 2.2. This includes 1 slot for NB-IoT, and also include one symbol for NB-IoT / eMTC.

On Nokia comment, the UE capability ue-NBIOT-NeedSegmentedPrecompensationGaps will need to be specified in TS 36.306 / TS 36.331 by RAN2 to the moderator understanding. Then, as usual the understanding in RAN1 should be that the UE report its capability and the network configures UE accordingly. If there is support from companies in RAN1, the text in the single UE capability discussed in RAN1#108e (and copied in Section 2.2) could replace the text “one of the following durations” with the text “one or more of the following durations” in the sentence above the sub-bullets listing the durations for NB-IoT / eMTC.

On SONY comment, for the NB-IoT proposal the text “after transmissions and/or postponements due to NPRACH of Nsegmentprecompensation time units” is to align with similar text for the leagacy UL compensation gap in TS 36.211 Section 10.1.3.6 Mapping to physical resources which specifies “After transmissions and/or postponements due to NPRACH of 256.30720Ts time units, for frame structure type 1, a gap of  40.30720Ts time units shall be inserted where the NPUSCH transmission is postponed”.

Moderator view for FIRST ROUND discussion:

- The RAN1 TPs seem agreable by 8 companies , with 2 companies supported TPs with minor revisions.

- The single UE capability and TPs were further discussed in offline  on Friday 5:00-6:00 AM UTC to further align company views with revised proposals as below

- Since there is now consensus from 7 companies (Ericsson, Nokia, ZTE, Lenovo, Qualcomm, SONY, MediaTek) in the offline, moderator makes the FL recommendation as a proposal by1st check point: May 13

Proposal 2a for 1st Checkpoint May 13:

- The single UE capability that governs UE behavior w.r.t gaps between segments for PUSCH, PUCCH and NPUSCH, when the UE performs segmented pre-compensation, is as follows:

- When a single capability is signalled: UE drops one or more of the following durations of uplink transmission between segments (indicated by the capability):

  - 1 slot (applicable to eMTC)

  - 1 subframe (applicable to eMTC)

  - 1 slot (applicable to NB-IoT)

  - 2 slots (applicable to NB-IoT)

  - 1 symbol (applicable to both eMTC and NB-IoT)

  - UE follows legacy behaviour at slot boundaries due to TA adjustment

- When capability is NOT signalled: UE follows legacy behaviour at slot boundaries due to TA adjustment

Proposal 2b for 1st Checkpoint May 13:

- Send LS to RAN2 (cc RAN4) for specification of single UE capability (in TS 36.306, TS 36.331)

Proposal 2c for 1st Checkpoint May 13:

- Adopt TP#1  on TS 36.211 Section 5.3.4 for PUSCH for segmented transmission

<TP1, Section 5.3.4, TS 36.211>

For BL/CE UEs communicating over NTN, for PUSCH transmission, for frame structure type 1, after a transmission duration of Nsegmentprecompensation time units (which may include subframes that are not BL/CE UL subframes), a transmission gap of Ngapprecompensationtime units shall be counted for the PUSCH resource mapping but not used for transmission of the PUSCH, according to the single UE capability ue-CE-NeedSegmentedPrecompensationGaps, as specified in 3GPP TS 36.331. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity of Ngapprecompensation is based on the UE capability as configured by the higher layers.

<END TP1>

Proposal 2d for 1st Checkpoint May 13:

- Adopt TP#2  on TS 36.211 Section 5.4.3 for PUCCH for segmented transmission

<TP2, Section 5.4.3, TS 36.211>

For BL/CE UEs communicating over NTN, for PUCCH transmission, for frame structure type 1, after a transmission duration of Nsegmentprecompensation time units (which may include subframes that are not BL/CE UL subframes), a transmission gap of Ngapprecompensation time units shall be counted for the PUCCH resource mapping but not used for transmission of the PUCCH, according to the single UE capability ue-CE-NeedSegmentedPrecompensationGaps, as specified in 3GPP TS 36.331 [9]. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity Ngapprecompensation is based on the UE capability as configured by the higher layers.

<END TP2>

Proposal 2e for 1st Checkpoint May 13:

- Adopt TP#3  on TS 36.211 Section 10.1.3.6 for NPUSCH for segmented transmission

<TP3, Section 10.1.3.6, TS 36.211>

For a UE communicating over NTN, after transmissions (and/or postponements due to NPRACH) of Nsegmentprecompensation time units, for frame structure type 1, a transmission gap of Ngapprecompensation time units shall be counted for the NPUSCH resource mapping but not used for transmission of the NPUSCH according to the UE capability ue-NBIOT-NeedSegmentedPrecompensationGaps , as specified in 3GPP TS 36.331. The quantity Nsegmentprecompensation is provided by higher layers, , and the quantity of Ngapprecompensation is based on the UE capability as configured by the higher layers.

<END TP3>

## THIRD ROUND Discussion

The guidance on proposal 2b from Session Chair is that the new FGs for the newly agreed UE capability will be sent by LS to RAN2 from 8.16.14 Session Chair, where the agreement will be provided in the UE features table. Hence, Proposal 2b seems redundant for RAN2.

It was discussed on the RAN1 reflector by ZTE and SONY that an independent LS for "applying different pre-compensation values per segment" for modification on RAN4 spec (36.133) is needed. As background information, the LS to RAN4 could include Proposals 2a, 2c, 2d, 2e.

3rd Round Proposal – 2.4

•	Send LS to RAN4  for "applying different pre-compensation values per segment" (in TS 36.133)

| Companies | Comments |
| --- | --- |

# [ACTIVE] NTN SIB accumulation

The issues in contributions submitted to RAN1#109e for Re-17 IoT NTN for time and frequency synchronization aspects for NTN SIB accumulation across SI windowswere summarized by the moderator in [2], and copied below in Table XX.

Table XX – Issues on time/frequency sync for NTN SIB accumulation across SI windows

| Issue# | Issue | References | FL initial assessment | Company inputs (if any) |
| --- | --- | --- | --- | --- |
| 1-2 | NTN SIB accumulation across SI windows FL Note: RAN1 can discuss if NTN-specific SIB is updated per SI window / accumulated across SI windows | R1-2203232, 3386, 3632 | H | [Companies will fill input their views here] [ZTE]: We are fine to discuss this issue and think SI should be updated per SI window to avoid ambiguity of epoch time indication. [Thales]: Agree with FL initial assessment [SONY] Agree that this is high priority. We also discuss this issue in section 3 of 3722 [Lockheed Martin] Agree with the FL. [Qualcomm]: OK to discuss. [Spreadtrum]: Agree with the FL. [Xiaomi]: Supportive to discuss this issue. [Huawei, HiSilicon]: Support SIB can be accumulated across SI windows. It can avoid extra restriction to the gNB and align with the current specification design. The end of the first SI window is the epoch time during update period of assistance information [Mediatek]: Agree with priority and FL note. |

## Company views

The agreement on implicit epoch time indication method was revised in RAN1#108e for NR NTN.

Agreement

Modify second bullet of RAN1#107-e agreement on Epoch time as follows:

Otherwise, when Epoch time is not explicitly indicated in SIB (other than SIB1), epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is implicitly known as the end of the SI window during which the NTN-specific SIB SI message is transmitted.

ZTE discussed that in legacy NB-IoT, UE is allowed to combine SI across multiple SI windows. If implicit epoch time indication is applied, UE cannot figure out which SI window is used to indicate the epoch time. To avoid ambiguity without significant specification impact, ZTE proposed the NTN-specific SIB can be updated per SI window. MediaTek, Xiaomi similarly proposed that the UE shall not assume that the NTN-specific SIB is constant across SI windows.

Ericsson provided some analysis for SI configuration details for eMTC and NB-IoT as provided in Table below. Additionally, the UEs can possibly accumulate SIBs across multiple SI windows if needed for decoding (except for the SIBs that change frequently such as SIB16). It is beneficial to allow SIB accumulation over those windows to overcome poor coverage. Network should have the ability to optionally indicate if NTN SIB accumulation across SI windows is allowed or not. The network can also indicate a reference time (e.g., subframe, SFN, H-SFN) in SI to indicate the end of the SI accumulation window. This would eliminate any ambiguity about which NTN SIBs to accumulate across once the UE attempts to acquire SI.

Table 2 was provided Ericsson to sum up pros and cons of NTN-specific SIB is updated per SI window / accumulated across SI windows

Table 1:  SI window configuration parameters for eMTC and NB-IoT [5].

|  | SI window length | Repetition pattern within SI window | SI periodicity |
| --- | --- | --- | --- |
| eMTC | {1, 2, 5, 10, 15, 20, 40, 60, 80, 120, 160, 200} ms | Every radio frame or every {2nd, 4th, 8th} radio frame | {8, 16, 32, 64, 128, 256, 512} radio frames |
| NB-IoT | {160, 320, 480, 640, 960, 1280, 1600} ms | Every {2nd, 4th, 8th, 16th} radio frame | {64, 128, 256, 512, 1024, 2040, 4096} radio frames |

100 repetitions within SI window of 1600 ms, 1 repetition every 100 ms

SI window with 4096 periodicity

Table 2 Way forward for enabling NTN SIB accumulation for IoT NTN [5].

|  | Pros | Cons | Spec. impact |
| --- | --- | --- | --- |
| Option 1: Prohibit NTN SIB accumulation across SI windows | Only meaningful in NTN scenarios where NTN SIB will be updated during each SI window. | Too restrictive for many NTN scenarios where SI window periodicity is shorter than NTN SIB update frequency, e.g., the maximum SI periodicity is 5.12 s for eMTC and 40.96 s for NB-IoT which is much shorter than the NTN SIB update frequency in GEO. Large SI transmission overhead as network may need to configure a larger SI window to support a greater number of repetitions in the cell to compensate for the lack of SIB accumulation. Coverage-limited UEs cannot access the network if NTN SIB cannot be decoded. Not synergistic with legacy eMTC/NB-IoT specification which supports SIB accumulation. | Minimal |
| Option 2: Network dynamically indicates if NTN SIB accumulation is allowed in a cell | Network has the flexibility to dynamically allow/disallow NTN SIB accumulation depending on the NTN scenario and the SI configuration: - If allowed, UEs in deep coverage can leverage SIB accumulation to decode the NTN SIB. - When SIB accumulation is not feasible, the network may prohibit SIB accumulation and configure a larger number of repetitions instead. Lean SI transmission possible since network need not waste additional resources (i.e., longer SI windows with more repetitions) to cater to UEs in deep coverage since such UEs can accumulate NTN SIBs. | 1-bit indication needed in SI. | Low |
| Option 2a: Network dynamically indicates if NTN SIB accumulation is allowed in a cell AND indicates a reference time for SIB accumulation window | Same as Option 2. UEs will know exactly how many SI windows it can accumulate across, e.g., similar to epoch time, a reference time can be signalled to mark the end of the SI accumulation period. | Same as Option 2. Reference time and/or number of indentical NTN SIBs needs to be indicated in SI. | Low |
| Option 3: Increase SI window size for IoT NTN | Enables the network to configure a larger number of SIB repetitions to provide robust coverage. Especially useful in scenarios where NTN SIB accumulation is impossible e.g., certain LEO scenarios where network updates the NTN SIB in every SI window. Can be used in conjunction with Option 2/2a to provide a greater flexibility. | UEs in poor coverage may still require NTN SIB accumulation across multiple SI windows. | Low |

## FIRST ROUND Round Discussion

Moderator view is that Option 2, Option 2a requires new agreement in RAN1 beyond the scope of maintenance phase. Option 3 also requires spec change. A compromise for Option 1 with minimum impact on specs  is that the UE shall not assume that the NTN-specific SIB is constant across SI windows. This does not prevent the eNB from updating the NTN SIB contents per SI window, or broadcast the same NTN SIB contents across SI windows. The legacy NB-IoT, UE is allowed to combine SI across multiple SI windows. The issue of NTN SIB accumulation is specific to oT NTN.

In IoT NTN, there is currently no agreement on implicit epoch time. In RAN1#108-e it was discussed to re-use NR NTN agreement for implicit epoch time. There was no agreement because discussions in NR NTN on epoch time are still on-going. Once RAN1 has concluded discussions on epoch time in NR NTN, RAN1 should conclude on implicit epoch time for IoT NTN. A working assumption to adopt NR NTN solution for implicit epoch time could be considered.

First Round proposal – 3.2-1: Companies are encouraged to comment on options for NTN SIB accumulation

- Option 1: Prohibit NTN SIB accumulation across SI windows (or alternatively UE shall not assume that the NTN-specific SIB is constant across SI windows)

- Option 2: Network dynamically indicates if NTN SIB accumulation is allowed in a cell

- Option 2a: Network dynamically indicates if NTN SIB accumulation is allowed in a cell AND indicates a reference time for SIB accumulation window

- Option 3: Increase SI window size for IoT NTN

First Round proposal – 3.2-2:

- Adopt NR NTN solution for implicit epoch time as baseline for IoT NTN.

| Companies | Comments |
| --- | --- |
| Nokia, NSB | Based on legacy spec, 36.331 section 5.2.1.2a “The UE is not required to accumulate several SI messages in parallel but may need to accumulate a SI message across multiple SI windows, depending on coverage condition.” i.e. UE may accumulate the SIB across SI windows for coverage PoV. Network do not know how many SI window UE will combine the repetitions of the SIB. The ephemeris will be updated by satellite control center, instead of eNB. Both UE implementation for coverge and satellite control center based NSN SIB update should be considered. Considering above issues, we think implicit signaling of epoch time in IoT NTN is not workable, i.e. maybe only explicit signaling on epoch time is supported in IoT NTN. |
| ZTE | For 3.2-1, we support the alternative option 1, i.e., UE shall not assume that NTN-specific SIB is constant across SI windows, which resolves the ambiguity issue of implicit epoch time indication with minimized the spec effect. For 3.2-2, we support to reuse NR-NTN solution as baseline. |
| Lenovo | For 3.2-1 Support Option 1. Since eNB can provide explicit indicaton of the epoch time, this is the baseline option. If the explicit indication is not configured, implicit indication can be a supplement with the assumption that UE shall not assume that the NTN-specific SIB is constant across SI windows. This gives the eNB the choice/configuraton flexibility. For3.2-2 Support |
| Apple | For 3.2-1: Option 1 At the maintenance phase, we prefer not introducing a new scheme/signaling to optimize the performance. For 3.2-2: Support The NR NTN design is reused as much as possible. |
| Ericsson | For 3.2-1: We would like to reiterate that SIB accumulation is supported in terrestrial NB-IoT/eMTC specification for coverage enhancement. Since NTN SIB is as essential SIB, we strongly prefer to allow NTN SIB accumulation where possible. We support Option 2/2-a due to reasons mentioned in Table 2 in this section. As a compromise, if Option 1 is selected, then such a restriction should not be introduced for GEO as the NTN SIB does not need to be updated that aggressively and the UEs should be allowed to accumulate NTN SIB across multiple SI windows if needed. For 3.2-2: Support |
| Lockheed Martin | For 3.2-1 Support Option 1. For 3.2-2: Support |
| Qualcomm | For 3.2-1: We should give the network flexibility to change this SIB on the fly (so as to not have ephemeris information go “stale”, which would limit validity timer durations, etc.). So, the UE should generally not assume that this SIB remains same across SI windows. From the DL link budget analyses that were carried out in Release 17, DL coverage generally seemed pretty decent for all cases: accumulation “across windows” seems not needed. Proponents: could you provide a pointer to any coverage analysis with the assumptions in TR, that imply that SIB accumulation beyond a SI window will be required (apologies if we missed it). For 3.2-2: OK to follow NR-NTN. |
| Spreadtrum | For 3.2-1 Prefer Option 1. For 3.2-2: Support |
| Nordic | For 3.2-1 Prefer Option 1. For 3.2-2: Support |
| SONY | For 3.2-1. We do not support option 1. Coverage is challenging. We can support SIB accumulation across SI windows in TN and the same functionality should be supported in NTN. Option 2a works as it allows accumulation of SIB across multiple SI windows, but doesn’t require the NTN SIB to be the same for the whole modification period. We assume “reference time for SIB accumulation window” means that the UE will know set of SI windows across which it can assumulate SIB. For 3.2-2: we need to agree on 3.2-1 first. If the UE is able to accumulate SIB across multiple SI windows then adopting the NR NTN agreement would lead to the question of which of the SI windows in which a the SIB is transmitted implicitly marks the epoch time. IoT-NTN would need to be specific that the implicit epoch time is the “the end of the last SI window of a set of SI Windows transmitting the same SIB during which the NTN-specific SIB SI message is transmitted.” |
| Xiaomi | For 3.2-1, we prefer option 1, however, we are also open to discuss other options if coverage issue exists. For 3.2.2, support in principle. |

## SECOND ROUND Discussion

On proposal 3.2-1

- Option 1 is supported by 9 companies: ZTE, Lenovo, Apple, Lockheed Martin, Qualcomm, Spreadtrum, Nordic, Xiaomi, MediaTek

- Option 2/2-a is supported by 2 companies: Ericsson, SONY

On proposal 3.2-2, it supported by 10 companies ZTE, Lenovo, Apple, Ericsson, Lockheed Martin, Qualcomm,  Spreadtrum, Nordic, Xiomi, MediaTek .Nokia does not think implicit signalling of epoch time in IoT NTN is workable,SONY proposed  that IoT-NTN would need to be specific that the implicit epoch time is the “the end of the last SI window of a set of SI Windows transmitting the same SIB during which the NTN-specific SIB SI message is transmitted.”

Moderator view on Proposal 3.2-1: Based on comments, Option 1 is supported by 9 companies. If there is no agreement on SIB accumulation, then to the moderator understanding the UE implementation will assume that the SIB accumulation is not possible. The UE needs to know about the Epoch time first to determine the corresponding ephemeris and common TA parameters. Since the epoch time is indicated explicitly in NTN SIB, the UE first needs to decode the NTN SIB without accumulation. It would be a desirable clarification if Option 1 is agreed. There is not enough support for Option 2/2a

Moderator view on Proposal 3.2-2: RAN1 can adopt NR NTN solution for implicit epoch time as baseline for IoT NTN. If agreeable, a note as proposed by SONY text revision can be added. Based on Option 1, we remove “of a set of SI Windows transmitting the same SIB”

Second Round proposal – 3.2-1:  RAN1 agree Option 1: Prohibit NTN SIB accumulation across SI windows (or alternatively UE shall not assume that the NTN-specific SIB is constant across SI windows)

Second Round proposal – 3.2-2:

- Adopt NR NTN solution for implicit epoch time as baseline for IoT NTN.

Note: For IoT-NTN, the implicit epoch time is the the end of the last SI window of a set of SI Windows transmitting the same SIB during which the NTN-specific SIB is transmitted.”

| Companies | Comments |
| --- | --- |
| Moderator | We revised the note based on proposal 3.2-1 |
| Ericsson | 3.2-1: Please note that SIB accumulation is already supported in terrestrial eMTC/NB-IoT. This is very convenient from a coverage point of view considering that the UEs in a given cell may undergo very different coverage conditions. What should a network do in such a case: Approach 1: Configure a very long SI window with a large number of repetitions to ensure coverage to worst-case UEs an overkill, inefficient and leads to waste of resources Approach 2: Configure SI window/number of repetitions targeting the typical UEs, and leave the worst-case UEs to rely on SIB accumulation to decode the SIB an efficient approach, especially convenient for cells in good coverage (e.g., also applicable to many NTN scenarios) Therefore, we strongly think that Option 2/2-a can be considered to make the legacy SI accumulation work in NTN. In case there is no consensus, as a compromise, Option 1 should only apply to LEO, and SIB accumulation should not be prohibited for GEO since the ephemeris need not be updated in every SIB. |
| ZTE | For 3.2-1, support. For 3.2-2, we do not see the necessity of adding the note if option-1 of proposal 3.2-1 is agreed. |
| Xiaomi | For proposal 3.2-1, although we are fine to accept option 1. This will definitely hurt the coverage. Basically the eNB have to use explicit signaling to inform UE of the epoch time once there is coverage issue. We are willing to discuss potential solutions to resolve this issue for implicit indication. One clarification question on proposal 2 and 2-a is how the UE can know whether it should accumulate the SI or not. |
| Nokia, NSB | Coverage is one important requirement for IoT with large pathloss in high possibility, which should be guaranteed with high priority. As mentioned in first round, IoT UE may or may not do accumulation of SIB across SI windows for coverage PoV as legacy spec. And it is UE implementation but network do not know. It is not reasonable to limit the coverage of IoT NTN by prohibiting UE accumulation for SIB, especially when explicit signaling of epoch time is already supported and used at least once as mentioned by Xiaomi. The option with last SI window for SIB is also not good for IoT NTN, as it may cause significant reduction of validity timer when SIB do not change for long time. It may also cause the time large difference between current time and epoch time, thus may make ephemeris much less effective in time domain. Considering above issues, we think implicit signaling of epoch time in IoT NTN is not workable, we propose only explicit signaling on epoch time is supported in IoT NTN. |
| SONY | 3.2.1 Agree with Ericsson that SIB accumulation is already supported in terrestrial eMTC / NB-IoT. This helps to support operation in poor coverage in a terrestrial network. We should also support this feature in NTN. For the Ericsson approach 2, our understanding is that this would require option 2/2-a. Agree with Ericsson that Option 2/2s can be considered to make the legacy SI accumulation work in NTN. We would be OK with the compromise from Ericsson that Option 1 should only apply to LEO and that option 2/2a can apply to GEO. However, our preference is that Option 2/2a is supported both in LEO and GEO (do we really need to have signalling optimisations between different orbits??). In response to Xiaomi’s question: “how the UE can know whether it should accumulate the SI or not”: this could be indicated in SIB1. The UE needs to read SIB1 anyway and SIB1 should have good coverage. We do not understand the moderator’s comment that, “The UE needs to know about the Epoch time first to determine the corresponding ephemeris and common TA parameters. Since the epoch time is indicated explicitly in NTN SIB, the UE first needs to decode the NTN SIB without accumulation.”. Our understanding is that the UE needs to know the epoch time in order to interpret the ephemeris and common TA parameters. The UE does not need to first decode the NTN SIB without accumulation to determine epoch time: the UE can decode NTN SIB with accumulation in order to determine both the ephemeris, common TA and epoch time. 3.2.2 We think that 3.2.1 needs to be agreed before 3.2.1 is agreed. If there is a prohibition of accumulation across SI windows (according to the moderator proposal 3.2.1), the assumption is that the NTN-SIB changes every SI window. In this case, “For IoT-NTN, the implicit epoch time is the the end of the last SI window of a set of SI Windows transmitting the same SIB during which the NTN-specific SIB is transmitted.”. However, based on our view above (3.2.1), we think that accumulation between SI Windows should be possible for NTN-SIB. In this case, to avoid ambiguity in terms of SI Window, we would still suggest that the following is appropriate: Note: For IoT-NTN, the implicit epoch time is the the end of the last SI window of a set of SI Windows transmitting the same SIB during which the NTN-specific SIB is transmitted.” |

# [CLOSED] Synchronization aspects common to IoT NTN and NR NTN

The issues in contributions submitted to RAN1#109e for Re-17 IoT NTN for time and frequency synchronization aspects common to NR NTN and IoT NTN were summarized by the moderator in [2], and copied below in Table XX.

Table XX – Issues on time/frequency sync common to IoT NTN and IoT NTN

| Issue# | Issue | References | FL initial assessment | Company inputs (if any) |
| --- | --- | --- | --- | --- |
| 1-4 | SFN indicating Epoch time FL Note: RAN1#108-e working assumption is “adopt NR NTN solution for interpretation SFN indicating Epoch time”. Discuss in 8.4 NR NTN | R1-2203232, 3289, 3386, 3632, 3722, 3769, 3839, 4217 | H | [Thales]: Agree with FL initial assessment and FL Note |
| 1-5 | Negative TACommonDriftVariation values FL Note: RAN1#108-e working assumption is “adopt NR NTN solution for negative TACommonDriftVariation values”. Discuss in 8.4 NR NTN | R1-2203386, 3632, 3722, 4997 | H | [Thales]: Agree with FL initial assessment and FL Note |
| 1-6 | Common Delay formula in TS 36.213 FL Note: Adopt NR NTN agreement for IoT NTN. Discuss in 8.4 NR NTN | R1-2203386, 3721 | H | [Thales]: Agree with FL initial assessment and FL Note |
| 1-7 | Reference Frame for Ephemeris Set 2 – Orbital parameters FL Note: This is new issue also for NR NTN. Adopt NR NTN agreement for IoT NTN. Discuss in 8.4 NR NTN | R1-2203386 | H | [Thales]: Agree with FL initial assessment and FL Note |

RAN1#108-e already agreed the following:

Working assumption

Adopt NR NTN solution for negative TACommonDriftVariation values.

Working assumption

Adopt NR NTN solution for interpretation SFN indicating Epoch time.

## Company views

## FIRST ROUND Discussion

FL recommendation: discuss the following issues  in [2] in 8.14 NR NTN first

- Issue 1-4: SFN indicating Epoch time

- Issue 1-5: Negative TACommonDriftVariation values

- Issue 1-6: Common Delay formula in TS 36.213

- Issue 1-7: Reference Frame for Ephemeris Set 2 – Orbital parameters

Conclusions and agreements for these issues can then be re-used for IoT NTN

| Companies | Comments |
| --- | --- |
| Nokia, NSB | OK |
| ZTE | Support |
| Apple | Fine |
| Ericsson | Support |
| Lockheed Martin | Support |
| Qualcomm | Agree. |
| Spreadtrum | Support |
| Nordic | Support |
| SONY | Support |
| Xiaomi | Support |

## FIRST ROUND Discussion Conclusion

Since there is consensus from 10 commenting companies, moderator makes the FL recommendation as a proposal by1st check point: May 13 (any RRC impact by May 12)

Proposal 4.3 for 1st Checkpoint May 13:

Conclusions and agreements for the following issues as discussed in 8.14 NR NTN can be re-used for IoT NTN

- SFN indicating Epoch time

- Negative TACommonDriftVariation values

- Common Delay formula in TS 36.213

- Reference Frame for Ephemeris Set 2 – Orbital parameters

# Conclusions

## Proposals for 1st Checkpoint – May 13

The following proposals for 1st Checkpoint – May 13 are stable as discussed on RAN1 reflector and can be endorsed in RAN1

Proposal 2a for 1st Checkpoint May 13:

- The single UE capability that governs UE behavior w.r.t gaps between segments for PUSCH, PUCCH and NPUSCH, when the UE performs segmented pre-compensation, is as follows:

- When a single capability is signalled: UE drops one or more of the following durations of uplink transmission between segments (indicated by the capability):

  - 1 slot (applicable to eMTC)

  - 1 subframe (applicable to eMTC)

  - 1 slot (applicable to NB-IoT)

  - 2 slots (applicable to NB-IoT)

  - 1 symbol (applicable to both eMTC and NB-IoT)

  - UE follows legacy behaviour at slot boundaries due to TA adjustment

- When capability is NOT signalled: UE follows legacy behaviour at slot boundaries due to TA adjustment

Proposal 2c_rev1 for 1st Checkpoint May 13:

- Adopt TP#1  on TS 36.211 Section 5.3.4 for PUSCH for segmented transmission

<TP1, Section 5.3.4, TS 36.211>

For BL/CE UEs communicating over NTN, for PUSCH transmission, for frame structure type 1, after a transmission duration of Nsegmentprecompensation time units (which may include subframes that are not BL/CE UL subframes), a transmission gap of Ngapprecompensationtime units shall be counted for the PUSCH resource mapping but not used for transmission of the PUSCH, according to the single UE capability ue-CE-NeedSegmentedPrecompensationGaps, as specified in 3GPP TS 36.331. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity of Ngapprecompensation is based on the UE capability as is configured by the higher layers based on the UE capability if signalled.

<END TP1>

“reason for change” Clarify UE behaviour w.r.t gaps between segments for PUSCH when the UE performs segmented pre-compensation

“summary of change” a transmission gap of Ngapprecompensationtime units shall be counted for the PUSCH resource mapping but not used for transmission of the PUSCH, according to the single UE capability ue-CE-NeedSegmentedPrecompensationGaps, as specified in 3GPP TS 36.331. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity of Ngapprecompensation is based on the UE capability as configured by the higher layers.

“consequence if not approved” Release 17 eMTC UE cannot communicate via GEO and NGSO NTNs

Proposal 2d_rev1 for 1st Checkpoint May 13:

- Adopt TP#2  on TS 36.211 Section 5.4.3 for PUCCH for segmented transmission

<TP2, Section 5.4.3, TS 36.211>

For BL/CE UEs communicating over NTN, for PUCCH transmission, for frame structure type 1, after a transmission duration of Nsegmentprecompensation time units (which may include subframes that are not BL/CE UL subframes), a transmission gap of Ngapprecompensation time units shall be counted for the PUCCH resource mapping but not used for transmission of the PUCCH, according to the single UE capability ue-CE-NeedSegmentedPrecompensationGaps, as specified in 3GPP TS 36.331. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity Ngapprecompensation is based on the UE capability as is configured by the higher layers based on the UE capability if signalled.

<END TP2>

“reason for change” Clarify UE behaviour w.r.t gaps between segments for PUCCH when the UE performs segmented pre-compensation

“summary of change” a transmission gap of Ngapprecompensation time units shall be counted for the PUCCH resource mapping but not used for transmission of the PUCCH, according to the single UE capability ue-CE-NeedSegmentedPrecompensationGaps, as specified in 3GPP TS 36.331

“consequence if not approved” Release 17 eMTC UE cannot communicate via GEO and NGSO NTNs

Proposal 2e_rev1 for 1st Checkpoint May 13:

- Adopt TP#3  on TS 36.211 Section 10.1.3.6 for NPUSCH for segmented transmission

<TP3, Section 10.1.3.6, TS 36.211>

For a UE communicating over NTN, after transmissions (and/or postponements due to NPRACH) of Nsegmentprecompensation time units, for frame structure type 1, a transmission gap of Ngapprecompensation time units shall be counted for the NPUSCH resource mapping but not used for transmission of the NPUSCH according to the UE capability ue-NBIOT-NeedSegmentedPrecompensationGaps , as specified in 3GPP TS 36.331. The quantity Nsegmentprecompensation is provided by higher layers,  and the quantity of Ngapprecompensation is based on the UE capability as is configured by the higher layers based on the UE capability if signalled.

<END TP3>

“reason for change” Clarify UE behaviour w.r.t gaps between segments for NPUSCH when the UE performs segmented pre-compensation

“summary of change” a transmission gap of Ngapprecompensation time units shall be counted for the NPUSCH resource mapping but not used for transmission of the NPUSCH according to the UE capability ue-NBIOT-NeedSegmentedPrecompensationGaps , as specified in 3GPP TS 36.331

“consequence if not approved” Release 17 NB-IoT UE cannot communicate via NGSO NTNs

# References

- R1-2203089, Huawei, Maintenance on NB-IoT/eMTC support for Non-Terrestrial Network, RAN1#109-e, e-Meeting, May 9th – 20th, 2022

- R1-2203232, ZTE, Remaining issues on IoT-NTN, RAN1#109-e, e-Meeting, May 9th – 20th, 2022

- R1-2203386, Spreadtrum Communications, Maintenance on NB-IoT/eMTC support for Non-Terrestrial Network, RAN1#109-e, e-Meeting, May 9th – 20th, 2022

- R1-2203386, MediaTek, Maintenance on NB-IoT/eMTC to support NTN, RAN1#109-e, e-Meeting, May 9th – 20th, 2022

- R1-2203632, Ericsson, On IoT NTN maintenance issues, RAN1#109-e, e-Meeting, May 9th – 20th, 2022

- R1-2203722, SONY, Maintenance of IoT-NTN, RAN1#109-e, e-Meeting, May 9th – 20th, 2022

- R1-2203769, Xiaomi, Remaining issues on IoT NTN, RAN1#109-e, e-Meeting, May 9th – 20th, 2022

- R1-2203839, Nokia, Nokia Shanghai Bell, Maintenance on NB-IoT/eMTC support for Non-Terrestrial Network, RAN1#109-e, e-Meeting, May 9th – 20th, 2022

- R1-2203991, OPPO, Discussion on remaining issues for NB-IOT/eMTC NTN, RAN1#109-e, e-Meeting, May 9th – 20th, 2022

- R1-2204217, Apple, On remaining issues of IoT NTN, RAN1#109-e, e-Meeting, May 9th – 20th, 2022

- R1-2204997, Qualcomm, Maintenance on IoT-NTN, RAN1#109-e, e-Meeting, May 9th – 20th, 2022

# Appendix

In the Table below, company proposals for time and frequency synchronization are listed

| Contribution | Observation/Proposals |
| --- | --- |
| Huawei (R1-2203089) | Observation 1: The description on the timing relationships is typically based on logical timing and there is no need to introduce a physical timing of TA. Proposal 1: For IoT NTN, adopt NR NTN solutions for the definition of epoch time and the UE behaviour related to the validity timer of the assistance information. |
| ZTE (R1-2203232) | - Proposal 1: The details of how to apply the segment pre-compensation should be captured in the specification. - Proposal 2: Adopt following TPs in TS 36.213 to ensure the support of segment pre-compensation. <TP1, Section 4.2.3, TS 36.213> For a BL/CE UE, if a segment duration is configured by higher layer parameter TransmissionDurationPRACH-r17, the [36.211] shall be adjusted per segment during the transmission of physical random access preamble. For a BL/CE UE, if a segment duration is configured by higher layer parameter TransmissionDurationPUSCH-r17, the [36.211] shall be adjusted per segment during the transmission of physical uplink shared channel. - <END TP1> <TP2, Section 16.1.2, TS 36.213> If a segment duration is configured by higher layer parameter TransmissionDurationNPRACH-NB-r17, the [36.211] shall be adjusted per segment during the transmission of narrowband physical random access preamble. If a segment duration is configured by higher layer parameter TransmissionDurationNPUSCH-NB-r17, the [36.211] shall be adjusted per segment during the transmission of narrowband physical uplink shared channel. - <END TP2> - Proposal 3: An LS to RAN4 is needed to inform the RAN1 required updates on TS 36.133 to support segment pre-compensation, i.e., allow UE to adjust UL timing during an ongoing repetition period at least in NTN. - Proposal 4: UE should follow legacy behavior at slot boundaries due to TA adjustment if single UE capability that governs UE behavior w.r.t gaps between segments for PUSCH and NPUSCH, when the UE performs segmented pre-compensation, is not indicated. - Proposal 5: Support the TPs on TS 36.211 proposed by the feature leader of synchronization issue in RAN1#108e, which are as follows: <TP3, Section 5.3.4, TS 36.211> For BL/CE UEs communicating over NTN, for PUSCH transmission, for frame structure type 1, after a transmission duration of Nsegmentprecompensation time units (which may include subframes that are not BL/CE UL subframes), a transmission gap of Ngapprecompensationtime units shall be counted for the PUSCH resource mapping but not used for transmission of the PUSCH, according to the UE capability ue-CE-NeedSegmentedPrecompensationGaps, as specified in 3GPP TS 36.331. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity Ngapprecompensationis one subframe, one slot, or one symbol may be configured by network based on the quantity supported by the UE if indicated by UE in ue-CE-NeedSegmentedPrecompensationGaps. <END TP3> <TP4, Section 10.1.3.6, TS 36.211> For a UE communicating over NTN, after transmissions and/or postponements due to NPRACH of Nsegmentprecompensation time units, for frame structure type 1, a transmission gap of Ngapprecompensation time units shall be shall be counted for the NPUSCH resource mapping but not used for transmission of the PUSCH according to the UE capability ue-NBIOT-NeedSegmentedPrecompensationGaps , as specified in 3GPP TS 36.331. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity Ngapprecompensation is one subframe, one slot or 2 slots, or one symbol may be configured by network based on the quantity supported by the UE if indicated by UE in ue-CE-NeedSegmentedPrecompensationGaps. <END TP4> <TP5, Section 5.3.4, TS 36.211> For BL/CE UEs communicating over NTN, for PUSCH transmission, for frame structure type 1, after a transmission duration of Nsegmentprecompensation time units (which may include subframes that are not BL/CE UL subframes), a transmission gap of Ngapprecompensation time units shall be counted for the PUCCH resource mapping but not used for transmission of the PUCCH, according to the UE capability ue-CE-NeedSegmentedPrecompensationGaps, as specified in 3GPP TS 36.331. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity Ngapprecompensation is one subframe, one slot, or one symbol may be configured by network based on the quantity supported by the UE if indicated by UE in ue-CE-NeedSegmentedPrecompensationGaps. - <END TP5> - Proposal 6: The NTN-specific SIB is expected to be updated per SI window in IoT-NTN. |
| Spreadtrum (R1-2203316) | Proposal 1: Adopt text proposal 1 in section 2.1. ------------------------------------------ Start of Text Proposal 1-------------------------- < Unchanged parts are omitted > For BL/CE UEs communicating over NTN, for PUSCH transmission, for frame structure type 1, after a transmission duration of Nsegmentprecompensation time units (which may include subframes that are not BL/CE UL subframes), a transmission gap of Ngapprecompensation time units shall be shall be counted for the PUSCH resource mapping but not used for transmission of the PUSCH, according to the UE capability ue-CE-NeedSegmentedPrecompensationGaps, as specified in 3GPP TS 36.331. BL/CE UL subframes within the gap of Ngapprecompensation time units shall be counted for the PUSCH resource mapping but not used for transmission of the PUSCH. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity Ngapprecompensation is one subframe , one slot, or one symbol may be configured by network based on the quantity supported by the UE if indicated by UE in ue-CE-NeedSegmentedPrecompensationGaps. For BL/CE UEs in CEModeB, for PUSCH transmission not associated with Temporary C-RNTI, for frame structure type 1, after a transmission duration of time units (which may include subframes that are not BL/CE UL subframes), a gap of time units shall be inserted, according to the UE capability ue-CE-NeedULGaps, as specified in 3GPP TS 36.331 [9]. BL/CE UL subframes within the gap of time units shall be counted for the PUSCH resource mapping but not used for transmission of the PUSCH. < Unchanged parts are omitted > ------------------------------ End of Text Proposal 1------------------------ Proposal 2: Adopt text proposal 2 in section 2.2. < Unchanged parts are omitted > ---------------------------------- Start of Text Proposal 2----------------------- For a UE communicating over NTN, after transmissions and/or postponements due to NPRACH of Nsegmentprecompensation time units, for frame structure type 1, a gap of Ngapprecompensation time units shall be shall be counted for the NPUSCH resource mapping but not used for transmission of the PUSCH according to the UE capability ue-NBIOT-NeedSegmentedPrecompensationGaps , as specified in 3GPP TS 36.331. UL slots within the gap of Ngapprecompensation time units shall be counted for the NPUSCH resource mapping but not used for transmission of the NPUSCH. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity Ngapprecompensation is one subframe, one slot or 2 slots, or one symbol may be configured by network based on the quantity supported by the UE if indicated by UE in ue-CE-NeedSegmentedPrecompensationGaps. The portion of a postponement due to NPRACH which coincides with a gap is counted as part of the gap. NPRACH gaps as defined in clause 10.1.6.1 are not part of the NPRACH resource. For frame structure type 2, the valid uplink subframes which are not used for NPRACH transmission when it is not possible to map G symbol groups back-to-back are not part of the NPRACH resource. The mapping of is then repeated until slots have been transmitted. After transmissions and/or postponements due to NPRACH of time units, for frame structure type 1, a gap of time units shall be inserted where the NPUSCH transmission is postponed. The portion of a postponement due to NPRACH which coincides with a gap is counted as part of the gap. < Unchanged parts are omitted > --------------------------------- End of Text Proposal 2------------------------- |
| MediaTek (R1-2203386) | Proposal 1: Adopt new range for TACommonDrift - 262143… + 262143 (i.e: 52.42 µs/s … + 52.42 µs/s ) and new range for TACommonDriftVariation 0… 32767 (0… 0.65 µs/s2). Proposal 2: confirm RAN1#107-e working for TACommon, TACommonDrift, TACommonDriftVariation with revised range for TACommonDrift Parameter name; Value range; Granularity; Bits allocation. TACommon; 0 ... 8316827 (i.e: 0… 270.73 ms); 32.55208 x 10-3 µs; 23 bits. TACommonDrift; - 261935… + 261935- 262143… + 262143 (i.e: -53.33 µs/s … +53.33 µs/s -52.42 µs/s … +52.42 µs/s); 0.2 x 10-3 µs/s; 19 bits. TACommonDriftVariation; 0…2947032767 (0…0.600.65 µs/s2 ); 0.2 x 10-4 µs/ s2; 15 bits. - Value ranges are given in unit of corresponding granularity. Proposal 3: Re-use TP on Delaycommont in TS 38.213 Section 4.2.3 in NR NTN for IoT NTN in TS 36.213 Section 4.2.3 Proposal 4: For GEO for IoT NTN: - TACommonDrift with granularity 0.2 * 1e-4 us/s and range +/-5.24 us/s, bits allocation 19 bits - TACommonDriftVariation with granularity 2 * 1e-7 us/s^2 and range +/-3.27 ns/s^2, bits allocation 15 bits Observation 1: RAN1#108-e proposal 15 Rev 3 “If indicated explicitly by a SFN and subframe number, the UE considers this frame to be the frame which is nearest to the frame where the message is received” cannot solve SFN wrapping ambiguity if UE decodes SFN for Epoch time (Epoch time SFN 500) at SIBx SFN (SFN 1012). Observation 2: RAN1#108-e proposal 15 Rev 4 “Indicated SFN for Epoch time is current SFN or the next upcoming SFN after the frame where the message indicating the Epoch time is received.” requires longer predicition time with an additional 10.24 s if UE decodes SFN for Epoch time (Epoch time SFN 0) at SIBx SFN (SFN 1). Proposal 5: Indicated SFN for Epoch time is, - if (Epoch time SFN- SIBx SFN) is positive choose next epoch time after SIBx SFN (i.e. SFN for epoch time is in the future). - if (Epoch time SFN- SIBx SFN) is zero choose SIBx SFN (i.e. SFN for epoch time is SIBx SFN ). - if (Epoch time SFN- SIBx SFN) is negative choose previous epoch time before SIBx SFN (i.e. SFN for epoch time is in the past). Note 1: SIBx SFN is the last frame where the message indicating the Epoch time is received. Note 2: The UE shall not assume that the NTN-specific SIB is constant across SI windows Observation 3: UE should report the smallest duration of uplink transmission it drops between segments to minimize SNR loss. Proposal 6: The single UE capability that governs UE behavior w.r.t gaps between segments for PUSCH, PUCCH and NPUSCH, when the UE performs segmented pre-compensation, is as follows: When a single capability is signaled: UE drops one of the following durations of uplink transmission between segments (indicated by the capability): - 1 slot (applicable to eMTC) - 1 subframe (applicable to eMTC) - 1 slot (applicable to NB-IoT) - 2 slots (applicable to NB-IoT) - 1 symbol (applicable to both eMTC and NB-IoT) When capability is NOT signalled: UE follows legacy behavior at slot boundaries due to TA adjustment. Proposal 7: RAN1 can discuss the single UE capability for UL segmented transmission in AI 8.16.14 UE features for IoT over NTN. Proposal 8: For set 2, RAN1 agree on orbital parameters α , e, ω , Ω , I, and M in Earth Centered Inertial (ECI) Frame - The ECI and ECEF coincide at Epoch time (e.g. x,y,z axis in ECEF are aligned with x,y,z axis in ECI) |
| Ericsson (R1-2203632) | Observation 1: NTN SIB may need to be updated much more frequently for LEO than for GEO. Observation 2:NTN SIB accumulation across SI windows may be more beneficial for GEO than for LEO scenarios. Observation 3: The NTN SIB update frequency and the SI window configuration jointly determine if NTN SIB accumulation will be beneficial. Proposal 1: Adopt TP on Delaycommont in TS 38.213 Section 4.2.3 in NR NTN for IoT NTN in TS 36.213 Section 4.2.3. Proposal 2: Adopt the following proposal for PUSCH in TS 36.211 Clause 5.3.4: -------------------- Start of TP for 3GPP TS 36.211 -------------------- ## 5.3.4 Mapping to physical resources <Unchanged Text Omitted> For BL/CE UEs communicating over NTN, for PUSCH transmission, for frame structure type 1, after a transmission duration of Nsegmentprecompensation time units (which may include subframes that are not BL/CE UL subframes), a transmission gap of Ngapprecompensationtime units shall be counted for the PUSCH resource mapping but not used for transmission of the PUSCH, according to the UE capability ue-CE-NeedSegmentedPrecompensationGaps, as specified in 3GPP TS 36.331 [9]. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity Ngapprecompensationis one subframe, one slot, or one symbol, based on the quantity supported by the UE if indicated by the UE in ue-CE-NeedSegmentedPrecompensationGaps. For BL/CE UEs in CEModeB, for PUSCH transmission not associated with Temporary C-RNTI, for frame structure type 1, after a transmission duration of time units (which may include subframes that are not BL/CE UL subframes), a gap of time units shall be inserted, according to the UE capability ue-CE-NeedULGaps, as specified in 3GPP TS 36.331 [9]. BL/CE UL subframes within the gap of time units shall be counted for the PUSCH resource mapping but not used for transmission of the PUSCH. -------------------- End of TP for 3GPP TS 36.211 -------------------- Proposal 3: Adopt the following proposal for PUCCH in TS 36.211 Clause 5.4.3: -------------------- Start of TP for 3GPP TS 36.211 -------------------- ## 5.4.3 Mapping to physical resources <Unchanged Text Omitted> For BL/CE UEs communicating over NTN, for PUCCH transmission, for frame structure type 1, after a transmission duration of Nsegmentprecompensation time units (which may include subframes that are not BL/CE UL subframes), a transmission gap of Ngapprecompensation time units shall be counted for the PUCCH resource mapping but not used for transmission of the PUCCH, according to the UE capability ue-CE-NeedSegmentedPrecompensationGaps, as specified in 3GPP TS 36.331 [9]. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity Ngapprecompensation is one subframe, one slot, or one symbol based on the quantity supported by the UE if indicated by the UE in ue-CE-NeedSegmentedPrecompensationGaps. -------------------- End of TP for 3GPP TS 36.211 -------------------- Proposal 4: Adopt the following proposal for NPUSCH in TS 36.211 Clause 10.1.3.6: -------------------- Start of TP for 3GPP TS 36.211 -------------------- ## 10.1.3.6 Mapping to physical resources <Unchanged Text Omitted> For a UE communicating over NTN, after transmissions and/or postponements due to NPRACH of Nsegmentprecompensationtime units, for frame structure type 1, a transmission gap of Ngapprecompensation time units shall be shall be counted for the NPUSCH resource mapping but not used for transmission of the NPUSCH according to the UE capability ue-NBIOT-NeedSegmentedPrecompensationGaps, as specified in 3GPP TS 36.331 [9]. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity Ngapprecompensation is one slot, two slots, or one symbol based on the quantity supported by the UE if indicated by the UE in ue-NBIOT-NeedSegmentedPrecompensationGaps. The portion of a postponement due to NPRACH which coincides with a gap is counted as part of the gap. -------------------- End of TP for 3GPP TS 36.211 -------------------- Proposal 5: For IoT NTN, adopt the same definition for explicit epoch time as for NR NTN. Proposal 6: Network should have the ability to optionally indicate if NTN SIB accumulation across SI windows is allowed or not. Proposal 7: RAN1 to discuss and decide the best solution for NTN SIB accumulation while considering the technical merits as summarized in Table 2. Table 2 Way forward for enabling NTN SIB accumulation for IoT NTN. Pros; Cons; Spec. impact. Option 1: Prohibit NTN SIB accumulation across SI windows; Only meaningful in NTN scenarios where NTN SIB will be updated during each SI window.; Too restrictive for many NTN scenarios where SI window periodicity is shorter than NTN SIB update frequency, e.g., the maximum SI periodicity is 5.12 s for eMTC and 40.96 s for NB-IoT which is much shorter than the NTN SIB update frequency in GEO. Large SI transmission overhead as network may need to configure a larger SI window to support a greater number of repetitions in the cell to compensate for the lack of SIB accumulation. Coverage-limited UEs cannot access the network if NTN SIB cannot be decoded. Not synergistic with legacy eMTC/NB-IoT specification which supports SIB accumulation.; Minimal. |
| SONY (R1-2203722) | Proposal 1: Specify capability-based uplink gaps due to segmented pre-compensation in TS 36.211, during which a UE indicating the need of such gaps is not expected to transmit in the uplink for the requisite number of uplink slots. - These may be described in the “Mapping to physical resources” sections for PUSCH (Section 5.3.4) and NPUSCH (Section 10.1.3.6), analogous to the existing 40 ms gap after 256 ms of continuous uplink transmission. Proposal 2: For UL segmented transmissions, the UE signals which gap insertion method(s) it is capable of and the eNB configures the UE with the gap insertion method to it should use. Proposal 3: When Epoch time is not explicitly indicated in SIB, epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is implicitly known as the end of the Nth SI window of a set of N SI windows during which an identical NTN-specific SI message is transmitted. Observation: No specification changes are required for cases 7-11 of the “NPDCCH monitoring restrictions”. Proposal 4: The agreed equation of 〖Delay〗_common (t) and epoch time t_epoch definition in RAN1 107-e should be captured in specification. If this issue is agreed in NR NTN, IoT NTN should also capture the agreement in the TS36.213 specification. Proposal 5: Support negative values for the NTACommon-NB-r17 RRC parameter to support compensation for TA overestimation. |
| Xiaomi (R1-2203769) | Proposal 1: The epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is always provided explicitly via SIBx/dedicated signalling. Proposal 2: If implicit indication is supported, epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is implicitly known as the end of the SI window during which the NTN-specific SIB is transmitted. UE shall not assume that the NTN-specific SIB is constant across SI windows |
| Nokia, Nokia Shanghai Bell (R1-2203839) | Observation 1: The more dropping, the more performance loss for the dropped symbol/subframe. Observation 2: Dropping of entire subframe in each segment will cause large performance reduction, especially for the case when segment size is small, e.g. segment size as 2/4/8 subframe. Observation 3: Considering the longer CP of first symbol, it will be good to dropping starting from first symbol of the segment. Observation 4: For dropping samples in IoT NTN, the simplest way is to reuse the legacy method of dropping samples. Observation 5: Both reporting or not reporting for UE to report supporting of legacy dropping sample method can be possible and workable. Observation 6: Implicit indication of the Epoch time of assistance information in NTN SIB does not work, because the assistance information may not necessarily be updated every SI window. Proposal 1: For PUSCH, when dropping is not exceeding the CP, samples dropping of partial CP can be considered, otherwise one-symbol to be dropped should be utilized. Proposal 2: For PRACH, when dropping is not exceeding the CP, samples dropping of parital CP can be considered, otherwise dropping of CP + one sequence should be utilized. Proposal 3: Legacy dropping samples should be mandatory for IoT NTN. Proposal 4: UE to report capability and network to configure which method to be used according to UE’s capability and default supported method if any, so that no additional impact on standard in terms of UE capability utilization. Proposal 5: The proposed definition for UE capability on dropping method and the TP should be discussed and added in NR specification. Proposal 6: Only explicit signaling of Epoch time for assistance information shall be specified for IoT NTN. |
| Apple (R1-2204217) | Proposal 1: In IoT NTN, when epoch time is not explicitly indicated in SIB, epoch time of assistance information is implicitly known as the end of the SI window during which the NTN-specific SIB is transmitted. Proposal 2: A single value X is indicated in SIB for the uplink transmission segment duration for NPRACH for NB-IoT and PRACH for eMTC. Proposal 3: In uplink transmissions, - if UE is to drop samples between two successive segments, it drops the tail samples of the earlier segment. - if UE is to insert samples between two successive segments, it repeats the last sample of the earlier segment. - if UE is to blank subframe/slot/repetition unit between two successive segments, it blanks the last subframe/slot/repetition unit of the earlier segment. |
| Qualcomm (R1-2204997) | Proposal 1: Specify capability-based uplink gaps due to segmented pre-compensation in TS 36.211, during which a UE indicating the need of such gaps is not expected to transmit in the uplink for the requisite number of uplink slots. - These may be described in the “Mapping to physical resources” sections for PUSCH (Section 5.3.4) and NPUSCH (Section 10.1.3.6), analogous to the existing 40 ms gap after 256 ms of continuous uplink transmission. - Candidate TPs capturing this—TP1 and TP2 in this contribution—may be endorsed. Proposal 2: Modify the specification for DL monitoring restrictions after NPUSCH transmission to account for uplink transmission timing. Proposal 3: Clarify the use of logical/physical time in different places of the specifications. - Option 1: For half-duplex monitoring restrictions (such as Example 1.2 in this contribution), use logical time to index uplink transmissions with UL indices, DL reception with DL indices, and use a TA term to link the two. - Option 2: Include a table in the specifications to state which relationships use logical time, and which use physical time. - Currently, only the half-duplex monitoring restrictions appear to use physical time, while other timing relationships use logical time. Proposal 4: For GEO NTNs, a UE applies common TA only in accordance with the common TA term, TACommon, and is not required to process the drift/variation parameters TACommonDrift and TACommonDriftVariation. Proposal 5: Describe the concept of (NTN-specific) frequency pre-compensation/adjustment for uplink transmission in the physical layer specifications (3GPP TS 36.211), analogous to the current description of NTN-specific timing pre-compensation in Section 8.1 of TS 36.211 on Timing Advance. - Option 1: Create a new section to describe frequency pre-compensation/adjustment for uplink transmission - Option 2: Describe frequency pre-compensation in the sections on SC-FDMA signal generation (5.6 and 10.1.5) |