3GPP TSG RAN WG1 Meeting #109-e	R1-2203390

Agenda Item:	9.12.2

Source:	MediaTek Inc.

Title:	Disabling of HARQ for IoT NTN

Document for:	Discussion and Decision

- 1 Introduction

In RAN#95e [1], the revised WID on IoT NTN enhancements was endorsed for Release 18. In the WID, it is proposed to define further enhancements for NB-IoT NTN and eMTC NTN to improve performance in terms of throughput under assumption of transparent payload based on GEO and NGSO network scenarios addressing at least 3GPP power class 3 UE with GNSS capability in both Earth fixed &/or moving cell configurations. Rel-17 IoT-NTN is considered as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objective for disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates.

This contribution aims to discuss aspects related to disabling of HARQ feedback for IoT NTN.

- 2 Discussion on disabling of HARQ feedback

According to the WID, disabling of HARQ feedback for IoT NTN has been agreed in RAN meetings.

This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below: - Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2] - Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption [RAN1]

Observation 1: for IoT NTN, disabling of HARQ feedback is supported in Rel-18 IoT NTN WID.

The main concerns for disabling HARQ process are:

- HARQ stalling impact on peak data rates.

- HARQ re-transmissions impact on latency

- Reliability via RLC ARQ re-transmission with fast RLC configuration

## 2.1 HARQ Stalling impact on peak data rates

In LTE eMTC and NB-IoT specifications, the HARQ process with adaptive retransmissions is asynchronous on DL and UL. In both directions, the receiver tries to decode the transport block (TB) with soft combining of HARQ re-transmissions. In case the transmission is a new TB, the soft buffer should be flushed. In case a retransmission occurred, soft combining of current HARQ re-transmission and previous HARQ transmissions should be performed. The UE knows a scheduled TB is re-transmitted from the new-data indicator (NDI) as indicated on DCI.

In satellite systems, larger values of RTD can be typically experienced due to the large channel propagation delay between the UE and the satellite, which depends on the satellite orbit and the elevation angle. HARQ stalling occurs due to the longer satellite RTT between the device and the gNB, where the UE needs to wait longer for the reception of a packet or the UL grant to transmit a new packet with HARQ re-transmissions.

In Rel-14 NB-IoT, the max TBS is 2536 bits. Table 1 shows the maximum data rates of NB-IoT for Cellular, LEO, and GEO. The impact of the satellite RTD on the NB-IoT data rates in GEO due to HARQ stalling is in the order of 95% reduction; for LEO it is about a 49% reduction (we took the average in Table 1). NB-IoT data rate reduction in LEO is relatively much smaller than that in GEO.

|  | Rel-13 1 HARQ Process, TBS=680 bits (DL) TBS 1000 bits (UL) |  | Rel-14 1 HARQ Process TBS=2536 bits |  | Rel-14 2-HARQ Processes TBS=2536 bits |  | Rel-17 2-HARQ Processes TBS=2536 bits 16 QAM |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DL | UL | DL | UL | DL | UL | DL | UL |
| Cellular | 26.1 kbps | 58.8 kbps | 79.2 kbps | 115.2 kbps | 126.8 kbps | 158.5 kbps | 253.6 kbps | 317.0 kbps |
| LEO | 13.3 kbps (49.0%) | 22.2 kbps (62.2%) | 43.7 kbps (44.8%) | 50.7 kbps (55.9%) | 74.6 kbps (41.2%) | 84.5 kbps (46.7%) | 149.2 kbps (41.2%) | 169.0 kbps (46.7%) |
| GEO | 1.2 kbps (95.4%) | 1.8 kbps (96.9%) | 4.4 kbps (94.4%) | 4.5 kbps (96.0%) | 8.7 kbps (93.1%) | 8.8 kbps (94.4%) | 17.4 kbps (93.1%) | 17.6 kbps (94.4%) |

Table 1: Data rates and reduction in data rates due to HARQ stalling

Observation 2: for NB-IoT, HARQ stalling reduces data rates by approximately 95% and 49% for GEO and LEO respectively without disabling of HARQ feedback.

## 2.2 HARQ re-transmissions impact on latency

The latency with 1, 2, and 4 HARQ transmissions are shown in Table 2. The worst case for latency is GEO with 2 HARQ process and 4 HARQ re-transmissions is 2264 ms.

|  | Num HARQ Tx | Rel-13 1 HARQ Process, TBS=680 bits (DL) TBS 1000 bits (UL) |  | Rel-14 1 HARQ Process TBS=2536 bits |  | Rel-14 2-HARQ Processes TBS=2536 bits |  | Rel-17 2-HARQ Processes TBS=2536 bits 16 QAM |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL | UL | DL | UL | DL | UL | DL | UL |
| Cellular | 1 | 26 ms | 17 ms | 32 ms | 22 ms | 40 ms | 32 ms | 40 ms | 32 ms |
|  | 2 | 52 ms | 34 ms | 64 ms | 44 ms | 80 ms | 64 ms | 80 ms | 64 ms |
|  | 4 | 104 ms | 208 ms | 108 ms | 88 ms | 160 ms | 128 ms | 160 ms | 128 ms |
| LEO | 1 | 51 ms | 43 ms | 58 ms | 50 ms | 68 ms | 60 ms | 68 ms | 60 ms |
|  | 2 | 102 ms | 86 ms | 116 ms | 100 ms | 136 ms | 120 ms | 136 ms | 120 ms |
|  | 4 | 204 ms | 172 ms | 232 ms | 200 ms | 272 ms | 240 ms | 272 ms | 320 ms |
| GEO | 1 | 567 ms | 549 ms | 574 ms | 556 ms | 584 ms | 566 ms | 584 ms | 566 ms |
|  | 2 | 1134 ms | 1098 ms | 1148 ms | 1112 ms | 1168 ms | 1132 ms | 1168 ms | 1132 ms |
|  | 4 | 2268 ms | 2196 ms | 2296 ms | 2224 ms | 2336 ms | 2264 ms | 2336 ms | 2264 ms |

Table 2: Latency due to HARQ stalling and HARQ re-transmissions

Observation 3: for NB-IoT, the maximum latency with 2 HARQ processes with up to 4 HARQ re-transmissions is 2264 ms.

## 2.3 Disabling of HARQ Feedback

The data rates achievable in LEO and GEO with disabling of HARQ feedback can be consistent with cellular, as shown for NB-IoT in Table 3. Disabling of HARQ feedback may also reduce latency as HARQ re-transmissions are not done at the MAC layer.

Observation 4: for NB-IoT, disabling of HARQ feedback avoids the issue of HARQ stalling and allows data rates consistent with cellular NB-IoT / eMTC.

|  | Rel-13 1 HARQ Process, TBS=680 bits (DL) TBS 1000 bits (UL) |  | Rel-14 1 HARQ Process TBS=2536 bits |  | Rel-14 2-HARQ Processes TBS=2536 bits |  | Rel-17 2-HARQ Processes TBS=2536 bits 16 QAM |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DL | UL | DL | UL | DL | UL | DL | UL |
| Cellular | 26.1 kbps | 58.8 kbps | 79.2 kbps | 115.2 kbps | 126.8 kbps | 158.5 kbps | 253.6 kbps | 317.0 kbps |
| LEO with HARQ feedback Disabling | 26.1 kbps | 58.8 kbps | 79.2 kbps | 115.2 kbps | 126.8 kbps | 158.5 kbps | 253.6 kbps | 317.0 kbps |
| GEO with HARQ feedback Disabling | 26.1 kbps | 58.8 kbps | 79.2 kbps | 115.2 kbps | 126.8 kbps | 158.5 kbps | 253.6 kbps | 317.0 kbps |

Table 3: Data rates with HARQ feedback disabling

Disabling of HARQ feedback was discussed in Rel-17 IoT NTN study phase, and also discussed in Rel-16 NR NTN study phase / Rel-17 NR NTN Work Item. There were agreements in Rel-17 IoT NTN on disabling of HARQ feedback.

Agreement (RAN1 104-e): - In relation to HARQ operation in NTN IoT, further study at least - The necessity, potential benefits and drawbacks of any other potential HARQ feedback mechanisms - The necessity, potential benefits and drawbacks of reduced PDCCH monitoring - Further discuss the potential benefits and/or drawbacks of disabling HARQ feedback for NB-IoT and eMTC, and consider at least the following number of HARQ processes for the analysis - NB-IoT: - Total: 2, disabled: {1,2} - eMTC: - Total: 2, disabled: {1,2} - Total: 8, disabled: {1,2,7,8} - Other values for number of HARQ processes below the maximum value can be discussed - FFS: whether to consider separately LEO and GEO scenarios - FFS: whether to allow disabling of HARQ feedback in case of single HARQ process - FFS: whether to allow disabling of all HARQ feedback - FFS: other details for the evaluation/analysis Further study to identify whether HARQ stalling happens at least in the GEO satellite scenario. Agreement (RAN1 104b-e): Increasing the number of HARQ processes for NB-IoT and for eMTC in NTN is recommended not to be supported in Rel-17.

There were agreements in Rel-17 NR NTN on disabling of HARQ feedback.

Agreement (RAN1 102-e): Enabling/disabling on HARQ feedback for downlink transmission should be at least configurable per HARQ process via UE specific RRC signaling

For disabling of HARQ feedback, reuse NR NTN solutions in IoT NTN can be assumption.

Proposal 1: Enabling/disabling on HARQ feedback for downlink transmission is configurable per HARQ process via UE specific RRC signalling in IoT NTN.

Based on eNB configuration for disabling of HARQ feedback per HARQ process per UE, the eNB will know for a given HARQ process that the UE will not transmit the HARQ feedback on NPUSCH format 2 for NB-IoT or PUCCH for eMTC. In cellular case, the expected eNB transmitted behaviour is to wait for ACK/NACK of current DL packet transmission before scheduling a new/retransmission.

Observation 5: In case of disabling of HARQ feedback, it seems natural if eNB transmitter does not wait for ACK/NACK of current DL packet transmission before scheduling a new/retransmission since the UE does not anyway send ACK/NACK for the current NPDSCH/PDSCH for the corresponding HARQ process.

For the ending of the received NPDSCH in subframe n, UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+12 in cellular case. Considering disabling of HARQ feedback per HARQ process per UE, to align with cellular, UE is not required to monitor another NPDCCH in any subframe starting from subframe n+1 to subframe n+12.

TS 36.213 Section 16.6 If a NB-IoT UE receives a NPDSCH transmission ending in subframe n, and if the UE is not required to transmit a corresponding NPUSCH format 2, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+12.

Proposal 2:  When HARQ feedback for the HARQ process ID is disabled, if a NB-IoT UE receives a NPDSCH transmission ending in subframe n, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+12.

## 2.4 Reliability via RLC ARQ re-transmission with fast RLC configuration

Assuming the eNB schedules a conservative MCS for the transmission of the packet, the impact of disabling of HARQ feedback on latency could overall be beneficial compare to cellular. The impact on resource utilization is not expected to be significant since the NTN channel profile has small delay spread. It can be observed from simulations with TDL-D channel profile that a steep BLER curve was obtained. An SNR gain in the order of 1 dB is required to achieve a BLER target of e-3 compare to BLER target of e-1. Though LDPC codes were simulated for NR NTN, we expect similar observations with turbo codes for NB-IoT / eMTC. More details can be found in [2]. Network can ensure very high reliability based on RLC ARQ re-transmission with fast RLC configuration.

Observation 6: High reliability can be ensured by RLC ARQ re-transmission.

Observation 7: Impact on resource utilisation with lower BLER target for first transmission with HARQ feedback disabling can be further studied.

Hence based on observation in the above, we make the following proposal:

Proposal 3: RAN2 can discuss fast RRC configuration for RLC ARQ for IoT NTN.

# 4 Conclusion

In this contribution, the following proposals were made

Observation 1: for IoT NTN, disabling of HARQ feedback is supported in Rel-18 IoT NTN WID.

Observation 2: for NB-IoT, HARQ stalling reduces data rates by approximately 95% and 49% for GEO and LEO respectively without disabling of HARQ feedback.

Observation 3: for NB-IoT, the maximum latency with 2 HARQ processes with up to 4 HARQ transmissions is 2264 ms.

Observation 4: for NB-IoT, disabling of HARQ feedback avoids the issue of HARQ stalling and allows data rates consistent with cellular NB-IoT / eMTC.

Observation 5: In case of disabling of HARQ feedback, it seems natural if eNB transmitter does not wait for ACK/NACK of current DL packet transmission before scheduling a new/retransmission since the UE does not anyway send ACK/NACK for the current NPDSCH/PDSCH for the corresponding HARQ process.

Observation 6: High reliability can be ensured by RLC ARQ re-transmission.

Observation 7: Impact on resource utilisation with lower BLER target for first transmission with HARQ feedback disabling can be further studied.

Proposal 1: Enabling/disabling on HARQ feedback for downlink transmission is configurable per HARQ process via UE specific RRC signalling in IoT NTN.

Proposal 2:  When HARQ feedback for the HARQ process ID is disabled, if a NB-IoT UE receives a NPDSCH transmission ending in subframe n, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+12.

Proposal 3: RAN2 can discuss fast RRC configuration for RLC ARQ for IoT NTN.

# 5 References

- RP-220979, Moderator (MediaTek), Revised WID on IoT NTN enhancements, March 17 - 23, 2022

- R1-1912125, MediaTek, Delay-tolerant re-transmission mechanisms in NR-NTN, RAN1#99, November 2019