3GPP TSG-RAN WG1 Meeting #109-e	Tdoc R1-2203393

e-Meeting, May 9th – 20th, 2022

Agenda Item:	5

Source:	Ericsson

Title:	Discussion of SA2 LS on RAN feedback for Low Latency

Document for:	Discussion, Decision

# 1	Introduction

SA2 has sent an LS to RAN2 (CC RAN1, RAN3) on possible RAN feedback to better support low latency applications [1]. The questions are highly relevant to RAN1 specifications as well, especially the dynamic slot format for TDD.

In this paper, we discuss the questions in the LS from SA2 about RAN feedback for low latency [1].

# 2	Discussion

The LS is about the need to have feedback from RAN to adapt downstream scheduling at the application layer. The purpose is for 5GS to meet low latency (e.g., 2 milliseconds) requirement. The principle is that the application layer may adjust (if possible) the time when the message is generated so that when the message arrives at the base station, there is a minimum buffering delay. The focus is on downlink scheduling.

## 2.1 Problem 1 – TDD cycle

One identified problem is that the arrival time of the packets does not fit well with the TDD cycle used in the network. For example, if a downlink packet arrives at an uplink slot, then it has to wait for the first downlink slot to be transferred and vice versa. This creates additional delay (e.g., more buffering time) to the traffic flows. The additional (maximum) delay is the duration of an UL in one TDD cycle.

For NR, a slot format includes downlink symbols, uplink symbols, and flexible symbols. For a serving cell, tdd-UL-DL-ConfigurationCommon provides the cell specific Uplink/Downlink TDD configuration. As per the Annex, the periodicity of a static UL-DL pattern (per cell) is configurable by the network with the below values

- {0.5ms, 0.625ms, 1ms, 1.25ms, 2ms, 2.5ms, 3ms, 4ms, 5ms, 10ms}

If the tdd-UL-DL-ConfigurationCommon only provides a pattern1 (with periodicity P) without pattern2 (optional), then the periodicity P with possible values above applies. On the other hand, if the tdd-UL-DL-ConfigurationCommon provides both pattern1 (with periodicity P) and pattern2 (with periodicity P2), then the periodicity is (P+ P2), where (P+ P2) divides 20 msec and can take the following values:

- {1 ms, 1.25 ms, 2 ms, 2.5 ms, 4 ms, 5 ms, 10 ms, 20 ms }

Combining the possible periodicity values of P and P+P2, there are a total of 11 possible periodicities that the gNB can configure:

- {0.5 ms, 0.625 ms, 1 ms, 1.25 ms, 2 ms, 2.5 ms, 3 ms, 4 ms, 5 ms, 10 ms, 20 ms}

Respond to SA2 LS that the possible periodicity values of semi-static TDD pattern is {0.5 ms, 0.625 ms, 1 ms, 1.25 ms, 2 ms, 2.5 ms, 3 ms, 4 ms, 5 ms, 10 ms, 20 ms}.

The network can also configure a UE-specific UL/DL TDD configuration as in TDD-UL-DL-ConfigDedicated. The network can configure, within each slot, how many symbols are for downlink (in the beginning of a slot) and how many symbols are for uplink (in the end of a slot) by the IE TDD-UL-DL-SlotConfig.

Together tdd-UL-DL-ConfigurationCommon and TDD-UL-DL-ConfigDedicated provide the semi-static TDD configuration. For symbols indicated as downlink or uplink in the semi-static pattern, the direction of transmission is thus determined. For symbols indicated as flexible in the semi-static pattern, the transmission direction (downlink or uplink) is according to the DCI that schedules the transmission, when the UE is not configured to monitor PDCCH for DCI format 2_0.

Beyond tdd-UL-DL-ConfigurationCommon and TDD-UL-DL-ConfigDedicated, the gNB can also use DCI format 2_6 to additionally provide dynamic TDD uplink/downlink pattern. The dynamic pattern by DCI format 2_6 can further specify some (or all) of the flexible symbols in the semi-static TDD pattern to be downlink or uplink. In TS 38.213, Table 11.1.1-1 provides 55 slot formats for normal cyclic prefix.

From the above description, it can be observed that there are a huge number of possible downlink/uplink TDD patterns in NR. Also, due to the provision of flexible symbols, the final transmission direction is determined by DCI, including DCI format 2_0 and DCIs that schedule downlink or uplink transmission.

For the purpose of supporting low latency applications (e.g., PDB<=5ms), it is expected that the gNB will configure tdd-UL-DL-ConfigurationCommon, TDD-UL-DL-ConfigDedicated, and DCI format 2_6 (SlotFormatIndicator) properly. Additionally, the gNB need to send the content of DCI format 2_6 properly to provide dynamic slot format pattern. The gNB is expected to ensure that the waiting time due to TDD pattern is not an excessive proportion of the PDB for the worst case of packet arrival time.

Respond to SA2 LS that the flexible symbols in the semi-static TDD pattern can be further modified by dynamic control information.

Per the above two proposals, it is clear that RAN has considered the impacts of the TDD pattern on the packet delivery latency, and provides numerous tools (i.e., a very short semi-static TDD pattern, the possibility to define flexible symbols, and the possibility of using DCI to adjust DL/UL symbols). Thus, we propose that:

Respond to SA2 LS that there is no need to adjust BAT in TDD scenario.

## 2.2 Problem 2 – FDD

For the FDD, the network and the UE can operate in full duplex mode and so there is no waiting time for an UL transmission or DL transmissions as in TDD. The gNB can, for example, activate SPS/CG just after the burst arrival time information provided by the core network. Moreover, the minimum periodicity of these resources is 1 slot for SPS, and 2 symbol for CG.

This means that there is no issue at all for FDD.

Respond to SA2 LS that there is no need to adjust BAT in FDD scenario.

## 2.3 Problem 3 – multiple flows

The last problem is that the arrival times of multiple flows in the same RAN node are not coordinated. If the arrival time can be coordinated, then it is possible to adjust them so that the transmission of these multiple flows does not overlap. In other words, from the RAN node point of view, the best outcome would be that at each time instance, only one flow is served. In the worst case (without coordination), the RAN node may need to serve multiple flows at the same time (and thus leading to reject the admission as indicated in the LS).

If the end-to-end latency of a flow is the same as or larger than the transfer interval of that flow, then it is expected that RAN node may have to schedule transmissions for that flow all the time. In other words, it is not useful at all to adjust the BAT.

If the end-to-end latency is smaller than the transfer interval, then the RAN node may drop a packet if the end-to-end latency (i.e., PDB) has passed. The RAN node may schedule other flows between the end of the PDB of the previous flow and the arrival of the next message of the previous flow. Only in this case, it may be useful (but only theoretically) to adjust the BAT so that the “active” transmission time of different flows do not overlap. Even if possible, the theorical gain is limited, since the end-to-end latency would not be much shorter than the transfer interval, assuming the low latency of 2 msecs use case in the SA2 LS. Additionally, whether RAN to discard the packet after PDB is up-to RAN implementation. Lastly, as far as we know, there is no such requirement of a flow with e2e latency smaller than the transfer interval for low latency of 2 msecs (as indicated in the SA2 LS) in the TS 22.104.

All in all, the benefits to adjust BAT to allow more admission of QoS flows are not clear.

Respond to SA2 LS that RAN does not see any use case in which the adjustment of BAT allows more admission of QoS flows.

# 3. Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Respond to SA2 LS that the possible periodicity values of semi-static TDD pattern is {0.5 ms, 0.625 ms, 1 ms, 1.25 ms, 2 ms, 2.5 ms, 3 ms, 4 ms, 5 ms, 10 ms, 20 ms}.

Proposal 2	Respond to SA2 LS that the flexible symbols in the semi-static TDD pattern can be further modified by dynamic control information.

Proposal 3	Respond to SA2 LS that there is no need to adjust BAT in TDD scenario.

Proposal 4	Respond to SA2 LS that there is no need to adjust BAT in FDD scenario.

Proposal 5	Respond to SA2 LS that RAN does not see any use case in which the adjustment of BAT allows more admission of QoS flows.

# 4. References

S2-2201767, LS on RAN feedback for low latency, SA2, SA2#149e

# 5. Annex – RRC spec for TDD

#### –	TDD-UL-DL-ConfigCommon

The IE TDD-UL-DL-ConfigCommon determines the cell specific Uplink/Downlink TDD configuration.

TDD-UL-DL-ConfigCommon information element

-- ASN1START

-- TAG-TDD-UL-DL-CONFIGCOMMON-START

TDD-UL-DL-ConfigCommon ::=          SEQUENCE {

referenceSubcarrierSpacing          SubcarrierSpacing,

pattern1                            TDD-UL-DL-Pattern,

pattern2                            TDD-UL-DL-Pattern                                                       OPTIONAL, -- Need R

...

}

TDD-UL-DL-Pattern ::=               SEQUENCE {

dl-UL-TransmissionPeriodicity       ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10},

nrofDownlinkSlots                   INTEGER (0..maxNrofSlots),

nrofDownlinkSymbols                 INTEGER (0..maxNrofSymbols-1),

nrofUplinkSlots                     INTEGER (0..maxNrofSlots),

nrofUplinkSymbols                   INTEGER (0..maxNrofSymbols-1),

...,

[[

dl-UL-TransmissionPeriodicity-v1530     ENUMERATED {ms3, ms4}                                               OPTIONAL -- Need R

]]

}

-- TAG-TDD-UL-DL-CONFIGCOMMON-STOP

-- ASN1STOP

TDD-UL-DL-ConfigCommon field descriptions

referenceSubcarrierSpacing Reference SCS used to determine the time domain boundaries in the UL-DL pattern which must be common across all subcarrier specific carriers, i.e., independent of the actual subcarrier spacing using for data transmission. Only the following values are applicable depending on the used frequency: FR1: 15, 30, or 60 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz The network configures a not larger than any SCS of configured BWPs for the serving cell. The network or SL-PreconfigGeneral configures a not larger than the SCS of (pre-)configured SL BWP.See TS 38.213 [13], clause 11.1.

TDD-UL-DL-Pattern field descriptions

dl-UL-TransmissionPeriodicity Periodicity of the DL-UL pattern, see TS 38.213 [13], clause 11.1. If the dl-UL-TransmissionPeriodicity-v1530 is signalled, UE shall ignore the dl-UL-TransmissionPeriodicity (without suffix).

nrofDownlinkSlots Number of consecutive full DL slots at the beginning of each DL-UL pattern, see TS 38.213 [13], clause 11.1. In this release, the maximum value for this field is 80.

nrofDownlinkSymbols Number of consecutive DL symbols in the beginning of the slot following the last full DL slot (as derived from nrofDownlinkSlots). The value 0 indicates that there is no partial-downlink slot. (see TS 38.213 [13], clause 11.1).

nrofUplinkSlots Number of consecutive full UL slots at the end of each DL-UL pattern, see TS 38.213 [13], clause 11.1. In this release, the maximum value for this field is 80.

nrofUplinkSymbols Number of consecutive UL symbols in the end of the slot preceding the first full UL slot (as derived from nrofUplinkSlots). The value 0 indicates that there is no partial-uplink slot. (see TS 38.213 [13], clause 11.1).

#### –	TDD-UL-DL-ConfigDedicated

The IE TDD-UL-DL-ConfigDedicated determines the UE-specific Uplink/Downlink TDD configuration.

TDD-UL-DL-ConfigDedicated information element

-- ASN1START

-- TAG-TDD-UL-DL-CONFIGDEDICATED-START

TDD-UL-DL-ConfigDedicated ::=       SEQUENCE {

slotSpecificConfigurationsToAddModList      SEQUENCE (SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotConfig       OPTIONAL, -- Need N

slotSpecificConfigurationsToReleaseList     SEQUENCE (SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotIndex        OPTIONAL, -- Need N

...

}

TDD-UL-DL-ConfigDedicated-IAB-MT-r16::=         SEQUENCE {

slotSpecificConfigurationsToAddModList-IAB-MT-r16  SEQUENCE (SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotConfig-IAB-MT-r16      OPTIONAL, -- Need N

slotSpecificConfigurationsToReleaseList-IAB-MT-r16 SEQUENCE (SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotIndex                  OPTIONAL, -- Need N

...

}

TDD-UL-DL-SlotConfig ::=            SEQUENCE {

slotIndex                           TDD-UL-DL-SlotIndex,

symbols                             CHOICE {

allDownlink                         NULL,

allUplink                           NULL,

explicit                            SEQUENCE {

nrofDownlinkSymbols                 INTEGER (1..maxNrofSymbols-1)                                   OPTIONAL, -- Need S

nrofUplinkSymbols                   INTEGER (1..maxNrofSymbols-1)                                   OPTIONAL  -- Need S

}

}

}

TDD-UL-DL-SlotConfig-IAB-MT-r16::=    SEQUENCE {

slotIndex-r16                           TDD-UL-DL-SlotIndex,

symbols-IAB-MT-r16                      CHOICE {

allDownlink-r16                         NULL,

allUplink-r16                           NULL,

explicit-r16                            SEQUENCE {

nrofDownlinkSymbols-r16                 INTEGER (1..maxNrofSymbols-1)                               OPTIONAL, -- Need S

nrofUplinkSymbols-r16                   INTEGER (1..maxNrofSymbols-1)                               OPTIONAL  -- Need S

},

explicit-IAB-MT-r16                     SEQUENCE {

nrofDownlinkSymbols-r16                 INTEGER (1..maxNrofSymbols-1)                               OPTIONAL, -- Need S

nrofUplinkSymbols-r16                   INTEGER (1..maxNrofSymbols-1)                               OPTIONAL  -- Need S

}

}

}

TDD-UL-DL-SlotIndex ::=             INTEGER (0..maxNrofSlots-1)

-- TAG-TDD-UL-DL-CONFIGDEDICATED-STOP

-- ASN1STOP

TDD-UL-DL-ConfigDedicated field descriptions

slotSpecificConfigurationsToAddModList The slotSpecificConfigurationToAddModList allows overriding UL/DL allocations provided in tdd-UL-DL-configurationCommon, see TS 38.213 [13], clause 11.1.

TDD-UL-DL-ConfigDedicated-IAB-MT field descriptions

slotSpecificConfigurationsToAddModList-IAB-MT The slotSpecificConfigurationToAddModList-IAB-MT allows overriding UL/DL allocations provided in tdd-UL-DL-configurationCommon with a limitation that effectively only flexible symbols can be overwritten in Rel-16.

slotSpecificConfigurationsToReleaseList-IAB-MT The slotSpecificConfigurationsToReleaseList-IAB-MT allows release of a set of slot configuration previously add with slotSpecificConfigurationToAddModList-IAB-MT.

TDD-UL-DL-SlotConfig field descriptions

nrofDownlinkSymbols Number of consecutive DL symbols in the beginning of the slot identified by slotIndex. If the field is absent the UE assumes that there are no leading DL symbols. (see TS 38.213 [13], clause 11.1).

nrofUplinkSymbols Number of consecutive UL symbols in the end of the slot identified by slotIndex. If the field is absent the UE assumes that there are no trailing UL symbols. (see TS 38.213 [13], clause 11.1).

slotIndex Identifies a slot within a slot configuration period given in tdd-UL-DL-configurationCommon, see TS 38.213 [13], clause 11.1.

symbols The direction (downlink or uplink) for the symbols in this slot. Value allDownlink indicates that all symbols in this slot are used for downlink; value allUplink indicates that all symbols in this slot are used for uplink; value explicit indicates explicitly how many symbols in the beginning and end of this slot are allocated to downlink and uplink, respectively.

TDD-UL-DL-SlotConfig-IAB-MT field descriptions

symbols-IAB-MT The symbols-IAB-MT is used to configure an IAB-MT with the SlotConfig applicable for one serving cell. Value allDownlink indicates that all symbols in this slot are used for downlink; value allUplink indicates that all symbols in this slot are used for uplink; value explicit indicates explicitly how many symbols in the beginning and end of this slot are allocated to downlink and uplink, respectively; value explicit-IAB-MT indicates explicitly how many symbols in the beginning and end of this slot are allocated to uplink and downlink, respectively.

maxNrofSymbols-1                        INTEGER ::= 13      -- Maximum index identifying a symbol within a slot (14 symbols, indexed from 0..13)

maxNrofSlots                             INTEGER ::= 320     -- Maximum number of slots in a 10 ms period

maxNrofSlots-1                          INTEGER ::= 319     -- Maximum number of slots in a 10 ms period minus 1

# 6. Annex – TS 38.213 V17.1.0 (2022-03)

## 11.1	Slot configuration

- …

A slot configuration period and a number of downlink symbols, uplink symbols, and flexible symbols in each slot of the slot configuration period are determined from tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated and are common to each configured BWP.

A UE considers symbols in a slot indicated as downlink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated to be available for receptions and considers symbols in a slot indicated as uplink by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated to be available for transmissions.

If a UE is not configured to monitor PDCCH for DCI format 2_0, for a set of symbols of a slot that are indicated as flexible by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are not provided to the UE

-	the UE receives PDSCH or CSI-RS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format

-	the UE transmits PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format, a RAR UL grant, fallbackRAR UL grant, or successRAR

- …

### 11.1.1	UE procedure for determining slot format

- …

If a UE is configured by higher layers with parameter SlotFormatIndicator, the UE is provided an SFI-RNTI by sfi-RNTI and with a payload size of DCI format 2_0 by dci-PayloadSize.

- …

A SFI-index field value in a DCI format 2_0 indicates to a UE a slot format for each slot in a number of slots for each DL BWP or each UL BWP starting from a slot where the UE detects the DCI format 2_0. The number of slots is equal to or larger than a PDCCH monitoring periodicity for DCI format 2_0. The SFI-index field includes  bits where maxSFIindex is the maximum value of the values provided by corresponding slotFormatCombinationId. A slot format is identified by a corresponding format index as provided in Table 11.1.1-1 where 'D' denotes a downlink symbol, 'U' denotes an uplink symbol, and 'F' denotes a flexible symbol.

- …

Table 11.1.1-1: Slot formats for normal cyclic prefix

| Format | Symbol number in a slot |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| 0 | D | D | D | D | D | D | D | D | D | D | D | D | D | D |
| 1 | U | U | U | U | U | U | U | U | U | U | U | U | U | U |
| 2 | F | F | F | F | F | F | F | F | F | F | F | F | F | F |
| 3 | D | D | D | D | D | D | D | D | D | D | D | D | D | F |
| 4 | D | D | D | D | D | D | D | D | D | D | D | D | F | F |
| 5 | D | D | D | D | D | D | D | D | D | D | D | F | F | F |
| 6 | D | D | D | D | D | D | D | D | D | D | F | F | F | F |
| 7 | D | D | D | D | D | D | D | D | D | F | F | F | F | F |
| 8 | F | F | F | F | F | F | F | F | F | F | F | F | F | U |
| 9 | F | F | F | F | F | F | F | F | F | F | F | F | U | U |
| 10 | F | U | U | U | U | U | U | U | U | U | U | U | U | U |
| 11 | F | F | U | U | U | U | U | U | U | U | U | U | U | U |
| 12 | F | F | F | U | U | U | U | U | U | U | U | U | U | U |
| 13 | F | F | F | F | U | U | U | U | U | U | U | U | U | U |
| 14 | F | F | F | F | F | U | U | U | U | U | U | U | U | U |
| 15 | F | F | F | F | F | F | U | U | U | U | U | U | U | U |
| 16 | D | F | F | F | F | F | F | F | F | F | F | F | F | F |
| 17 | D | D | F | F | F | F | F | F | F | F | F | F | F | F |
| 18 | D | D | D | F | F | F | F | F | F | F | F | F | F | F |
| 19 | D | F | F | F | F | F | F | F | F | F | F | F | F | U |
| 20 | D | D | F | F | F | F | F | F | F | F | F | F | F | U |
| 21 | D | D | D | F | F | F | F | F | F | F | F | F | F | U |
| 22 | D | F | F | F | F | F | F | F | F | F | F | F | U | U |
| 23 | D | D | F | F | F | F | F | F | F | F | F | F | U | U |
| 24 | D | D | D | F | F | F | F | F | F | F | F | F | U | U |
| 25 | D | F | F | F | F | F | F | F | F | F | F | U | U | U |
| 26 | D | D | F | F | F | F | F | F | F | F | F | U | U | U |
| 27 | D | D | D | F | F | F | F | F | F | F | F | U | U | U |
| 28 | D | D | D | D | D | D | D | D | D | D | D | D | F | U |
| 29 | D | D | D | D | D | D | D | D | D | D | D | F | F | U |
| 30 | D | D | D | D | D | D | D | D | D | D | F | F | F | U |
| 31 | D | D | D | D | D | D | D | D | D | D | D | F | U | U |
| 32 | D | D | D | D | D | D | D | D | D | D | F | F | U | U |
| 33 | D | D | D | D | D | D | D | D | D | F | F | F | U | U |
| 34 | D | F | U | U | U | U | U | U | U | U | U | U | U | U |
| 35 | D | D | F | U | U | U | U | U | U | U | U | U | U | U |
| 36 | D | D | D | F | U | U | U | U | U | U | U | U | U | U |
| 37 | D | F | F | U | U | U | U | U | U | U | U | U | U | U |
| 38 | D | D | F | F | U | U | U | U | U | U | U | U | U | U |
| 39 | D | D | D | F | F | U | U | U | U | U | U | U | U | U |
| 40 | D | F | F | F | U | U | U | U | U | U | U | U | U | U |
| 41 | D | D | F | F | F | U | U | U | U | U | U | U | U | U |
| 42 | D | D | D | F | F | F | U | U | U | U | U | U | U | U |
| 43 | D | D | D | D | D | D | D | D | D | F | F | F | F | U |
| 44 | D | D | D | D | D | D | F | F | F | F | F | F | U | U |
| 45 | D | D | D | D | D | D | F | F | U | U | U | U | U | U |
| 46 | D | D | D | D | D | F | U | D | D | D | D | D | F | U |
| 47 | D | D | F | U | U | U | U | D | D | F | U | U | U | U |
| 48 | D | F | U | U | U | U | U | D | F | U | U | U | U | U |
| 49 | D | D | D | D | F | F | U | D | D | D | D | F | F | U |
| 50 | D | D | F | F | U | U | U | D | D | F | F | U | U | U |
| 51 | D | F | F | U | U | U | U | D | F | F | U | U | U | U |
| 52 | D | F | F | F | F | F | U | D | F | F | F | F | F | U |
| 53 | D | D | F | F | F | F | U | D | D | F | F | F | F | U |
| 54 | F | F | F | F | F | F | F | D | D | D | D | D | D | D |
| 55 | D | D | F | F | F | U | U | U | D | D | D | D | D | D |
| 56 – 254 | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 255 | UE determines the slot format for the slot based on tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, if any, on detected DCI formats |  |  |  |  |  |  |  |  |  |  |  |  |  |

# 7. Annex – TS 22.104

Table 5.2-1: Periodic deterministic communication service performance requirements

| Characteristic parameter |  |  |  | Influence quantity |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Communication service availability: target value (note 1) | Communication service reliability: mean time between failures | End-to-end latency: maximum (note 2) (note 12a) | Service bit rate: user experienced data rate (note 12a) | Message size [byte] (note 12a) | Transfer interval: target value (note 12a) | Survival time (note 12a) | UE speed (note 13) | # of UEs | Service area (note 3) | Remarks |
| 99,9999 % | – | < 5 ms | 1 kbit/s (steady state) 1,5 Mbit/s (fault case) | < 1500 | < 60 s (steady state) ≥ 1 ms (fault case) | transfer interval | stationary | 20 | 30 km x 20 km | Electrical Distribution – Distributed automated switching for isolation and service restoration (A.4.4); (note 5) |
| 99,9999 % to 99,999999 % | ~ 10 years | < 0,5 * transfer interval | 2,5 Mbit/s | 250 500 with localisation information | > 5 ms >2,5 ms >1,7 ms (note 10) | 0 transfer interval 2 * transfer interval (note 10) | ≤6 km/h (linear movement) | 2 to 8 | 10 m x 10 m x 5 m; 50 m x 5 m x 5 m (note 11 | Cooperative carrying – fragile work pieces; (ProSe communication) |
| 99,9999 % to 99,999999 % | ~ 10 years | < 0,5 * transfer interval | 2,5 Mbit/s | 250 500 with localisation information | > 5 ms >2,5 ms >1,7 ms (note 10) | 0 transfer interval 2 * transfer interval (note 10) | ≤12 km/h (linear movement) | 2 to 8 | 10 m x 10 m x 5 m; 50 m x 5 m x 5 m (note 11) | Cooperative carrying – elastic work pieces; (ProSe communication) |
| NOTE 1: One or more retransmissions of network layer packets may take place in order to satisfy the communication service availability requirement. NOTE 2: Unless otherwise specified, all communication includes 1 wireless link (UE to network node or network node to UE) rather than two wireless links (UE to UE). NOTE 3: Length x width (x height). NOTE 4: (void) NOTE 5: Communication includes two wireless links (UE to UE). NOTE 6: This covers different transfer intervals for different similar use cases with target values of 1 ms, 1 ms to 10 ms, and 10 ms to 50 ms. NOTE 7: The transfer interval deviates around its target value by < ± 25 %. NOTE 8: The transfer interval deviates around its target value by < ± 5 %. NOTE 9: Communication may include two wireless links (UE to UE). NOTE 10: The first value is the application requirement, the other values are the requirement with multiple transmission of the same information (two or tree times respectively). NOTE 11: Service Area for direct communication between UEs. The group of UEs with direct communication might move throughout the whole factory site (up to several km²). NOTE 12: Maximum straight-line distance between UEs. NOTE 12a: It applies to both UL and DL unless stated otherwise. NOTE 13: It applies to both linear movement and rotation unless stated otherwise. NOTE 14: The mobile operation panel is connected wirelessly to the 5G system. If the mobile robot/production line is also connected wirelessly to the 5G system, the communication includes two wireless links. |  |  |  |  |  |  |  |  |  |  |

Table 5.2-2: Communication service performance requirements for industrial wireless sensors

| Characteristic parameter |  |  |  |  |  | Influence quantity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Communication service availability: target value | Communication service reliability: mean time between failure | End-to-end latency (note 6) | Transfer interval (note 1) (note 7) | Service bit rate: user experienced data rate (note 2) (note 7) | Battery lifetime [year] (note 3) | Message Size [byte] (note 7) | Survival time (note 7) | UE speed | UE density [UE / m²] | Range [m] (note 4) | Remarks |
| 99,99 % | ≥ 1 week | < 100 ms | 100 ms – 60 s | ≤ 1 Mbit/s | ≥5 | 20 (note 5) | 3 x transfer interval | Stationary | Up to 1 | <500 | Process monitoring, e.g. temperature sensor (A.2.3.2) |
| 99,99 % | ≥ 1 week | < 100 ms | ≤ 1 s | ≤ 200 kbit/s | ≥5 | 25 k | 3 x transfer interval | Stationary | Up to 0,05 | <500 | Asset monitoring, e.g. vibration sensor (A.2.3.2) |
| 99,99 % | ≥ 1 week | < 100 ms | ≤ 1 s | ≤ 2 Mbit/s | ≥5 | 250 k | 3 x transfer interval | Stationary | Up to 0,05 | <500 | Asset monitoring, e.g. thermal camera (A.2.3.2) |
| NOTE 1: The transfer interval deviates around its target value by < ± 25 %. NOTE 2: The traffic is predominatly mobile originated. NOTE 3: Industrial sensors can use a wide variety of batteries depending on theuse case, but in general they are highly constrained in terms of battery size. NOTE 4: Distance between the gNB and the UE. NOTE 5: The application-level messages in this use case are typically transferred over Ethernet. For small messages, the minimum Ethernet frame size of 64 bytes applies and dictates the minimum size of the PDU sent over the air interface. NOTE 6: It applies to both UL and DL unless stated otherwise. NOTE 7: It applies to UL. |  |  |  |  |  |  |  |  |  |  |  |