3GPP TSG RAN WG1 #109-e	Tdoc R1-2203396

e-Meeting, May 9th – 20th, 2022

Agenda Item:	7.2.5

Source:	Ericsson

Title:	Resolving Collision of HARQ-ACK with Repetition

Document for:	Discussion, Decision

# 1	Introduction

In RAN1#108-e, the issue of collision handling among PUCCH with repetition was discussed. While a conclusion was reached, the issue of two non-overlapping HARQ-ACK was left for further clarification.

In this contribution, we discuss the problem and provide the possible solutions.

# 2	Discussion

In RAN1#108-e, the following conclusion was reached:

Conclusion For collision handling among PUCCH with repetitions with the same UCI type priority, - the starting slot of PUCCH is the PUCCH slot before deferral, i.e. the starting slot of PUCCH is - the slot determined as described in clause 9.2.3 for HARQ-ACK reporting. - the slot determined as described in clause 9.2.4 for SR reporting - the slot determined as described in clause 5.2.1.4 of TS 38.214 for CSI reporting. - if a UE would transmit at least two PUCCHs with UCI of same UCI type priority that start in different slots and the earliest PUCCH is with repetitions, the UE drops transmission of a later PUCCH in every slot where the transmission of the later PUCCH would overlap with a repetition of the earliest PUCCH transmission. - Note: The related handling of at least two non-overlapping PUCCHs with HARQ-ACK within a PUCCH slot requires further clarification.

First, the conclusion should be clarified that the “PUCCH slot” refers to a full slot if sub-slot is not configured, and refers to a sub-slot if sub-slot is configured (i.e., a UE is provided subslotLengthForPUCCH). This is because, section 9 sentence below governs over the section 9.2.6, which describes the PUCCH repetition procedure.

TS 38.213 V17.1.0 section 9: In the remaining of this clause, if a UE is provided subslotLengthForPUCCH, a slot for an associated PUCCH resource of a PUCCH transmission with HARQ-ACK information includes a number of symbols indicated by subslotLengthForPUCCH, unless stated otherwise.

The “PUCCH slot” refers to a full slot if subslotLengthForPUCCH is not configured, and refers to a sub-slot if subslotLengthForPUCCH is configured.

Then, when the UE is not provided ackNackFeedbackMode = separate, one PUCCH slot can only carry one PUCCH for HARQ-ACK since Rel-15, see below. Therefore, if there are two non-overlapping PUCCHs carrying HARQ-ACK in a PUCCH slot, this is not expected by UE.

TS 38.213 V17.1.0 section 9.2.3: A UE does not expect to transmit more than one PUCCH with HARQ-ACK information in a slot per priority index, if the UE is not provided ackNackFeedbackMode = separate.

With the above understanding, there is no difference if the two PUCCHs in the same PUCCH slot occupying the same symbol(s) or not. As long as the two PUCCHs are mapped to the same PUCCH slot, then this is considered a collision. Procedures are to be applied to resolve the collision so that there is at most one PUCCH carrying HARQ-ACK in a PUCCH slot, when PUCCH repetition is configured.

The two PUCCHs are considered overlapping, and collision handling is to be applied, when two PUCCHs carrying HARQ-ACK are mapped to the same PUCCH slot, even if the two PUCCHs occupy different symbols.

(A). Collision involving HARQ-ACK of dynamically scheduled PDSCH

There are two sub-cases of collision involving HARQ-ACK of dynamically scheduled PDSCH:

- HARQ-ACK of dynamically scheduled PDSCH#1 vs. HARQ-ACK of dynamically scheduled PDSCH#2;

- HARQ-ACK of dynamically scheduled PDSCH#1 vs. HARQ-ACK of SPS PDSCH#2;

Since the gNB scheduler can dynamically control the transmission of HARQ-ACK of dynamically scheduled PDSCH, the collision can be avoided in principle. If collision still occurs, then the gNB already expects that some HARQ-ACK will be dropped.

For both sub-cases of (A), the conclusion of RAN1#108e (see above) applies, i.e.,

- “the UE drops transmission of a later PUCCH in every slot where the transmission of the later PUCCH would overlap with a repetition of the earliest PUCCH transmission.”

The conclusion from RAN1#108e applies to the PUCCH collision when one of the PUCCH carries HARQ-ACK of dynamically scheduled PDSCH.

(B). Collision involving HARQ-ACK of two or more SPS PDSCHs

As pointed out in RAN1#108-e discussion, Rel-16 introduces SPS configuration with reduced periodicity (min periodicity is reduced from 10ms to 1 slot). Furthermore, multiple SPS configurations can be activated simultaneously. Different SPS configurations can have different configured parameters, including different periodicities to support traffic stream of different characteristics. When repetition is configured for a PUCCH carrying HARQ-ACK of SPS PDSCH, collision may occur.  As observed above, collision occurs when two PUCCHs carrying HARQ-ACK of SPS PDSCH are mapped to the same PUCCH slot, even if the two PUCCHs do not use the same symbol(s).

Since SPS PDSCH and corresponding HARQ-ACK recur periodically, it is not possible to define which PUCCH is earlier, which PUCCH is later. This is illustrated with an example below. Thus the procedure in the conclusion of RAN1#108-e cannot be applied directly.

Figure 1. Example of problematic configuration of HARQ-ACK repetition for SPS PDSCH. Periodicity of the SPS configuration = 1 slot. Number of repetitions for the HARQ-ACK PUCCH resources = 2.

For HARQ-ACK for SPS PDSCH a given SPS configuration index, it is reasonable to expect that the gNB should configure properly so that consecutive HARQ-ACKs do not overlap. This is illustrated with an example below.

Figure 2. Example of acceptable configuration of HARQ-ACK repetition for SPS PDSCH. Periodicity of the SPS configuration = 2 slot. Number of repetitions for the HARQ-ACK PUCCH resources = 2.

Thus, it is proposed that the UE expects the gNB to configure properly for a given SPS configuration to avoid PUCCH collision.

For a given SPS configuration, the UE does not expect PUCCHs carrying HARQ-ACK of two adjacent SPS PDSCHs to overlap.

For HARQ-ACK for SPS PDSCH of two different SPS configuration indices, HARQ-ACK collision may occur as illustrated in the example below.

Figure 3. Example of problematic configuration of HARQ-ACK repetition for PDSCH of two SPS configurations. Periodicity of the both SPS configurations = 2 slot. Number of repetitions for the HARQ-ACK PUCCH resources = 2 for both SPS configurations.

For HARQ-ACK of different SPS configuration indices, there are two alternatives to resolve the PUCCH collision:

- PUCCH collision is not expected. That is, gNB configure the two or more SPS configurations properly, so that PUCCH collision do not occur between HARQ-ACK of two different SPS configurations.

- PUCCH collision is expected. Then a procedure is applied so that the UE resolves the PUCCH collision. One approach is, to resolve collision of PUCCH with repetitions carrying HARQ-ACK of SPS PDSCH, the lowest SPS configuration index of HARQ-ACK carried by each PUCCH is compared. PUCCH carrying HARQ-ACK for smaller SPS configuration index is transmitted, while PUCCH carrying HARQ-ACK for larger SPS configuration index is dropped.

Both Alternative 1 and Alternative 2 can work. Alternative 1 is cleaner but puts constraints on SPS configuration. Alternative 2 has more specification impact, requires extra UE processing, but allows better flexibility in SPS configuration. Considering the pros and cons, Alternative 1 is preferred in our view.

The bottom line is, RAN1 need to discuss and decide on an alternative to resolve the PUCCH collision.

For multiple SPS configurations, RAN1 decide on an approach to resolve the PUCCH collision carrying HARQ-ACK of different SPS configuration indices.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The “PUCCH slot” refers to a full slot if subslotLengthForPUCCH is not configured, and refers to a sub-slot if subslotLengthForPUCCH is configured.

Observation 2	The two PUCCHs are considered overlapping, and collision handling is to be applied, when two PUCCHs carrying HARQ-ACK are mapped to the same PUCCH slot, even if the two PUCCHs occupy different symbols.

Based on the discussion above, the following proposals are made to resolve collision for repeated PUCCH carrying HARQ-ACK:

Proposal 1	The conclusion from RAN1#108e applies to the PUCCH collision when one of the PUCCH carries HARQ-ACK of dynamically scheduled PDSCH.

Proposal 2	For a given SPS configuration, the UE does not expect PUCCHs carrying HARQ-ACK of two adjacent SPS PDSCHs to overlap.

Proposal 3	For multiple SPS configurations, RAN1 decide on an approach to resolve the PUCCH collision carrying HARQ-ACK of different SPS configuration indices.

# References

3GPP TS 38.213 V16.8.0 (2022-03), “Physical layer procedures for control (Release 16)”.

Chair notes, RAN1#108-e.