3GPP TSG-RAN WG1 Meeting #109-e	Tdoc R1-2203398

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.3.2

Source:	Ericsson

Title:	Remaining Issues of Intra-UE Multiplexing/Prioritization for IIoT/URLLC

Document for:	Discussion, Decision

# 1	Introduction

In RAN#108-e, significant progress has been made for the topic of intra-UE multiplexing/prioritization for IIoT/URLLC. Only a few minor issues are not resolved yet.

In the following, we discuss the remaining issues of intra-UE multiplexing and prioritizations. Resolutions are presented for each open issue.

# 2	Discussion

## 2.1	Multiplexing UCI of different priorities onto PUCCH

In RAN1#107bis and RAN1#108-e, the following agreements were made for resolving the collision of HP PUCCH carrying {SR, HARQ-ACK} and a LP PUCCH:

Agreement (RAN1#107bis) When a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 2/3/4 overlaps with a PUCCH carrying LP HARQ-ACK, information bits for K HP SRs are appended to HP HARQ-ACK bits, and treat them as HP UCI, where K (K≥1) PUCCHs semi-statically configured for K HP SRs overlap with the original PUCCH carrying the HP HARQ-ACK. - The number of HP UCI bits is , same as Rel-15; - FFS: PF0, PF1 - Reuse other procedures for multiplexing of LP HARQ-ACK and HP HARQ-ACK on PUCCH resource with PF 2/3/4, i.e. separate coding, PRB determination, rate matching and power control. - If the HP HARQ-ACK is a dynamic HARQ-ACK, a PUCCH resource indicated by PRI is used for multiplexing. - If the HP HARQ-ACK is a SPS HARQ-ACK, a PUCCH resource determined from the PUCCH resource(s) provided by sps-PUCCH-AN-List is used for multiplexing. Agreement (RAN1#108) To resolve overlapping between a HP PUCCH carrying SR and HP HARQ-ACK with PUCCH format 2/3/4 and a LP PUCCH carrying HARQ-ACK and CSI/SR, only LP HARQ-ACK is multiplexed with HP UCI (i.e., SR and HARQ-ACK) and LP CSI/SR are dropped. - FFS HP PUCCH with PUCCH format 0/1

The overlapping scenario where HP PUCCH carrying SR and HP HARQ-ACK with PUCCH format 0/1 still needs to be resolved.

For the case of PUCCH format 0/1 carrying {SR, HARQ-ACK}, while multiple SR resources may overlap with the HARQ-ACK resources, the UE ensures that only one SR is triggered at most. Thus only one SR bit is to be considered in the multiplexing procedure. It’s worth noting that for this case, TS 38.213 section 9.2.5.1 uses phrases like “If a UE would transmit a PUCCH with positive SR”, “If the UE would transmit negative SR”, “If a UE would transmit SR”. That is, the specification assumes a single SR bit.

When a PUCCH carrying HP SR and HP HARQ-ACK with PF 0/1, the number of SR bits is K=1.

Since the number of HARQ bits transmitted on PF 0/1 is at most 2 bits, the 1-bit SR can be appended to the 1 or 2-bit HARQ-ACK, and the result treated as if the HP PUCCH carried a 2 or 3-bit HARQ-ACK.

Then in the step of resolving the collision with LP PUCCH carrying HARQ-ACK, the procedure agreed for HP PUCCH with PF2/3/4 can be reused. That is, the overall procedure can be describe as below.

When a HP PUCCH carrying SR and HARQ-ACK with PF0/1 overlaps with a LP PUCCH carrying HARQ-ACK,

- 1-bit HP SR is appended to 1 or 2-bit HP HARQ-ACK, and treated together as a 2 or 3-bit HP HARQ-ACK;

- Reuse other procedures for multiplexing LP HARQ-ACK and HP HARQ-ACK onto a PUCCH resource with PF 2/3/4, i.e. separate coding, PRB determination, rate matching and power control.

- If the HP HARQ-ACK is a dynamic HARQ-ACK, a PUCCH resource indicated by PRI is used for multiplexing.

- If the HP HARQ-ACK is an SPS HARQ-ACK, a PUCCH resource determined from the PUCCH resource(s) provided by sps-PUCCH-AN-List is used for multiplexing.

For the related case where the HP PUCCH carrying SR and HP HARQ-ACK with PF0/1 and a LP PUCCH carrying HARQ-ACK and CSI/SR, the same principle as PF2/3/4 can be reused, i.e., only LP HARQ-ACK is multiplexed with HP UCI (i.e., SR and HARQ-ACK) and LP CSI/SR are dropped.

When a HP PUCCH carrying SR and HARQ-ACK with PF0/1 overlaps with a LP PUCCH carrying HARQ-ACK and CSI/SR, only LP HARQ-ACK is multiplexed with HP UCI (i.e., SR and HARQ-ACK) and LP CSI/SR are dropped.

## 2.2	Multiplexing HARQ-ACK onto a CG-PUSCH with CG-UCI

The following agreements have been reached which are related to multiplexing HARQ-ACK onto a CG-PUSCH with CG-UCI.

Agreement When performing Intra-UE multiplexing procedure, if a PUCCH with HARQ-ACK overlaps with a CG-PUSCH and the cg-RetransmissionTimer is configured: - If the HARQ-ACK and the CG-PUSCH have the same priority and the CG-PUSCH is selected for HARQ-ACK multiplexing: - If cg-UCI-Multiplexing is enabled for that CG-PUSCH, HARQ-ACK would be multiplexed in CG-PUSCH. - Otherwise, CG-PUSCH would be dropped. - If the HARQ-ACK and the CG-PUSCH have different priority and the CG-PUSCH is selected for HARQ-ACK multiplexing: - If multiplexing HARQ-ACK on the CG-PUSCH with different priroity is not indicated, - The LP channel between PUCCH or CG-PUSCH would be dropped as in Rel-16. - If multiplexing HARQ-ACK on the CG-PUSCH with different priroity is indicated, - If cg-UCI-Multiplexing is enabled for that CG-PUSCH, HARQ-ACK would be multiplexed in CG-PUSCH. - Otherwise, the LP channel would be dropped. Agreement If LP HARQ-ACK without HP HARQ-ACK would be transmitted on HP PUSCH, down-select from the options: - Option 2: UE follows the same behaviour as that in case of PUSCH with HP HARQ-ACK assuming two bits - FFS for CG-UCI PUSCH.

The cases of a PUCCH carrying HARQ-ACK overlapping with a CG-PUSCH with CG-UCI are not resolved, when the PUCCH and PUSCH have different priorities. Specifically, the scenario is that the CG-PUSCH is selected for multiplexing with the PUCCH carrying HARQ-ACK of different priorities, when higher layer parameters cg-UCI-Multiplexing and UCI-MuxWithDifferentPriority are configured.

In the existing multiplexing procedure in TS 38.212, when there are both CG-UCI and HARQ-ACK, HARQ-ACK is appended to the CG-UCI to form a single UCI sequence, before channel coding and rate matching are applied. Thus, it can be understood that CG-UCI is treated the same as HARQ-ACK bits from the perspective of UCI processing (i.e., channel coding, rate matching, and mapping to RE of PUSCH).

When performing inter-priority multiplexing with CG-UCI, the following procedure can be applied to have a unified solution.

When a HP (or LP) CG-PUSCH carrying CG-UCI overlaps with a LP (or HP) PUCCH carrying HARQ-ACK,

Step 1: Treat the CG-PUSCH with CG-UCI as a CG-PUSCH with a HARQ-ACK sequence.

- When a HP (or LP) CG-PUSCH carries CG-UCI without HARQ-ACK: CG-UCI is treated as HP (or LP) HARQ-ACK.

- When a HP (or LP) CG-PUSCH carries CG-UCI with HP (or LP) HARQ-ACK: append the HP (or LP) HARQ-ACK to the HP (or LP) CG-UCI, and treat the outcome as one HP (or LP) HARQ-ACK sequence.

Step 2. Apply the existing procedure for multiplexing a HP (or LP) PUSCH carrying HARQ-ACK with a LP (or HP) PUCCH carrying HARQ-ACK, including the procedure for handling CSI/SR (if any).

In summary, the following is proposed for handling CG-UCI.

When a HP (or LP) CG-PUSCH carrying CG-UCI overlaps with a LP (or HP) PUCCH carrying HARQ-ACK, treat the CG-PUSCH with CG-UCI as a CG-PUSCH with a HARQ-ACK sequence of the same priority. Then apply the existing procedure for multiplexing a HP (or LP) PUSCH carrying HARQ-ACK with a LP (or HP) PUCCH carrying HARQ-ACK, including the procedure for handling CSI/SR (if any).

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	When a PUCCH carrying HP SR and HP HARQ-ACK with PF 0/1, the number of SR bits is K=1.

Based on the discussion in the previous sections we propose the following:

Proposal 1	When a HP PUCCH carrying SR and HARQ-ACK with PF0/1 overlaps with a LP PUCCH carrying HARQ-ACK,

- 1-bit HP SR is appended to 1 or 2-bit HP HARQ-ACK, and treated together as a 2 or 3-bit HP HARQ-ACK;

- Reuse other procedures for multiplexing LP HARQ-ACK and HP HARQ-ACK onto a PUCCH resource with PF 2/3/4, i.e. separate coding, PRB determination, rate matching and power control.

- If the HP HARQ-ACK is a dynamic HARQ-ACK, a PUCCH resource indicated by PRI is used for multiplexing.

- If the HP HARQ-ACK is an SPS HARQ-ACK, a PUCCH resource determined from the PUCCH resource(s) provided by sps-PUCCH-AN-List is used for multiplexing.

Proposal 2	When a HP PUCCH carrying SR and HARQ-ACK with PF0/1 overlaps with a LP PUCCH carrying HARQ-ACK and CSI/SR, only LP HARQ-ACK is multiplexed with HP UCI (i.e., SR and HARQ-ACK) and LP CSI/SR are dropped.

Proposal 3	When a HP (or LP) CG-PUSCH carrying CG-UCI overlaps with a LP (or HP) PUCCH carrying HARQ-ACK, treat the CG-PUSCH with CG-UCI as a CG-PUSCH with a HARQ-ACK sequence of the same priority. Then apply the existing procedure for multiplexing a HP (or LP) PUSCH carrying HARQ-ACK with a LP (or HP) PUCCH carrying HARQ-ACK, including the procedure for handling CSI/SR (if any).

# References

3GPP TS38.213 V17.1.0, 2022-03.

R1-2202770, “Summary #3 of email thread [108-e-R17-IIoT-URLLC-03],” Moderator (CATT).

R1-2202830, “Summary#3 of email thread [108-e-R17-IIoT-URLLC-04],” Moderator (OPPO).