3GPP TSG-RAN WG1 Meeting #109-e	Tdoc R1-2203400

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.16.3

Source:	Ericsson

Title:	Rel-17 UE Features for IIoT/URLLC

Document for:	Discussion, Decision

# Introduction

In this contribution, the remaining issues of Rel-17 UE Features for IIoT/URLLC are discussed [1][2][3]. For each enhancement aspect, the candidate UE features are proposed in each section below.

# Discussion

The remaining open issues from the FL summary [3] are discussed in the following sections.

## Type of Remaining FG 25-X

A question was raised whether to consider FSPC or BC as the Type for these features. From our perspective, the corresponding functionality of the features under FG 25-X are not affected when testing for different bands. Therefore, it is not clear that band differentiation would impact the feature and the corresponding testing. Therefore, we suggest indicating “Per UE” as the Type, and not accept other proposed alternatives such as FSPC. This suggestion is also in line with RAN2 recommendation, that FSPC indication should be avoided as much as possible (R2-2002378).

Adopt “Per UE” type for remaining FG 25-X.

## UE Feedback Enhancements for HARQ-ACK

### FG 25-1: SPS HARQ-ACK deferral

It has been discussed whether/how to revise the prerequisite feature groups for FG 25-1. In our view, no revision is needed since the components of this feature require only the support of DL SPS and are not dependent on periodicity or multiple configurations of DL SPS.

Confirm FG 5-18 as prerequisite of FG 25-1.

### FG 25-2 to 25-3b: PUCCH repetition enhancements

It has been discussed whether/how to revise the prerequisite feature groups for FGs 25-2 to 25-3b.

| 25-2 | Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8 | Repetitions for PUCCH format 0 and 2 over multiple slots with K = 2, 4, 8 | [4-23] |
| --- | --- | --- | --- |
| 25-3 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with RRC configured repetition factor K = 2, 4, 8 Note: The support of FG 25-3 doesn’t imply an increase of the maximum number of PUCCHs per slot that supported by the UE | [4-23] [11-3] |
| 25-3a | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots based on dynamic repetition indication. Note: Dynamic PUCCH repetition factor indication is only supported for HARQ-ACK | [25-3 30-5] |
| 25-3b | Inter-subslot frequency hopping for PUCCH repetitions | 1. Support inter-subslot frequency hopping for PUCCH repetition operation of PUCCH Formats 0, 1, 2, 3 and 4 for 7OS subslot-based PUCCH configurations. 2. Support inter-subslot frequency hopping for PUCCH repetition operation of PUCCH Format 0 and Format 2 for 2OS subslot-based PUCCH configurations | [TBD] |

In order to examine the dependency between these feature groups to determine the corresponding prerequisites, FG 4-23 and FG 30-5 are provided below as well.

| 4-23 | Repetitions for PUCCH format 1, 3, and 4 over multiple slots with K = 2, 4, 8 | Repetitions for PUCCH format 1, 3, and 4 over multiple slots with K = 2, 4, 8 |  |
| --- | --- | --- | --- |
| 30-5 | Slot based dynamic PUCCH repetition indication | Support slot based dynamic PUCCH repetition indication for PUCCH formats 0/1/2/3/4 support slot based dynamic PUCCH repetition for PUCCH formats 0/1/2/3/4 | [4-23 and/or 25-2] |

- FG 25-2: Regarding FG 4-23 as the prerequisite, we observe that 4-23 is the PUCCH repetition for long PUCCH. However, the PUCCH formats in 25-2 are short PUCCHs. Therefore, with respect to PUCCH format, there is no reason to consider FG 4-23 as prerequisite for FG 25-2. On the other hand, in previous releases PUCCH repetition was only supported for long PUCCH. This implies that if a UE supports FG 4-23, it means that it is capable to support deferral and other PUCCH related functionalities. Therefore, it is reasonable to consider FG 4-23 as the prerequisite of FG 25-2.

- FG 25-3: Similar to the previous case, the functionalities needed to support this feature is PUCCH repetition related functionaries (i.e. FG 4-23) and sub-slot related functionalities (i.e. FG 11-3) while the format of PUCCH is not affecting the related functionalities of this feature. Therefore, it is reasonable to consider FG 4-23 and FG 11-3 as the prerequisite of FG 25-3.

- FG 25-3a: For dynamic PUCCH repetition, FG 30-5 as prerequisite is needed which in turn has FG 4-23 as prerequisite. For sub-slot related functionalities, FG 11-3 should be considered as prerequisite. There is no need to include FG 25-3 as the prerequisite of FG 25-3a and introduce additional functionalities specific for 25-3 and not related to 25-3a.

- FG 25-3b: It seems that no capability is introduced for inter-slot frequency hopping in Rel-15. Hence, regarding this feature the UE should be able to support inter-slot frequency PUCCH hopping and would need FG 4-23 and FG 11-3 to support PUCCH repetition in addition to sub-slot related functionalities.

Confirm FG 4-23 as prerequisite of FG 25-2.

Confirm FG 4-23 and FG 11-3 as prerequisites of FG 25-3.

Support FG 30-5 and FG 4-23 as prerequisites for FG 25-3a.

Support FG 11-3 and FG 4-23 as prerequisites for FG 25-3b.

There is a typo in component description of 25-3b where slot-based PUCCH configurations is used instead of sub-slot based PUCCH configurations for 7/2 symbols sub-slot.

Fix the typo in component description of FG 25-3b as “sub-slot based PUCCH configurations”.

### 25-4 to 25-7: Retransmission of cancelled HARQ-ACK

It has been discussed whether/how to revise the prerequisite feature groups for FGs 25-4/5 and 25-7. In our view the current prerequisites for FG 25-4/5 are adequate and no prerequisite is needed for FG 25-7 since the corresponding component descriptions already clarify the needed information.

Confirm the prerequisite feature groups of FG 25-4 and 25-5. No prerequisite is needed for FG 25-7.

### 25-9 to 25-10a: PUCCH cell switching

With respect to FGs for PUCCH cell switching, there is an open issue whether/how to indicate the capability for support of PUCCH cell switch in two PUCCH groups. PUCCH cell switching is supported for both primary cell group and secondary cell group if available. There is no dependency across cell groups for the PUCCH cell switching functionalities within a PUCCH group. Hence, if a secondary PUCCH group is enabled, the same functionalities as for the primary PUCCH group can be applied there as well. Hence, there is no differentiation is needed and two PUCCH groups should be supported by these feature groups.

FGs 25-9 to 25-10a support the corresponding PUCCH cell switching functionalities for one PUCCH group as well as two PUCCH groups.

## CSI Feedback Enhancements

For FG 25-11 in the preliminary list [1], no prerequisite FG is stated. However, the 4-bit subband CQI reporting should be an enhancement of basic CSI feedback. Hence FG 2-32 “Basic CSI feedback” should be added as a prerequisite as shown below.

Additionally, the 4-bit subband CQI should be confirmed as a “per UE” capability.

Table 1: Proposed FG for enhanced CSI feedback.

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-11 | 4-bits subband CQI | Subband CQI reporting with 4 bits per subband | 2-32 |

Add FG 2-32 “Basic CSI feedback” as a prerequisite for FG 25-11 “4-bits subband CQI”.

Confirm FG 25-11 “4-bits subband CQI” as a “Per UE” capability.

## Enhancements for Unlicensed Band URLLC/IIoT

The remain issue with respect to UE features for operation on shared spectrum was whether to update the component description of FG 25-12 as the following:

Table 2: FG 25-12 with proposed updates for component description

| 25-12 | UE initiating a semi-static channel occupancy with configurations dependent on gNB semi-static channel access configurations | 1. Support initiating a semi-static channel access occupancy by the UE where the corresponding period is the same as, integer multiple of, or inter-factor of the period configured for a semi-static channel occupancy that can be initiated by gNB. 2. Sensing to initiate a semi-static CO or transmit after a gap greater than 16us from any transmission burst within a UE-initiated CO. 3. Determination of COT initiator assumption based on rules for configured UL. 4. Validating COT initiator assumption indicated in UL scheduling DCI. |
| --- | --- | --- |

From our perspective, the proposed additions are important to be included and tested to ensure proper intra-operability.

Adopt the proposed changes in red in Table 2 for FG 25-12.

## Intra-UE Multiplexing/Prioritization

### Prioritization of overlapping DG-PUSCH and CG-PUSCH

In RAN1#107e AI 8.3, the following agreements were made:

Agreement (RAN1#107e) For collision of LP DG-PUSCH and HP CG-PUSCH of different priorities, the cancellation is applied per actual repetition, if LP DG-PUSCH and/or HP CG-PUSCH is repeated. Agreement (RAN1#107e) For collision of HP DG-PUSCH and LP CG-PUSCH, the cancellation is applied per actual repetition, if HP DG-PUSCH and/or LP CG-PUSCH is repeated.

As shown in the agreements of AI 8.3, the UE is expected to apply cancellation per actual repetition, if (LP/HP) DG-PUSCH and/or (HP/LP) CG-PUSCH is repeated. Thus it’s necessary to add the support of PUSCH with repetition in 25-14 and 25-15.

Add support of PUSCH with repetition in the components for FG 25-14 and 25-15: “2. Support cancellation per actual repetition if LP DG-PUSCH and/or HP CG-PUSCH is repeated”.

In summary, 25-14 and 25-15 are to be updated as follows.

| 25-14 | PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH | - Support PHY prioritization for the case where low-priority DG-PUSCH collides with high-priority CG-PUSCH - Support cancellation per actual repetition if LP DG-PUSCH and/or HP CG-PUSCH is repeated |
| --- | --- | --- |

### Simultaneous PUCCH and PUSCH transmission

In RAN1#107e AI 8.3, the following conclusions were made:

Conclusion There is no consensus in RAN1 to support simultaneous PUCCH/PUSCH transmission of same priority over different cells in Rel-17. Conclusion There is no consensus in RAN1 to support simultaneous PUCCH/PUSCH transmissions on different cells for intra-band CA in Rel-17.

Thus, Rel-17 only supports simultaneous PUCCH/PUSCH transmission of different priorities on different cells for inter-band CA. Accordingly FG 25-18 can be finalized as below:

| 25. NR_IIOT_URLLC_enh | 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH transmissions of different priorities on different cells [at least] for inter-band CA. |
| --- | --- | --- | --- |

FG 25-18a component description is revised to: “Support simultaneous PUCCH/PUSCH transmissions of different priorities on different cells for inter-band CA.”

## Propagation Delay Compensation

For the topic of propagation delay compensation, the following feature groups were agreed as of RAN1#108e.

| 25. NR_IIOT_URLLC_enh | 25-19 | RTT-based Propagation delay compensation based on CSI-RS for tracking and SRS | Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on CSI-RS for tracking and SRS | [2-51, 2-53] |
| --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-19a | RTT-based Propagation delay compensation based on DL PRS and SRS | 1. Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on DL PRS and SRS 2. Max number of DL PRS Resources in DL PRS Resource Set for PDC Values = {1, 2, 4, 8, 16, 32, 64} Note: 16, 32, 64 are only applicable to FR2 bands FFS whether to add necessary components and corresponding notes in FG 13-1 to FG 25-19a or to add FG 13-1 as a prerequisite FG for FG 25-19a - With potential different value ranges from FG 13-1 | [25-19, 13-1, 2-53] |
| 25. NR_IIOT_URLLC_enh | 25-20 | Propagation delay compensation based on legacy TA procedure | Support propagation delay compensation based on legacy TA procedure | FFS |

In our view, 25-19 and 25-19a are parallel, independent features. FG 25-19a can be supported without supporting 25-19, especially for a UE already with PRS processing capability for positioning purpose. Thus, 25-19 should be deleted from the prerequisite list of 25-19a.

FG 25-19 is not included as a prerequisite of FG 25-19a.

Regarding the question of 13-1 as prerequisite for 25-19a, it is preferable that 13-1 is not a perquisite, since a UE supporting 25-19a for the purpose of time synchronization do not need to support 13-1 which is for positioning. PRS reception for positioning is much more demanding than PRS reception for propagation delay compensation (PDC). For example, for PDC purpose, the PRS is within the active BWP, and use the same pointA, SCS, cyclic prefix as the PCell. Imposing 13-1 as a perquisite of 25-19a will unnecessarily bind the time synchronization use case to the positioning use case. This unnecessarily removes the freedom of UE implementing 25-19a as an independent feature for time synchronization without having positioning capability.

FG 13-1 is not included as a prerequisite of FG 25-19a.

With the understanding that FG 13-1 is not a prerequisite of FG 25-19a, a question arises if any components of FG 13-1 should be copied over to FG 25-19a. Our view is, no component of 13-1 is needed for 25-19a.

- For component 1 of 13-1, the maximum BW of PRS should be the DL BW that UE already reports (FG 2-1). Thus no need to report again in FG 25-19a.

Agreement

For PDC purpose, the UE is not expected to measure DL PRS outside the active BWP.

- For component 2-3 of 13-1, processing capability (N, T) corresponds to (K, P) in 38.214. But (K, P) are defined for positioning purpose, e.g., reference signal time difference (RSTD) between two PRS which are sent by two gNBs. Indeed none of the yellow-highlighted concepts in the excerpt of 38.214 below exist for PDC PRS.

TS 38.214 section 5.1.6.5:

- For component 4 of 13-1, the newly added component 2 provides the maximum number of PRS resources already. Furthermore, component 4 of 13-1 is intended to cover PRS resources from multiple resource sets. However, for PDC, there is only a single PRS resource set.

Thus, the existing components 1 and 2 are adequate for 25-19a. The components of 13-1 are either unnecessary or inappropriate for 25-19a.

Components of FG 13-1 are not included in FG 25-19a.

# Clarification on Type of an FG and Type of Its Prerequisite

A recurring general question has been the relationship of the Type of a FG and Type of its Prerequisite. Specifically, some companies argue that if the prerequisite has a narrower Type (e.g., “FSPC”), then the FG cannot have a broader Type (e.g., “BC”). As a reference, the hierarchy of the Type are described in TS 38.306 as shown below.

TS 38.306 Section 4.2.1: UE capability parameters have hierarchical structure. In the table of UE capability parameter in subsequent clauses, "Per" indicates the level the associated parameter is included. "UE" in the column indicates the associated parameter is signalled per UE, "Band" indicates it is signalled per band, "BC" indicates it is signalled per band combination, "FS" indicates it is signalled per feature set (per band per band combination), "FSPC" indicates it is signalled per feature set per component carrier (per CC per band per band combination), and "FD" in the column indicates to refer the associated field description.

However, checking of existing feature groups indicate that Type of a FG can be broader than its prerequisite. The following are some examples:

- (NR_L1enh_URLLC) FG 11-4b is per UE while its prerequisite FG 11-4 is per FS.

- (NR IIoT) FG 12-1a is per UE while its prerequisite FG 11-1 is per FS.

- (NR_eMIMO) FG 16-2a-4, 16-2a-4a, 16-2a-8 are per UE while the prerequisite FG 16-2a is per FSBC.

Numerous examples in a variety of topics show that Type of an FG can be broader than the Type of Its Prerequisite.

Then a question is, what should be understanding if Type of FG-x is broader than Type of its prerequisite FG-y? Our understanding is, for such FG-x (e.g., Type = “UE”) and FG-y (e.g., Type=“BC”), for a given scenario (e.g., a given set of band combinations) where UE supports FG-y, then {FG-x, FG-y} are supported for the given scenario, if UE reports FG-x for the broader scenario. This is shown with an example in Table 3.

Table 3. Example of FG-x has a broader Type than Type of its perquisite FG-y

|  | Type of FG-x = Per UE | Type of prerequisite FG-y = Per BC | Interpretation |
| --- | --- | --- | --- |
| Case 1 | UE supports FG-x | UE supports FG-y on (BC#1, BC#2) | UE supports {FG-x, FG-y} on (BC#1, BC#2) |
| Case 2 | UE does not support FG-x | UE supports FG-y on (BC#1, BC#2) | UE does not support FG-x on any band combination |

In contrast, if FG-x and FG-y have the same Type (e.g.,  Type=”BC”), then for a given scenario (e.g., the set of band combinations (BC#1, BC#2)) where UE supports FG-y, the UE may or may not support {FG-x, FG-y} for the given scenario (e.g., (BC#1, BC#2)). The UE support {FG-x, FG-y} for (BC#1, BC#2) only if UE also reports FG-x for (BC#1, BC#2). One example is shown in Table 4.

Table 4. Example of FG-x has a same Type as the Type of its perquisite FG-y

|  | Type of FG-x = Per BC | Type of prerequisite FG-y = Per BC | Interpretation |
| --- | --- | --- | --- |
| Case 1 | UE supports FG-x on (BC#1, BC#2) | UE supports FG-y on (BC#1, BC#2) | UE supports {FG-x, FG-y} on (BC#1, BC#2) |
| Case 2 | UE supports FG-x on (BC#1) | UE supports FG-y on (BC#1, BC#2) | UE supports {FG-x, FG-y} on (BC#1) UE does not support FG-x on BC#2 |
| Case 3 | UE supports FG-x on (BC#3) | UE supports FG-y on (BC#1, BC#2) | UE does not support FG-x on BC#1, BC#2 or BC#3 Note: This may be considered an erroneous report for FG-x |

It is proposed that the understanding is formally captured (e.g., in TS 38.306) if Type of FG-x is broader than Type of its prerequisite FG-y.

Formally capture that Type of an FG-x can be broader than Type of Its Prerequisite FG-y.

Formally capture the interpretation when Type of FG-x is broader than Type of Its Prerequisite FG-y: for a narrower scenario where UE supports FG-y, {FG-x, FG-y} are supported for the narrower scenario, if UE reports FG-x for the broader scenario.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Adopt “Per UE” type for remaining FG 25-X.

Proposal 2	Confirm FG 5-18 as prerequisite of FG 25-1.

Proposal 3	Confirm FG 4-23 as prerequisite of FG 25-2.

Proposal 4	Confirm FG 4-23 and FG 11-3 as prerequisites of FG 25-3.

Proposal 5	Support FG 30-5 and FG 4-23 as prerequisites for FG 25-3a.

Proposal 6	Support FG 11-3 and FG 4-23 as prerequisites for FG 25-3b.

Proposal 7	Fix the typo in component description of FG 25-3b as “sub-slot based PUCCH configurations”.

Proposal 8	Confirm the prerequisite feature groups of FG 25-4 and 25-5. No prerequisite is needed for FG 25-7.

Proposal 9	FGs 25-9 to 25-10a support the corresponding PUCCH cell switching functionalities for one PUCCH group as well as two PUCCH groups.

Proposal 10	Add FG 2-32 “Basic CSI feedback” as a prerequisite for FG 25-11 “4-bits subband CQI”.

Proposal 11	Confirm FG 25-11 “4-bits subband CQI” as a “Per UE” capability.

Proposal 12	Adopt the proposed changes in red in Table 2 for FG 25-12.

Proposal 13	Add support of PUSCH with repetition in the components for FG 25-14 and 25-15: “2. Support cancellation per actual repetition if LP DG-PUSCH and/or HP CG-PUSCH is repeated”.

Proposal 14	FG 25-18a component description is revised to: “Support simultaneous PUCCH/PUSCH transmissions of different priorities on different cells for inter-band CA.”

Proposal 15	FG 25-19 is not included as a prerequisite of FG 25-19a.

Proposal 16	FG 13-1 is not included as a prerequisite of FG 25-19a.

Proposal 17	Components of FG 13-1 are not included in FG 25-19a.

Proposal 18	Formally capture that Type of an FG-x can be broader than Type of Its Prerequisite FG-y.

Proposal 19	Formally capture the interpretation when Type of FG-x is broader than Type of Its Prerequisite FG-y: for a narrower scenario where UE supports FG-y, {FG-x, FG-y} are supported for the narrower scenario, if UE reports FG-x for the broader scenario.

# References

R1-2202928		Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e; Moderators (AT&T, NTT DOCOMO, INC.)

R1-2202927		LS on updated Rel-17 RAN1 UE features list for NR; RAN1.

R1-2201514		Summary on UE features for enhanced IIoT and URLLC, Moderators (NTT DOCOMO, INC.)

# Annex. Existing FGs for Positioning: 13-1, 13-4x

| 13-1 | Common DL PRS Processing Capability | 1. Maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} 2. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 3. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 4. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) of no more than 30%. |  |
| --- | --- | --- | --- |
| 13-4 | DL PRS Resources for Multi-RTT | 1. Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE. Values = {1, 2} 2. Max number of TRPs across all positioning frequency layers per UE. Values = {4, 6, 12, 16, 24, 32, 64, 128, 256} 3. Max number of positioning frequency layers UE supports Values = {1, 2, 3, 4} | 13-1 |
| 13-4a | DL PRS Resources for Multi-RTT on a band | 1. Max number of DL PRS Resources per DL PRS Resource Set Values = {1, 2, 4, 8, 16, 32, 64} Note: 16, 32, 64 are only applicable to FR2 bands 2. Max number of DL PRS Resources per positioning frequency layer. Values = {6, 24, 32, 64, 96, 128, 256, 512, 1024} Note: 6 is only applicable to FR1 bands | 13-1 |
| 13-4b | DL PRS Resources for Multi-RTT on a band combination | 1. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1-only. Values = {6, 24, 64, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR1 only BC. 2. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2-only. Values = {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR2 only BC 3. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1 in FR1/FR2 mixed operation. Values = {6, 24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands 4. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2 in FR1/FR2 mixed operation. Values = {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands | 13-1 |