3GPP TSG RAN WG1 #109-e		R1-2203401

e-Meeting, May 09th – 20th, 2022

Source:	Panasonic

Title: 	Discussion on PDSCH/PUSCH enhancements for NR 52.6-71 GHz

Agenda Item:		8.2.3

Document for:	Discussion

# Introduction

This document provides our view on the remaining items on multi-PDSCH/PUSCH scheduling for extending NR up to 71 GHz [1].

# Discussion on multi-PDSCH/PUSCH scheduling

## Collision handling between PUSCH and CORESET#0

In RAN1#106bis-e meeting, the following agreement was made, however, the highlighted yellow part is still not concluded on how to handle a possible collision between PUSCH and flexible symbol(s) indicated in MIB for a CORESET with Type0-PDCCH CSS set.

Agreement: (RAN1#106bis-e) For multiple PDSCHs (or PUSCHs) scheduled by a single DCI, - Rel-15/16 behavior that is described in TS 38.213 Clauses 11 and 11.1 for a PDSCH (or PUSCH) indicated by DCI also applies for multiple PDSCHs (or PUSCHs) schedule by a single DCI. - If one of multiple PDSCHs (or PUSCHs) scheduled by the DCI collides with a flexible symbol (indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated), - If that PUSCH is collided with SSB symbols indicated by ssb-PositionsInBurst [or symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set], the HARQ process number increment is skipped for the PUSCH. - Otherwise, the HARQ process number increment is not skipped for that PDSCH (or PUSCH).

We think the actual SSB symbols indicated by ssb-PositionsInBurst is sufficient and it is not required to consider the pdcch-ConfigSIB1 in MIB. Therefore, we propose the following

Proposal 1: Support to update the previous agreement with the following change

For multiple PDSCHs (or PUSCHs) scheduled by a single DCI,

- Rel-15/16 behavior that is described in TS 38.213 Clauses 11 and 11.1 for a PDSCH (or PUSCH) indicated by DCI also applies for multiple PDSCHs (or PUSCHs) schedule by a single DCI.

- If one of multiple PDSCHs (or PUSCHs) scheduled by the DCI collides with a flexible symbol (indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated),

  - If that PUSCH is collided with SSB symbols indicated by ssb-PositionsInBurst [or symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set], the HARQ process number increment is skipped for the PUSCH.

  - Otherwise, the HARQ process number increment is not skipped for that PDSCH (or PUSCH).

## Whether to combine HARQ-disabling feature introduced in Rel-17 NTN with multi-PDSCH scheduling

Regarding HARQ-disabling feature, it has been specifically introduced in Re. 17 NTN. The HARQ-disabling can increase throughput, but the radio efficiency is decreased because of an assumption of lower BLER target. It can be used for the back-haul usage of FR2-2, but it might not be used for the generic use case. We are not so supportive of the feature of HARQ-disabling for FR2-2, but we are not objecting the proposal.

# Conclusions

In this contribution, we made the following proposal.

Proposal 1: Support to update the previous agreement with the following change

For multiple PDSCHs (or PUSCHs) scheduled by a single DCI,

- Rel-15/16 behavior that is described in TS 38.213 Clauses 11 and 11.1 for a PDSCH (or PUSCH) indicated by DCI also applies for multiple PDSCHs (or PUSCHs) schedule by a single DCI.

- If one of multiple PDSCHs (or PUSCHs) scheduled by the DCI collides with a flexible symbol (indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated),

  - If that PUSCH is collided with SSB symbols indicated by ssb-PositionsInBurst [or symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set], the HARQ process number increment is skipped for the PUSCH.

  - Otherwise, the HARQ process number increment is not skipped for that PDSCH (or PUSCH).

# Reference

[1]. R1-2200744, " Summary #2 of PDSCH/PUSCH enhancements (Scheduling/HARQ)", Moderator (LG Electronics), RAN WG1 #107bis-e.