3GPP TSG RAN WG1 #109-e		R1-2203402

e-Meeting, May 9th – 20th, 2022

Source:	Panasonic

Title: 	Discussion on joint channel estimation for PUSCH and PUCCH

Agenda Item:		8.8.2

Document for:	Discussion

# Introduction

NR coverage enhancements work item in RAN1 is finalized [1]. For the mechanism(s) to enable joint channel estimation for enhancing coverage of PUSH and PUCCH, some details need to be finalized as the corrections. This document provides our view on these aspects for enabling joint channel estimation.

# Discussion on remaining issues

## Precoding cycling

In RAN1#106-bis-e, it was agreed to define events that violate power consistency and phase continuity in the following agreement. In RAN1#107-e, a few FFS points of this agreement have been solved, e.g., UL beam switching for multi-TRP operation, TPC command, TA adjustment, actual TDW reaches the maximum duration, frequency hopping. We will discuss on precoding cycling aspect in the following.

Agreement - Support at least the following events that violate power consistency and phase continuity. - Dropping/cancellation based on Rel-15/16 collision rules. - FFS: Rel-17 collision rules. - DL slot or DL reception/monitoring based on semi-static DL/UL configuration for unpaired spectrum. - FFS: Other UL transmission in between PUSCH/PUCCH transmissions. - Gap between two PUSCH/PUCCH transmissions exceeds 13 symbols. - FFS: Transmission parameters need to be changed due to network-indicated operations, including: Tx power, UL beam/TPMI, and RB allocation. - FFS: TPC command. - FFS: TA adjustment. - FFS: The actual TDW reaches the maximum duration. - FFS: Frequency hopping. - FFS: Precoder cycling. - FFS: other events. - FFS: whether events are semi-static events or dynamic events. - FFS: the time duration of an event.

During email-discussions in RAN1#107-e, there were different understandings for precoder cycling between companies. In our understanding, the precoder cycling is a transparent diversity scheme, where a UE might or might not have any information of the channel propagation in general, it will generate a set of precoders or select precoders from the codebook, each of which concentrates the transmitted signal power in one certain direction around gNB. The same precoding is applied to across all allocated PUSCH RBs. It is necessary to use the same precoder of precoding cycling within an actual TDW by UE to have joint channel estimation over actual TDW at gNB.

For the interaction among inter-slot frequency hopping (FH), precoding cycling, and joint channel estimation, our view is the same precoder should be applied for actual TDW within a hop of the inter-slot FH for PUCCH/PUSCH for joint channel estimation at gNB. This can be achieved by a proper respective configuration of the period. No additional specification is necessary for the interaction among inter-slot FH, precoding cycling, and joint channel estimation. Therefore, we propose the following.

Proposal 1: To specify that a UE expects to perform the same precoder of precoding cycling within an actual TDW.

## Dynamic and semi-static events

In RAN#107bis-e and RAN1#108-e meetings, the following two cases have been discussed in order to clarify UE behaviour as follows

- Case 1: A semi-static event is triggered after one or multiple dynamic events. Whether a new actual TDW is created after the semi-static event?

- Case 2: A semi-static event overlaps with a dynamic event. Whether a new actual TDW is created after the semi-static event?

We think that a new actual TDW is created in both cases 1 and 2 because UE is mandatory to support restarting DM-RS bundling due to semi-static event as shown in the following agreement.

Agreement: - If DM-RS bundling is supported, UE is mandatory to support restarting DM-RS bundling due to semi-static events. UE capability of restarting DMRS bundling is applied only to dynamic events. - An event is regarded as a dynamic event if it is triggered by a DCI or MAC-CE, otherwise it is regarded as a semi-static event. Note: At least frequency hopping event is considered as semi-static event.

## Group common TPC commands

For the action of group common TPC commands with format 2_2, a working assumption has been reached that it does not constitute an event that violates power consistency and phase continuity as follows

Working assumption: - The action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity. - If UE is configured to accumulate TPC commands, - If UE receives TPC commands that would take into effect during a configured TDW, UE accumulates TPC commands without taking effect during the current configured TDW. TPC commands take effect after the current configured TDW. - If UE is not configured to accumulate TPC commands - the last TPC command that would take effect within a configured TDW supersedes all previous TPC commands that take effect within that configured TDW and only the last TPC command is applied by the UE after the current configured TDW. - FFS: no more than 1 TPC command is expected to take effect during a configured TDW.

In RAN1#108-e, it has been discussed on how to capture the above working assumption for DG-PUSCH into the specification as shown in the following 5 options.

- Option 1: Legacy definition of KPUSCHi is preserved for PUSCH transmissions without DM-RS bundling. Redefine KPUSCHi for PUSCH transmissions within a nominal TDW in case of DM-RS bundling. e.g., KPUSCHi is a number of symbols from K symbols before the start of the first repetition within the nominal time domain window including the transmission occasion i and before a first symbol of the transmission occasion i. - FFS: the value of K, e.g., K is “a number of KPUSCH,min symbols equal to the product of a number of symbols per slot, Nsymbslot, and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP b of carrier f of serving cell c”. - Option 2a: Modify the TPC command value set Di, e.g. if transmission occasion i is not the first transmission occasion within a nominal time domain window, then any TPC command values received via DCI format 2_2 contained in the set Di are deleted and added to the set Dj where j is a transmission occasion occurring after the end of the nominal time domain window and j>0 is the smallest integer for which KPUSCH(i) symbols before PUSCH transmission occasion i is earlier than KPUSCH(j) symbols before PUSCH transmission occasion j. - Option 3: For group common TPC commands with format 2_2, if UE is configured to accumulate TPC commands, - For a transmission occasion i occurs within a nominal time domain window, fi=f i1, where transmission occasion i1 is a first transmission occasion within the nominal time domain window. - For the first transmission occasion k occurring after the nominal time domain window, fk=fi1+δj, where δj is all the TPC command values that would take effect for the transmission occasions occurring after transmission occasion i1 and no later than transmission occasion k (i.e. including occasion k itself). - Option 3a: For group common TPC commands with format 2_2, if UE is configured to accumulate TPC commands, - For a transmission occasion i occurs within a nominal time domain window, fi=f i1, where transmission occasion i1 is a first transmission occasion within the nominal time domain window. - For the first transmission occasion k occurring after the nominal time domain window, fk=fi1+δj, where δj is all the TPC command values that would take effect for the transmission occasions occurring after are received between KPUSCHi1 symbols before transmission occasion i1 and no later than KPUSCHk symbols before transmission occasion k (i.e. including occasion k itself). - Note: If a UE is configured with DMRS bundling, for a given PUSCH power control adjustment state l in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission occasion m in symbol p and is configured with any CG PUSCH transmission occasion n ending before symbol p, the UE is not expected to receive a TPC command by DCI format 2_2 arrived later than KPUSCHm symbols before transmission occasion m but no later than KPUSCHn symbols before transmission occasion n. - Combined Option 1&3a: For group common TPC commands with format 2_2, if UE is configured to accumulate TPC commands and configured with DMRS bundling, - For a transmission occasion i occurs within a nominal time domain window, fi=f i1, KPUSCHi for PUSCH transmissions occasion i from K KPUSCHi1 symbols before the start a first symbol of the transmission occasion i1 before to a first symbol of the transmission occasion i where transmission occasion i1 is a first transmission occasion within the nominal time domain window for the same transport block as transmission occasion i and KPUSCHi1 reuses the same Rel-15/16 definition of KPUSCHx. - The definition of set for in Rel-15/16 TS 38.213 is reused. For the first transmission occasion k occurring after the nominal time domain window, fk=fi1+δj, where δj is all the TPC command values that are received between KPUSCHi1 symbols before transmission occasion i1 and KPUSCHk symbols before transmission occasion k. - Note: If a UE is configured with DMRS bundling, for a given PUSCH power control adjustment state l in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission occasion m in symbol p and is configured with any CG PUSCH transmission occasion n ending before symbol p, the UE is not expected to receive a TPC command by DCI format 2_2 arrived later than KPUSCHm symbols before transmission occasion m but no later than KPUSCHn symbols before transmission occasion n.

The views of companies on these options are diverged because they can work in principle. There might still be unclear issue for all options in case of interlacing between configured grant (CG) and dynamic grant (DG), but it is not clear for now. Therefore, FL proposed the following alternatives

- Alt 1: It’s up to Editor how to capture it into the specification.

- Alt 2: The action of group common TPC commands with format 2_2 is regarded as an event.

- Alt 3: UE does not expect to receive group common TPC commands with format 2_2 that would take effect during the actual TDWs.

Our view is to support that the action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity. If UE is configured to accumulate TPC commands and it receives TPC commands that would take into effect during a configured TDW, the UE accumulates TPC commands without taking effect during the current configured TDW. TPC commands take effect after the current configured TDW. On the other hand, for sake of progress, we can support Alt. 1 (It’s up to Editor how to capture it into the specification).

Proposal 2: It’s up to Editor how to capture the following working assumption into the specification

Working assumption: - The action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity. - If UE is configured to accumulate TPC commands, - If UE receives TPC commands that would take into effect during a configured TDW, UE accumulates TPC commands without taking effect during the current configured TDW. TPC commands take effect after the current configured TDW. - If UE is not configured to accumulate TPC commands - the last TPC command that would take effect within a configured TDW supersedes all previous TPC commands that take effect within that configured TDW and only the last TPC command is applied by the UE after the current configured TDW. - FFS: no more than 1 TPC command is expected to take effect during a configured TDW.

# Conclusions

In this contribution, we provide our view on joint channel estimation for PUSCH and PUCCH. We made the following proposals.

Proposal 1: To specify that a UE expects to perform the same precoder of precoding cycling within an actual TDW.

Proposal 2: It’s up to Editor how to capture the following working assumption into the specification

Working assumption: - The action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity. - If UE is configured to accumulate TPC commands, - If UE receives TPC commands that would take into effect during a configured TDW, UE accumulates TPC commands without taking effect during the current configured TDW. TPC commands take effect after the current configured TDW. - If UE is not configured to accumulate TPC commands - the last TPC command that would take effect within a configured TDW supersedes all previous TPC commands that take effect within that configured TDW and only the last TPC command is applied by the UE after the current configured TDW. - FFS: no more than 1 TPC command is expected to take effect during a configured TDW.

# Reference

[1] RP-213684, “Status report of WI: NR coverage enhancements,” China Telecom, RAN#94e, Dec. 2021.