3GPP TSG RAN WG1 #109-e                            		R1-2203408

e-Meeting, May 9th – 20th, 2022

Source:	CATT

Title:	Discussion on UE/TRP TEG framework

Agenda item:	5

Document for:	Discussion and Decision

Introduction

RAN4 LS R1-2203024 [1] informs RAN1 that RAN4 has made the following agreements for UE/TRP Rx TEG framework, and asks RAN1 to provide feedback if any issues are identified.

- The framework of UE/TRP Rx TEG: - Define multiple candidate timing error margin values {TE1, TE2, …, TEN} in the spec. - The number of candidate values (i.e. N) and the exact values of {TE1, TE2, …, TEN} will be decided in Perf part. - UE/TRP selects one value M from {TE1, TE2, …, TEN} based on its implementation and indicate to LMF. - For UE that supports multiple Rx TEGs (TEG#1, TEG#2, …), the associated timing error margin value of each Rx TEG is M, which means the timing error difference between the measurements within the same Rx TEG is within the margin M. - The applicability of reported UE Rx TEG is limited to the measurements contained within the measurement report in which the Rx TEG information is provided, and only to measurements that are tagged with the corresponding TEG ID. - The RRM accuracy requirements corresponding to the candidate timing error margin values {TE1, TE2, …, TEN} will be defined in Perf part. - The framework of UE/TRP Rx TEG can be also applied for UE/TRP RxTx TEG - Note: if additional issues are identified based on RAN1/2 progress, then this agreement can be revised.

In this contribution, we present our views on the response to RAN4 LS.

Discussion

Based on RAN4’s agreement in R1-2203024 [1], it is clear that:

Multiple candidate timing error margins will be defined in Rel-17

  - The number of candidate values (i.e. N) and the exact values of {TE1, TE2, …, TEN} will be decided by RAN4

UE/TRP informs the selected timing error margin to LMF

When multiple Rx/RxTx TEGs are supported by the UE/TRP, all of these Rx/RxTx TEGs have the same timing error margin.

The lifecycle of reported UE Rx/RxTx TEG information is only within one measurement report, i.e., the same value of Rx/RxTx TEG IDs may not mean the same Rx/RxTx TEG for different measurement reports.

In our view, there are still some issues we may need to check with RAN4:

Issue 1: During previous RAN1 discussions, some companies suggested considering the possible changes in the error margins of  Tx/Rx/TxRx TEGs with the time.  RAN4’s LS does not mention whether the error margin will change. Thus, it might be better to check with RAN4 on which of the following understanding is correct:

  - Regardless of the number of Rx TEGs supported by a UE/TRP, all of the Rx TEGs have the same error margin value, which will not change with time. In this case, a UE/TRP only needs to indicate the error margin to LMF one time.

  - Regardless of the number of Rx TEGs supported by a UE/TRP, all of the Rx TEGs have the same error margin value. But, the error margins may change, i.e., the value can be different in different measurement reports. In this case, a UE/TRP may need to provide an update on the timing error margin to LMF whenever the error margins change.

- Issue 2: RAN4’s LS indicates the framework of UE/TRP Rx TEG can be also applied for UE/TRP RxTx TEG. But,  the LS does not indicate whether the error margin for the Rx TEGs and that for RxTx TEGs is the same or can be different. In our understanding, for a UE or a TRP, all of its Rx TEGs have the same error margin, and all of its RxTx TEGs have the same error margin, but the error margins for the Rx TEGs and RxTx TEGs can be the same or different. This may need to be confirmed by RAN4.

- Based on the above discussion, we have the following proposal as feedback to RAN4’s LS.

Proposal 1: Send a reply LS to RAN4 with the following questions of clarification:

Question 1: Does a UE/TRP always support the same timing error margin value for Rx TEGs (or RxTx TEGs), or can a UE/TRP support different timing error margin values at different times?

Question 2: If a UE/TRP supports both Rx and RxTx TEGs, will the selected timing error margin value for Rx TEG(s) be the same as the timing error margin value for RxTx TEG(s), or can a UE/TRP select different timing error margin values for Rx TEG(s) and RxTx TEG(s)?

Conclusions

In this contribution, we discussed the feedback to RAN4’s LS (R1-2203024 (R4-2206998). Based on the discussion, we have the following proposal. The corresponding draft response LS is also prepared in [2].

Proposal 1: Send a reply LS to RAN4 with the following questions of clarification:

Question 1: Does a UE/TRP always support the same timing error margin value for Rx TEGs (or RxTx TEGs), or can a UE/TRP support different timing error margin values at different times?

Question 2: If a UE/TRP supports both Rx and RxTx TEGs, will the selected timing error margin value for Rx TEG(s) be the same as the timing error margin value for RxTx TEG(s), or can a UE/TRP select different timing error margin values for Rx TEG(s) and RxTx TEG(s)?

References

- R1-2203024 (R4-2206998), LS on the UE/TRP TEG framework, RAN4 (CATT)

- R1-220xxxx, Draft Reply LS on the UE/TRP TEG framework, CATT