3GPP TSG RAN WG1 #109-e              	                       R1-2203406

e-Meeting, May 9th – 20th, 2022

Source:	CATT

Title:	Discussion on lower Rx beam sweeping factor for latency improvement

Agenda item:	5

Document for:	Discussion and Decision

Introduction

In the RAN4 LS R4-2203022 [1], RAN4 discussed whether UE needs to be configured by LMF to perform PRS measurements in FR2 with a reduced Rx beam sweeping factor and reached the following agreements:

Agreements: - FFS on whether UE needs to be configured by LMF to perform measurements with a reduced Rx beam sweeping factor? - Option 1: - Yes - Option 2: - No - A LS can be send to RAN1 to check whether LMF needs to reqeuest UE to perform PRS measurements with a reduced Rx beams sweeping factor in FR2

Therefore, RAN4 wants RAN1 to confirm whether UE needs to be configured by LMF to perform PRS measurements in FR2 with a reduced Rx beam sweeping factor.

This contribution discusses and gives our views on the issue in the RAN4 LS.

Discussion

About the issue of whether UE needs to be configured by LMF to perform PRS measurements in FR2 with a reduced Rx beam sweeping factor, we prefer Option 1 in the above agreement, i.e., UE need to be configured by LMF to perform PRS measurements in FR2 with a reduced Rx beam sweeping factor.

The motivation of the introduction of reduced Rx beam sweeping factor in RAN1 is positioning latency reduction. However, even though RAN4 will not define separated accuracy performance requirements for reduced Rx beam sweeping factor, the actual positioning accuracy may be degaded when the UE uses a reduced number of Rx beams. In other words, the cost of reducing positioning latency may be the degradation of positioning accuracy. Then, we see the necessity of configuration and indication by LMF to perform PRS measurements in FR2 with a reduced Rx beam sweeping factor for a UE. At least such indication can provide flexibility on the network configuration of Rx beam sweeping factor.

In addition, the candidate values of number of lower Rx beam sweeping factors in UE capability include {1,2,4,6}, which had been proposed by RAN4’s previous LS [2] and confirmed by RAN1 [3], as shown in the following table:

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown 27. NR_pos_enh; 27-9; Support of lower Rx beam sweeping factor; 1. Support of the lower Rx beam sweeping factor than 8 for FR2 2. Number of Rx beam sweeping factors; No; UE only supports 8 as the Rx beam sweeping factor defined by RAN4.; Per band; n/a; n/a FR2 only; n/a; Component 2 candidate values: {1,2,4,6} FFS Need for location server to know if the feature is supported; Optional with capability signaling.

we prefer UE can report only one value for the reduced beam sweeping factor instead of multiple ones, i.e., UE will select one value from the set of {1,2,4,6}and report it to LMF as the number of lower Rx beam sweeping factor. And a UE will use this value as the configured lower Rx beam sweeping factor if the UE received the indication from LMF, otherwise, the UE will use Rel-16 Rx beam sweeping factor (i.e. 8) in case of no indication from LMF.

Proposal 1: UE need to be configured by LMF to perform PRS measurements in FR2 with a reduced Rx beam sweeping factor.

Proposal 2: UE can report only one value for the reduced beam sweeping factor instead of multiple ones in UE capability reporting.

Conclusions

In this contribution, we discuss the issue in the RAN4 LS, and give the following proposals:

Proposal 1: UE need to be configured by LMF to perform PRS measurements in FR2 with a reduced Rx beam sweeping factor.

Proposal 2: UE can report only one value for the reduced beam sweeping factor instead of multiple ones in UE capability reporting.

References

- R4-2206980, “LS on lower Rx beam sweeping factor for latency improvement”, RAN4, RAN4#102-e.

- R4-2202678, “Reply LS on lower Rx beam sweeping factor for latency improvement”, RAN4, RAN4#101bis-e.

- R1-2202853, “Summary of UE features for NR positioning enhancements	Moderator”, AT&T, RAN1#108-e.