3GPP TSG RAN WG1 #109-e                            		R1-2203412

e-Meeting, May 9th – 20th, 2022

Source:	CATT

Title:	Discussion on questions concerning the implementation of RAN1 agreements in NRPPa

Agenda item:	5

Document for:	Discussion and Decision

Introduction

In LS R1-2203040 [1], RAN3 requested RAN1’s inputs on the issues in the following table:

Table 1: Issues requiring RAN1 inputs [1]:

| Topic | Issue | Required RAN1 work |
| --- | --- | --- |
| UL-SRS-RSRPP | RAN1 has defined a new measurement for UL SRS-RSRPP: Agreement#6: Definition UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power of the received UL SRS signal configured for the measurement at the i-th path delay of the channel response, where UL SRS-RSRPP for 1st path delay is the power corresponding to the first detected path. For frequency range 1, the reference point for the UL SRS-RSRPP shall be the antenna connector of the gNB. For frequency range 2, UL SRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. Note: The following two options are supported by gNB to LMF: Option 1 (RX diversity for the first path UL SRS-RSRPP) - The same RX branch(es) as applied for the first path UL SRS-RSRPP measurements are used for the additional paths UL SRS-RSRPP measurements if those are provided together - For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-RSRPP measurements, then reported UL SRS-RSRPP value for the first path shall not be lower than the corresponding UL SRS-RSRPP for the first path of any of the individual receiver branches Option 2 (RX diversity for UL SRS-RSRP) The same RX branch(es) as applied for UL SRS-RSRP measurements are used for UL SRS-RSRPP measurements (i.e., the first and additional paths UL SRS-RSRPP if those are provided). RAN3 will define the value range of the UL-SRS-RSRPP measurement for the i-th path as an INTEGER (0..126). Some companies question whether the information on the RX Diversity options, captured as a note in the RAN1 agreement, need to be signalled to LMF along with the UL-SRS-RSRPP measurement. | RAN1 to feedback if information on the Rx Diversity options needs to be signalled to LMF and to comment on the UL-SRS-RSRPP value range, if needed? |
| TEG | Some companies in RAN3 believe it is beneficial to signal the SRS port index to the LMF, so that LMF can group measurements based on Port index. RAN3 would like to know if SRS Port Index needs to be signalled to the LMF when SRS resource for MIMO is used? | RAN1 to feedback if information on the SRS port index needs to be signalled to LMF when SRS resource for MIMO is used. |
| On demand PRS | RAN3 has observed the following RAN1 agreement on On-demand PRS ON/OFF indicator (for LMF initiated request only): “In "On-demand PRS information for LMF-initiated on-demand DL PRS requests"; either per resource, or per resource set, or per UE”. RAN3 would like to inform RAN1 that the procedures defined by RAN3 for on-demand PRS transmission are non-UE-associated (i.e., cell specific) and thus an ON/OFF indication per UE does not make sense. Instead, it is considered to have an OFF indication per TRP to minimize the transmission power. | RAN1 to take into account and update their agreed parameter lists for On-demand PRS. |

In this contribution, we present our views on the response to RAN3’s questions.

Discussion

UL-SRS-RSRPP

For the reporting of the UL SRS-RSRPP with RX diversity, RAN1 decides to support the following two options:

Option 1 (RX diversity for the first path UL SRS-RSRPP)

-	The same RX branch(es) as applied for the first path UL SRS-RSRPP measurements are used for the additional paths UL SRS-RSRPP measurements if those are provided together

-	For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-RSRPP measurements, then reported UL SRS-RSRPP value for the first path shall not be lower than the corresponding UL SRS-RSRPP for the first path of any of the individual receiver branches

Option 2 (RX diversity for UL SRS-RSRP)

The same RX branch(es) as applied for UL SRS-RSRP measurements are used for UL SRS-RSRPP measurements (i.e., the first and additional paths UL SRS-RSRPP if those are provided)

For these two options, RAN3 asks RAN1 to provide feedback on whether to support gNB to signal the Rx Diversity options to the LMF. In addition, RAN3 as RAN1 to provide feedback on the UL-SRS-RSRPP value range.

In our understanding, the two options were agreed upon with the consideration of potential implementation convenience in gNB. RAN1 has so far no evaluation on the potential impact of the two options on the positioning performance of the UL-AoA or other NR positioning methods. In another word, it is unclear what is the difference between these two options on the positioning performance, which may depend on the positioning algorithms used in LMF.

Although the benefit for gNB to report the option is not evaluated in RAN1, we think that the information could be especially useful when AI/ML algorithms are used for UL SRS-RSRPP for positioning, because of the different implications of these two options. Having the information on which option is used may be useful for the establishment, training and verification of the AI/ML model for positioning in the LMF. Therefore, our suggestion is to support gNB to report the option that is used for deriving the UL SRS-RSRPP.

For the reporting value ranges of the first path UL SRS-RSRPP, we have the following agreement in RAN1#108e, which is also included in the updated higher-layer parameter list [3]. The reporting of the additional path RSRPP was discussed in the previous meeting without conclusion. It is expected the issue will be resolved in this meeting.

- Agreement The gNB can be requested to measure and report to the LMF the UL SRS-RSRPP of the first path using at least the following option: - Option 1: if the UL SRS-RSRPP is reported only for the first path (and UL SRS-RSRP is not reported), then the UL SRS-RSRPP of the first path is reported using the absolute reporting as defined in the mapping Table 13.3.1-1 of TS 38.133 for SRS-RSRP. - Agreement If both UL SRS-RSRP and UL SRS-RSRPP of the first path are reported, then UL SRS-RSRPP of the first path is reported using the absolute reporting as defined in the mapping Table 13.3.1-1 of TS 38.133 for SRS-RSRP.

Proposal 1:  Provide the following response on RAN3’s question on UL-SRS-RSRPP:

- Support gNB to signal the information on the Rx Diversity option used for the UL-SRS-RSRPP to LMF.

- Absolute reporting will be used for the first path UL-SRS-RSRPP as defined in the mapping Table 13.3.1-1 of TS 38.133 for SRS-RSRP.

SRS Port Index

The proposal of reporting SRS port IDs with the UL positioning measurements has been discussed in previous RAN1 meetings without conclusion. In RAN1#108e, the proposal of letting RAN3 decide whether to support SRS port ID reporting was discussed, for which the number of supporting companies equals the number of not supporting companies [2]. Thus, for RAN3’s request of providing feedback on the issue, our view is that RAN1 can simple inform RAN3 that RAN1 does not reach the consensus to support it, and no further discussion on this issue in Rel-17.

Proposal 2:  Provide the following response to RAN3’s question on the reporting of the SRS Port Index:

- RAN1 has discussed, but cannot reach the consensus, the support of reporting the SRS port index to LMF when SRS resource for MIMO is used for positioning measurements. RAN1 decides no further discussion on this issue in Rel-17.

On-demand PRS

In RAN3’s LS, RAN3 inform RAN1 that on-demand PRS transmission is non-UE-associated (i.e., cell specific) and thus an ON/OFF indication per UE does not make sense. Thus, it is considered to have an OFF indication per TRP to minimize the transmission power.

Agreement - From RAN1 perspective, for LMF-initiated request of on-demand DL PRS, the following group of on-demand DL PRS parameters is defined and signaled - per resource set per positioning frequency layer per FR - DL PRS Periodicity - DL PRS Resource Bandwidth - DL PRS Resource Repetition Factor - Number of DL PRS Resource Symbols per DL PRS Resource - DL-PRS CombSizeN - Two options for indication of DL PRS QCL-Info, either - Option 1: per resource set per positioning frequency layer per FR - LMF recommends a list of QCL sources - Option 2: per resource set per positioning frequency layer per FR - LMF requests to provide the QCL information in the assistance data in NRPPa - per FR - Number of DL PRS frequency layers - either per resource set per positioning frequency layer or per UE - Start/end time of DL PRS transmission - either per resource, or per resource set, or per UE - ON/OFF indicator (for LMF initiated request only)

We basically share the similar view that “ON/OFF indicator” can be per TRP, although the on-demand PRS may be triggered due to the need to support the positioning of particular UE(s). In addition, “the Start/end time of DL PRS transmission” should also be per TRP, i.e., which means request a TRP for the transmission of all DL PRS resources instead of some of the DL PRS resources.

Proposal 3:  Make the following modifications on the previous agreement and provide the updated agreement as the response to RAN3’s question on the On-demand PRS:

- From RAN1 perspective, for LMF-initiated request of on-demand DL PRS, the following group of on-demand DL PRS parameters is defined and signaled - per resource set per positioning frequency layer per FR - DL PRS Periodicity - DL PRS Resource Bandwidth - DL PRS Resource Repetition Factor - Number of DL PRS Resource Symbols per DL PRS Resource - DL-PRS CombSizeN - Two options for indication of DL PRS QCL-Info, either - Option 1: per resource set per positioning frequency layer per FR - LMF recommends a list of QCL sources - Option 2: per resource set per positioning frequency layer per FR - LMF requests to provide the QCL information in the assistance data in NRPPa - per FR - Number of DL PRS frequency layers - either per resource set per positioning frequency layer or per TRPUE - Start/end time of DL PRS transmission - either per resource, or per resource set, or per TRPUE - ON/OFF indicator (for LMF initiated request only)

Conclusions

In this contribution, we discussed the responses to the questions in RAN3 LS (R1-2203040 (R3-222721). Based on the discussion, we have the following proposals. The corresponding draft response LS is also prepared in [4].

Proposal 1:  Provide the following response on RAN3’s question on UL-SRS-RSRPP:

- Support gNB to signal the information on the Rx Diversity option used for the UL-SRS-RSRPP to LMF.

- Absolute reporting will be used for the first path UL-SRS-RSRPP as defined in the mapping Table 13.3.1-1 of TS 38.133 for SRS-RSRP.

Proposal 2:  Provide the following response to RAN3’s question on the reporting of the SRS Port Index:

- RAN1 has discussed, but cannot reach the consensus, the support of reporting the SRS port index to LMF when SRS resource for MIMO is used for positioning measurements. RAN1 decides no further discussion on this issue in Rel-17.

Proposal 3:  Make the following modifications on the previous agreement and provide the updated agreement as the response to RAN3’s question on the On-demand PRS:

- From RAN1 perspective, for LMF-initiated request of on-demand DL PRS, the following group of on-demand DL PRS parameters is defined and signaled - per resource set per positioning frequency layer per FR - DL PRS Periodicity - DL PRS Resource Bandwidth - DL PRS Resource Repetition Factor - Number of DL PRS Resource Symbols per DL PRS Resource - DL-PRS CombSizeN - Two options for indication of DL PRS QCL-Info, either - Option 1: per resource set per positioning frequency layer per FR - LMF recommends a list of QCL sources - Option 2: per resource set per positioning frequency layer per FR - LMF requests to provide the QCL information in the assistance data in NRPPa - per FR - Number of DL PRS frequency layers - either per resource set per positioning frequency layer or per TRPUE - Start/end time of DL PRS transmission - either per resource, or per resource set, or per TRPUE - ON/OFF indicator (for LMF initiated request only)

References

- R1-2203040 (R3-222721), “Questions concerning the implementation of RAN1 agreements in NRPPa,” RAN3 (Ericsson)

- R1-2202500, “FL Summary #4 for accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays,” Moderator (CATT)

- R1-2202542, “LS on updated Rel-17 LTE and NR higher-layers parameter list”, RAN1

- R1-220xxxx, “Draft Reply LS on Questions concerning the implementation of RAN1 agreements in NRPPa”, CATT