3GPP TSG RAN WG1 #109-e			R1-2203413

e-Meeting, May 9th – 20th, 2022

Title:	Draft reply LS on questions concerning the implementation of RAN1 agreements in NRPPa

Response to:	R1-2203040 (R3-2222721) LS on questions concerning the implementation of RAN1 agreements in NRPPa

Release:	Rel-17

Work Item:	NR_pos_enh-Core

Source:	CATT[to be RAN1]

To:	RAN3

Cc:	RAN2, RAN4

Contact Person:

#### Name:	Ren Da

E-mail Address:	renda@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

1. Overall Description:

RAN1 thanks RAN3 LS (R1-2203040/R3-2222721), which asks RAN1 to provide feedback on the issues included in the following table.

Table 1: Issues requiring RAN1 inputs:

| Topic | Issue | Required RAN1 work |
| --- | --- | --- |
| UL-SRS-RSRPP | RAN1 has defined a new measurement for UL SRS-RSRPP: Agreement#6: Definition UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power of the received UL SRS signal configured for the measurement at the i-th path delay of the channel response, where UL SRS-RSRPP for 1st path delay is the power corresponding to the first detected path. For frequency range 1, the reference point for the UL SRS-RSRPP shall be the antenna connector of the gNB. For frequency range 2, UL SRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. Note: The following two options are supported by gNB to LMF: Option 1 (RX diversity for the first path UL SRS-RSRPP) - The same RX branch(es) as applied for the first path UL SRS-RSRPP measurements are used for the additional paths UL SRS-RSRPP measurements if those are provided together - For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-RSRPP measurements, then reported UL SRS-RSRPP value for the first path shall not be lower than the corresponding UL SRS-RSRPP for the first path of any of the individual receiver branches Option 2 (RX diversity for UL SRS-RSRP) The same RX branch(es) as applied for UL SRS-RSRP measurements are used for UL SRS-RSRPP measurements (i.e., the first and additional paths UL SRS-RSRPP if those are provided). RAN3 will define the value range of the UL-SRS-RSRPP measurement for the i-th path as an INTEGER (0..126). Some companies question whether the information on the RX Diversity options, captured as a note in the RAN1 agreement, need to be signalled to LMF along with the UL-SRS-RSRPP measurement. | RAN1 to feedback if information on the Rx Diversity options needs to be signalled to LMF and to comment on the UL-SRS-RSRPP value range, if needed? |
| TEG | Some companies in RAN3 believe it is beneficial to signal the SRS port index to the LMF, so that LMF can group measurements based on Port index. RAN3 would like to know if SRS Port Index needs to be signalled to the LMF when SRS resource for MIMO is used? | RAN1 to feedback if information on the SRS port index needs to be signalled to LMF when SRS resource for MIMO is used. |
| On demand PRS | RAN3 has observed the following RAN1 agreement on On-demand PRS ON/OFF indicator (for LMF initiated request only): “In "On-demand PRS information for LMF-initiated on-demand DL PRS requests"; either per resource, or per resource set, or per UE”. RAN3 would like to inform RAN1 that the procedures defined by RAN3 for on-demand PRS transmission are non-UE-associated (i.e., cell specific) and thus an ON/OFF indication per UE does not make sense. Instead, it is considered to have an OFF indication per TRP to minimize the transmission power. | RAN1 to take into account and update their agreed parameter lists for On-demand PRS. |

RAN1 has discussed above issues in RAN1#109e, and would provide the following responses to RAN3’s questions.

For RAN3’s question on UL-SRS-RSRPP, RAN1 would provide the following response:

- Support gNB to signal the information on the Rx Diversity option used for the UL-SRS-RSRPP to LMF.

- Absolute reporting will be used for the first path UL-SRS-RSRPP as defined in the mapping Table 13.3.1-1 of TS 38.133 for SRS-RSRP.

For the RAN3’s question on SRS Port Index, RAN1 would provide the following response:

- RAN1 has discussed, but cannot reach the consensus, the support of reporting the SRS port index to LMF when the SRS resource for MIMO is used for positioning measurements. RAN1 decides no further discussion on this issue in Rel-17.

For the RAN3’s question on the On-demand PRS, RAN1 would provide the following response:

- RAN1 has made the following modification to the previous RAN1 agreement.

- From RAN1 perspective, for LMF-initiated request of on-demand DL PRS, the following group of on-demand DL PRS parameters is defined and signaled - per resource set per positioning frequency layer per FR - DL PRS Periodicity - DL PRS Resource Bandwidth - DL PRS Resource Repetition Factor - Number of DL PRS Resource Symbols per DL PRS Resource - DL-PRS CombSizeN Two options for indication of DL PRS QCL-Info, either - Option 1: per resource set per positioning frequency layer per FR - LMF recommends a list of QCL sources - Option 2: per resource set per positioning frequency layer per FR - LMF requests to provide the QCL information in the assistance data in NRPPa - per FR - Number of DL PRS frequency layers - either per resource set per positioning frequency layer or per TRP UE - Start/end time of DL PRS transmission - either per resource, or per resource set, or per TRP UE - ON/OFF indicator (for LMF initiated request only)

2. Actions:

To RAN2

ACTION: 	RAN1 respectfully requests RAN3 to take the above feedback on RAN3’s questions into account in their future work.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #109-e	16 May – 27 May 2022		E-Meeting

TSG-RAN WG1 Meeting #110	22 Aug – 26 Aug 2022		Toulouse, France