3GPP TSG RAN WG1 #109-e		                                                      R1-2203419

e-Meeting, May 9th – 20th, 2022

Source:	CATT

Title:	Discussion on intra-UE multiplexing/prioritization for eURLLC

Agenda Item:	7.2.5

Document for:	Discussion and Decision

Introduction

For intra-UE prioritization, option 2/3/5 as given below were extensively discussed in RAN1#108-e to clarify the UE behavior [1]. Some issues were raised and discussed for option 5, and no benefit was observed for option 5 compared with option 2/3.

- Option 2 (v2): The UE does not use the outcome of intermediate multiplexing for HP channels to cancel LP channels. - Any HP channel with a corresponding DCI that overrides or overlaps with a HP channel that overlaps with a LP channel shall meet the cancellation timeline, namely all HP DCIs corresponding to these HP channels must arrive Tproc,2+d1 before the earliest symbol of the LP channel that would be cancelled by the any of these HP channels. - All HP PUCCH/PUSCH channels except the final HP PUCCH/PUSCH that gets transmitted by the UE are intermediate channel - Option 2’ (updated): The UE does not use the outcome of intermediate multiplexing for HP channels to cancel LP channels. (from Samsung) - Any HP PUCCH channel that overrides or overlaps with a HP PUCCH channel that overlaps with a LP channel shall meet the cancellation timeline, namely all HP DCIs must arrive Tproc,2+d1 before the earliest symbol that would be cancelled by the final HP PUCCH channel. - If a UE detects a first DCI format indicating a first resource for a PUCCH transmission with corresponding HARQ-ACK information in a slot and also detects at a later time a second DCI format indicating a second resource for a PUCCH transmission with corresponding HARQ-ACK information in the slot, UE does not expect the second resource starts earlier than the start of the first resource. - All HP PUCCH/PUSCH channels except the final HP PUCCH/PUSCH that gets transmitted by the UE are intermediate channels. - Option 3: [No change from the spec is needed.] Clarify that the “before or after” term in Claus 9 in 38.213 is interpreted as: - the UE checks overlapping between HP and LP channel for each HP grant it receives, including any intermediate HP channel that results from UCI multiplexing and PUCCH overriding triggered by each of the HP grant. - Option 3a: [No change from the spec is needed.] Clarify that the “before or after” term in Claus 9 in 38.213 is interpreted as: - A UE checks the overlap between a HP channel and a low priority channel before multiplexing. If there is an overlap, the LP channel gets cancelled. If not, a UE performs multiplexing across the HP PUCCH channels. If then there is an overlap with a LP channel, the LP channel gets cancelled. Then, multiplexing between PUCCH and PUSCH is performed. If then there is an overlap with a LP channel, the LP channel gets cancelled - Option 3b: - Cancellation timeline needs to be satisfied for a group of overlapping HP channels as long as one of the HP channels overlaps with a LP channel. - HP PUCCH for HARQ-ACK indicated by each DCI can cancel LP. - Final HP PUCCH or PUSCH is used to cancel LP. - Option 4: whether the intermediate HP channels is used to cancel the LP channels is left to UE implementation. - Option 5: The UE makes a determination about canceling the LP channel at the cancellation deadline. This determination is based on the multiplexing/overriding of the HP channels that are determined up to the cancellation deadline. - Multiplexing/overriding of the HP channels are performed based on their associated timelines defined in R15. - Each and every dynamically scheduled HP channel as well as the HP channels that are the result of the HP channel multiplexing/overriding and are overlapping with a LP channel should satisfy the cancellation timeline, i.e., the gap between the ending symbol of the HP DCI to the starting symbol of that HP channel should be at least Tproc,2+d1. - The UE cancels the LP channel starting from the first symbol that overlaps with the HP channel at the latest, i.e., the current specification wording is kept. - Once a LP channel is determined to be cancelled at the cancellation deadline, a UE is not expected to revert its decision.

In this contribution, we further discuss our understanding on option 2 and option 3.

Discussion

Option 3 is aligned with previous agreements for intra-UE multiplexing and prioritization, but it was considered with extra complexity since UE should determine all the intermediate channels to make sure that gNB and UE have same understanding on whether a LP channel would be cancelled. UE vendors have concern on this since they consider a reasonable UE implementation should not be required to generate unnecessary intermediate channels.

Then, option 2 was proposed to reduce the UE implementation complexity, since UE does not need to check cancellation timeline and not need to determine all the HP intermediate channels. In option 2, gNB should make sure that all HP channels with corresponding DCIs that override or overlap with a HP channel that overlaps with a LP channel shall meet the cancellation timeline, namely all DCIs corresponding to these HP channels must arrive Tproc,2+d1 before the earliest symbol of these HP channels.

As shown in Figure 1, a PUCCH with HP HARQ-ACK overlaps with a LP PUCCH, and a PUCCH with positive HP SR overlaps with the PUCCH with HP HARQ-ACK, but does not overlap with the LP PUCCH. According to the description of option 2, DCI for the HP HARQ-ACK should be earlier than time point T1, which is Tproc,2+d1 before the start of the PUCCH with HP HARQ-ACK.

Figure 1 Start time of cancellation timeline for PUCCH overlapping

For another case shown in Figure 2, a PUCCH with HP HARQ-ACK-1 overlaps with a LP PUCCH, and the PUCCH with HP HARQ-ACK-1 is overridden by HP HARQ-ACK-2, but PUCCH with HP HARQ-ACK-2 does not overlap with the LP PUCCH. According to the description of option 2, DCI for the HP HARQ-ACK-1 and HP HARQ-ACK-2 should be earlier than time point T1, which is Tproc,2+d1 before the start of the PUCCH with HP HARQ-ACK-2.

Figure 2 Start time of cancellation timeline for PUCCH overriding

Although we think option 3 is clear and has minimum scheduling restriction, we are willing to compromise to option 2 to address the concern on implementation complexity. Hence, we have the following proposal.

Proposal 1: Either option 2 or option 3 is adopted for Rel-16 intra-UE prioritization.

Conclusion

In this contribution, we discuss some considerations for intra-UE multiplexing/prioritization and give the following proposal.

Proposal 1: Either option 2 or option 3 is adopted for Rel-16 intra-UE prioritization.

References

- R1-2112889, Summary of email discussion [107-e-NR-L1enh-URLLC-05] on UCI multiplexing and prioritization in Rel-16, Moderator (Apple)