3GPP TSG RAN WG1 #109-e		   R1-2203423

e-Meeting, May 9th – 20th, 2022

Source: 	CATT

Title:	Maintenance issues on SRS

Agenda Item:	8.1.3

Document for:	Discussion and Decision

- Introduction

In this contribution, some maintenance issues on Rel-17 SRS are analyzed.

- Discussion

  - Collision handling of AP-SRS

For Rel-17 flexible AP-SRS triggering, an SRS resource set is transmitted in the t-th available slot counting from n+k, where n is the slot triggering the set, and t is selected by DCI from multiple values configured by RRC. The determination of “available slot” is related to the slot format when the SRS resource set is triggered. Since the slot format can be dynamically changed by SFI indication and the scheduling of DL/UL signals in flexible slots, it is difficult for gNB to ensure that the two AP-SRS resource sets configured with the same trigger state are always in different slots. Therefore collision handling for multiple AP-SRS resource sets is necessary. The priority of SRS resource sets can be determined by CC ID first and then set ID. That is, for two SRS resource sets in different CCs, the SRS resource set in the CC with lower ID is of higher priority than the other SRS resource set, and for two SRS resource sets in the same CC, the SRS resource set with lower set ID is of higher priority than the other SRS resource set.

Proposal 1:

- Introduce the following dropping rule when collision happens among multiple aperiodic SRS resource sets in a same CC or different CCs:

  - Rule 2– Based on set ID and CC ID.

On collision of Rel-17 flexible aperiodic SRS and PUSCH/PUCCH/PRACH, collision handling is also desired to increase the scheduling flexibility. For the collision in the same CC, same handling rule as that specified in Rel-16 can be used. For the collision in different CCs, similar dropping rule as that specified for collision handling of aperiodic SRS for antenna switching and other UL signals can be used, i.e., if the SRS overlaps with PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH, the SRS in the overlapped symbols are dropped, otherwise the other UL signal is dropped.

Proposal 2:

- Introduce the following dropping rule when overlapping happens between a aperiodic SRS resource in a CC and a PUSCH/PUCCH/PRACH in another CC and result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability:

  - If the SRS overlaps with PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH, the SRS in the overlapped symbols are dropped;

  - Otherwise, the PUSCH/PUCCH is dropped.

  - Flexible AP-SRS triggering

In previous meetings, AP-SRS triggering with DCI format 0_1/0_2 without data and without CSI request is supported. It is natural to support AP-SRS triggering with DCI format 0_1/0_2 without data but with a non-zero "CSI request" where the associated "reportQuantity" in CSI-ReportConfig set to "none" for all CSI report(s) triggered by "CSI request" in this DCI format 0_1 or 0_2.

Proposal 3:

- Support DCI format 0_1 and 0_2 to trigger aperiodic SRS without data but with a non-zero "CSI request" where the associated "reportQuantity" in CSI-ReportConfig set to "none" for all CSI report(s) triggered by "CSI request" in this DCI format 0_1 or 0_2.

We can capture the proposal in the spec by clarify in TS 38.214 that upon detection of a DCI format 0_1 or 0_2  with 'UL-SCH indicator' set to '0' and with a non-zero 'CSI request' where the associated reportQuantity in CSI-ReportConfig set to 'none' for all CSI report(s) triggered by 'CSI request' in this DCI format 0_1 or 0_2, the UE ignores all fields in this DCI except the 'CSI request' and 'SRI request' and the UE shall not transmit the corresponding PUSCH as indicated by this DCI format 0_1 or 0_2.

The corresponding TP is as follows:

--------------------------------------- Start of TP for TS38.214-------------------------------------------

6.1	UE procedure for transmitting the physical uplink shared channel

<Unrelated part omitted>

A UE shall upon detection of a PDCCH with a configured DCI format 0_0, 0_1 or 0_2 transmit the corresponding PUSCH as indicated by that DCI unless the UE does not generate a transport block as described in [10, TS38.321]. Upon detection of a DCI format 0_1 or 0_2  with 'UL-SCH indicator' set to '0' and with a non-zero 'CSI request' where the associated reportQuantity in CSI-ReportConfig set to 'none' for all CSI report(s) triggered by 'CSI request' in this DCI format 0_1 or 0_2, the UE ignores all fields in this DCI except the 'CSI request' and 'SRI request' and the UE shall not transmit the corresponding PUSCH as indicated by this DCI format 0_1 or 0_2. ……

--------------------------------------- End of TP for TS38.214----------------------------------------

  - TPs on available slot for flexible SRS triggering

TP 1:

Reason for change:

For Rel-17 flexible AP-SRS triggering, it is specified in TS 38.214 that from the first symbol carrying the SRS request DCI to the last symbol of the triggered SRS resource set, UE does not expect to receive SFI indication, UL cancellation indication or dynamic scheduling of DL channel/signal(s) on flexible symbol(s) that may change the determination of available slot. Which symbol is the first symbol carrying the SRS request DCI is not clear.

Summary for change:

Change “the first symbol carrying the SRS request DCI” to “the first symbol of the PDCCH carrying the triggering command”.

Consequences if not approved

It is not clear when the UE does not expect to receive SFI indication, UL cancellation indication or dynamic scheduling of DL channel/signal(s) on flexible symbol(s) that may change the determination of available slot.

Proposed TP

--------------------------------------- Start of TP for TS38.214------------------------------------------

6.2.1	UE sounding procedure

<Unrelated part omitted>

-	An available slot is a slot satisfying there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. From the first symbol of the PDCCH carrying the triggering command to the last symbol of the triggered SRS resource set, UE does not expect to receive SFI indication, UL cancellation indication or dynamic scheduling of DL channel/signal(s) on flexible symbol(s) that may change the determination of available slot.

<Unrelated part omitted>

--------------------------------------- End of TP for TS38.214----------------------------------------

TP 2:

Reason for change:

For Rel-17 flexible AP-SRS triggering, it is specified in TS 38.214 that for SRS resource set configured with availableSlotOffset parameter, each of resource set is configured with K values of availableSlotOffset parameter. The definition of K in the spec is not clear.

Summary for change:

Remove K and clarify that for SRS resource set configured with availableSlotOffset parameter, each of resource set is configured with same number of values of availableSlotOffset parameter

Consequences if not approved

How to configure availableSlotOffset parameters for SRS resource set configured with availableSlotOffset parameter is not clear.

Proposed TP

--------------------------------------- Start of TP for TS38.214-------------------------------------------

6.2.1	UE sounding procedure

<Unrelated part omitted>

-	t is configured via higher layer parameter availableSlotOffset with up to four different values for each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission. When one or more SRS resource sets across all configured BWPs in a component carrier are configured, and at least one resource set is configured with availableSlotOffset parameter of more than one values, the indicated value of availableSlotOffset is indicated by SOI field in DCI scheduling PUSCH/PDSCH and DCI 0_1/0_2 without data and without CSI request described in [5, TS 38.212]. The UE shall apply indicated value of availableSlotOffset set specifically for those sets with configured availableSlotOffset parameter. When one or more SRS resource sets across all configured BWPs in a component carrier are configured and at least one resource set is configured with availableSlotOffset parameter, and the availableSlotOffset parameter for each SRS resource set has only one value, the UE shall apply the configured value of availableSlotOffset specifically for those sets with configured availableSlotOffset parameter. For SRS resource set configured with availableSlotOffset parameter, each of resource set is configured with same number of values of availableSlotOffset parameter. For SRS resource set configured without availableSlotOffset parameter, t = 0 is applied for each of resource set.

<Unrelated part omitted>

--------------------------------------- End of TP for TS38.214----------------------------------------

  - TPs on SRS for antenna switching

TP1:

Reason for change:

In TS 38.214, for 1T=1R, or 2T=2R, or 4T=4R, if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets], configuring with SRS resource set configured with resourceType in SRS-ResourceSet set to 'aperiodic' is not allowed.

Summary for change:

Clarify in TS 38.214 that for 1T=1R, or 2T=2R, or 4T=4R, if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets], up to two AP-SRS resource sets, or two semi-persistent SRS resource set and one periodic SRS resource sets can be configured.

Consequences if not approved

Configuring aperiodic SRS resource set for a UE indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets] for 1T=1R, or 2T=2R, or 4T=4R is not allowed, which is not aligned with meeting agreement.

Proposed TP

--------------------------------------- Start of TP for TS38.214-------------------------------------------

6.2.1.2	UE sounding procedure for DL CSI acquisition

<Unrelated part omitted>

-	For 1T=1R, or 2T=2R, or 4T=4R, up to two SRS resource sets each with one SRS resource, where the number of SRS ports for each resource is equal to 1, 2, or 4, or two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets], where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, or

<Unrelated part omitted>

--------------------------------------- End of TP for TS38.214----------------------------------------

TP2:

Reason for change:

For antenna switching with xTyR in Rel-16, when multiple AP-SRS resource sets are configured, the multiple AP-SRS resource sets should be configured with the same values of alpha, p0, pathlossReferenceRS, and srs-PowerControlAdjustmentStates in SRS-ResourceSet. And the value of aperiodicSRS-ResourceTrigger or the value of an entry in AperiodicSRS-ResourceTriggerList in each SRS-ResourceSet should be the same, and the value of slotOffset in each SRS-ResourceSet should be different. These restrictions are not included in the spec for 1T2R, 2T4R, 1T6R, 1T8R, 2T6R, 2T8R, and 4T8R in Rel-17.

Summary for change:

Clarify in TS 38.214 that for 1T2R, 2T4R, 1T6R, 1T8R, 2T6R, 2T8R, and 4T8R, when multiple AP-SRS resource sets are configured, the multiple AP-SRS resource sets are configured with the same values of alpha, p0, pathlossReferenceRS, and srs-PowerControlAdjustmentStates in SRS-ResourceSet. And the value of aperiodicSRS-ResourceTrigger or the value of an entry in AperiodicSRS-ResourceTriggerList in each SRS-ResourceSet is the same, and the value of slotOffset in each SRS-ResourceSet is different.

Consequences if not approved

Accurate DL CSI acquisition from aperiodic SRS for antenna switching for 1T2R, 2T4R, 1T6R, 1T8R, 2T6R, 2T8R, and 4T8R cannot be guaranteed.

--------------------------------------- Start of TP for TS38.214-------------------------------------------

6.2.1.2	UE sounding procedure for DL CSI acquisition

<Unrelated part omitted>

-	For 1T2R, if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets] and/or a capability for [extension of aperiodic antenna switching SRS configuration]:

<Unrelated part omitted>

-	zero or one SRS resource set with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is not indicating a capability for [extension of aperiodic antenna switching SRS configuration], or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is indicating a capability for [extension of aperiodic antenna switching SRS configuration], where in the case of one resource set, a total of two SRS resources transmitted in different symbols of one slot and where the SRS port of the second resource in the given set is associated with a different UE antenna port than the SRS port of the first resource in the same set. In the case of two resource sets, a total of two SRS resources transmitted in different symbols of two different slots, and where the SRS port of each SRS resource in the given two sets is associated with a different UE antenna port. The two sets are each configured with one SRS resource. The UE shall expect that the two SRS resource sets are both configured with the same values of the higher layer parameters alpha, p0, pathlossReferenceRS, and srs-PowerControlAdjustmentStates in SRS-ResourceSet. The UE shall expect that the value of the higher layer parameter aperiodicSRS-ResourceTrigger or the value of an entry in AperiodicSRS-ResourceTriggerList in each SRS-ResourceSet is the same, and the value of the higher layer parameter slotOffset in each SRS-ResourceSet is different, or

<Unrelated part omitted>

-	For 2T4R, if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets] and/or a capability for [extension of aperiodic antenna switching SRS configuration]:

<Unrelated part omitted>

-	zero or one SRS resource set configured with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is not indicating a capability for [extension of aperiodic antenna switching SRS configuration], or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is indicating a capability for [extension of aperiodic antenna switching SRS configuration], where in the case of one resource set, a total of two SRS resources transmitted in different symbols in the same slot, each SRS resource in a given set consisting of two SRS ports, and the SRS port pair of the second resource is associated with a different UE antenna port pair than the SRS port pair of the first resource. In the case of two resource sets, a total of two SRS resources transmitted in different symbols of two different slots, and where the SRS port pair of each SRS resource in the given two sets is associated with a different UE antenna port pair. The two sets are each configured with one SRS resource. The UE shall expect that the two SRS resource sets are both configured with the same values of the higher layer parameters alpha, p0, pathlossReferenceRS, and srs-PowerControlAdjustmentStates in SRS-ResourceSet. The UE shall expect that the value of the higher layer parameter aperiodicSRS-ResourceTrigger or the value of an entry in AperiodicSRS-ResourceTriggerList in each SRS-ResourceSet is the same, and the value of the higher layer parameter slotOffset in each SRS-ResourceSet is different, or

<Unrelated part omitted>

-	For 1T6R, zero or one or two or three SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic', where in the case of one resource set a total of six SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of each resource in the set is associated with a different UE antenna port. In the case of two resource sets a total of six SRS resources transmitted in different symbols of two different slots, and where the SRS port of each SRS resource in the given two sets is associated with a different UE antenna port. In the case of three resource sets a total of six SRS resources transmitted in different symbols of three different slots, and where the SRS port of each SRS resource in the given three sets is associated with a different UE antenna port. The UE shall expect that the two or three SRS resource sets are configured with the same values of the higher layer parameters alpha, p0, pathlossReferenceRS, and srs-PowerControlAdjustmentStates in SRS-ResourceSet. The UE shall expect that the value of the higher layer parameter aperiodicSRS-ResourceTrigger or the value of an entry in AperiodicSRS-ResourceTriggerList in each SRS-ResourceSet is the same, and the value of the higher layer parameter slotOffset in each SRS-ResourceSet is different, or

<Unrelated part omitted>

-	For 1T8R, zero or two or three or four SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic', where in the case of two resource sets a total of eight SRS resources transmitted in different symbols of two different slots, and where the SRS port of each SRS resource in the given two sets is associated with a different UE antenna port. In the case of three resource sets a total of eight SRS resources transmitted in different symbols of three different slots, and where the SRS port of each SRS resource in the given three sets is associated with a different UE antenna port. In the case of four resource sets a total of eight SRS resources transmitted in different symbols of four different slots, and where the SRS port of each SRS resource in the given four sets is associated with a different UE antenna port. The UE shall expect that the two or three or four SRS resource sets are configured with the same values of the higher layer parameters alpha, p0, pathlossReferenceRS, and srs-PowerControlAdjustmentStates in SRS-ResourceSet. The UE shall expect that the value of the higher layer parameter aperiodicSRS-ResourceTrigger or the value of an entry in AperiodicSRS-ResourceTriggerList in each SRS-ResourceSet is the same, and the value of the higher layer parameter slotOffset in each SRS-ResourceSet is different,, or

<Unrelated part omitted>

-	For 2T6R, zero or one or two or three SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic', where in the case of one resource set has three SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a two SRS ports, and the SRS port pair of the resource in the set is associated with a different UE antenna port pair. In the case of two resource sets a total of three SRS resources transmitted in different symbols of two different slots, and where the SRS port pair of each SRS resource in the given two sets is associated with a different UE antenna port pair. One set is configured with two SRS resources and another set with one resource. In the case of three resource sets a total of three SRS resources transmitted in different symbols of three different slots, and where the SRS port pair of each SRS resource in the given three sets is associated with a different UE antenna port pair. The UE shall expect that the two or three SRS resource sets are configured with the same values of the higher layer parameters alpha, p0, pathlossReferenceRS, and srs-PowerControlAdjustmentStates in SRS-ResourceSet. The UE shall expect that the value of the higher layer parameter aperiodicSRS-ResourceTrigger or the value of an entry in AperiodicSRS-ResourceTriggerList in each SRS-ResourceSet is the same, and the value of the higher layer parameter slotOffset in each SRS-ResourceSet is different, or

<Unrelated part omitted>

-	For 2T8R, zero or one or two or three or four SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic', where in the case of one resource set has four SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a two SRS ports, and the SRS port pair of the  resource in the set is associated with a different UE antenna port pair. In the case of two resource sets a total of four SRS resources transmitted in different symbols of two different slots, and where the SRS port pair of each SRS resource in the given two sets is associated with a different UE antenna port pair. In the case of three resource sets a total of four SRS resources transmitted in different symbols of three different slots, and where the SRS port pair of each SRS resource in the given three sets is associated with a different UE antenna port pair. Two sets are configured with one SRS resource in each set and one resource set is configured with two resources. In the case of four resource sets a total of four SRS resources transmitted in different symbols of four different slots, and where the SRS port pair of each SRS resource in the given four sets is associated with a different UE antenna port pair. Four sets are configured with one SRS resource in each set. The UE shall expect that the two or three or four SRS resource sets are configured with the same values of the higher layer parameters alpha, p0, pathlossReferenceRS, and srs-PowerControlAdjustmentStates in SRS-ResourceSet. The UE shall expect that the value of the higher layer parameter aperiodicSRS-ResourceTrigger or the value of an entry in AperiodicSRS-ResourceTriggerList in each SRS-ResourceSet is the same, and the value of the higher layer parameter slotOffset in each SRS-ResourceSet is different, or

<Unrelated part omitted>

-	For 4T8R, zero or one or two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic', where in the case of one resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a four SRS ports, and the SRS ports of the resource in the set are associated with a different UE antenna ports. In the case of two resource sets a total of two SRS resources are transmitted in different symbols of two different slots, and where the SRS ports of each SRS resource in the given two sets are associated with a different UE antenna ports. The UE shall expect that the two SRS resource sets are both configured with the same values of the higher layer parameters alpha, p0, pathlossReferenceRS, and srs-PowerControlAdjustmentStates in SRS-ResourceSet. The UE shall expect that the value of the higher layer parameter aperiodicSRS-ResourceTrigger or the value of an entry in AperiodicSRS-ResourceTriggerList in each SRS-ResourceSet is the same, and the value of the higher layer parameter slotOffset in each SRS-ResourceSet is different.

<Unrelated part omitted>

--------------------------------------- End of TP for TS38.214----------------------------------------

TP3:

Reason for change:

The capability for antenna switching with xTyR is denoted as “txry” in Rel-16. In Rel-17, '1t8r' for 1T8R, '2t6r' for 2T6R, '2t8r' for 2T8R, '4t8r' for 4T8R are used in the spec, which is not aligned with that for xTyR in Rel-16.

Summary for change:

Change “ '1t8r' for 1T8R, '2t6r' for 2T6R, '2t8r' for 2T8R, '4t8r' for 4T8R” to “ 't1r8' for 1T8R, 't2r6' for 2T6R, 't2r8' for 2T8R, 't4r8' for 4T8R” in TS 38.214.

Consequences if not approved

The parameter of UE’s capability in TS38.214 for 1T8R, 2T6R, 2T8R, and 4T8R would not be aligned with the specs for UE’s capability.

Proposed TP

--------------------------------------- Start of TP for TS38.214-------------------------------------------

6.2.1.2	UE sounding procedure for DL CSI acquisition

When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set as 'antennaSwitching', the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitch ('t1r2' for 1T2R, 't1r1-t1r2' for 1T=1R/1T2R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't1r6' for 1T6R, 't1r8' for 1T8R, 't2r6' for 2T6R, 't2r8' for 2T8R, 't4r8' for 4T8R, 't1r1-t1r2-t1r4' for 1T=1R/1T2R/1T4R, 't1r4-t2r4' for 1T4R/2T4R, 't1r1-t1r2-t2r2-t2r4' for 1T=1R/1T2R/2T=2R/2T4R, 't1r1-t1r2-t2r2-t1r4-t2r4' for 1T=1R/1T2R/2T=2R/1T4R/2T4R, 't1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r1-t2r2' for 1T=1R/2T=2R, 't4r4' for 4T=4R, or 't1r1-t2r2-t4r4' for 1T=1R/2T=2R/4T=4R):

--------------------------------------- End of TP for TS38.214----------------------------------------

- Conclusions

In this contribution, we provide our views on maintenance issues of Rel-17 SRS enhancements. We propose several TPs on flexible AP-SRS triggering and SRS for antenna switching. Besides, we have the following proposals on flexible AP-SRS triggering:

Proposal 1:

- Introduce the following dropping rule when collision happens among multiple aperiodic SRS resource sets in a same CC or different CCs:

  - Rule 2– Based on set ID and CC ID.

Proposal 2:

- Introduce the following dropping rule when overlapping happens between a aperiodic SRS resource in a CC and a PUSCH/PUCCH/PRACH in another CC and result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability:

  - If the SRS overlaps with PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH, the SRS in the overlapped symbols are dropped;

  - Otherwise, the PUSCH/PUCCH is dropped.

Proposal 3:

- Support DCI format 0_1 and 0_2 to trigger aperiodic SRS without data but with a non-zero "CSI request" where the associated "reportQuantity" in CSI-ReportConfig set to "none" for all CSI report(s) triggered by "CSI request" in this DCI format 0_1 or 0_2.