3GPP TSG RAN WG1 #109-e              	                            R1-2203424

e-Meeting, May 9th – 20th, 2022

Source:	CATT, GOHIGH

Title:	Maintenance on resource allocation for power saving

Agenda Item:	8.11.1

Document for:	Discussion and Decision

Introduction

At RAN1#108-e meeting, the following agreements were made related to resource allocation for power saving in sidelink enhancements [1].

Agreement The lower bound of M value for CPS in the case of periodic transmission (contiguousSensingWindowPeriodic) for both resource (re)selection and re-evaluation / pre-emption checking is a non-zero value (lower bound for M is 5) Note: CATT indicated that they do not agree to the technical benefits of this agreement Agreement When a UE is triggered to perform re-evaluation and pre-emption checking for aperiodic transmission (Prsvp_TX=0) in slot n and the minimum M slots for CPS cannot be guaranteed, - UE senses in all available slots starting from the resource (re)selection trigger slot of the same TB to Tproc,0SL+Tproc,1SL slots earlier than tyiSL. - The UE re-evaluation and pre-emption checking is based on all available sensing results after n-T0 Conclusion The existing Step 5 and 5a are applicable for UE configured for partial sensing by its higher layer. Agreement In Step 6 c) of TS38.214 Section 8.1.4, when UE is configured with partial sensing by its higher layer, adopt the following changes: - t'n'SL=tyiSL-Tproc,1SL if slot tyiSL-Tproc,1SL belongs to the set t'0SL,t'1SL,...,t'T'max-1SL, otherwise, slot t'n'SL is the first slot after slot tyiSL-Tproc,1SL belonging to the set t'0SL,t'1SL,...,t'T'max-1SL. - Option D: Tscal=tyLSL-(tyiSL-Tproc,1SL) converted to milliseconds, where slot tyLSL is the last slot of the Y or Y’ candidate slots. - Slot tyiSL is the first slot of the selected/remaining set of Y or Y’ candidate slots.

In this contribution, the remaining issues of resource allocation mechanisms on power saving schemes for maintenance were discussed.

Discussion on remaining issues on sidelink enhancements

Remaining issues of PBPS and CPS combined RA scheme

For PBPS and CPS combined RA scheme, in RAN1 #106-e meeting, two cases were discussed and agreements were achieved as follows:

Agreement When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled, - For a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n, - The resource selection window (RSW) is [n+T1, n+T2], and T1 and T2 are defined in the same way according to step 1) of Rel-16 TS 38.214 Sec. 8.1.4 - FFS whether UE determines a new set of Y candidate slots within the RSW and monitors corresponding periodic sensing occasions between slot n and the first slot of the new Y candidate slots subject to processing constraints - FFS how to initialize a set of candidate resource (SA) for the triggered resource (re)selection procedure and which partial sensing scheme(s) and results can be used for resource exclusion in the resource (re)selection procedure - FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX - Note, re-evaluation and pre-emption checking based on periodic-based and contiguous partial sensing schemes is considered separately Agreement When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled, - For a resource (re)selection procedure triggered by periodic transmission (Prsvp_TX≠0) in slot n - A set of candidate resource (SA) is initialized to the set of selected Y candidate slots of PBPS - UE performs contiguous partial sensing in [n+TA, n+TB] for resource exclusion from the initialized candidate resource set (SA) - FFS details of TA and TB based on the agreement(s) from previous RAN1 meetings - Note, re-evaluation and pre-emption checking based on periodic-based and contiguous partial sensing schemes is considered separately FFS: The condition under which UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enable.

It was raised in previous discussion that the following agreement:

Agreement Conditions in which contiguous partial sensing is performed by UE, when at least all of the followings are met: - L1 [is expected to be or] is triggered by higher layer to report resources for resource (re-)selection in a mode 2 Tx pool - FFS: When the trigger will be received by L1 - The resource pool is (pre-)configured to enable partial sensing - Partial sensing is configured by higher layer in the UE

However, the agreement above for CPS is only about required conditions, and is not a necessary condition:

This is because it says that “Conditions in which contiguous partial sensing is performed by UE, when at least all of the followings are met”. This means even when all the pre-conditions (conditions described in the sub-bullet of the agreement) are met, the UE is still not mandatory to perform CPS. In fact, whether to perform CPS or not can be based on other criterions.

Observation 1:   In a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled, RAN1 agreement indicates the UE is not mandatory to perform both periodic-based and contiguous partial sensing. UE does not need to perform CPS if PBPS already provides enough sensing result.

Note this is also aligned with the current specification TS 38.214 [2], where it is described as follows,

TS 38.214: When a UE is triggered by higher layer to report resources for resource (re-)selection in a mode 2 Tx pool, the resource pool is (pre-)configured with allowedResourceSelectionConfig including partial sensing, and partial sensing is configured by higher layer, the UE may perform contiguous partial sensing.

Proposal 1: Since the current specification’s description regarding UE behaviour it is not mandatory to perform both periodic-based and contiguous partial sensing is aligned with previous RAN1 agreement, there is no change needed.

Remaining issues of re-evaluation and pre-emption checking for aperiodic transmission

In RAN1#107bis-e, the value zero was made as a working assumption for the lower bound of the (pre-)configured M value in CPS for a resource (re)selection procedure and re-evaluation/pre-emption checking triggered by aperiodic transmission. Based on this working assumption, the corresponding RRC parameter is set to a value range of [0]...30. However, in RAN1#108-e, there is no agreement on confirmation of the FFS.

Agreement (from RAN1#107bis-e): When UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure and re-evaluation/pre-emption checking triggered by aperiodic transmission (Prsvp_TX=0) in slot n, - For minimum size M of the CPS monitoring window [n+TA, n+TB]: - By default, M is 31 unless (pre-)configured with another value The range of (pre-)configured M is from 0 (working assumption) to 30

The deadlock was caused because RAN1 reached a completely different agreement in RAN1#108-e regarding UE behavior in the case of periodic transmission:

Agreement The lower bound of M value for CPS in the case of periodic transmission (contiguousSensingWindowPeriodic) for both resource (re)selection and re-evaluation / pre-emption checking is a non-zero value (lower bound for M is 5) Note: CATT indicated that they do not agree to the technical benefits of this agreement

To facilitate UE implementation, same parameter setting is preferred for both cases. In addition, the argument for the M value setting is same for both cases, it would be strange two different conclusions are made out of same underling argument. Therefore, after RAN1 reached the agreement of the M value for aperiodic transmission, the same M value selection for periodic case should be aligned with previous agreement in the case of aperiodic case.  However, this is not the case because some companies incorrectly linked this agreement with another discussion that if UE is allowed to only perform PBPS in the case of periodic transmission. It should be noted there are two different issues. Even is M cannot be set to 0, UE can still chose to perform only PBPS for periodic transmission as the current specification indicate. The decision of whether to perform CPS together with PBPS is not based on the M value setting. M is only considered if UE decide to perform both.

Considering the fact that agreement for the periodic case is already reached, to facilitate simplified RRC configuration, it is preferred that the aperiodic case also adopt the same M value setting as the periodic case.

Proposal 2: To facilitate simplified RRC configuration, it is preferred that the same M value setting as the periodic transmission is also adopted for aperiodic transmission.

Remaining issues of partial sensing mechanism

In RAN1 #106-e meeting, the following agreement was achieved regarding the condition of UE selecting Y candidate slot[3]:

Agreement When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled, - For a resource (re)selection procedure triggered by periodic transmission (Prsvp_TX≠0) in slot n - A set of candidate resource (SA) is initialized to the set of selected Y candidate slots of PBPS - UE performs contiguous partial sensing in [n+TA, n+TB] for resource exclusion from the initialized candidate resource set (SA) - FFS details of TA and TB based on the agreement(s) from previous RAN1 meetings - Note, re-evaluation and pre-emption checking based on periodic-based and contiguous partial sensing schemes is considered separately FFS: The condition under which UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled

In RAN1 #107-e meeting, the following agreement was achieved regarding CPS mechanism[5]:

Agreement When UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n, - The UE selects a set of Y’ candidate slots with corresponding PBPS and/or CPS results (if available) within the RSW. - If the total number of Y’ candidate slots is less than a (pre-)configured threshold Y’min, - How UE includes other candidate slots is up to UE implementation - Candidate resource set (SA) is initialized to the set of all single-slot candidate resources in the selected Y’ candidate slots. - For the CPS monitoring window [n+TA, n+TB]: - TA and TB are both selected such that UE has sensing results starting at M consecutive logical slots before ty0 and ending at Tproc,0 + Tproc,1 slots earlier than ty0. - FFS: By default, M is 31 unless (pre-)configured with another value, or where M is (pre-)configured based on transmission priority - FFS: The range of (pre-)configured M from a TBD lowest value up to 30 - When the minimum M slots for CPS cannot be guaranteed, support both - Option A, the UE ensures the Y’min criterion is fulfilled - Option B: UE performs random resource selection - When the UE performs Option A or Option B is up to UE implementation

According to the above agreements, it can be observed that the UE selects a set of Y candidate slot when UE performs PBPS and Prsvp_TX≠0, while the UE selects a set of Y’ candidate slot when UE performs at least CPS and Prsvp_TX=0. However, the corresponding description is somehow ambiguous in the current specification[2] and the above yellow high-light parts were also missed.

In order to capture RAN1 previous agreements clearly and correctly, the following TP#1 for section 8.1.4 of TS 38.214 is proposed.

- TP#1

| Reason for change: | Corrections on NR Sidelink enhancements |
| --- | --- |
| Summary of change: | In clause 8.1.4, capture entire agreements regarding Y/Y’ candidate slots in RAN1 #106-e and #107-e meeting. |
| Consequences if not approved: | Incorrect capture of RAN1 previous agreements |

----------------------------------------------TP to TS 38.214 clause 8.1.4 starts------------------------------------------------

8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<<< UNCHANGED PARTS OMITTED >>>

1)	A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot t'ySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing full sensing, in a set of Y candidate slots within the time interval n+T1,n+T2 for UE performing periodic-based partial sensing if Prsvp_TX≠0, correspond to one candidate single-slot resource, or in a set of Y' candidate slots within the time interval [n+T1,n+T2] for UE performing at least contiguous partial sensing if Prsvp_TX=0, correspond to one candidate single-slot resource, where

-	selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL  , where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP;

-	if  T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to  T2min ≤ T2 ≤ remaining packet delay budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots).

-	Y is selected by UE where Y≥Ymin.

-	Y' is selected by UE where Y'≥Ymin'. When the UE performs at least contiguous partial sensing and if Prsvp_TX=0, the UE selects a set of Y’ candidate slots with corresponding PBPS and/or CPS results (if available). If the number of candidate single-slot resources Y' is smaller than Ymin', it is up to UE implementation to include other candidate slots.

The total number of candidate single-slot resources is denoted by Mtotal.

<<< UNCHANGED PARTS OMITTED >>>

-----------------------------------------------TP to TS 38.214 clause 8.1.4 ends------------------------------------------------

In RAN1 #106b-e meeting, the following agreement was also achieved regarding CPS mechanism[4], but the yellow high-light part below was missed in RAN1 #107-e agreement as well as in the current specification. Meanwhile, the condition for adoption of CPS monitoring window also needs to be corrected.

Agreement When UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n, TA and TB for CPS monitoring window and a candidate resource set (SA) is initialized according to potentially one of the following approaches (final decision in RAN1#107-e). Other approaches are not precluded and the details in each approach can still be updated. - Approach 1: (SA is initialized based on at least slots with PBPS and/or CPS results and guarantee a minimum of M slots for CPS) - The UE selects a set of Y’ candidate slots with corresponding PBPS and/or CPS results (if available) within the RSW. - FFS how to handle the case if the total number of Y’ candidate slots is less than a (pre-)configured threshold Y’min without dropping the aperiodic transmission - FFS whether the Y’ candidate slots for aperiodic transmission is the same as the Y candidate slots in PBPS for periodic transmission of another TB(s) - FFS whether/how to prioritize/select resources based on partial sensing results. - FFS: How to select Y’ in case of CPS only - Candidate resource set (SA) is initialized to the set of all single-slot candidate resources in the selected Y’ candidate slots. - For the CPS monitoring window [n+TA, n+TB]: - TA and TB are both selected such that UE has sensing results for a minimum of M consecutive logical slots before ty0, where ty0 is the first slot of the selected Y’ candidate slots. - FFS: By default, M is 31 unless (pre-)configured with another value, or M is (pre-)configured based on transmission priority - FFS the range of (pre-)configured M from a TBD lowest value up to 30 - FFS: how to handle the case when the minimum M slots for CPS cannot be guaranteed - FFS: RSW in case of CPS only

Therefore, the following TP#2 for section 8.1.4 of TS 38.214 is proposed.

- TP#2

| Reason for change: | Corrections on NR Sidelink enhancements |
| --- | --- |
| Summary of change: | In clause 8.1.4, capture the agreements regarding Y’ candidate slots in RAN1 #106b-e meeting. In clause 8.1.4, current the condition for adoption of the defined CPS monitoring window. |
| Consequences if not approved: | Incorrect capture of RAN1 previous agreements |

----------------------------------------------TP to TS 38.214 clause 8.1.4 starts------------------------------------------------

8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<<< UNCHANGED PARTS OMITTED >>>

When the UE performs at least contiguous partial sensing and if Prsvp_TX=0, the sensing window is defined by the range of slots [n+TA, n+TB]. TA and TB are both selected such that the UE has sensing results starting at M consecutive logical slots before ty0SL and ending at Tproc,0SL+Tproc,1SL slots earlier than ty0SL, where ty0SL is the first slot of the selected Y’ candidate slots. The value of M is (pre-)configured with the contiguousSensingWindowAperiodic. If contiguousSensingWindowAperiodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection.

<<< UNCHANGED PARTS OMITTED >>>

-----------------------------------------------TP to TS 38.214 clause 8.1.4 ends------------------------------------------------

In RAN1 #106b-e meeting, the following agreement was achieved regarding the definition of CPS monitoring window:

Agreement When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled, - For a resource (re)selection procedure triggered by periodic transmission (Prsvp_TX≠0) in slot n, TA and TB for the CPS monitoring window is defined according to one of the followings: - n+TA is M logical slots earlier than slot ty0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than ty0SL, where ty0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. - By default, M is 31 unless (pre-)configured with another value.

From the agreement, it is clear that the above definition of CPS monitoring window can be applied only when “UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled” (i.e., condition #1) and “a resource (re)selection procedure is triggered by periodic transmission (Prsvp_TX≠0) in slot n” (i.e., condition #2) are both met. However, according to the current description in TS 38.214 section 8.1.4, the above definition of CPS monitoring window is adopted when condition #1 is met regardless of condition #2, which is not aligned with the previous RAN1 agreement.

Furthermore, under the above condition #1 and condition #2, there is not any agreement corresponding to the operation when the minimum M slots for CPS cannot be guaranteed. The operation only applies to the condition when the UE performs at least contiguous partial sensing and Prsvp_TX=0. Thus, the corresponding description in the current specification should be deleted.

Based on the above analysis, the following TP#3 for section 8.1.4 of TS 38.214 is proposed.

- TP#3

| Reason for change: | Corrections on NR Sidelink enhancements |
| --- | --- |
| Summary of change: | In clause 8.1.4, correct the condition for adoption of the defined CPS monitoring window and the operation of CPS. |
| Consequences if not approved: | Incorrect capture of RAN1 previous agreements |

----------------------------------------------TP to TS 38.214 clause 8.1.4 starts------------------------------------------------

8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<<< UNCHANGED PARTS OMITTED >>>

When the UE performs periodic-based partial sensing and contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) enabled and if  Prsvp_TX≠0, the sensing window is defined by the range of slots [n+TA, n+TB]. n+TA is M consecutive logical slots earlier than slot ty0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than ty0SL, where ty0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. Prsvp_TX≠0  The value of M is (pre-)configured with the contiguousSensingWindowPeriodic. If contiguousSensingWindowPeriodic is not (pre-)configured, M equals to 31. Prsvp_TX=0

<<< UNCHANGED PARTS OMITTED >>>

-----------------------------------------------TP to TS 38.214 clause 8.1.4 ends------------------------------------------------

In TS 38.214 section 8.1.4 step 2), UE behaviours of performing sensing are specified. Currently, the behaviours of four sensing mechanisms are described respectively. For periodic-based partial sensing, two paragraphs were added to specify the sensing slots. Considering the definition of Preserve and k are juxtaposed, the indentation positions should be the same. Hence, the following TP#4 for section 8.1.4 of TS 38.214 is proposed.

- TP#4

| Reason for change: | Corrections on NR Sidelink enhancements |
| --- | --- |
| Summary of change: | In clause 8.1.4, correct the format of the description of periodic-based partial sensing. |
| Consequences if not approved: | Editing error in the specfication |

----------------------------------------------TP to TS 38.214 clause 8.1.4 starts------------------------------------------------

8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<<< UNCHANGED PARTS OMITTED >>>

2)	The sensing window is defined by the range of slots [n –T0,n–Tproc,0SL), when the UE performs full sensing, where T0 is defined above and Tproc,0SL is defined in slots in Table 8.1.4-1 where μSL is the SCS configuration of the SL BWP. The UE shall monitor slots which belongs to a sidelink resource pool within the sensing window except for those in which its own transmissions occur. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.

When the UE performs periodic-based partial sensing, the UE shall monitor slots at ty-k×PreserveSL, where tySL is a slot of the selected candidate slots. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.

The value of Preserve corresponds to periodicSensingOccasionReservePeriodList if configured, otherwise, the values correspond to all periodicity from sl-ResourceReservePeriodList.

The UE monitors k sensing occasions determined by additionalPeriodicSensingOccasion, as previously described, and not earlier than n –T0. For a given periodicity Preserve, the values of k correspond to the most recent sensing occasion earlier than ty0SL-(Tproc,0SL+Tproc,1SL ) if additionalPeriodicSensingOccasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if additionalPeriodicSensingOccasion is (pre-)configured. ty0SL is the first slot of the selected Y candidate slots of PBPS.

<<< UNCHANGED PARTS OMITTED >>>

-----------------------------------------------TP to TS 38.214 clause 8.1.4 ends------------------------------------------------

Proposal 3: Modify the description of current specification regarding the remaining issues for partial sensing and adopt TP#1 ~#4.

Conclusion

In this contribution, the remaining issues of resource allocation mechanisms on power saving schemes for maintenance are discussed. Our proposals are given as follows:

Proposal 1: Since the current specification’s description regarding UE behaviour it is not mandatory to perform both periodic-based and contiguous partial sensing is aligned with previous RAN1 agreement, there is no change needed.

Proposal 2: To facilitate simplified RRC configuration, it is preferred that the same M value setting as the periodic transmission is also adopted for aperiodic transmission.

Proposal 3: Modify the description of current specification regarding the remaining issues for partial sensing and adopt TP#1 ~#4.

References

3GPP TSG RAN WG1 Meeting RAN1 #108-e Chairman's Notes, March. 2022.

3GPP TS 38.214, “NR; Physical layer procedures for data”, v17.1.0, 2022-03.

3GPP TSG RAN WG1 Meeting RAN1 #106-e Chairman's Notes, Aug. 2021.

3GPP TSG RAN WG1 Meeting RAN1 #106bis-e Chairman's Notes, Oct. 2021.

3GPP TSG RAN WG1 Meeting RAN1 #107-e Chairman's Notes, Nov. 2021.