3GPP TSG RAN WG1 #109-e                                                   R1-2203425

e-Meeting, May 9th –20th, 2022

Source:	CATT, GOHIGH

Title:	Maintenance on inter-UE coordination for mode 2 enhancements

Agenda Item:	8.11.2

Document for:	Discussion and Decision

Introduction

In RAN#95-e meeting, the following remaining issues were listed in RP-220945[1] and was endorsed to be solved in the maintenance stage:

- Finalization of UE-B’s behavior when it receives both preferred resource set and non-preferred resource set from the same UE-A or different UE-As

- Finalization of relationship between start/end slots of resource selection window used for sidelink transmission carrying inter-UE coordination information and start/end slots of resource selection window for determining the set of resources

In this contribution, we will further discuss and give our preference on the two remaining issues above.

Discussion on remaining issues of inter-UE coordination scheme 1

Remaining issues of UE-B’s behavior after receiving multiple resource sets

In RAN1#108-e meeting, the following agreements related to UE-B’s behaviors when receiving multiple preferred resource sets and/or non-preferred resource sets were achieved [2].

- Agreement - For UE-B’s behavior when UE-B receives multiple preferred resource sets from the same UE-A - It is up to UE-B implementation to use one or multiple of them in its resource (re)selection - Conclusion: UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the same UE-A - No RAN1 specification change to TS38.214 is deemed necessary in RAN1#108-e - For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A - FFS: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection - Agreement - For UE-B’s behavior when UE-B receives multiple preferred resource sets from the different UE-As, - UE-B uses each received preferred resource set for its resource selection for each TB to be transmitted to each UE-A providing the preferred resource set. - Conclusion: UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the different UE-As. - No RAN1 specification change to TS38.214 is deemed necessary in RAN1#108-e (except for the processing timeline) - For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the different UE-As, - FFS: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

Regarding UE-B’s behaviors after receiving both preferred resource set and non-preferred set from the same UE-A, the following three options were raised in RAN1#108-e meeting:

- Option 1: UE-B uses both the received preferred resource set and non-preferred resource set from the same UE-A for its resource selection for a TB to be transmitted to the UE-A.

- Option 2: UE-B does not expect to receive both preferred resource set and non-preferred resource set from the same UE-A for its resource selection for the same TB transmission to be transmitted to the UE-A.

- Option 3: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

For request-based scheme 1 Alt.1, where the resource set type is determined by UE-B, it is more reasonable that only one kind of resource set type indicator will be confirmed and indicated in request signaling to guarantee the same SCI content for UE-B’s different transmissions. Because only one resource set type indicator is presented in request signaling to UE-A for a TB, so only one type of resource set will be feedback. For request-based scheme 1 Alt.2 and condition-based scheme 1 where the resource set type is decided by UE-A, from the perspective of judging criteria of deciding resource set type (such as signaling overhead), either preferred resource set or non-preferred resource set will be finally identified. And considering that channel environment will not change hugely within a short time, UE-A can continue transmitting one type of resource set to UE-B for the same TB. Besides, we think that every resource set should be a complete one including all the preferred resources or non-preferred resources to show current status. Thus, we think that UE-B does not expect to receive both preferred resource set and non-preferred resource from the same UE-A for the same TB and the resource set should not be split.

Proposal 1: UE-B does not expect to receive both preferred resource set and non-preferred resource set from the same UE-A for the same TB and the resource set should not be split.

Regarding UE-B’s behaviors after receiving both preferred resource set and non-preferred set from different UE-As, the following options were discussed in RAN1#108-e meeting:

- Option 1: UE-B uses the received preferred resource set for its resource selection for a TB to be transmitted to the UE-A providing the preferred resource set. UE-B uses the received non-preferred resource set for its resource selection for a TB to be transmitted to the UE-A providing the non-preferred resource set.

- Option 2: UE-B uses both the received preferred resource set and non-preferred resource set from different UE-As for its resource selection for a TB to be transmitted to the UE-A providing the preferred resource set. UE-B uses the received non-preferred resource set for its resource selection for a TB to be transmitted to the UE-A providing the non-preferred resource set.

- Option 3: UE-B uses both the received preferred resource set and non-preferred resource set from different UE-As for its resource selection for a TB to be transmitted to any UE

- Option 4: UE-B uses all or a subset of the received preferred resource set and non-preferred resource set from different UE-As by its implementation for its resource selection for TB(s) to be transmitted to UE-A(s) providing the preferred resource set or non-preferred resource set

- Option 5: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

Different scenarios should be considered as shown in Figure 1 for this issue. In Figure 1-(a), UE-C1 and UE-C2 will cause higher interference to UE-A1, and the resources reserved by UE-C1 and UE-C2 will be identified as non-preferred resource set by UE-A1 for TB1 transmission. Besides, UE-A2 will transmit preferred resource set to UE-B for TB2 transmission. But because long distance is existed between UE-A1 and UE-A2, so the preferred resource set constructed by UE-A2 may not be suitable for TB1’s transmission, and the non-preferred resource set identified by UE-A1 is not necessarily unusable for TB2 transmission. So UE-B can only use the corresponding resource set for different TB transmissions. In Figure 1-(b), the non-preferred resource set constructed by UE-A1 will also be non-preferred resource set for TB2 transmission because of the similar sensing result. Thus, both preferred resource set and non-preferred resource set can be considered in TB2 transmission. Taking all the various applied scenarios into account, the best way is leave it to UE-B’s implementation.

- Different UE-As have different sensing result

- Different UE-As have similar sensing result

Figure 1: UE-B receives both preferred resource set and non-preferred resource set from different UE-As

Proposal 2: It is up to UE-B implementation to use one or multiple of preferred resource set and non-preferred resource set from different UE-As.

Remaining issues of resource selection window for determining resource set

In RAN1#108-e meeting, the definition of resource selection window for determining resource set was achieved with the following agreement [2].

- Agreement - Notations: - (n+T_1) – Start slot of resource selection window for determining the set of resources - For inter-UE coordination information triggered by UE-B’s explicit request, this value of (n+T_1) is provided by UE-B’s request as per the existing agreement - For inter-UE coordination information triggered by a condition other than explicit request reception, this value of (n+T_1) is determined by UE-A’s implementation as per the existing agreement - (n+T_2) – End slot of resource selection window for determining the set of resources - For inter-UE coordination information triggered by UE-B’s explicit request, this value of (n+T_2) is provided by UE-B’s request as per the existing agreement - For inter-UE coordination information triggered by a condition other than explicit request reception, this value of (n+T_2) is determined by UE-A’s implementation as per the existing agreement - (n’+T’_1) – Start slot of resource selection window used for sidelink transmission carrying inter-UE coordination information - (n’+T’_2) – End slot of resource selection window used for sidelink transmission carrying inter-UE coordination information - n' is the slot where UE procedure of determining TX resources of sidelink transmission carrying inter-UE coordination information is triggered - For inter-UE coordination information triggered by UE-B’s explicit request - Alt 1-1: - X1 ≤ (n’+T’_1) - (n’+T’_2) ≤ X2 - For inter-UE coordination information triggered by a condition other than explicit request reception, - Alt 2-2: - (n’+T’_2) < X3 - FFS: Values for X1, X2, X3

Regarding the relationship between UE-A’s resource selection window for coordination information transmission and resource set determination window for request-based scheme 1, the following options were given in RAN1#108-e meeting:

- Alt 1-1: (n+T_1) ≤ (n’+T’_1) & (n’+T’_2) ≤ (n+T_2)

- Alt 1-2: (n’+T’_1 – Tproc,1 – T2,min) ≤ (n+T_1)

- Alt 1-3: (n’+T’_2) < (n+T_1) – (Tproc,0+Tproc,1)

- Alt 1-4: (n’+T’_1) < (n+T_1)

- Alt 1-5: (n’+T’_1 – Tproc,1) ≤ (n+T_1)

- Alt 1-6: Up to UE-A’s implementation

As shown in Figure 2, the UE-B’s resource selection window [n+T_1, n+T_2] is included in request information and transmitted to UE-A, and then UE-A will determine the preferred resource set in the corresponding window. In order not to reduce the duration of resource selection window, UE-B should perform resource selection at n+T_1-Tproc,1. And thus the deadline of coordination information transmission should be n+T_1-Tproc,1-Tproc,0 considering the coordination information processing time. If the coordination information is transmitted later than the deadline, the resource selection occasion will be later than n+T_1-Tproc,1 and some portion of UE-B’s resource selection window will be invalid. So in order to guarantee the duration of UE-B’s resource selection window, the relationship between UE-A’s resource selection window for coordination information and resource set determination window should be (n’+T’_2) < (n+T_1) – (Tproc,0+Tproc,1).

Proposal 3: For request-based scheme 1, the relationship between UE-A’s resource selection window for coordination information transmission and resource set determination window should be (n’+T’_2) < (n+T_1) – (Tproc,0+Tproc,1).

Figure 2: The procedure of request-based scheme 1

Regarding the same issue in condition-based scheme 1, the following options were given in RAN1#108-e meeting:

- Alt 2-1: (n'+T’_1) < (n+T_1)

- Alt 2-2: (n’+T’_2) < (n+T_1) – (Tproc,0+Tproc,1)

- Alt 2-3: (n'+T’_1) ≥ (n+T_1)

- Alt 2-4: Up to UE-A’s implementation

Unlike request-based scheme 1, UE-A itself determines the resource set determination window [n+T_1, n+T_2]. Sharing the same views as request-scheme 1, in order to guarantee all the feedback resource set can be used by UE-B, both the UE-B’s reception processing time and the transmission processing time should be considered. So the relationship between UE-A’s resource selection window for coordination information and resource set determination window should also be (n’+T’_2) < (n+T_1) – (Tproc,0+Tproc,1) for condition-based scheme 1.

Proposal 4: For condition-based scheme 1, the relationship between UE-A’s resource selection window for coordination information and resource set determination window should be (n’+T’_2) < (n+T_1) – (Tproc,0+Tproc,1).

The following TP for Section 8.1.4 of TS 38.214 is proposed:

| Reason for change: | Corrections on NR Sidelink enhancements |
| --- | --- |
| Summary of change: | In clause 8.1.4, adding the restrictions on n+T2 if PSCCH/PSSCH transmissions carrying preferred resource set |
| Consequences if not approved: | The FFS part of previous agreements is unsloved and specification is incomplete |

----------------------------------------------TP to TS 38.214 clause 8.1.4 starts------------------------------------------------------------

8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<<< UNCHANGED PARTS OMITTED >>>

The following steps are used:

1)	A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot t'ySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing full sensing, in a set of Y candidate slots within the time interval n+T1,n+T2 for UE performing periodic-based partial sensing correspond to one candidate single-slot resource, or in a set of Y' candidate slots within the time interval [n+T1,n+T2] for UE performing contiguous partial sensing if Prsvp_TX=0, correspond to one candidate single-slot resource, where

-	selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL  , where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP;

-	if  T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to  T2min ≤ T2 ≤ remaining packet delay budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots).

- If the resource selection procedure is performed for PSCCH/PSSCH transmissions carrying preferred resource set, n +T2 should also be restricted to (n+ T2) < TRSW – (Tproc,0SL+Tproc,1SL), where TRSW is the starting time of resource set determination window

-	Y is selected by UE where Y≥Ymin.

-	Y' is selected by UE where Y'≥Ymin'. When the UE performs contiguous partial sensing and if Prsvp_TX=0, if the number of candidate single-slot resources Y' is smaller than Ymin', it is up to UE implementation to include other candidate slots.

The total number of candidate single-slot resources is denoted by Mtotal.

<<< UNCHANGED PARTS OMITTED >>>

-----------------------------------------------TP to TS 38.214 clause 8.1.4 ends------------------------------------------------------------

Conclusions

In this contribution, the remaining issues on inter-UE coordination are discussed. Particularly, we have following proposals.

Proposal 1: UE-B does not expect to receive both preferred resource set and non-preferred resource set from the same UE-A for the same TB and the resource set should not be split.

Proposal 2: It is up to UE-B implementation to use one or multiple of preferred resource set and non-preferred resource set from different UE-As.

Proposal 3: For request-based scheme 1, the relationship between UE-A’s resource selection window for coordination information transmission and resource set determination window should be (n’+T’_2) < (n+T_1) – (Tproc,0+Tproc,1).

Proposal 4: For condition-based scheme 1, the relationship between UE-A’s resource selection window for coordination information transmission and resource set determination window should be (n’+T’_2) < (n+T_1) – (Tproc,0+Tproc,1).

References

- RP-220945, “Status report for Rel-17 Sidelink WI”, LGE, RAN#95-e meeting.

- 3GPP TSG RAN WG1 Meeting RAN1#108-e Chairman’s Notes. 2022.