3GPP TSG RAN WG1 #109-e                                                                R1-2203432

e-Meeting, May 9th – 20th, 2022

Source:	CATT

Title:	Remaining issues on PDSCH/PUSCH enhancements for up to 71GHz operation

Agenda Item:	8.2.3

Document for:	Discussion and Decision

- Introduction

The work item on supporting NR from 52.6 GHz to 71 GHz was updated and approved in RAN 90 meeting [1], and objects of WI are listed as in the following.

- Physical layer aspects including [RAN1]: - In addition to 120 KHz SCS, specify new SCS, 480 KHz and 960 KHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. Note: Except for timing line related aspects, a common design framework shall be adopted for 480 KHz to 960 KHz - Time line related aspects adapted to 480 KHz and 960 KHz, e.g., BWP and beam switching timing, HARQ timing, and UE processing, preparation and computation timelines for PDSCH, PUSCH/SRS and CSI, respectively. - Support of up to 64 SSB beams for licensed and unlicensed operation in this frequency range. - Supports 120 KHz SCS for SSB and 120 KHz SCS for initial access related signals/channels in an initial BWP. - Study and specify, if needed, additional SCS (240 KHz, 480 KHz, 960 KHz) for SSB, and additional SCS (480 KHz, 960 KHz) for initial access related signals/channels in initial BWP. - Study and specify, if needed, additional SCS (480 KHz, 960 KHz) for SSB for cases other than initial access. - Note: coverage enhancement for SSB is not pursued. - Specify timing associated with beam-based operation to new SCS (i.e., 480 KHz and/or 960 KHz), study, and specify if needed, potential enhancement for shared spectrum operation - Study which beam management will be used as a basis: R15/16 or R17 in RAN #91-e - Support enhancement for PUCCH format 0/1/4 to increase the number of RBs under PSD limitation in shared spectrum operation. - Support enhancements for multi-PDSCH/PUSCH scheduling and HARQ support with a single DCI Note: coverage enhancement for multi-PDSCH/PUSCH scheduling is not pursued - Support enhancement to PDCCH monitoring, including blind detection/CCE budget, and multi-slot span monitoring, potential limitation to UE PDCCH configuration and capability related to PDCCH monitoring. - Specify support for PRACH sequence lengths (i.e. L=139, L=571 and L=1151) and study, if needed, specify support for RO configuration for non-consecutive RACH occasions (RO) in time domain for operation in shared spectrum - Evaluate, and if needed, specify the PTRS enhancement for 120 KHz SCS, 480 KHz SCS and/or 960 KHz SCS, as well as DMRS enhancement for 480 KHz SCS and/or 960 KHz SCS.

In this document, the remaining issues on PDSCH/PUSCH enhancements for up to 71GHz operation are discussed, which include PDSCH/PUSCH aggregation, collision between PUSCH and CORESET#0, and HARQ-ACK codebook generation.

- Discussion on remain issues on PDSCH/PUSCH enhancements

Issue 1: Application of PDSCH-AggregationFactor/PUSCH-AggregationFactor in pdsch-Config/ pusch-Config for DCI format 1_1/0_1 with CS-RNTI

Agreement (RAN1#107-e) - If a UE is configured with a TDRA table in which one or more rows contain multiple SLIVs for PDSCH for DCI format 1_1, the UE does not expect to be configured with repetition Number for the TDRA table, and if pdsch-AggregationFactor is configured in PDSCH-config, it does not apply to DCI format 1_1. - Note: repetition Number cannot be configured with pdsch-TimeDomainAllocationListDCI-1-2 as in Rel-16. - Note: Under agenda item 8.2.4, in RAN1#106-bis, it was already agreed that within the TDRA table for multi-PDSCH scheduling, the UE does not expect to be configured with the higher layer parameter repetition Number. - Note: These does not preclude pdsch-AggregationFactor can be configured and applies to DCI format 1_2 - If a UE is configured with a TDRA table in which one or more rows contain multiple SLIVs for PUSCH for DCI format 0_1, the UE does not expect to be configured with numberOfRepetitions for the TDRA table, and if pusch-AggregationFactor is configured in PUSCH-config, it does not apply to DCI format 0_1. - Note: These does not preclude numberOfRepetitions is configured for TDRA table corresponding to DCI format 0_2 - Note: These does not preclude pusch-AggregationFactor can be configured and applies to DCI format 0_2 Agreement (RAN1#107b-e) A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or a configured UL grant Type 2 PDCCH if the PDCCH indicates a TDRA row index including only one SLIV. Agreement (RAN1#108-e) - When re-transmission of DL SPS is indicated by DCI format 1_1, the PDCCH indicates a TDRA row index including only one SLIV. - When re-transmission of UL CG is indicated by DCI format 0_1, the PDCCH indicates a TDRA row index including only one SLIV.

In the last meeting [2], the issue of clarification on whether pdsch-AggregationFactor/pusch-AggregationFactor in pdsch-Config/pusch-Config can be applicable to DCI format 1_1/0_1 (that can schedule multiple PDSCHs/PUSCHs) scrambled with CS-RNTI or not was raised, but it was not discussed fully and no conclusion was reached.

As shown in agreements above, the repetition transmission of PDSCH/PUSCH is not supported by DCI format 1_1/0_1 scheduling if Multi-PDSCH/PUSCH TDRA is configured. Even if the scheduling row index includes only one SLIV, only one transmission of PDSCH/PUSCH is assumed. Moreover, for format 1_1/0_1 signaling, it was also agreed that only one SLIV can be scheduled for DL SPS/configured UL grant Type 2 operation, including activation and re-transmission. So it is reasonable that repetition transmission is not supported by DCI format 1_1/0_1 if the multiple PDSCH(s)/PUSCH(s) scheduling is configured. The above agreement should also apply to DCI format 1_1/0_1 scrambled by CS-RNTI.

It is noted that the repetition transmission can be scheduled by format 2_2/0_2 if pusch-AggregationFactor/ numberOfRepetitions is configured.

- Proposal 1：pdsch-AggregationFactor/pusch-AggregationFactor in PDSCH-Config/PUSCH-Config cannot be applicable to DCI format 1_1/0_1 (that can schedule multiple PDSCHs/PUSCHs) scrambled by CS-RNTI.

Issue 2: How to handle collision between PUSCH and CORESET#0:

Agreement: (RAN1#106bis-e) For multiple PDSCHs (or PUSCHs) scheduled by a single DCI, - Rel-15/16 behavior that is described in TS 38.213 Clauses 11 and 11.1 for a PDSCH (or PUSCH) indicated by DCI also applies for multiple PDSCHs (or PUSCHs) schedule by a single DCI. - If one of multiple PDSCHs (or PUSCHs) scheduled by the DCI collides with a flexible symbol (indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated), - If that PUSCH is collided with SSB symbols indicated by ssb-PositionsInBurst [or symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set], the HARQ process number increment is skipped for the PUSCH. Otherwise, the HARQ process number increment is not skipped for that PDSCH (or PUSCH).

For PDSCH transmission, if the time/frequency domain resource is collided with SSB symbols indicated by ssb-PositionsInBurst, those PRBs are regarded as not available for PDSCH, and whether the CORESET for Type0-PDCCH CCS set is available or not will depend on rateMatchPatternGroup configuration as described in TS 38.214 section 5.1.4.

For PUSCH transmission, if the time/frequency domain resource includes the Type0-PDCCH symbol(s), it means that it also includes one or more flexible symbols that used for downlink/uplink switching by RF-IC. Then the exact number of symbols used for RF receiving/transmitting switching depends on UE implementation, and gNB may not know the first valid symbols of PUSCH transmission. If the PUSCH is permitted to transmit, new problems may be introduced, such as DMRS issues, since RF switching may require puncturing the DMRS symbols of PUSCH. Considering there is not much time to discuss and solve all the potential issues, it is proposed that PUSCH transmission can be skipped if the scheduled PUSCH is collided with symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set.

Proposal 2：If a PUSCH is collided with symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the PUSCH transmission is skipped and the HARQ process number increment is skipped.

Issue 3) Combination of time domain bundling and PDSCH repetition

In last meeting [2], the issue of PDSCH repetition on type-1 HARQ-ACK was discussed, the text of related procedure is copied as following

TS 38.231 section 9.1.2 while r<CR if the UE is not provided enableTimeDomainHARQ-Bundling and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if PDSCH-TimeDomainResourceAllocationListForMultiPDSCH is provided and HARQ-ACK information for PDSCH time resource derived by row r in slot nU-K1,k⋅2μDL-μUL+nD cannot be provided in slot nU R=R\r; elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot nU-K1,k⋅2μDL-μUL+nD- ∆K0,r(d), at least one symbol of each PDSCH time resource derived by row r of set R' is configured as UL, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r. R=R\r; R'=R'\r; else r=r+1; end if end while

When gNB configures time domain bundling (enableTimeDomainHARQ-Bundling) for multi-PDSCH scheduling for a DCI format 1_1, and gNB doesn’t configure PDSCH repetition (e.g., NPDSCHrepeat,max>1)  for single-PDSCH scheduling by DCI format 1_2, the above green highlight pseudo-code can work without problem.

To enable flexible gNB scheduling, gNB can configure PDSCH repetition (e.g., NPDSCHrepeat,max>1) for single-PDSCH scheduling by DCI format 1_2. In this scenario, the issue on type-1 HARQ code book generation is raised. Theoretically, if enableTimeDomainHARQ-Bundling is provided, PDSCH repetition will not triggered by DCI format 1_1 scheduling, and only DCI format 1_2 can schedule PDSCH repetition. Restrictions can be added to make more row of TDRA removed to improve efficiency of HARQ-ACK feedback. However, this will cause more complexity for UE implementation; therefore such optimization scheme is not preferred.

Proposal 3：Further optimization is not preferred for type-1 HARQ-ACK generation when both  NPDSCHrepeat,max are more than 1 and enableTimeDomainHARQ-Bundling is provided. Approve TP-1 in Appendix.

Issue 4: Applicable for UL DAI Indication in multiple PUSCH(s) scheduling

In the current R15/R16 specification, the HARQ-ACK codebook may multiplex on PUSCH when the PUCCH carrying HARQ-ACK information is overlapping with PUSCH. To prevent the last DCI missing problem, the UL DAI is indicated in format DCI 0_1 and 0_2. If a UE is configured with a TDRA table in which one or more rows contain multiple SLIVs for PUSCH for DCI format 0_1, it is necessary to clarify how to apply the UL DAI to multiple PUSCHs.

When gNB configures PUSCH repetition transmission, and if the PUSCH transmission over the multiple slots is scheduled by a DCI format that includes a UL DAI field, the UL DAI can be applied on each slot PUSCH, the text of specification is copied as in the following .

TS 38.213 section 9 If the PUSCH transmission over the multiple slots is scheduled by a DCI format that includes a DAI field, the value of the DAI field is applicable for multiplexing HARQ-ACK information in the PUSCH transmission in any slot from the multiple slots where the UE multiplexes HARQ-ACK information.

For multiple PUSCHs transmission, the same principle can be reused if multi-PUSCH scheduling signalling includes UL DAI. When gNB configures multiple PUSCH(s) scheduling by a DCI, and if more than one PUSCHs transmission over the multiple slots is scheduled by a DCI format that includes a UL DAI field, the UL DAI can be applied on each PUSCH;

Proposal 4： If more than one PUSCH is scheduled by a DCI format that includes a UL DAI field, the UL DAI is applied on each PUSCH. Approve TP-2 in Appendix.

- Conclusion

This contribution discussed the remaining issues on PDSCH/PUSCH enhancements for up to 71GHz operation. These include PDSCH/PUSCH aggregation, collision between PUSCH and CORESET#0, and HARQ-ACK codebook generation, and some proposals are given as following.

- Proposal 1：pdsch-AggregationFactor/pusch-AggregationFactor in PDSCH-Config/PUSCH-Config cannot be applicable to DCI format 1_1/0_1 (that can schedule multiple PDSCHs/PUSCHs) scrambled by CS-RNTI.

Proposal 2：If a  PUSCH is collided with symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the PUSCH transmission can skipped, the HARQ process number increment is skipped

Proposal 3：No optimization is preferred for type-1 HARQ-ACK generation when both  NPDSCHrepeat,max are more than 1 and enableTimeDomainHARQ-Bundling is provided, and TP-1 in appendix is adopted.

Proposal 4： If more than one PUSCH is scheduled by a DCI format that includes a UL DAI field, the UL DAI is applied on each PUSCH.and TP-2 in appendix is proposed.

- Reference

RP-202925, “Revised WID:  Extending current NR operation to 71 GH”, CMCC

- [2]      3GPP TSG RAN WG1 Meeting RAN1 #108-e February 21st – March 3rd, 2022

- Appendix

- TP#1 for TS 38.213

------------------------------------------Start of TP#1 for TS 38.213 Clause 9.1.2.1 ---------------------------------------------

- 9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

============================== Unchanged Text Omitted ==============================

While r<CR

if the UE is not provided enableTimeDomainHARQ-Bundling and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if PDSCH-TimeDomainResourceAllocationListForMultiPDSCH is provided and HARQ-ACK information for PDSCH time resource derived by row r in slot nU-K1,k⋅2μDL-μUL+nD cannot be provided in slot nU

R=R\r;

else if the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot nU-K1,k⋅2μDL-μUL+nD- ∆K0,r(d), at least one symbol of each PDSCH time resource derived by row r of set R' is configured as UL, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r. and for each slot from n0,k+nD-NPDSCHrepeat,max+1 to slotn0,k+nD, at least one symbol of the PDSCH time resource derived by row r of set R is configured as UL

R=R\r;

R'=R'\r;

else

r=r+1;

end if

end while

- ------------------------------------------------------------End of TP#1---------------------------------------------------------------

- TP#2 for TS 38.213

- ------------------------------------------Start of TP#2 for TS 38.213 Clause 9 ---------------------------------------------

9 UE procedure for reporting control information

============================== Unchanged Text Omitted ==============================

If the PUSCH transmission over the multiple slots is scheduled by a DCI format that includes a DAI field, the value of the DAI field is applicable for multiplexing HARQ-ACK information in the PUSCH transmission in any slot from the multiple slots where the UE multiplexes HARQ-ACK information.

If PUSCH-TimeDomainResourceAllocationListForMultiPUSCH is provided, and if the more than one PUSCH are scheduled by a DCI format that includes a DAI field, the value of the DAI field is applicable for multiplexing HARQ-ACK information in the each PUSCH scheduled where the UE multiplexes HARQ-ACK information.

When a UE would multiplex HARQ-ACK information in a PUSCH transmission that is configured by a ConfiguredGrantConfig, and includes CG-UCI [5, TS 38.212], the UE multiplexes the HARQ-ACK information in the PUSCH transmission if the UE is provided cg-UCI-Multiplexing; otherwise, if the HARQ-ACK information and the PUSCH have same priority index, the UE does not transmit the PUSCH and multiplexes the HARQ-ACK information in a PUCCH transmission or in another PUSCH transmission; if the HARQ-ACK information and the PUSCH have different priority indexes, the UE does not transmit the channel with the smaller priority index.

- ------------------------------------------------------------End of TP#2---------------------------------------------------------------