3GPP TSG RAN WG1 #109-e  		      R1-2203433

e-Meeting, May 9th – 20th, 2022

Source:	CATT

Title:	Remaining issues on channel access mechanism for up to 71GHz operation

Agenda Item:	8.2.4

Document for:	Discussion and Decision

- Introduction

In RAN#90-e meeting, a Rel-17 WI on extending current NR operation to 71GHz was approved [1]. This WI extends NR operation up to 71GHz according to the outcome of the study item [2], with the following objectives on channel access mechanism for up to 71GHz operations.

- Physical layer procedure(s) including [RAN1]: - Channel access mechanism assuming beam based operation in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6GHz and 71GHz. - Specify both LBT and No-LBT related procedures, and for No-LBT case no additional sensing mechanism is specified. - Study, and if needed specify, omni-directional LBT, directional LBT and receiver assistance in channel access - Study, and if needed specify, energy detection threshold enhancement

In this document, we share our views on remaining issues of indication for Japan vs outside Japan, Type 2 channel access and Contention Exempt Short Control Signaling rules.

Discussion

  - Indication for Japan vs outside Japan

In the RAN1#108e meeting, it has discussed a bundle of solutions to identify if LBT is required for UL transmission in email discussion [3] as followings.

##### Proposal 2.15-1 (new) Down-select between the next two alternatives - Alt 1. Introduce one bit in SIB1 indicates whether LBT is required for all UL transmissions - If the bit is set to true, msg1 and msgA cannot be transmitted with Contention Exempt Short Control Signaling based transmission. If the bit is set to false, msg1 and msgA can be transmitted by with Contention Exempt Short Control Signaling based transmission if requirement of 10% over 100ms is satisfied - It is a separate discussion if the requirement of 10% over 100ms is per UE or per cell - For fallback DCI formats 0_0 and 1_0 and RAR UL grant, for FR2-2 operation, the ChannelAccess-Cpext field in DCI indicates the channel access type only. A new table similar to Table 7.3.1.1.1-4 is introduced with entries “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213 or Type 3 channel access in 4.4.3 of 37.213”. For the 2nd entry, if the bit is set to true, it will be interpreted as “Type 2 channel access in 4.4.2 of 37.213”. If the bit is set to false, it will be interpreted as “Type 3 channel access in 4.4.3 of 37.213” - For an UL transmission indicated or configured to use Type 1 channel access, if the UE later finds out, through DCI 2_0 detection, the transmission falls in a gNB COT, the UE can change the channel access type to Type 2 channel access if the bit is set to true and the UE can change the channel access type to Type 3 channel access if the bit is set to false - Alt 1A (From Ericsson and Apple as replacement for Alt 1). Introduce one bit in SIB1 indicates whether LBT is required for all UL transmissions - If the bit is set to true, msg1 and msgA cannot be transmitted with Contention Exempt Short Control Signaling based transmission. If the bit is set to false, msg1 and msgA can be transmitted by with Contention Exempt Short Control Signaling based transmission if requirement of 10% over 100ms is satisfied - It is a separate discussion if the requirement of 10% over 100ms is per UE or per cell - For an UL transmission indicated or configured to use Type 1 channel access, if the UE later finds out, through DCI 2_0 detection, the transmission falls in a gNB COT, the UE can change the channel access type to Type 2 channel access if the bit is set to true and the UE can change the channel access type to Type 3 channel access if the bit is set to false - UE as initiating device can resume transmission within maximum COT without a type 2 LBT, no matter how long the gap is from the previous transmission from initiating device or responding device, if the bit is set to false. If the bit is set to true, UE as initiating device can resume transmission after a type 2 LBT if device is capable. - Alt 1B. Introduce one bit in SIB1 indicates whether LBT is required for all UL transmissions - If the bit is set to true, msg1 and msgA cannot be transmitted with Contention Exempt Short Control Signaling based transmission. If the bit is set to false, msg1 and msgA can be transmitted by with Contention Exempt Short Control Signaling based transmission if requirement of 10% over 100ms is satisfied - It is a separate discussion if the requirement of 10% over 100ms is per UE or per cell - For fallback DCI formats 0_0 and 1_0 and RAR UL grant, for FR2-2 operation, the ChannelAccess-Cpext field in DCI indicates the channel access type only. A new table similar to Table 7.3.1.1.1-4 is introduced with entries “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213” and “Type 3 channel access in 4.4.3 of 37.213”, and “reserved”. If the bit in SIB is set to true, UE does not expect a DCI indicating “Type 3 channel access in 4.4.3 of 37.213” - For an UL transmission indicated or configured to use Type 1 channel access, if the UE later finds out, through DCI 2_0 detection, the transmission falls in a gNB COT, the UE can change the channel access type to Type 2 channel access if the bit is set to true and the UE can change the channel access type to Type 3 channel access if the bit is set to false - Alt 2. - gNB provides separate RRC configuration in SIB1 to indicate if msg1 or msgA transmission with Contention Exempt Short Control Signaling based transmission is allowed. - It is a separate discussion if the requirement of 10% over 100ms is per UE or per cell - For fallback DCI formats 0_0 and 1_0 and RAR UL grant, for FR2-2 operation, the ChannelAccess-Cpext field in DCI indicates the channel access type only. A new table similar to Table 7.3.1.1.1-4 is introduced with entries “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213” and “Type 3 channel access in 4.4.3 of 37.213”, and “reserved”. - Note: This option requires 2 bis in fallback DCI - TP 2.9-C - For an UL transmission indicated or configured to use Type 1 channel access, if the UE later finds out, through DCI 2_0 detection, the transmission falls in a gNB COT, the UE can change the channel access type to Type 2 channel access or Type 3 channel access. - RRC configuration is introduced to indicate either Type 2 channel access or Type 3 channel access will be used, subject to UE capability - FFS: For UE as initiating device, if additional RRC configuration is introduced to indicate if the UE can resume transmission within maximum COT without a type 2 LBT, no matter how long the gap is from the previous transmission from initiating device or responding device, or this is left for UE implementation

According to the regulation of 60GHz in Japan, the UEs need perform LBT before all the UL transmission. However, in the other regions except for Japan, the UE is allowed to transmit UL transmission without LBT in the certain cases. The main issue is that how to inform the UE if LBT is required in these certain cases.

Msg1 and msgA:

It was agreed that Contention Exempt Short Control Signaling rule can be applied for msg1 and msgA if requirement of 10% over 100ms is satisfied. For the regions except for Japan, the transmission of msg1 and msg A is allowed without LBT. But, in Japan, msg1 and msg A can’t be transmitted without LBT. Therefore, one bit field in SIB1 can be introduced to inform UE if LBT is required for the transmission of msg1 and msgA. For example, if the one bit field is true, msg1 and msgA cannot be transmitted with Contention Exempt Short Control Signaling based transmission. Otherwise, msg1 and msgA can be transmitted by with Contention Exempt Short Control Signaling based transmission if requirement of 10% over 100ms is satisfied.

Type1 channel access update:

For Type1 channel access update in Rel-16 NR-U, if a UE determines the duration in time domain and the location in frequency domain of a remaining channel occupancy initiated by the gNB from a DCI format 2_0, the UE may switch from Type1 channel access procedure to Type 2A channel access procedure for the indicated or configured UL transmission within the gNB COT. The same mechanism can be reused for the 60GHz NR-U. For the regions except for Japan, channel access Type1 can be switched to Type 3 channel access. For the Japan, channel access Type1 can be switched to Type 2 channel access. Per our understanding, one bit filed in SIB1 introduced for msg1 and msg A can be reused to inform UE that Type 1 channel access is changed to Type 2 channel access or Type 3 channel access for the Type1 channel access update mechanism. For example, if the one bit field is true, the UE can change the channel access type 1 to Type 2 channel access; otherwise, the UE can change the channel access type 1 to Type 3 channel access.

LBT Type indication in fallback DCI and RAR UL grant

When a UE is scheduled by a fallback DCI or RAR UL grant to transmit a UL transmission, the UE will determine the LBT type according to the LBT Type indication in fallback DCI or RAR UL grant. For 60GHz NR-U, the LBT types include Type1 channel access, Type 2 channel access and Type 3 channel access. Although Type 2 channel access is motived by the Japan regulations, it is not restricted for use only in Japan. In our view, a new table similar to Table 7.3.1.1.1-4 can be introduced with entries “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213” and “Type 3 channel access in 4.4.3 of 37.213”, and “reserved”.

To summarize, for a bundle of solutions to identify if LBT is required for UL transmission of the Proposal 2.15-1 in RAN1#108e, Alt 1B is our preferred solution:

- Alt 1B. Introduce one bit in SIB1 indicates whether LBT is required for all UL transmissions

  - If the bit is set to true, msg1 and msgA cannot be transmitted with Contention Exempt Short Control Signaling based transmission. If the bit is set to false, msg1 and msgA can be transmitted by with Contention Exempt Short Control Signaling based transmission if requirement of 10% over 100ms is satisfied

    - It is a separate discussion if the requirement of 10% over 100ms is per UE or per cell

  - For fallback DCI formats 0_0 and 1_0 and RAR UL grant, for FR2-2 operation, the ChannelAccess-Cpext field in DCI indicates the channel access type only. A new table similar to Table 7.3.1.1.1-4 is introduced with entries “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213” and “Type 3 channel access in 4.4.3 of 37.213”, and “reserved”. If the bit in SIB is set to true, UE does not expect a DCI indicating “Type 3 channel access in 4.4.3 of 37.213”

  - For an UL transmission indicated or configured to use Type 1 channel access, if the UE later finds out, through DCI 2_0 detection, the transmission falls in a gNB COT, the UE can change the channel access type to Type 2 channel access if the bit is set to true and the UE can change the channel access type to Type 3 channel access if the bit is set to false

Proposal 1: To identify if LBT is required for UL transmission, Alt 1B is our preferred solution as following:

- Alt 1B. Introduce one bit in SIB1 indicates whether LBT is required for all UL transmissions

  - If the bit is set to true, msg1 and msgA cannot be transmitted with Contention Exempt Short Control Signaling based transmission. If the bit is set to false, msg1 and msgA can be transmitted by with Contention Exempt Short Control Signaling based transmission if requirement of 10% over 100ms is satisfied

    - It is a separate discussion if the requirement of 10% over 100ms is per UE or per cell

  - For fallback DCI formats 0_0 and 1_0 and RAR UL grant, for FR2-2 operation, the ChannelAccess-Cpext field in DCI indicates the channel access type only. A new table similar to Table 7.3.1.1.1-4 is introduced with entries “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213” and “Type 3 channel access in 4.4.3 of 37.213”, and “reserved”. If the bit in SIB is set to true, UE does not expect a DCI indicating “Type 3 channel access in 4.4.3 of 37.213”

  - For an UL transmission indicated or configured to use Type 1 channel access, if the UE later finds out, through DCI 2_0 detection, the transmission falls in a gNB COT, the UE can change the channel access type to Type 2 channel access if the bit is set to true and the UE can change the channel access type to Type 3 channel access if the bit is set to false

  - Type 2 channel access

In RAN1#108e meeting [4], for gNB as initiating device to resume transmission within the COT, it was agreed to support both of following alternatives. It was noted that the one alternative is motivated by the region where LBT is not required before each transmission and the other alternative is motivated by the region where LBT is required before each transmission.

Agreement (RAN1#108e) Support gNB as the initiating device to resume transmission within maximum COT without a Cat 2 LBT, no matter how long the gap is from the previous transmission from initiating device or responding device - Note: This is motivated by regions where LBT is not required before each transmission (say outside Japan)? - Note: This should only be used when allowed by local regulation Agreement(RAN1#108e) Support gNB as the initiating device to resume transmission with a Cat 2 LBT if there is gap longer than Y us from the previous transmission from initiating device or responding device - Note this is motivated by regions where LBT is required before each transmission (say Japan) - Y is left for initiating device implementation and should comply with local regulation but no less than 8us

The remaining issue is that, for UE as initiating device, how to resume the transmission within the COT. In our view, the alternatives applied for gNB as initiating device to resume the transmission can be extended to the UE as following.

- Alt.1: UE as the initiating device to resume transmission within maximum COT without a Cat 2 LBT, no matter how long the gap is from the previous transmission from initiating device or responding device.

- Note: This is motivated by regions where LBT is not required before each transmission

- Alt.2: UE as the initiating device to resume transmission with a Cat 2 LBT if there is gap longer than Y us from the previous transmission from initiating device or responding device

- Note this is motivated by regions where LBT is required before each transmission

Proposal 2: On UE as initiating device to resume transmission within the COT, support two alternatives as follows.

- Alt.1: UE as the initiating device to resume transmission within maximum COT without a Cat 2 LBT, no matter how long the gap is from the previous transmission from initiating device or responding device.

- Note: This is motivated by regions where LBT is not required before each transmission

- Alt.2: UE as the initiating device to resume transmission with a Cat 2 LBT if there is gap longer than Y us from the previous transmission from initiating device or responding device

- Note this is motivated by regions where LBT is required before each transmission

  - Contention Exempt Short Control signaling rules

Agreement: (RAN1#105-e) - Contention Exempt Short Control Signaling rules apply to the transmission of msg1 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS. - Note restriction for short control signalling transmissions apply (10% over any 100ms intervals) - Alt 1: The 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell - Alt 2: The 10% over any 100ms interval restriction is applicable to the msg1/msgA transmission from one UE perspective - FFS: Other UL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as msg3, SRS, PUCCH, PUSCH without user plain data, etc Conclusion: (RAN1#107-e) In Rel.17, there is no consensus to apply contention exemption short control signalling to UL transmissions other than msg1 and msgA.

It was agreed in [5] that the Contention Exempt Short Control Signaling (CESCS) rules can be applied to the transmission of msg1 for the 4 step PRACH and MsgA for the 2-step RACH for all supported SCS. And according to the conclusion achieved in RAN1#107-e, there is no consensus to apply contention exemption short control signalling to UL transmissions other than msg1 and msgA. One remaining issue is that whether the 10% over any 100ms interval restriction should be counted from the cell perspective or UE perspective. If the 10% over any 100ms interval restriction is counted from UE perspective, the total amount of UL signals in a cell which applies to Contention Exempt Short Control Signaling rule may be too large to interfere with other systems. Therefore, it is more reasonable to count the 10% limit from the perspective of the cell, which can control the amount of the Contention Exempt Short Control Signals in the cell.

Proposal 3: The 10% over any 100ms interval restriction should be applicable to all Contention Exempt Short Control Signals from cell perspective.

- Conclusions

In this contribution, we provide our view on remaining issues of channel access mechanism for up to 71GHz operation. The proposals are summarized as follows:

Proposal 1: To identify if LBT is required for UL transmission, Alt 1B is our preferred solution as following:

- Alt 1B. Introduce one bit in SIB1 indicates whether LBT is required for all UL transmissions

  - If the bit is set to true, msg1 and msgA cannot be transmitted with Contention Exempt Short Control Signaling based transmission. If the bit is set to false, msg1 and msgA can be transmitted by with Contention Exempt Short Control Signaling based transmission if requirement of 10% over 100ms is satisfied

    - It is a separate discussion if the requirement of 10% over 100ms is per UE or per cell

  - For fallback DCI formats 0_0 and 1_0 and RAR UL grant, for FR2-2 operation, the ChannelAccess-Cpext field in DCI indicates the channel access type only. A new table similar to Table 7.3.1.1.1-4 is introduced with entries “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213” and “Type 3 channel access in 4.4.3 of 37.213”, and “reserved”. If the bit in SIB is set to true, UE does not expect a DCI indicating “Type 3 channel access in 4.4.3 of 37.213”

  - For an UL transmission indicated or configured to use Type 1 channel access, if the UE later finds out, through DCI 2_0 detection, the transmission falls in a gNB COT, the UE can change the channel access type to Type 2 channel access if the bit is set to true and the UE can change the channel access type to Type 3 channel access if the bit is set to false

Proposal 2: On UE as initiating device to resume transmission within the COT, support two alternatives as following.

- Alt.1: UE as the initiating device to resume transmission within maximum COT without a Cat 2 LBT, no matter how long the gap is from the previous transmission from initiating device or responding device.

- Note: This is motivated by regions where LBT is not required before each transmission

- Alt.2: UE as the initiating device to resume transmission with a Cat 2 LBT if there is gap longer than Y us from the previous transmission from initiating device or responding device

- Note this is motivated by regions where LBT is required before each transmission

Proposal 3: The 10% over any 100ms interval restriction should be applicable to all Contention Exempt Short Control Signals from cell perspective.

- Reference

RP-202925, Revised WID: Extending current NR operation to 71 GHz, CMCC, RAN#90-e meeting

3GPP TR 38.808 v1.0.0: Study on supporting NR from 52.6 GHz to 71 GHz

R1-2202685, FL summary on channel access for NR beyond 52.6GHz, version 2	Moderator (Qualcomm Incorporated)

3GPP TSG RAN WG1 Meeting RAN1#108-e Chair’s Notes, February 21st – March 3rd, 2022.

3GPP TSG RAN WG1 Meeting RAN1#105-e Chair’s Notes, May 19th –May 27th , 2021