3GPP TSG RAN WG1 #109-e		                                                      R1-2203435

e-Meeting, May 9th – 20th, 2022

Source:	CATT

Title:	Maintenance on intra-UE multiplexing and prioritization

Agenda Item:	8.3.2

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss remaining issues for intra-UE multiplexing and prioritization.

Discussion

HP SR + HP HARQ-ACK with PF 0/1 vs. LP HARQ-ACK (+ LP CSI/SR)

For handling overlapping PUCCHs/PUSCHs with different priorities in R17, the following was agreed for multiplexing between a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 2/3/4 and a LP PUCCH carrying HARQ-ACK (and CSI/SR).

Agreement When a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 2/3/4 overlaps with a PUCCH carrying LP HARQ-ACK, information bits for K HP SRs are appended to HP HARQ-ACK bits, and treat them as HP UCI, where K (K≥1) PUCCHs semi-statically configured for K HP SRs overlap with the original PUCCH carrying the HP HARQ-ACK. - The number of HP UCI bits is OUCI=OACK+log2K+1, same as Rel-15; - FFS: PF0, PF1 - Reuse other procedures for multiplexing of LP HARQ-ACK and HP HARQ-ACK on PUCCH resource with PF 2/3/4, i.e. separate coding, PRB determination, rate matching and power control. - If the HP HARQ-ACK is a dynamic HARQ-ACK, a PUCCH resource indicated by PRI is used for multiplexing. - If the HP HARQ-ACK is a SPS HARQ-ACK, a PUCCH resource determined from the PUCCH resource(s) provided by sps-PUCCH-AN-List is used for multiplexing. Agreement To resolve overlapping between a HP PUCCH carrying SR and HP HARQ-ACK with PUCCH format 2/3/4 and a LP PUCCH carrying HARQ-ACK and CSI/SR, only LP HARQ-ACK is multiplexed with HP UCI (i.e., SR and HARQ-ACK) and LP CSI/SR are dropped. - FFS HP PUCCH with PUCCH format 0/1

For a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, the following proposal was given in last meeting:

Proposal for 6th round discussion: When a PUCCH carrying explicit/implicit HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs which are all negative or include at least one positive, - In Step 1, 1-bit HP SR is multiplexed to HP HARQ-ACK bits (based on Rel-15 rules) for determining the resultant PUCCH resource. - Note: The description of Step 1 here is only for information purpose, which has no spec impact. - In Step 2, down-select from the two options: - Option 0: LP HARQ-ACK is dropped. - Option 2: 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1 - The resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is either of PF2, PF3, or PF4.

For option 0, LP HARQ-ACK is dropped. It should be clarified that whether LP HARQ-ACK is dropped for positive SR only or LP HARQ-ACK is dropped regardless of whether SR is positive or negative. If LP HARQ-ACK is dropped for positive SR only, and is multiplexed with HP HARQ-ACK for negative SR, gNB cannot differentiate whether UE transmits negative SR and LP HARQ-ACK or positive SR and dropped LP HARQ-ACK on PUCCH format 0 [1]. If LP HARQ-ACK is dropped regardless of whether SR is positive or negative, it seems not reasonable since LP HARQ-ACK multiplexing with HP A/N+SR in PF 2/3/4 has already been supported.

For option 2, the case is almost the same as the case of HP PUCCH with 2 bits HARQ-ACK overlapping with LP HARQ-ACK and Option 2 basically follows the same rule by treating the HP SR the same as HP HARQ-ACK. Option 2 is also aligned with the previous agreement for HP PUCCH with HP SR and HP HARQ-ACK with PF 2/3/4. If Option 2 is adopted, then for a HP PUCCH with HP SR and HP HARQ-ACK regardless of the PUCCH format, LP HARQ-ACK is multiplexed with HP UCI by treating both HP SR and HP HARQ-ACK and HP UCI.

Hence, it is preferred that PUCCH format 2/3/4 is used for multiplexing HP SR and HP HARQ-ACK carried in a PUCCH with PUCCH format 0/1 and LP HARQ-ACK.

In addition, only 1 bit HP SR can be multiplexed with HP HARQ-ACK with PF0/1, then if there are more than one HP SRs overlapping with the original PUCCH carrying the HP HARQ-ACK, it should be clarified whether one bit or multiple bits for HP SR are multiplexed with HP HARQ-ACK and LP HARQ-ACK. From the implementation perspective, multiplexing one bit SR would be simpler since UE does not need to record and check the state of all the HP SRs which overlap with the initial HP HARQ-ACK, hence it is preferred that only one bit HP SR is multiplexed with HP HARQ-ACK and LP HARQ-ACK.

Proposal 1: If a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, LP HARQ-ACK is multiplexed with HP SR and HP HARQ-ACK where 1 bit HP SR is appended to HP HARQ-ACK bits and treated as HP UCI.

Similarly, if the LP PUCCH carries SR and/or CSI in addition to HARQ-ACK, it is proposed to drop LP SR and/or CSI and multiplex LP HARQ-ACK with HP SR and HARQ-ACK carried in PUCCH format 0/1 as proposal 1.

Proposal 2: If a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK and LP CSI/SR, only LP HARQ-ACK is multiplexed with HP UCI and LP CSI/SR are dropped.

LP HARQ-ACK multiplexing on PUSCH

For LP HARQ-ACK multiplexing with HP HARQ-ACK on PUCCH, the following was agreed in RAN1#107-e meeting, LP HARQ-ACK is not partially dropped or compressed when there is no enough resource for LP HARQ-ACK on HP PUCCH.

Agreement For determining the PUCCH resource to carry the multiplexed high-priority and low-priority HARQ-ACKs, if OHP_UCI+OCRC, HP_UCIrHP_UCI∙Qm+OLP_UCI+OCRC, LP_UCIrLP_UCI∙Qm >MRBPUCCH∙Nsc, ctrlRB∙Nsymb-UCIPUCCH - The number of RBs is MRBPUCCH. Then follow Rel-15 procedure, i.e., LP HARQ-ACK is mapped to the rest REs after HP HARQ-ACK.

For LP HARQ-ACK multiplexing on HP PUSCH, if the amount of REs available for LP HARQ-ACK mapping on the HP PUSCH is not sufficient to satisfy the coding rate of the LP HARQ-ACK based on the corresponding beta offset, it is proposed to follow the same principle of UCI multiplexing on PUCCH, , i.e. LP HARQ-ACK is mapped to the rest REs after mapping HP HARQ-ACK or HP CSI.

Proposal 3: If the amount of REs available for LP HARQ-ACK mapping on the HP PUSCH is not sufficient to satisfy the coding rate of the LP HARQ-ACK based on the corresponding beta offset, LP HARQ-ACK is mapped to the rest REs after mapping HP HARQ-ACK or HP CSI .

Beta-offset indicator in DCI

In Rel-16, separate sets of beta offset values are configured for multiplexing UCI on PUSCH for LP and HP respectively. If priority indicator is configured for DCI format 0_1 or DCI format 0_2, bit width of beta_offset indicator is aligned by inserting a number of MSB with ‘0’ to smaller beta_offset indicator when needed.

In Rel-17, for UCI multiplexing on PUSCH with different priority, 2 new set of beta-offset values were introduced according to the following agreement.

Agreement In NR Rel-17, [at least] 2 new set of beta offset values can be configured to the UE to indicate separate beta_offset values for the following cases: - Multiplexing LP HARQ-ACK on HP PUSCH - Multiplexing HP HARQ-ACK on LP PUSCH

Therefore, there are four separate sets of beta-offset values configured to the UE for the following four cases:

- Multiplexing LP HARQ-ACK on LP PUSCH

- Multiplexing HP HARQ-ACK on HP PUSCH

- Multiplexing LP HARQ-ACK on HP PUSCH

- Multiplexing HP HARQ-ACK on LP PUSCH

The bit widths of beta_offset indicator in DCI format 0_1 or DCI format 0_2 based on different sets of beta-offset values can be different.

Two options can be considered to resolve the issue, one way is to mandate the configuration of beta-offset sets for different cases would lead to the same bit width of the beta_offset indicator in DCI format 0_1/0_2. The other way is to define a rule to align the bit width of the beta_offset indicator in DCI format 0_1/0_2 as in Rel-16. We prefer the latter option to for better configuration flexibility.

As a regular practice, a number of most significant bits with value set to '0' can be inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format for the HARQ-ACKs with different priorities multiplexing on one PUSCH are the same.

Proposal 4: Adopt the following text proposal for section 7.3.1.1.2 and 7.3.1.1.3 of TS38.212.

----------------------------------------------------- Start of text proposal ------------------------------------------------------

- 7.3.1.1.2	Format 0_1

DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback information (CG-DFI) to a UE.

The following information is transmitted by means of the DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

*** Unchanged text is omitted ***

-	Second PTRS-DMRS association – 2 bits if PTRS-DMRS association field and SRS resource set indicator field are present and maxRank>2; 0 bit otherwise. Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second precoding information field when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field.

-	beta_offset indicator – 0 if the higher layer parameter betaOffsets = semiStatic; otherwise 2 bits as defined by Table 9.3-3 in [5, TS 38.213].

When two HARQ-ACK codebooks are configured for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-1 is configured, if the bit width of the beta_offset indicator in DCI format 0_1 for one HARQ-ACK codebook is not equal to that of the beta_offset indicator in DCI format 0_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_1 for the two HARQ-ACK codebooks are the same.

When higher layer parameter UCI-MuxWithDifferentPriority and priorityIndicatorDCI-0-1 are both configured, the bit width of the beta_offset indicator in DCI format 0_1 is determined by the maximum bit width of the beta_offset indicator in DCI format 0_1 for HARQ-ACK of larger priority index on PUSCH of smaller priority index, HARQ-ACK of smaller priority index on PUSCH of smaller priority index, HARQ-ACK of smaller priority index on PUSCH of higher priority index, HARQ-ACK of higher priority index on PUSCH of higher priority index. A number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_1 for HARQ-ACKs with different priorities multiplexing on one PUSCH with larger or smaller priority index are the same.

-	DMRS sequence initialization – 0 bit if transform precoder is enabled; 1 bit if transform precoder is disabled.

*** Unchanged text is omitted ***

- 7.3.1.1.3	Format 0_2

DCI format 0_2 is used for the scheduling of PUSCH in one cell.

The following information is transmitted by means of the DCI format 0_2 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

*** Unchanged text is omitted ***

-	Second PTRS-DMRS association – 2 bits if PTRS-DMRS association field and SRS resource set indicator field are present and maxRankDCI-0-2>2; 0 bit otherwise. Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second precoding information field when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field.

-	beta_offset indicator – 0 bit if the higher layer parameter betaOffsets = semiStatic; otherwise 1 bit if 2 offset indexes are configured by higher layer parameter dynamicDCI-0-2 as defined by Table 9.3-3A in [5, TS 38.213], and 2 bits if 4 offset indexes are configured by higher layer parameter dynamicDCI-0-2 as defined by Table 9.3-3 in [5, TS 38.213].

When two HARQ-ACK codebooks are configured for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-2 is configured, if the bit width of the beta_offset indicator in DCI format 0_2 for one HARQ-ACK codebook is not equal to that of the beta_offset indicator in DCI format 0_2 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_2 for the two HARQ-ACK codebooks are the same.

When higher layer parameter UCI-MuxWithDifferentPriority and priorityIndicatorDCI-0-2 are both configured, the bit width of the beta_offset indicator in DCI format 0_2 is determined by the maximum bit width of the beta_offset indicator in DCI format 0_2 for HARQ-ACK of larger priority index on PUSCH of smaller priority index, HARQ-ACK of smaller priority index on PUSCH of smaller priority index, HARQ-ACK of smaller priority index on PUSCH of higher priority index, HARQ-ACK of higher priority index on PUSCH of higher priority index. A number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_2 for HARQ-ACKs with different priorities multiplexing on one PUSCH with larger or smaller priority index are the same.

-	DMRS sequence initialization – 0 or 1 bit

-	0 bit if the higher layer parameter dmrs-SequenceInitializationDCI-0-2 is not configured or if transform precoder is enabled;

-	1 bit if transform precoder is disabled and the higher layer parameter dmrs-SequenceInitializationDCI-0-2 is configured.

*** Unchanged text is omitted ***

------------------------------------------------------ End of text proposal ------------------------------------------------------

Conclusion

In this contribution, we discuss remaining issues for intra-UE multiplexing/prioritization and give the following proposals.

Proposal 1: If a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, LP HARQ-ACK is multiplexed with HP SR and HP HARQ-ACK where 1 bit HP SR is appended to HP HARQ-ACK bits and treated as HP UCI.

Proposal 2: If a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK and LP CSI/SR, only LP HARQ-ACK is multiplexed with HP UCI and LP CSI/SR are dropped.

Proposal 3: If the amount of REs available for LP HARQ-ACK mapping on the HP PUSCH is not sufficient to satisfy the coding rate of the LP HARQ-ACK based on the corresponding beta offset, LP HARQ-ACK is mapped to the rest REs after mapping HP HARQ-ACK or HP CSI .

Proposal 4: Adopt the following text proposal for section 7.3.1.1.2 and 7.3.1.1.3 of TS38.212.

----------------------------------------------------- Start of text proposal ------------------------------------------------------

- 7.3.1.1.2	Format 0_1

DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback information (CG-DFI) to a UE.

The following information is transmitted by means of the DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

*** Unchanged text is omitted ***

-	Second PTRS-DMRS association – 2 bits if PTRS-DMRS association field and SRS resource set indicator field are present and maxRank>2; 0 bit otherwise. Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second precoding information field when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field.

-	beta_offset indicator – 0 if the higher layer parameter betaOffsets = semiStatic; otherwise 2 bits as defined by Table 9.3-3 in [5, TS 38.213].

When two HARQ-ACK codebooks are configured for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-1 is configured, if the bit width of the beta_offset indicator in DCI format 0_1 for one HARQ-ACK codebook is not equal to that of the beta_offset indicator in DCI format 0_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_1 for the two HARQ-ACK codebooks are the same.

When higher layer parameter UCI-MuxWithDifferentPriority and priorityIndicatorDCI-0-1 are both configured, the bit width of the beta_offset indicator in DCI format 0_1 is determined by the maximum bit width of the beta_offset indicator in DCI format 0_1 for HARQ-ACK of larger priority index on PUSCH of smaller priority index, HARQ-ACK of smaller priority index on PUSCH of smaller priority index, HARQ-ACK of smaller priority index on PUSCH of higher priority index, HARQ-ACK of higher priority index on PUSCH of higher priority index. A number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_1 for HARQ-ACKs with different priorities multiplexing on one PUSCH with larger or smaller priority index are the same.

-	DMRS sequence initialization – 0 bit if transform precoder is enabled; 1 bit if transform precoder is disabled.

*** Unchanged text is omitted ***

- 7.3.1.1.3	Format 0_2

DCI format 0_2 is used for the scheduling of PUSCH in one cell.

The following information is transmitted by means of the DCI format 0_2 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

*** Unchanged text is omitted ***

-	Second PTRS-DMRS association – 2 bits if PTRS-DMRS association field and SRS resource set indicator field are present and maxRankDCI-0-2>2; 0 bit otherwise. Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second precoding information field when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field.

-	beta_offset indicator – 0 bit if the higher layer parameter betaOffsets = semiStatic; otherwise 1 bit if 2 offset indexes are configured by higher layer parameter dynamicDCI-0-2 as defined by Table 9.3-3A in [5, TS 38.213], and 2 bits if 4 offset indexes are configured by higher layer parameter dynamicDCI-0-2 as defined by Table 9.3-3 in [5, TS 38.213].

When two HARQ-ACK codebooks are configured for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-2 is configured, if the bit width of the beta_offset indicator in DCI format 0_2 for one HARQ-ACK codebook is not equal to that of the beta_offset indicator in DCI format 0_2 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_2 for the two HARQ-ACK codebooks are the same.

When higher layer parameter UCI-MuxWithDifferentPriority and priorityIndicatorDCI-0-2 are both configured, the bit width of the beta_offset indicator in DCI format 0_2 is determined by the maximum bit width of the beta_offset indicator in DCI format 0_2 for HARQ-ACK of larger priority index on PUSCH of smaller priority index, HARQ-ACK of smaller priority index on PUSCH of smaller priority index, HARQ-ACK of smaller priority index on PUSCH of higher priority index, HARQ-ACK of higher priority index on PUSCH of higher priority index. A number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_2 for HARQ-ACKs with different priorities multiplexing on one PUSCH with larger or smaller priority index are the same.

-	DMRS sequence initialization – 0 or 1 bit

-	0 bit if the higher layer parameter dmrs-SequenceInitializationDCI-0-2 is not configured or if transform precoder is enabled;

-	1 bit if transform precoder is disabled and the higher layer parameter dmrs-SequenceInitializationDCI-0-2 is configured.

*** Unchanged text is omitted ***

------------------------------------------------------ End of text proposal ------------------------------------------------------

Reference

- R1-2202830, Summary#3 of email thread [108-e-R17-IIoT-URLLC-04], Moderator (OPPO)