3GPP TSG RAN WG1 #109-e                                                                               R1-2203439

e-Meeting, May 9th – 20th, 2022

Source:	CATT

Title:	Remaining issues on PUSCH enhancements in Rel-17

Agenda Item:	8.8.1

Document for:	Discussion and Decision

Introduction

In Rel-17, NR supports coverage enhancement (CE) techniques for UL channels, including PUSCH, PUCCH and Msg3 PUSCH. The latest WID can be found in [1]. For PUSCH, several directions were targeted, i.e. increasing the maximum repetition number, counting based on available slots, TB processing over multiple slots, joint channel estimation, etc.

During RAN1#108-e meeting, agreements were reached for coverage enhancement [2]. So far, it is declared that all fundamental L1 designs for coverage enhancement are completed. But to make L1 design as perfect as possible, several open issues are better to be addressed. In this contribution, we provide our views on these open issues.

Discussion

Frequency hopping for PUSCH repetition Type A and TBoMS

In RAN1#108-e, companies discuss whether/how to add back the intra-slot hopping for single slot PUSCH transmission (especially for PUSCH scheduled by DCI format 0_0) [3]. The following conclusion was proposed based on almost-common understanding.

The latest version of Proposed conclusion on Issue#2-1: - It is RAN1’s common understanding that: - For PUSCH scheduled (with or without repetitions) by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, the UE is not required to read frequencyHopping in pusch-Config for the determination of frequency hopping type or for the determination of whether the frequency hopping is configured for the PUSCH and is required to read the frequency hopping flag information field of the RAR UL grant or the DCI format 0_0 with CRC scrambled by TC-RNTI for the determination of whether the frequency hopping is configured for the PUSCH or not. - For PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, the conclusion for R1-2003594 in RAN1#101-e is applied.

For convenience, the conclusion for R1-2003594 in RAN1#101-e is copied as follows [4]:

Decision: R1-2003594 is not pursued for Rel-15. Conclusion Draft CR in R1-2003594 is not pursued for Rel-15 with the following understanding - For a PUSCH scheduled by a DCI format 0_0 with CRC scrambled by C-RNTI, MCS-C-RNTI or CS-RNTI, the frequency hopping field in the DCI format 0_0 is not expected to be set to 1 if frequencyHopping in PUSCH-Config is not provided or set to interSlot. - For Type 2 CG PUSCH activated by DCI format 0_0 with CRC scrambled by CS-RNTI, the frequency hopping field in the DCI format 0_0 is not expected to be set to 1 if frequencyHopping in configuredGrantConfig is not provided or set to interSlot.

Drawing a clear conclusion provides a clear understanding on the hopping issue, which is beneficial and avoids wasting the group’s time on interpreting the specification in the future. Unfortunately, the proposed conclusion was not approved due to last time comments in GTW2, asking whether the proposed conclusion is for Rel-17 or earlier releases (Rel-15/16).

In our opinion, it is no doubt that the proposed conclusion is for Rel-17. The first bullet explains the new hopping behavior when Msg3 is scheduled with or without repetition introduced in Rel-17, and the second bullet shows that legacy understanding is still valid for PUSCH scheduled by DCI format 0_0. Indeed, the proposed conclusion quotes the legacy understanding for legacy PUSCH scheduled by DCI format 0_0 in the second bullet, but it is not intended to change any of the legacy understanding in Rel-15/16. To make it clearer, we can consider adding a note to clarify that the Rel-17 approach of frequency hopping determination is follows the same approach in Rel-15/Rel-16.

Therefore, we have the following proposal:

Proposal 1: Approve the following conclusion,

- It is RAN1’s common understanding that for Rel-17:

  - For PUSCH scheduled (with or without repetitions) by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, the UE is not required to read frequencyHopping in pusch-Config for the determination of frequency hopping type or for the determination of whether the frequency hopping is configured for the PUSCH and is required to read the frequency hopping flag information field of the RAR UL grant or the DCI format 0_0 with CRC scrambled by TC-RNTI for the determination of whether the frequency hopping is configured for the PUSCH or not.

    - Note: This follows the same approach of frequency hopping determination for PUSCH scheduled (without repetitions) by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI in Rel-15/Rel-16.

  - For PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, the conclusion for R1-2003594 in RAN1#101-e is applied.

TP for frequency hopping

Several TPs were proposed [3] to add back the intra-slot frequency hopping scheduled by DCI format 0_0. The most concerned point is to distinguish the following two cases:

- Msg3 transmission/retransmission, w/wo repetition: The frequency hopping can be regarded as ‘enabled’ by natural, and the detailed hopping type (i.e. intra-slot or inter-slot) is determined by whether Msg3 is repeated or not, but not any dedicated RRC configuration. To determine the hopping is actually performed or not, the UE only need to read frequency hopping flag information field indicated in the RAR UL grant/DCI scrambled by TC-RNTI.

- Single PUSCH scheduled by DCI format 0_0: The existence of higher layer parameter frequencyHopping determines whether frequency hopping is enabled or not, which also configures detailed the hopping type (i.e. intra-slot or inter-slot). However, a UE can expect frequency hopping to be set to 1 (i.e. perform frequency hopping) only if the higher layer parameter frequencyHopping is provided in PUSCH-Config and set to intraSlot.

It can be seen from the above analysis that the logic of these two cases is different. Subsequently, it seems better to list them in a parallel manner, rather than putting them together. With this principle, the following TP, i.e. Variant#2 of TP#3 seems suitable to go with, which also well aligns the conclusion for R1-2003594.

- Variant#2 of TP#3 - 6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots For PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in configuredGrantConfig for configured PUSCH transmission. For PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, a UE is configured for frequency hopping by the higher layer frequencyHopping provided in pusch-Config set to ‘intraSlot’. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.

Additionally, we suggest a minor update on top of the change part in Variant#2 of TP#3, i.e. adding an ‘and’ between ‘pusch-Config’ and ‘set to’, which makes it consistent to the previous conclusion for R1-203594 in RAN1#101-e (i.e. frequencyHopping in PUSCH-Config is not provided or set to interSlot).

In summary, the following TP is considerable, with the additional information below:

Reason for change: Addback the intra-slot frequency hopping of PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI in Rel-17 TS 38.214, based on the conclusion for R1-2003594 in RAN1#101-e. Summary of change: Add a sentence to describe the intra-slot frequency hopping of PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI in Clause 6.3.1 of TS 38.214 in Rel-17. Consequence if not approved: PUSCH scheduled by DCI format 0_0 cannot perform intra-slot hopping.

- 6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots For PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in configuredGrantConfig for configured PUSCH transmission. For PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, a UE is configured for frequency hopping by the higher layer frequencyHopping provided in pusch-Config and set to ‘intraSlot’. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.

We have the following proposal:

Proposal 2: Adopt the following TP for frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots in Rel-17 TS 38.214:

- 6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots For PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in configuredGrantConfig for configured PUSCH transmission. For PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, a UE is configured for frequency hopping by the higher layer frequencyHopping provided in pusch-Config and set to ‘intraSlot’. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.

Conclusion

In this contribution, we discussed remaining open issues for PUSCH enhancement for Rel-17 coverage enhancement. The proposals are summarized as follows:

Proposal 1: Approve the following conclusion,

- It is RAN1’s common understanding that for Rel-17:

  - For PUSCH scheduled (with or without repetitions) by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, the UE is not required to read frequencyHopping in pusch-Config for the determination of frequency hopping type or for the determination of whether the frequency hopping is configured for the PUSCH and is required to read the frequency hopping flag information field of the RAR UL grant or the DCI format 0_0 with CRC scrambled by TC-RNTI for the determination of whether the frequency hopping is configured for the PUSCH or not.

    - Note: This follows the same approach of frequency hopping determination for PUSCH scheduled (without repetitions) by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI in Rel-15/Rel-16.

  - For PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, the conclusion for R1-2003594 in RAN1#101-e is applied.

Proposal 2: Adopt the following TP for frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots in Rel-17 TS 38.214:

- 6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots For PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in configuredGrantConfig for configured PUSCH transmission. For PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, a UE is configured for frequency hopping by the higher layer frequencyHopping provided in pusch-Config and set to ‘intraSlot’. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.

References

- RP-211566, Revised WID on NR coverage enhancements, China Telecom, RAN#92-e, e-Meeting, June 14th – 18th, 2021.

- Chairman's Notes RAN1#108-e, RAN1#108-e, e-Meeting, February 21st – March 3rd, 2022.

- R1-2202569, FL Summary #4 on Enhancements on PUSCH repetition type A, moderator (Sharp), RAN1#108-e, e-Meeting, February 21st – March 3rd, 2022.

- Chairman’s Notes RAN1#101-e, RAN1#101-e, e-Meeting, August 17th – 28th, 2020.