3GPP TSG RAN WG1 #109-e		R1- 2203440

e-Meeting, May 9th – 20th, 2022

Source:	CATT

Title:	Remaining issues on joint channel estimation in Rel-17

Agenda Item:	8.8.2

Document for:	Discussion and Decision

Introduction

The basic framework of joint channel estimation for both PUSCH and PUCCH has been completed after two-year hard work including the design on enabling/disabling DMRS bundling, the configuration of configured time domain window, the division of actual time domain window caused by semi-static or dynamic events and the new frequency hopping pattern, etc.

RAN1#108-e meeting is a closing point for work items in Rel-17 RAN1. Most remaining issues on joint channel estimation have been solved with efforts and appropriate compromise. A set of agreements were achieved during last meeting [1]. Unfortunately, there are still a few problems left to the maintenance stage.

In this contribution, we provide our views on how to capture the enhanced TPC command and how to calculate the physical slot index when determining the frequency hopping pattern for PUSCH.

Discussion

Remaining issue on TPC command

It is well understood that the action of the TPC adjustment would violate power consistency and phase continuity. Hence, a working assumption was achieved to modify the action time of the TPC command to maintain the power consistency and phase continuity when DMRS bundling is enabled in RAN1#107e-meeting [2].

Working assumption: - The action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity. - If UE is configured to accumulate TPC commands, - If UE receives TPC commands that would take into effect during a configured TDW, UE accumulates TPC commands without taking effect during the current configured TDW. TPC commands take effect after the current configured TDW. - If UE is not configured to accumulate TPC commands - the last TPC command that would take effect within a configured TDW supersedes all previous TPC commands that take effect within that configured TDW and only the last TPC command is applied by the UE after the current configured TDW. - FFS: no more than 1 TPC command is expected to take effect during a configured TDW.

Whether absolute TPC command can be supported for group common TPC with DCI format 2_2 was under discussion in [3] without consensus, companies focused on how to capture the enhancement on the accumulate TPC command in the specification. Three approaches with six variant options were proposed during last meeting, which were proved to be workable in general [4].

- Option 1: Legacy definition of KPUSCHi is preserved for PUSCH transmissions without DM-RS bundling. Redefine KPUSCHi for PUSCH transmissions within a nominal TDW in case of DM-RS bundling. e.g., KPUSCHi is a number of symbols from K symbols before the start of the first repetition within the nominal time domain window including the transmission occasion i and before a first symbol of the transmission occasion i. - FFS: the value of K, e.g., K is “a number of KPUSCH,min symbols equal to the product of a number of symbols per slot, Nsymbslot, and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP b of carrier f of serving cell c”. - Option 2a: Modify the TPC command value set Di, e.g. if transmission occasion i is not the first transmission occasion within a nominal time domain window, then any TPC command values received via DCI format 2_2 contained in the set Di are deleted and added to the set Dj where j is a transmission occasion occurring after the end of the nominal time domain window and j>0 is the smallest integer for which KPUSCH(i) symbols before PUSCH transmission occasion i is earlier than KPUSCH(j) symbols before PUSCH transmission occasion j. - Option 3: For group common TPC commands with format 2_2, if UE is configured to accumulate TPC commands, - For a transmission occasion i occurs within a nominal time domain window, fi=f i1, where transmission occasion i1 is a first transmission occasion within the nominal time domain window. - For the first transmission occasion k occurring after the nominal time domain window, fk=fi1+δj, where δj is all the TPC command values that would take effect for the transmission occasions occurring after transmission occasion i1 and no later than transmission occasion k (i.e. including occasion k itself). - Option 3a: For group common TPC commands with format 2_2, if UE is configured to accumulate TPC commands, - For a transmission occasion i occurs within a nominal time domain window, fi=f i1, where transmission occasion i1 is a first transmission occasion within the nominal time domain window. - For the first transmission occasion k occurring after the nominal time domain window, fk=fi1+δj, where δj is all the TPC command values that would take effect for the transmission occasions occurring after are received between KPUSCHi1 symbols before transmission occasion i1 and no later than KPUSCHk symbols before transmission occasion k (i.e. including occasion k itself). - Note: If a UE is configured with DMRS bundling, for a given PUSCH power control adjustment state l in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission occasion m in symbol p and is configured with any CG PUSCH transmission occasion n ending before symbol p, the UE is not expected to receive a TPC command by DCI format 2_2 arrived later than KPUSCHm symbols before transmission occasion m but no later than KPUSCHn symbols before transmission occasion n. - Option 3b: For group common TPC commands with format 2_2, if UE is configured to accumulate TPC commands, - For a transmission occasion i occurs within a nominal time domain window, fi=f i1, where transmission occasion i1 is a first transmission occasion within the nominal time domain window. - For the first transmission occasion i1 within the nominal time domain window, fi1=fi1-i1_0+δj, where δj is all the TPC command values that would take effect for the transmission occasions occurring after are received between KPUSCHi1 symbols before transmission occasion i1 and no later than KPUSCHi1-i1_0 symbols before transmission occasion i1-i1_0 (i.e. including occasion k itself) where transmission occasion i1-i1_0 is the first transmission occasion of another nominal time domain window with conditions that i1_0 is the smallest integer and KPUSCHi1-i1_0 symbols before PUSCH transmission occasion i1-i1_0 is earlier than KPUSCHi1 symbols before PUSCH transmission occasion i1 - Option 1&3a: For group common TPC commands with format 2_2, if UE is configured to accumulate TPC commands and configured with DMRS bundling, - For a transmission occasion i occurs within a nominal time domain window, fi=f i1, KPUSCHi for PUSCH transmissions occasion i from K KPUSCHi1 symbols before the start a first symbol of the transmission occasion i1 before to a first symbol of the transmission occasion i where transmission occasion i1 is a first transmission occasion within the nominal time domain window for the same transport block as transmission occasion i and KPUSCHi1 reuses the same Rel-15/16 definition of KPUSCHx. - The definition of set for in Rel-15/16 TS 38.213 is reused.

In addition, to find way out of the intense debate without consensus, the following proposal was suggested in the last meeting [4].

Proposal 9: - If no consensus on how to capture the working assumption can be reached in RAN1#108-e, the action of group common TPC commands with format 2_2 is regarded as an event.

According to the aforementioned situation, we propose to down select from the following two alternatives to deal with the problem:

- Alt 1: Adopt Option 3 to capture the enhancement on the accumulate TPC command.

- Alt 2: Not confirm the WA and treat the action of TPC command as an event.

For Alt 1, it is recalled that Option 3 is supported by the majority, which is the closest one to the formulation of the WA. We prefer to support Option 3 considering that it is the most readable and straightforward one with the description of WA.

On the other hand, it is recognized that possibly neither of the options can work correctly in the condition of CG-DG interlacing. It will be unfortunate that the above working assumption cannot be confirmed since CG-DG interlacing is not precluded by the current specification and is possible in reality after all. Then, treating the action of the TPC command with format 2_2 for CG-PUSCH repetitions as a dynamic event can be a fallback option.

Proposal 1: down select from the following two alternatives:

-	Alt 1: Adopt Option 3 to capture the enhancement on the accumulate TPC command.

-	Alt 2: Not confirm the WA and treat the action of TPC command as an event.

Remaining issue on frequency hopping pattern for PUSCH

Before discussing the issue on inter-slot frequency hopping pattern for PUSCH repetitions with DMRS bundling, a tiny modification should be considered for current inter-slot frequency hopping pattern. As shown in the latest specification in TS38.214 [5].

In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is not enabled, the starting RB during slot is given by: , where is the current slot number within a system radio frame, where a multi-slot PUSCH transmission can take place, is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) and is the frequency offset in RBs between the two frequency hops. In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is enabled, the starting RB during slot is given by: [RBstartns,fμ=RBstart RBstart+RBoffsetmodNBWPsize ns,fμ+nf∙Nslotframe,μNFH mod 2=0ns,fμ+nf∙Nslotframe,μNFH mod 2=1] where ns,fμ is the current slot number within a system radio frame, nf is the number of the system radio frame containing the current slot, Nslotframe,μ is the number of slots per frame for subcarrier spacing configuration μof the UL BWP that the PUSCH is transmitted on, NFH is the value of the higher layer parameter PUSCH-Frequencyhopping-Interval, is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) andis the frequency offset in RBs between the two frequency hops.

In case of inter-slot frequency hopping pattern for PUSCH repetitions without DMRS bundling, nsμ is defined as the slot index within a system radio frame, which is inconsisitent with the definition of the slot index within a subframe as specified in TS 38.211 [6]. In addition, ns,fμ is used as the current slot number within a system radio frame in case of inter-slot frequency hopping pattern for PUSCH repetitions with DMRS bundling. From the perspective of context consistency, we should replace nsμ with ns,fμ, where ns,fμ is the slot index within a system radio frame already defined in TS 38.211 too [6]. It should be noted that the parameter of the slot index highlighted in yellow when PUSCH-DMRS-Bundling is enabled must be modified to align with the parameter in the formula and subsequent explanation.

In summary, a text proposal is needed for inter-slot frequency hopping when DMRS bundling is disabled and for inter-slot frequency hopping when DMRS bundling is enabled.

Reason for change: When nsμ is defined as the slot index within a system radio frame, it is inconsisitent with the description of the slot index within a subframe as specified in TS 38.211. Considering ns,fμ is used in case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is enabled, the same parameter name should be used when PUSCH-DMRS-Bundling is not enabled. In addition, the misalignment even exists in the description of slot index when PUSCH-DMRS-Bundling is enabled. Hence, ns,fμ should be used to indicate the slot number where a multi-slot PUSCH transmission can take place, where ns,fμ is the slot index within a system radio frame already defined in TS 38.211.

Summary of change: Replace nsμ with ns,fμ in the formula of calculating the starting RB for inter-slot frequency hopping, where ns,fμ is the slot index within a system radio frame already defined in TS 38.211.

Consequences if not approved: The name of the parameter used as the current slot number within a system radio frame is misaliment in case of inter-slot frequency hopping pattern for PUSCH repetitions with DMRS bundling. In addition, the misalignment also exists between the PUSCH repetitions with DMRS bundling and the PUSCH repetitions without DMRS bundling.

Proposal 2: Adopt the following text proposal for inter-slot frequency hopping when DMRS bundling is not enabled and for inter-slot frequency hopping when DMRS bundling is enabled.

In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is not enabled, the starting RB during slot ns,fμ is given by: , RBstartns,fμ=RBstart RBstart+RBoffsetmodNBWPsize ns,fμ mod 2=0ns,fμ mod 2=1, where ns,fμ is the current slot number within a system radio frame, where a multi-slot PUSCH transmission can take place, is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) and is the frequency offset in RBs between the two frequency hops. In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is enabled, the starting RB during slot ns,fμis given by: [RBstartns,fμ=RBstart RBstart+RBoffsetmodNBWPsize ns,fμ+nf∙Nslotframe,μNFH mod 2=0ns,fμ+nf∙Nslotframe,μNFH mod 2=1] where ns,fμ is the current slot number within a system radio frame, nf is the number of the system radio frame containing the current slot, Nslotframe,μ is the number of slots per frame for subcarrier spacing configuration μof the UL BWP that the PUSCH is transmitted on, NFH is the value of the higher layer parameter PUSCH-Frequencyhopping-Interval, is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) andis the frequency offset in RBs between the two frequency hops.

Further, it was agreed that inter-slot frequency hopping pattern for PUSCH repetitions with DMRS bundling is determined based on physical slot index in RAN1 #108-e meeting.

Agreement Inter-slot frequency hopping pattern for PUSCH repetitions with DMRS bundling is determined based on physical slot index

However, in the latest TS 38.214 after RAN1 #108-e, square brackets were added to the calculation formula, considering there was no consensus on whether system frame number (SFN) should be included in the determination of physical slot index when calculating the starting RB in each hop [5].

In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is enabled, the starting RB during slot is given by: [RBstartns,fμ=RBstart RBstart+RBoffsetmodNBWPsize ns,fμ+nf∙Nslotframe,μNFH mod 2=0ns,fμ+nf∙Nslotframe,μNFH mod 2=1] where ns,fμ is the current slot number within a system radio frame, nf is the number of the system radio frame containing the current slot, Nslotframe,μ is the number of slots per frame for subcarrier spacing configuration μof the UL BWP that the PUSCH is transmitted on, NFH is the value of the higher layer parameter PUSCH-Frequencyhopping-Interval, is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) andis the frequency offset in RBs between the two frequency hops.

In our opinion, it is not enough to reuse ns,fμ as the physical slot index within a frame when calculating the starting RB in each hop. Considering some of the candidate hopping interval NFH would be larger than 10, ns,fμ cycles from zero to nine which cannot meet the requirement of slot index when SCS equals to 15 kHz. In addition, if NFH is configured as the same as the number of slots in a system radio frame, ns,fμ cycles from zero to NFH-1 results in  ns,fμ / NFH in each frame is always equal to zero. In other words, all of the PUSCH repetitions transmitted in the same frequecy domain position. Hence, we propose to use the system frame number, i.e. nf, to determine the physical slot index. Then, the physical slot index nindex equals to the ns,fμ plus the product of nf and Nslotframe,μ. Note that nf is already defined in TS 38.211 too [6].

Proposal 3: System frame number nf should be used to determine the physical slot index. Remove the square brackets of the formula of inter-slot frequency hopping with inter-slot bundling.

According to the above analysis, a text proposal is needed for the calculation formula of the starting RB in case of inter-slot frequency hopping when PUSCH-DMRS-Bundling is enabled.

Reason for change: The current formula can meet the requirement of slot index when SCS equals to 15 kHz even if the candidate hopping interval NFH is configured larger than 10. In addition, the current formula can ensure that the starting RB of of all PUCCH repetitions is distinguishable when the hopping interval NFH equals to the number of slots in a system radio frame. Hence, the current formula can be used to calculate the starting RB in case of inter-slot frequency hopping when PUSCH-DMRS-Bundling is enabled.

Summary of change: Directly remove the square brackes of the formula of inter-slot frequency hopping with inter-slot bundling.

Consequences if not approved: The formula of the starting RB continues to be pending in case of inter-slot frequency hopping when PUSCH-DMRS-Bundling is enabled.

Proposal 4: Adopt the following text proposal for the calculation formula of inter-slot frequency hopping with inter-slot bundling.

In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is enabled, the starting RB during slot is given by: [RBstartns,fμ=RBstart RBstart+RBoffsetmodNBWPsize ns,fμ+nf∙Nslotframe,μNFH mod 2=0ns,fμ+nf∙Nslotframe,μNFH mod 2=1] where ns,fμ is the current slot number within a system radio frame, nf is the number of the system radio frame containing the current slot, Nslotframe,μ is the number of slots per frame for subcarrier spacing configuration μof the UL BWP that the PUSCH is transmitted on, NFH is the value of the higher layer parameter PUSCH-Frequencyhopping-Interval, is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) andis the frequency offset in RBs between the two frequency hops.

Conclusion

In this contribution, we share our views on PUSCH coverage enhancement on joint channel estimation. The proposals are summarized as follows:

Proposal 1: down select from the following two alternatives:

-	Alt 1: Adopt Option 3 to capture the enhancement on the accumulate TPC command.

-	Alt 2: Not confirm the WA and treat the action of TPC command as an event.

Proposal 2: Adopt the following text proposal for inter-slot frequency hopping when DMRS bundling is not enabled and for inter-slot frequency hopping when DMRS bundling is enabled.

In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is not enabled, the starting RB during slot ns,fμ is given by: , RBstartns,fμ=RBstart RBstart+RBoffsetmodNBWPsize ns,fμ mod 2=0ns,fμ mod 2=1, where ns,fμ is the current slot number within a system radio frame, where a multi-slot PUSCH transmission can take place, is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) and is the frequency offset in RBs between the two frequency hops. In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is enabled, the starting RB during slot ns,fμis given by: [RBstartns,fμ=RBstart RBstart+RBoffsetmodNBWPsize ns,fμ+nf∙Nslotframe,μNFH mod 2=0ns,fμ+nf∙Nslotframe,μNFH mod 2=1] where ns,fμ is the current slot number within a system radio frame, nf is the number of the system radio frame containing the current slot, Nslotframe,μ is the number of slots per frame for subcarrier spacing configuration μof the UL BWP that the PUSCH is transmitted on, NFH is the value of the higher layer parameter PUSCH-Frequencyhopping-Interval, is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) andis the frequency offset in RBs between the two frequency hops.

Proposal 3: System frame number nf should be used to determine the physical slot index. Remove the square brackets of the formula of inter-slot frequency hopping with inter-slot bundling.

Proposal 4: Adopt the following text proposal for the calculation formula of inter-slot frequency hopping with inter-slot bundling.

In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is enabled, the starting RB during slot is given by: [RBstartns,fμ=RBstart RBstart+RBoffsetmodNBWPsize ns,fμ+nf∙Nslotframe,μNFH mod 2=0ns,fμ+nf∙Nslotframe,μNFH mod 2=1] where ns,fμ is the current slot number within a system radio frame, nf is the number of the system radio frame containing the current slot, Nslotframe,μ is the number of slots per frame for subcarrier spacing configuration μof the UL BWP that the PUSCH is transmitted on, NFH is the value of the higher layer parameter PUSCH-Frequencyhopping-Interval, is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) andis the frequency offset in RBs between the two frequency hops.

Reference

- Chairman's Notes RAN1#108-e, 3GPP TSG RAN WG1 Meeting #108-e, e-Meeting, February 21st – March 3rd, 2022.

- Chairman's Notes RAN1#107bis-e, 3GPP TSG RAN WG1 Meeting #107bis-e, e-Meeting, January 17th – 25th, 2022.

- R1-2202834, “Summary of [108-e-NR-CRs-03] Issue#4 Absolute TPC timeline and support of absolute TPC with DCI format 2_2”, Moderator (ZTE Corporation), RAN1 #108-e, February 21st – March 3rd, 2022.

- R1-2202869, “FL Summary#4 of joint channel estimation for PUSCH”, Moderator (China Telecom), RAN1 #108-e, February 21st – March 3rd, 2022.

- TS 38.214, Physical layer procedures for data, v17.1.0.

- TS 38.211, Physical channels and modulation, v17.0.0.