3GPP TSG RAN WG1 #109-e		  R1-2203442

e-Meeting, May 9th – 20th, 2022

Source:        CATT

Title:	      On Two TAs for UL multi-DCI for multi-TRP operation

Agenda Item:   9.1.1.2

Document for:  Discussion and Decision

- Introduction

As approved in RAN #94-e meeting, TA enhancements include the following objectives [1]:

- Study, and if justified, specify the following

  - Two TAs for UL multi-DCI for multi-TRP operation

- For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

In this contribution, TA related issues are discussed with the following outline: TA acquisition is discussed in section 2. In Section 3, TA indication are considered, which include the association between TA and TRP followed by TA updating. A brief conclusion is given in Section 4.

- TA Acquisition

For UL multi-DCI for multi-TRP operation, the propagation delay between different UE-TRP pair might lead to a degradation of uplink performance, it is shown in [2] that 0.2CP of receiving delay results in non-negligible performance degradation.

Fig.1. Illustration of propagation delay between UE-to-TRP1 and UE-to-TRP2

As illustrated in Fig.1., suppose there are two TRPs, TRP1 and TRP2. Table 1 shows the distance difference between UE-to-TRP1 and UE-to-TRP2 when 0.2CP propagation delay is required. SCS of 15KHz, 30KHz, 60KHz and 120KHz are assumed with FFT sampling rate 2048. From the table it can be seen that to avoid deterioration of the uplink performance, the distance difference between 2TRPs with 15KHz SCS should be less than 281.4m. For FR2(e.g. 60KHz), the difference between 2 links should be less than 70.5m.

Table.1. Distance difference of UE-to-TRP links when two TRPs are synchronized

| SCS(KHz) | 15KHz | 30KHz | 60KHz | 120KHz |
| --- | --- | --- | --- | --- |
| 0.2CP(us) | 0.938 | 0.469 | 0.235 | 0.117 |
| Dis. difference of 2 UE-to-TRP links(m) | 281.4 | 140.7 | 70.5 | 35.1 |

In current uplink Rel-17 multi-TRP transmission, one TA is used to adjust the timing of different uplink TRP-UE pairs, e.g. in the first transmission occasion, UE transmits PUSCH/PUCCH towards TRP1 with TA1. In another transmission occasion, UE transmits PUSCH/PUCCH towards TRP2, with the same timing advance TA1 applied. In this case, the propagation delay between two UE-to-TRP links as described above might lead to a degradation of the uplink performance.

Proposal-1: Consider TRP-specific TA to mitigate the possible performance loss due to imperfect UL synchronization between UE-TRP1 and UE-TRP2 transmission links.

In NR Rel-16/17, initial TA is obtained through initial Random Access procedure. In the CBRA, the UE randomly selects a preamble in the PRACH Occasion associated with the reference signal of the beam the UE wants to transmit on. This preamble is detected by the gNB receiver. With the detected preamble, the round trip delay is estimated by gNB and indicated to UE as TA by MAC RAR.

UE will receive RAR MAC PDU in the RAR window after sending the preamble and use the TA command(12bits) included in the MAC RAR to calculate the absolute value of Timing advance NTA. UE will use NTA together with a timing offset NTA_offset provided by gNB to perform timing adjustment of Msg3 on the basis of the downlink receiving frame. A time relation between uplink and downlink timing is shown in Fig.2.

Fig.2. Uplink-downlink timing relation

In NR Rel-18 UL multi-DCI for multi-TRP operation, two TAs are proposed to adjust the different UE-to-TRP uplink transmission. In the implementation, the first TA can be obtained by the legacy TA acquisition mechanism to keep spec impact as minimum as possible. For the second TA acquisition, following schemes can be considered:

Alt1:  TA acquisition based on UL reference signals including preamble, SRS, etc.

Alt2: TA acquisition based on DL receiving difference.

For Alt1, to obtain the second TA after gNB obtains the TA of one TRP(TA1), gNB can trigger UE to send a preamble to the second TRP. E.g. by DCI. After gNB detects the preamble, it will estimate TA of the second TRP(TA2) using similar method as the legacy one. In the implementation, the reference signal for the second TA acquisition can be other reference signals like SRS or DM RS of PUSCH. In this procedure, the beam information for the transmission to the second TRP might be added in the DCI.

For Alt2, the second TA is acquired by estimating DL receiving difference (timing offset) by UE and then report to gNB. Downlink reference signal like SSB/CSI-RS can be used for estimation. Upon the timing offset received from UE, gNB will figure out the TA of the second TRP(TA2) and indicate two TAs to UE for the subsequent uplink timing adjustment. For Alt2, the reporting scheme might need to be considered, e.g. using CSI report framework.

Proposal-2: For TA acquisition, consider UL reference signal(e.g. preamble, SRS, etc) based TA acquisition as well as downlink reference based TA acquisition schemes.

- TA indication

# 3.1 Association between TA and TRP

In UL multi-DCI for multi-TRP transmission, to ensure the estimated TA be applied to the corresponding TRP in PUSCH/PUCCH transmission, association between TA and TRP needs to be established with the following schemes:

Opt1:  Association between TA and PUSCH/PUCCH

Opt2:  Association between TA and CORESETPoolIndex

Opt3:  Association between TA and uplink transmission occasion(e.g. slot/subframe/symbol)

For Opt1, each TA can be associated with TCI state of the PUSCH/PUCCH transmission, or with the SRS resource set/PUCCH resource. When two TCI states are indicated for PUSCH/PUCCH transmission to two TRPs, the associated TA can be used accordingly.

For Opt2, each TA can be associated with the CORESETPoolIndex of the CORESET scheduling the PDSCH, through the association of TA with PDSCH, the TA is subsequently associated with the PUCCH carrying the HARQ/ACK of the PDSCH. In this case, whether the implementation of opt 2 can be applied to other PUCCH, e.g. PUCCH for CSI reporting needs to be further discussed. For PUSCH, each TA can be associated with the CORESETPoolIndex of the CORESET scheduling PUSCH.

For Opt3, predefined rules are needed considering the PUSCH/PUCCH transmission occasions. e.g. #TA1 #TA1 #TA2 #TA2 is applied corresponding to the PUSCH transmission slot. Opt 3 can be applied to certain transmission scenarios, e.g. Rel-17 TDM repetition for PUSCH/PUCCH.

Proposal-3: TA can be associated with TRP implicitly by TCI state/CORESETPoolIndex or by an explicit TASG ID, where TASG is introduced to represent TRP, each TAG consists of several TASG and each TASG corresponds to one TRP, UE maintains one TA for each TASG.

# 3.2 TA updating

In Rel-15/16, if uplink out-of-synchronization is detected, gNB will estimate timing advance by triggering an uplink reference signal(e.g. preamble) transmission. The estimated TA is indicated to UE by TA command MAC-CE, where TA Command is operated with TAG (Timing Advance Group) identified by TAG ID. Each TAG consists of a group of CCs sharing a same TA. Upon the indication from gNB, UE will calculate the absolute value of the timing advance and use it on the consequent PUSCH/PUCCH/SRS transmission, the absolute value is calculated by:

NTA_new = NTA_old + (TA-31)*16*64/2u. TA = 0, 1, 2, 3, …, 63.

where TA is indicated by gNB, 2u represents the SCS of 2u x 15K. NTA_old is the absolute value of TA for previous TA adjustment.

In UL multi-DCI for multi-TRP transmission, multiple TAs are needed to adjust the multiple uplink transmission to different TRPs. As described in section 3.1, each TA can be associated with one TRP explicitly by a TASG ID, with each TASG corresponding to one TRP, or implicitly by TCI state/CORESETPoolIndex. In the implementation, two MAC-CEs are indicated each consists of one TASG ID/CORESETPoolIndex and the corresponding TA command. E.g. gNB transmits two MAC-CEs, MAC-CE1 corresponds to TA1 toward TRP1 transmission, MAC-CE2 corresponds to TA2 toward TRP2 transmission. Alternatively, only one MAC-CE can be used to indicate two TAs, in this way, the offset between two TAs is included to differentiate two TAs.

Proposal-4: Use two TASG IDs/CORESETPoolIndexes each corresponding to one TA or one TASG/CORESETPoolIndex corresponding to two TAs.

- Conclusion

In this contribution, we focus on the enhancement on two TAs for uplink multi-DCI for multi-TRP operation, Initial TA acquisition and TA indication are mainly discussed. We have the following proposals:

Proposal-1: Consider TRP-specific TA to mitigate the possible performance loss due to imperfect UL synchronization between UE-TRP1 and UE-TRP2 transmission links.

Proposal-2: For TA acquisition, consider UL reference signal (e.g. preamble, SRS, etc) based TA acquisition as well as downlink reference based TA acquisition schemes.

Proposal-3: TA can be associated with TRP implicitly by TCI state/CORESETPoolIndex or by an explicit TASG ID, where TASG is introduced to represent TRP, each TAG consists of several TASGs and each TASG corresponds to one TRP, UE maintains one TA for each TASG.

Proposal-4: Use two TASG IDs/CORESETPoolIndexes each corresponding to one TA or one TASG/CORESETPoolIndex corresponding to two TAs.

- References

[1] RP-213598, New WID: MIMO Evolution for Downlink and Uplink, RAN #94-e.

[2] R1-062834, UL Timing Control Accuracy and Update Rate, Nokia.