3GPP TSG RAN WG1 #109-e			R1-2203443

e-Meeting, May 9th – 20th, 2022

Source: 	CATT

Title:	On Rel-18 CSI enhancements

Agenda Item:	9.1.2

Document for:	Discussion and Decision

Introduction

In RAN #94 e-meeting, the CSI enhancement for medium/high mobility and coherent-JT was agreed and the following objectives were approved in the WID for Rel-18 NR MIMO [1]:

Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - UE reporting of time-domain channel properties measured via CSI-RS for tracking Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

In this contribution, we will provide our views on CSI enhancement for high/medium mobility and coherent-JT in Rel-18.

CSI reporting enhancement for high/medium UE velocities

Motivation

For mobile UEs with high/medium velocities, if update rate of CSI cannot match channel coherence time, system performance would degrade due to CSI aging, especially when high accuracy CSI is used. Figure 1 shows the performance with increasing speed for the same CSI update rate and Table I in appendix provides the simulation assumptions.

Figure 1 Performance loss with increasing speed for the same CSI feedback rate

Observation-1:

- When the CSI feedback periodicity is 5ms, the average throughput of 60km/h has 22% loss and the 5% edge throughput of 60km/h has 45% loss compared with 3km/h.

In order to meet performance requirement, CSI needs to be updated frequently to match the fast changing channel. However, high CSI update rate leads to large feedback overhead and increases UE power consumption. Although the channel is changing, there is correlation among adjacent instant in time domain, or the channel can be considered to be nearly constant in Doppler domain. Time-domain correlation and/or Doppler-domain information can be exploited to reduce CSI feedback overhead and/or improve accuracy of CSI. For this purpose, the performance, overhead (reporting overhead and/or RS overhead), UE/gNB complexity and the specification impact of CSI enhancement for medium/high mobility shall be taken into account together.

Proposal-1:

- Support CSI enhancement for high/medium UE velocities by exploiting Doppler-domain information.

CSI prediction

As discussed in section 2.1, the channel variability in time domain caused by the UE medium/high movement makes it difficult for gNB to obtain accurate channel at all the time instants between two instants of CSI feedback for FDD or two instants of UL RS reception for TDD. Thus, if gNB/UE can obtain Doppler information, it might perform channel prediction for some time in the future. For example, the channel impulse response Hu,st at time 𝑡 for a transmitter and receiver element pair u,s is modelled as following.

Hu,st=kk+1Hu,sLOSt+1k+1Hu,sNLOSt

Hu,st=kk+1αlosexp(j2πfd,lost)exp (-j2πfτlos)+1k+1n=1Nm=1Mαm,nexp(j2πfd,m,nt)exp(-j2πfτn)

- k is the Rician factor-K

- αlos is the complex channel gain of LOS path, and αm,n is the complex channel gain of subpath m in cluster n

- fd,los is the Doppler shift of LOS path, and fd,m,nis the Doppler shift of subpath m in cluster n

- τlos is the delay of LOS path, and τn is the delay of all the subpaths in cluster n

It can be seen that the channel at time 𝑡 is the combination of multiple delay paths, including LOS path and multiple subpaths in different cluster. As a result, if we can assume that the Doppler shift and delay of each path are constant in a time interval, the only changes at different time instants are the phase shifts of each path caused by Doppler shift. Therefore, if gNB/UE can obtain Doppler shift of each path, in theory, DL CSI at any time can be predicted. And based on the predicted channel, gNB can calculate precoding matrix for future transmission. For example, the channel impulse response Hu,st+∆t at time t+∆t for the element pair u,s is given by the following formula.

Hu,st+∆t=kk+1αlosexp(j2πfd,los(t+∆t))exp(-j2πfτlos)+1k+1n=1Nm=1Mαm,nexp(j2πfd,m,n(t+∆t))exp(-j2πfτn)

Observation-2:

- If gNB or UE can estimate Doppler information of each path/subpath at time 𝑡, it might perform CSI prediction for 𝑡 +∆ 𝑡 in theory.

Refinement to Rel-16/17 Type II codebook

As discussed in section 2.1, the channel can be considered to be nearly constant in Doppler domain over a period of time. Therefore, if the Doppler shift of each path can be extracted by UE, UE can predict the future channel according to the current estimated channel at time instance t0. Based on the predicted channel, UE can calculate precoding matrix for future time (tn,   tn+dt, tn+2dt, …,  tn+(T-1)dt). tn is the time of the first PDSCH transmission using the estimated channel information at time instance t0.

The precoding matrix according to Rel-16/17 Type II codebook at time instance t is represented by

W(t)=W1W2(t)WfH

where W1∈CNtx×2L is an SD basis matrix, Wf∈CN3×M is an FD basis matrix, and W2(t)∈C2L×M is a combination coefficient matrix, W(t)∈CNtx×N3.

W(t) can be reconstructed to length-NtxN3 vector:

vec(Wt)=(Wf*⨂W1)vec(W2(t))

where vec(W(t)) ∈CNtxN3×1, vec(W2(t)) ∈C2LM×1, (Wf*⨂W1)∈CNtxN3×2LM.

The precoding matrix at time instance (tn,   tn+dt, tn+2dt, …,  tn+(T-1)dt) is represented by

W=vecWtn, vecW(tn+dt), vecW(tn+2dt), …, vecW(tn+(T-1)dt)

=Wf*⨂W1[vec(W2tn), vec(W2tn+dt), vec(W2tn+2dt), …, vec(W2(tn+(T-1)dt))]

Let W2=vec(W2tn), vec(W2tn+dt), vec(W2tn+2dt), …, vec(W2(tn+(T-1)dt)), and W2 can be compressed due to the correlation in the time domain:

W2=W2WtH

where W2∈C2LM×V is a combination coefficient matrix after compression in time domain. The feedback overhead of the coefficient can be reduced from 2LM×T to 2LM×V  after the compression. Wt∈CT×V is a basis matrix in time domain which is selected from CT×QT DFT vectors.

Wt=[ft,0,ft,1, ……, ft,V-1] , ft,v= 1,ej2πkv1QT, …,ej2πkvnQT,..,ej2πkv(T-1)QT T∈CT×1,

Q is oversampling factor, and it is used to control resolution of the Doppler information. When a larger Q value is used, the resolution of Doppler information is higher, but it will increase UE complexity. Both performance and complexity should be considered to decide the Q value. kv is index of the selected DFT vector.

As discussed above, the precoding matrix including Doppler information can be expressed by the following formula. Figure 2 shows the basic principles of time domain compression.

W=Wf*⨂W1W2WtH

Figure 2 Time domain compression

gNB uses the q-th column of W as the precoding matrix for downlink transmission at time tn+qdt. Figure 3 shows the timing of codebook based PDSCH transmission. The same precoding matrix will be used within time duration dt which is decided by performance.

Figure 3 Timing of codebook based PDSCH transmission

Proposal-2:

- The following codebook structure can be considered:

W=Wf*⨂W1W2WtH

There are two options to perform Doppler measurement.

- Option 1: CSI-RS burst

When using the CSI-RS burst for Doppler measurement, UE can extract the Doppler domain information in addition to angle-delay domain information. Figure 4 shows the CSI-RS burst with period P. A CSI-RS burst consists of L CSI-RS resources with interval d. Configuring L CSI-RS resources in a CSI-RS resource set can be considered to implement the CSI-RS burst. The overhead would be a problem if a large number of CSI-RS resources is needed and large number of antenna ports, e.g., 16, 32, are used.

Figure 4 Periodic CSI-RS burst

- Option 2: TRS

When using the TRS for Doppler measurement, UE can extract the Doppler domain information by TRS and extract angle-delay domain information by CSI-RS. The overhead is lower than CSI-RS burst if reusing the legacy TRS and CSI-RS to get Doppler domain information is possible. It is necessary to study whether the Doppler shift estimated by single-port TRS can represent the true channel condition of multiple-port CSI-RS.

Proposal-3:

- CSI-RS burst or TRS can be considered for Doppler measurement.

UE reporting of time-domain channel properties by TRS

In RAN #94 e-meeting, it was proposed that gNB acquires time-domain information by UE reporting without codebook refinement especially for TDD system. Since TRS resources are used for delay/Doppler tracking in NR system, UE reporting of time-domain channel properties by TRS was agreed as part of the study work.

Approaches for gNB acquiring Doppler information

For TDD system, gNB can estimate the DL channel according to the UL RS transmitted by UE utilizing the channel reciprocity, and then gNB can calculate the precoding matrix to match the current channel. Hence, there are two approaches for Doppler acquisition:

- Approach 1: Using SRS for Doppler acquisition by gNB

In Rel-15, one SRS resources can be configured in 1/2/4 consecutive symbols in the last 6 OFDM symbols of a slot. As shown in figure 5 below, two resources can be configured with comb-4 in 1 symbol and 1 slot, and each resource can be configured with the same period. Moreover, by combining four SRS resources configured in two continuous slots, the same pattern as TRS burst is achieved. However, according to the current specs, there is no restriction on transmit antennas for SRS resources in one or different slot. Hence, if there is a mechanism to ensure that same transmit antennas are used to transmit different SRS resources, gNB is able to accurately estimate the Doppler frequency offset.

Figure 5 One example pattern by combining four SRS resources configured in two continuous slots

Observation-3:

- For TDD system, combining different SRS resources in Rel-16 can achieve the same pattern as TRS burst. If there is a mechanism to ensure that same transmit antennas are used to transmit different SRS resources, gNB is able to accurately estimate the Doppler frequency offset.

According to the discussion above, our suggestion is to study gNB estimation of Doppler acquisition by SRS as the baseline for TDD system.

Proposal-4:

- Using SRS for Doppler acquisition by gNB shall be considered as the baseline for TDD system.

- Approach 2: UE reporting of Doppler information by TRS

According to the analysis of CSI prediction in section 2.2, the following process for channel prediction can be considered:

- gNB estimates DL channel Hu,stand precoder Wt at time t by SRS

- gNB predicts DL channel Hu,st+∆t with fd,los, fd,m,n reported by UE

Therefore, UE may need to report Doppler information of each subpath for accurate channel prediction in theory. However, since nearly dozens of clusters and hundreds of subpaths are modeled in complicated dense urban scenario, it is impossible to report Doppler information of each subpath by UE because of the feedback overhead and UE complexity. It is also questionable that a single or a few Doppler shifts reporting could provide satisfactory prediction performance. There is clearly a tradeoff among reporting overhead, complexity and performance. Therefore, the reporting content and its associated prediction algorithm shall be carefully studied.

Proposal-5:

- The reporting content and its associated prediction algorithm shall be carefully studied.

CSI enhancements for coherent-JT

In NR system, M-TRP transmission is an important technology to improve cell edge use throughput and provide a more balanced service quality for the serving cell. According to the mapping relationship of the layers to multiple TRPs, the M-TRP transmission schemes can be roughly divided into two types: C-JT (Coherent-Joint Transmission) schemes and NC-JT (Non-Coherent-Joint Transmission) schemes. For C-JT schemes, all the PDSCH/DMRS ports are jointly transmitted from multiple TRPs and signals from multiple TRPs are combined coherently; For NC-JT schemes, the PDSCH/DMRS ports are transmitted from each TRP respectively in SDM scheme.

In Rel-15/16, S-DCI and M-DCI based NC-JT transmission schemes have been standardized, but C-JT transmission can only be supported in a specs transparent manner. In Rel-17, SFN-ed schemes have been supported up to two TCI states for the PDSCH/DMRS ports. However, it is mainly for improving the demodulation performance of high-speed train deployment, and only TRS resources for HST-SFN have been enhanced. Without the CSI enhancement for C-JT, gNB cannot perform precoding per TRP according to the channel information of each TRP. Therefore, CSI acquisition for C-JT targeting FR1 was agreed to be further studied and discussed.

Codebook design for coherent-JT

To minimize the workload in Rel-18, the same number of antenna ports across TRPs and the max number of TRPs for C-JT is assumed. In such case, the first objective is Rel-16/17 Type-II codebook refinement for C-JT targeting FDD. In Rel-15, ‘typeI-MultiPanel’ codebook has been defined to support multi-panel transmission with up to 4 panels. Due to the similar antenna array structure between multi-panel and multi-TRP, the two following possible structures can be considered for distributed C-JT:

- Option 1: Per TRP Type II codebook + co-phase/amplitude factor across TRPs

As shown by the following formula, UE reports W1,W2,Wf for each TRP independently according to Rel-16 Type II codebook together with co-phase/amplitude factor across TRPs.

W=W1⋯0⋮⋱⋮0⋯W4∙W3

- Wi=Wf*⨂W1W2, i=1,2,3,4 denotes the Type II precoding for the ith TRP.

- W3=c1c2    c3c4, where ci, i =1,2,3,4 contains phase or amplitude compensation factor across different TRPs.

Based on this design, the feedback of combination coefficient does not need to be modified, and the gNB can obtain PMI of each TRP for more flexible scheduling for C-JT. But the overhead of CSI feedback would be greatly increased because of PMI reporting for 4 TRPs and co-phase/amplitude factor. Therefore, both performance and overhead of CSI feedback should be taken into account. In the following study, further enhancement on reducing CSI reporting overhead can be considered. For example, the dimension of W1,W2,Wf can be reduced and the frequency domain granularity of W3  can be further discussed considering the tradeoff among performance evaluation and reporting overhead.

- Option 2: Joint codebook across TRPs

For Option 2, joint codebook across TRPs shall be designed for matching coherent joint transmission. For example, the following structure can be considered as a start point for this option.

W=W1,1000…000W1,4*W2,1000…000W2,4* Wf,1000…000Wf,4

- W1,i, i=1,2,3,4 denotes the spatial beams for the ith TRP.

- W2,i is the combination coefficient matrix including the phase or amplitude compensation factor across different TRPs.

- Wf,i, i =1,2,3,4 denotes the frequency domain basis vector for ith TRP.

Considering the distributed M-TRP deployment, large-scale channel parameters from each TRP to the UE are obviously different. Hence, in our opinion, the spatial-domain beams (W1) and the frequency/delay-domain information (Wf) should be calculated individually for each TRP. Besides, the co-phase/amplitude factor across TRPs can be reported together in the combination coefficient matrix (W2). For this option, the total number of enhanced W1,Wf can be configured for UE and UE can allocate the number of SD basis and FD basis across all TRPs.

Proposal-6:

- Both joint codebook and per TRP Type II codebook together with co-phase/amplitude factor across TRPs shall be considered for coherent-JT.

CSI reporting enhancements for coherent-JT

In Rel-17, CSI enhancement for NC-JT schemes has been supported that the UE can be configured with Ks≥2 NZP CSI-RS resources in a CSI-RS resource set for CMR and N≥1 NZP CSI-RS resource pairs whereas each pair is used for a NCJT measurement hypothesis. Then UE can report one CSI for NC-JT and /or X =0, 1, 2 CSIs for S-TRP according to the CSI reporting mode 0/1. For CSI report quantity for NC-JT, UE reports 2 PMIs, 2 RIs, 2 LIs and one CQI per codeword based 2 TRPs NC-JT measurement hypotheses.

To minimize the impact on the current specs, the most intuitive approach is to extend the framework of CSI-RS resource configuration for NC-JT to 4 CSI-RS resources groups. Hence UE still can measure channel states based the CSI-RS resources grouped by TRP. Moreover, the optimal transmission schemes could be C-JT, NC-JT or S-TRP depending channel condition, traffic load etc. It is therefore beneficial for UE to report CSIs under multiple transmission scheme hypotheses under this common framework.

Although a UE can be configured with CSI-RS resources of 4 TRPs, C-JT transmission from 4 TRPs is not always suitable. For such case, if a UE is required to report the CSI for C-JT from all TRPs, it will cause unnecessary overhead. Therefore, TRP selection by UE can be considered for reducing feedback overhead and increasing scheduling flexibility.

Proposal-7:

- The framework of CSI-RS resource configuration for NC-JT can be extended to 4 CSI-RS resources groups, and the TRPs selection by UE shall be considered for reducing feedback overhead and scheduling flexibility.

Evaluations on coherent-JT

In order to align evaluation results among companies, our views on simulation methodology for C-JT together with preliminary simulation results are presented in this section.

Evaluation methodology

Since the performance of dense urban scenario is highly limited by the interference, it is expected that coherent joint transmission can provide remarkable gain in this scenario. Hence, dense urban scenario including Umi and Uma can be considered with higher priority for performance evaluation. Considering the overhead of CSI reporting and actual deployment cost, different number of TRPs can be evaluated and compared.

Proposal-8:

- For the evaluations on coherent-JT,

  - Dense urban scenario can be considered with higher priority;

  - The performance comparison of 2/3/4 TRPs can be considered.

Preliminary result

We provide our preliminary simulation results for C-JT deployment. For our simulations, 2 TRPs for intra-site topology in Uma were modeled. One TRP is deployed in the center of macro cell and another TRP is randomly distributed in each sector. Besides, the PMI, RI and CQI are calculated for each 4 PRBs assuming per TRP Rel-16 Type II codebook together with co-phasing factor as discussed in section 3.1. Detailed simulation assumption can be found from Table-II in the Appendix. Figure 6 shows the spectral efficiency (SE) gain of 2-TRP joint transmission for both cell average and cell edge. Comparing with S-TRP scheme, intra-site C-JT scheme can provide significant gain, both for the cell edge and cell average. Specifically, nearly 200% SE gains for the cell edge UEs, and 21% SE gains for the cell average are achieved.

Figure 6 Preliminary simulation result for 2-TRP coherent joint transmission

Proposal-9:

- CSI enhancement for coherent-JT should be supported in Rel-18.

Conclusions

In this contribution, we provided our views on the enhancements for DL CSI enhancements. We have the following observations and proposals:

Observation-1:

- When the CSI feedback periodicity is 5ms, the average throughput of 60km/h has 22% loss and the 5% edge throughput of 60km/h has 45% loss compared with 3km/h.

Observation-2:

- If gNB or UE can estimate Doppler information of each path/subpath at time 𝑡, it might perform CSI prediction for 𝑡 +∆ 𝑡 in theory.

Observation-3:

- For TDD system, combining different SRS resources in Rel-16 can achieve the same pattern as TRS burst. If there is a mechanism to ensure that same transmit antennas are used to transmit different SRS resources, gNB is able to accurately estimate the Doppler frequency offset.

Proposal-1:

- Support CSI enhancement for high/medium UE velocities by exploiting Doppler-domain information.

Proposal-2:

- The following codebook structure can be considered:

W=Wf*⨂W1W2WtH

Proposal-3:

- CSI-RS burst or TRS can be considered for Doppler measurement.

Proposal-4:

- Using SRS for Doppler acquisition by gNB shall be considered as the baseline for TDD system..

Proposal-5:

- The reporting content and its associated prediction algorithm shall be carefully studied.

Proposal-6:

- Both joint codebook and per TRP Type II codebook together with co-phase/amplitude factor across TRPs shall be considered for coherent-JT.

Proposal-7:

- The framework of CSI-RS resource configuration for NC-JT can be extended to 4 CSI-RS resources groups, and the TRPs selection by UE shall be considered for reducing feedback overhead and scheduling flexibility.

Proposal-8:

- For the evaluations on coherent-JT,

  - Dense Urban scenario can be considered with higher priority;

  - The performance comparison of 2/3/4 TRPs can be considered.

Proposal-9:

- CSI enhancement for coherent-JT should be supported in Rel-18.

References

[1] RP-213598, ‘New WID: MIMO Evolution for Downlink and Uplink’, RAN #94-e.

Appendix

Table-I System level simulation assumptions for high/medium UE velocities

| Parameter | Value |
| --- | --- |
| Duplex | FDD |
| Multiple access | OFDMA |
| Scenario | Dense Urban (Uma) |
| Carrier frequency | 4GHz |
| Inter-Macro BS distance | 500m |
| Antenna setup at gNB | 32Tx: (1,1,2,8,2), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup at UE | 2Rx: (1,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power | 41 dBm for 10MHz |
| BS antenna height | 25m |
| UE antenna height & gain | Follow TR36.873 |
| UE receiver noise figure | 9dB |
| Simulation bandwidth | 10 MHz with 15KHz |
| Maximum MU layers | 12 |
| CSI feedback period | 5 ms |
| Traffic model | FTP model 1 with packet size 0.5 Mbytes |
| UE distribution | 100% outdoor |
| Type II Codebook | Rel-16 regular {config-6} |
| UE receiver | MMSE-IRC as the baseline receiver |

Table-II System level simulation assumptions for Coherent-JT

| Parameter | Value |
| --- | --- |
| Duplex | FDD |
| Multiple access | OFDMA |
| Scenario | Dense Urban (Uma) |
| Carrier frequency | 2.2GHz |
| Inter-Macro BS distance | 200m |
| Antenna setup at gNB | 8Tx: (1,1,2,2,2), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup at UE | 2Rx: (1,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power | 41 dBm for 10MHz |
| BS antenna height | 25m |
| UE antenna height & gain | Follow TR36.873 |
| UE receiver noise figure | 9dB |
| Simulation bandwidth | 10 MHz with 15KHz |
| CSI feedback period | 5 ms |
| Traffic model | FTP model 1 with packet size 0.5 Mbytes |
| UE distribution | 80% outdoor, 20% indoor |
| UE receiver | MMSE-IRC as the baseline receiver |