3GPP TSG RAN WG1 #109-e                                                                                    R1-2203444

e-Meeting, May 9th – 20th, 2022

Source:	CATT

Title:	On increased number of orthogonal DMRS ports

Agenda Item:	9.1.3.1

Document for:	Discussion and Decision

Introduction

As approved in RAN #94-e meeting, DMRS enhancements include the following objectives [1]:

Study, and if justified, specify larger number of orthogonal DMRS ports for downlink and uplink MU-MIMO (without increasing the DMRS overhead), only for CP-OFDM, - Striving for a common design between DL and UL DMRS - Up to 24 orthogonal DMRS ports, where for each applicable DMRS type, the maximum number of orthogonal ports is doubled for both single- and double-symbol DMRS Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In this contribution, possible enhancements on increasing orthogonal DMRS ports for UL/DL MU-MIMO and 8 Tx UL SU-MIMO are discussed.

Enhancements on increasing number of orthogonal DMRS ports

  - Motivation

In Rel-15/16/17 DMRS transmission, the number of orthogonal antenna ports is up to 12. With the introduction of more advanced terminals (e.g., CPE/FWA) and more NR terminals, the number of simultaneous scheduled users and layers is expected to be increased. On the other hand, with the introduction of CJT, each data layer of one UE is served by multiple TRPs. In order to achieve the gain of CJT without sacrificing performance of other UEs, the number of orthogonal ports shall be increased. Due to the increasing need for multiplexing capacity of downlink and uplink DMRS from various use cases, DMRS enhancements shall be specified in Rel-18 to support more orthogonal ports.

Proposal 1:

- DMRS enhancements shall be supported in Rel-18 to support more orthogonal ports.

The following principles can be considered in DMRS pattern design.

- Principle 1: Rel-18 UE can be co-scheduled with Rel-15/16/17 UEs.

- Principle 2: PAPR is not worse than that in Rel-16.

  - Enhancements on Type 2 DMRS

According to WID, up to 24 orthogonal DMRS ports are expected and the maximum number of orthogonal ports is doubled for both single- and double-symbol DMRS without increasing the DMRS overhead. To achieve that, two direct extension methods are as follows:

- Pattern 1: Increase the number of ports in one CDM group

- Pattern 2: Increase the number of CDM groups.

In this section, detailed designs related to these two patterns are discussed.

Pattern 1: Increase the number of ports in one CDM group

Figure 1: An example of enhanced Type 2 DMRS pattern 1

Figure 1 shows an example of enhanced Type 2 DMRS pattern, where the number of CDM groups is 3 and the total number of DMRS ports can be up to 24. The number of DMRS ports in one CDM group is up to 8. Considering backward compatibility, correspondence between antenna port (1000+) 0~11 and CDM group 0/1/2 can be reused. Similar rule can be adopted to determine the correspondence between antenna port (1000+) 12~23 and these three CDM groups.

Cyclic shift or OCC can be enhanced to fulfil Principle 1 listed in Section 2.1. For cyclic shift enhancement, 2CSs or 4CSs can be considered. For OCC enhancement, 8OCC can be considered. Considering backward compatibility, FD-4OCC can be considered for single-symbol configuration, and FD-4OCC + TD-2OCC can be considered for double-symbol configuration.

This DMRS pattern is exactly the same as Rel-15 Type 2 DMRS pattern, which facilitates co-scheduling of Rel-18 UE and Rel-15/16/17 UE. Therefore, we propose to consider this DMRS pattern in Rel-18.

Proposal 2:

- DMRS pattern which is exactly the same as Rel-15 Type 2 DMRS pattern and includes 8 orthogonal ports in one CDM group can be considered.

  - OCC or cyclic shift can be enhanced to support co-scheduling of Rel-18 and Rel-15/16/17 UEs.

Pattern 2: Increase the number of CDM groups

Figure 2: An example of enhanced Type 2 DMRS pattern 2

Figure 2 shows another example of enhanced Type 2 DMRS pattern, where the number of CDM groups is 6 and the total number of DMRS ports can be up to 24. The number of DMRS ports in one CDM group is still up to 4. Considering backward compatibility, the correspondence between antenna port (1000+) 0~11 and CDM group 0/1/2 can be reused. Accordingly, antenna port (1000+) 12~23 can be allocated to CDM group 3/4/5.

With the introduction of more CDM groups, power allocation and DMRS sequence initialization methods should be enhanced for the increased CDM group 3/4/5. Similarly, two principles listed in Section 2.1 can be followed.

Proposal 3:

- Enhanced Type-2 DMRS pattern which includes 6 CDM groups and each CDM group includes 4 orthogonal ports can be considered.

  - Power allocation and DMRS sequence initialization methods should be enhanced for this pattern.

  - Enhancements on Type 1 DMRS

Pattern 1: Increase the number of ports in one CDM group

Figure 3: An example of enhanced Type 1 DMRS pattern 1

Similar to enhanced Type 2 DMRS pattern 1 in Section 2.2.1, the number of antenna ports in one CDM group can be increased to 8 for Type 1 DMRS, which is shown in Figure 3. The total number of antenna ports can be up to 16. OCC and cyclic shift can be enhanced to fulfil Principle 1 listed in Section 2.1. This DMRS pattern can also be considered since the pattern is exactly the same as Rel-15 Type 1 DMRS pattern.

In Rel-15 DMRS Type 1 with single-symbol, 2 REs in one RB form a CDM group, and each CDM group repeats three times in one RB. For enhanced Type 1 DMRS pattern 1, 4 REs in one RB form a CDM group, and each CDM group with a group of orthogonal weights (wf and wt) repeats three times in two RBs. If the number of scheduled PRBs is odd, whether the orthogonality among multiple scheduled ports can be kept or not is an issue. Therefore, with this pattern, how to perform frequency domain resource allocation should be studied.

Proposal 4:

- DMRS pattern which is exactly the same as Rel-15 Type 1 DMRS pattern and includes 8 orthogonal ports in one CDM group can be considered.

  - How to perform frequency domain resource allocation to ensure orthogonality among multiple scheduled ports should be studied.

Pattern 2: Increase the number of CDM groups

Figure 4: An example of enhanced Type 1 DMRS pattern 2

Similar to enhanced Type 2 DMRS pattern 2 in Section 2.2.2, the number of CDM groups can be increased to 4 for Type 1 DMRS, which is shown in Figure 4. The total number of antenna ports can be up to 16. Correspondingly, power allocation and DMRS sequence initialization methods should be enhanced.

Similar to enhanced Type 1 DMRS pattern 1, each CDM group with a group of orthogonal weights (wf and wt) repeats three times in two RBs. Therefore, how to perform frequency domain resource allocation to ensure orthogonality among multiple scheduled ports should be studied.

Proposal 5:

- Enhanced Type-1 DMRS pattern which includes 4 CDM groups and each CDM group includes 4 orthogonal ports can be considered to increase the maximum number of antenna ports.

  - Power allocation and DMRS sequence initialization methods should be enhanced for this pattern.

  - How to perform frequency domain resource allocation to ensure orthogonality among multiple scheduled ports should be studied.

Enhancements for UL 8Tx

In Rel-17, NR supports a maximum of 4 layers in UL and 8 layers in DL, hence there is a 100% gap between DL/UL peak data rate. It is desirable to bring UL peak data rate on par with DL to allow a balanced spectrum usage and user experience. Therefore up to 8 UL layers SU-MIMO should be supported, for both FR1 and FR2, and both codebook and non-codebook based PUSCH.

For both codebook and non-codebook based PUSCH transmission, the DMRS port(s) and PUSCH layer(s) are one-to-one mapped. Therefore if the maximum number of layers for UL is extended to 8 from 4, DMRS enhancement is needed.

Proposal 6:

- If more than 4 layers are supported for UL 8Tx, DMRS enhancement should be considered.

Conclusions

In this contribution, enhancements on increasing number of orthogonal DMRS ports and enhancements for UL 8Tx are discussed. Based on the above discussion, we have the following proposals:

Proposal 1:

- DMRS enhancement can be supported in Rel-18 to support more orthogonal ports.

Proposal 2:

- DMRS pattern which is exactly the same as Rel-15 Type 2 DMRS pattern and includes 8 orthogonal ports in one CDM group can be considered.

  - OCC or cyclic shift can be enhanced to support co-scheduling of Rel-18 and Rel-15/16/17 UEs.

Proposal 3:

- Enhanced Type-2 DMRS pattern which includes 6 CDM groups and each CDM group includes 4 orthogonal ports can be considered.

  - Power allocation and DMRS sequence initialization methods should be enhanced for this pattern.

Proposal 4:

- DMRS pattern which is exactly the same as Rel-15 Type 1 DMRS pattern and includes 8 orthogonal ports in one CDM group can be considered.

  - How to perform frequency domain resource allocation to ensure orthogonality among multiple scheduled ports should be studied.

Proposal 5:

- Enhanced Type-1 DMRS pattern which includes 4 CDM groups and each CDM group includes 4 orthogonal ports can be considered to increase the maximum number of antenna ports.

  - Power allocation and DMRS sequence initialization methods should be enhanced for this pattern.

  - How to perform frequency domain resource allocation to ensure orthogonality among multiple scheduled ports should be studied.

Proposal 6:

- If more than 4 layers are supported for UL 8Tx, DMRS enhancement should be considered.

References

[1] RP-213598, New WID: MIMO Evolution for Downlink and Uplink, RAN#94-e, Samsung.