3GPP TSG RAN WG1 #109-e			R1-2203447

e-Meeting, May 9th – 20th, 2022

Source: 	CATT

Title:	On SRI/TPMI enhancement for UL 8 TX

Agenda Item:	9.1.4.2

Document for:	Discussion and Decision

- Introduction

In RAN #94 e-meeting, enhancements for 8 Tx UL operations were agreed and the following objective was approved in the WID for Rel-18 NR MIMO [1]:

Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In this contribution, codebook design and SRI/TPMI enhancement for UL 8Tx operation are discussed.

- Discussion

  - Motivation

Since the power/form-factor/cost of CPE/FWA/vehicle/Industrial devices are not as limited as traditional handheld devices, they are able to support 8Tx antennas. Enabling 8Tx UL operation is helpful in improving data rate and UL coverage, and hence it is desirable to support 8Tx operation in Rel-18 for devices with UL 8Tx.

NR supports a maximum of 4 layers in the UL and 8 layers in the DL. There is a 100% gap between DL/UL peak data rate. It is desirable to bring UL peak data rate on par with DL to allow a balanced spectrum usage and user experience. Therefore up to 8 UL layers SU-MIMO also should be supported, for both FR1 and FR2, and both codebook and non-codebook based PUSCH.

Proposal 1:

- Support UL 8Tx with up to 8 layers in Rel-18.

  - Codebook design

    - General framework

CPE/FWA/vehicle/Industrial devices with 8Tx can be deployed in various scenarios. For example, the devices may be deployed outdoor or indoor, stationary or moving at a low/medium/high speed. Therefore the devices may have different sizes and different antenna structures. For example, the size of the devices ranges from several centimeters to several meters, the antenna structure of the devices can be one panel or multiple panels, directional polarized antennas or omnidirectional antennas, with non-coherent/partial-coherent/full-coherent coherency capabilities.

On the codebook design for UL 8Tx, the variety of antenna structures shall be considered. At least codebook designs for non-coherent, partial-coherent, and full-coherent devices should be considered for devices with different coherency capabilities.

Proposal 2:

- Codebook designs for non-coherent, partial-coherent and full-coherent devices are considered for UL 8Tx.

    - Non-coherent codebook

A non-coherent UE can only be configured with non-coherent codebook. Since a non-coherent UE is not capable of transmitting a single layer with multiple antennas coherently, each precoding matrix in a non-coherent codebook shall be a port selection precoding matrix, with one port selected for each layer.

For PUSCH transmission with DST-s-OFDM waveform, the transmission rank is 1. All of the port selection precoding matrices for rank 1 can be included in the 8Tx non-coherent codebook, as shown in Table 1.

Table 1 Non-coherent codebook for single-layer UL 8Tx, DFT-s-OFDM

| TPMI index | (ordered from left to right in increasing order of TPMI index) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 – 7 | 12210000000 | 12201000000 | 12200100000 | 12200010000 | 12200001000 | 12200000100 | 12200000010 | 12200000001 |

Proposal 3:

- For UL 8Tx with DFT-s-OFDM, precoding matrix in Table 1 is included in non-coherent codebook.

For the non-coherent codebook of 4Tx for CP-OFDM waveform, the total number of precoding matrices in non-coherent codebook is 12, with all of port selection precoding matrices for rank=1/2/4 included, and one precoding matrix for rank =3 included. For UL 8Tx in Rel-18, if all of port selection precoding matrices are included in the non-coherent codebook, the number of precoding matrices for each rank is calculated as follows:

- Rank 1: C81=8;

- Rank 2: C82=28;

- Rank 3: C83=56;

- Rank 4: C84=70;

- Rank 5: C85=56;

- Rank 6: C86=28;

- Rank 7: C87=8;

- Rank 8: C88=1.

The number of precoding matrices and the bit widths for TPMI field when maxRank = 1, 2, …, 8 is summarized in Table 2. It can be seen that 8 bits is needed when maxRank >= 4 for 8Tx. Compared to 4Tx, the bit width of the TPMI field is at least doubled. Note that for partial-coherent and full-coherent codebooks, all of precoding matrices in non-coherent codebook would be included. Therefore supporting too many non-coherent precoding matrices for 8Tx is not acceptable. How to down select precoding matrix for non-coherent codebook should be considered.

Table 2 Number of precoding matrices and bit widths of TPMI field for non-coherent codebook

| maxRank | Number of TPMIs | Bit widths for TPMI field (bits) |
| --- | --- | --- |
| 1 | 8 | 3 |
| 2 | 36 | 6 |
| 3 | 92 | 7 |
| 4 | 162 | 8 |
| 5 | 218 | 8 |
| 6 | 264 | 8 |
| 7 | 254 | 8 |
| 8 | 255 | 8 |

Proposal 4:

- For PUSCH transmission with CP-OFDM, down selection of port-selection precoding matrix shall be considered for non-coherent codebook for 8Tx.

    - Partial-coherent codebook

Various partial-coherent relationships among antennas can be supported by CPE/FWA/vehicle/Industrial devices with 8Tx. For example, some devices may have two coherent antenna groups, with four coherent antennas per group, while some other devices may have three coherent antenna groups, with one group comprises four coherent antennas and two groups comprises two coherent antennas per group. Designing codebooks for all possible partial-coherent relationships of antennas respectively incurs large workload of codebook design, which is not preferred. We prefer to design partial-coherent codebooks for UL 8Tx with one or several typical partial-coherent relationships. For example, partial-coherent relationship with two coherent groups with four coherent antennas per group, and partial-coherent relationship with four coherent groups with two coherent antennas per group are considered with high priority. On the design of precoding matrices for partial-coherent transmission, the precoding matrix for 2Tx/4Tx in Rel-15 can be considered for each coherent antenna group as a start.

Proposal 5:

- On the design of partial-coherent codebook(s) for UL 8Tx, one or more typical partial-coherent relationships of antennas are considered

  - Two coherent groups with four coherent antennas per group, and four coherent groups with two coherent antennas per group can be considered with high priority.

Proposal 6:

- On the design of precoding matrices for partial-coherent transmission for UL 8Tx, the precoding matrices for 2Tx/4Tx in Rel-15 can be considered for each coherent antenna group as a baseline.

    - Full-coherent codebook

For PUSCH transmission with DFT-s-OFDM in Rel-15, full-coherent precoding matrices  for UL 4Tx are obtained by computer search. Good distance properties (e.g. measured by the chordal distance) are considered to ensure good performance. On the design of full-coherent precoding matrix for UL 8Tx for DFT-s-OFDM waveform, similar approach can be considered.

Proposal 7:

- Full-coherent precoding matrix for UL 8Tx for DFT-s-OFDM can be designed by computer search with good distance properties.

For PUSCH transmission with CP-OFDM in Rel-15, full-coherent precoding matrices for UL 4Tx are obtained by down selecting precoding matrices from DL Type I codebook. For the full-coherent codebook for UL 8Tx in Rel-18, down selecting precoding matrices from DL Type I codebook also can be considered as a start.

Proposal 8:

- For the design of full-coherent precoding matrices for UL 8Tx for CP-OFDM, down selecting precoding matrices from DL Type I codebook can be considered as a start.

As multiple types of antenna structures are in consideration for UL 8Tx in Rel-18, supporting multiple codebooks for various types of antenna structures also can be considered.

Proposal 9:

- On full-coherent codebook design for UL 8Tx for CP-OFDM, supporting multiple codebooks for various types of antenna structures can be considered.

    - Size of codebook for UL 8Tx

In Rel-15, the maximum size of UL 4Tx codebook (i.e., full-coherent codebook with maxRank =4) is 64 (6 bits). For UL 8Tx, if all of non-coherent precoding matrices are supported, the maximum size of non-coherent codebook would be 255, which requires 2 more bits than 4Tx full-coherent codebook. When partial-coherent precoding matricesand full-coherent precoding matricesare also considered, the size of the codebook would be considerable large. Too many bits for TPMI indication in DCI is not expected. Therefore the size of codebooks for UL 8Tx shall be carefully determined.

Proposal 10:

- For UL 8Tx, the size of codebook should be carefully determined.

    - UL 8Tx with full power transmission

In Rel-16, UL full power transmission is supported. Large amount of CPE/FWA/vehicle/Industrial devices with 8Tx are not power sensitive. Therefore they may be capable of supporting full power transmission with part or all of antennas. In order to make full use of transmit power of the devices, UL full power transmission should be considered for these devices. Whether all of three full power transmission schemes in Rel-16 are supported can be considered.

Proposal 11:

- UL full power transmission is supported for UL 8Tx,

  - Whether all of UL full power transmission schemes in Rel-16 are supported can be considered.

  - SRI/TPMI indication

Codebook based PUSCH

The TPMI/SRI indication of UL 8 Tx for codebook based PUSCH is highly related to the design of codebook and SRS, as well as the presentation way of the codebook.

For example, if the codebook of 8Tx is a codebook with precoding matrices down selected from Rel-15 DL Type I codebook, and the codebook is generated by a formula similar as DL, it is possible that a two-stage TPMI indication can be considered. If a precoding matrix of 8Tx is constructed by combining two or more precoding matrices from Rel-17 codebook, enhancement for indicating multiple precoding matrices may be considered. If the codebook is just presented as a table, no matter how the UL codebook is obtained, enlarging the bit width of Rel-17 TPMI filed is sufficient.

Similarly, the SRI indication is related to the design of SRS for UL 8Tx. If 8-port SRS resources are used, enlarging the bit width of SRI field is sufficient. If 8 ports are realized by combining SRS ports of multiple SRS resources, SRI indication with indicating multiple SRS resources is needed.

Since TPMI/SRI indication of UL 8 Tx is highly related to the design of codebook and SRS, as well as the presentation way of the codebook, we propose to determine the design of TPMI/SRI indication for codebook based PUSCH after the codebook and SRS design are available.

Proposal 12:

- For UL 8Tx for codebook based PUSCH, TPMI/SRI indication is designed after the codebook and SRS design are available.

Non-codebook based PUSCH

For non-codebook based PUSCH, multiple one-port SRS resources are configured in the same SRS resource set and SRI indicating multiple SRS resources is supported. For UL 8Tx, it is natural to extend the number of SRS resources in the SRS resource set to 8 and extend the bit width of the SRI field.

Proposal 15:

- For UL 8Tx for non-codebook based PUSCH, extending the bit width of current SRI field to enable indicating up to 8 SRS resources.

- Conclusions

In this contribution, we provided our views on the enhancements for enabling UL 8Tx. We have the following proposals:

Proposal 1:

- Support UL 8Tx with up to 8 layers in Rel-18.

Proposal 2:

- Codebook designs for non-coherent, partial-coherent and full-coherent devices are considered for UL 8Tx.

Proposal 3:

- For UL 8Tx with DFT-s-OFDM, precoding matrix in Table 1 is included in non-coherent codebook.

Proposal 4:

- For PUSCH transmission with CP-OFDM, down selection of port-selection precoding matrix shall be considered for non-coherent codebook for 8Tx .

Proposal 5:

- On the design of partial-coherent codebook(s) for UL 8Tx, one or more typical partial-coherent relationships of antennas are considered

  - Two coherent groups with four coherent antennas per group, and four coherent groups with two coherent antennas per group can be considered with high priority.

Proposal 6:

- On the design of precoding matrices for partial-coherent transmission for UL 8Tx, the precoding matrices for 2Tx/4Tx in Rel-15 can be considered for each coherent antenna group as a baseline.

Proposal 7:

- Full-coherent precoding matrix for UL 8Tx for DFT-s-OFDM can be designed by computer search with good distance properties.

Proposal 8:

- For the design of full-coherent precoding matrices for UL 8Tx for CP-OFDM, down selecting precoding matrices from DL Type I codebook can be considered as a start.

Proposal 9:

- On full-coherent codebook design for UL 8Tx for CP-OFDM, supporting multiple codebooks for various types of antenna structures can be considered.

Proposal 10:

- For UL 8Tx, the size of codebook should be carefully determined.

Proposal 11:

- UL full power transmission is supported for UL 8Tx,

  - Whether all of UL full power transmission schemes in Rel-16 are supported can be considered.

Proposal 12:

- For UL 8Tx for codebook based PUSCH, TPMI/SRI indication is designed after the codebook and SRS design are available.

Proposal 13:

- For UL 8Tx for non-codebook based PUSCH, extending the bit width of current SRI field to enable indicating up to 8 SRS resources.

- References

[1] RP-213598, New WID: MIMO Evolution for Downlink and Uplink, RAN #94-e.