3GPP TSG RAN WG1 #109-e                                                                R1-2203448

e-Meeting, May 9th – 20th, 2022

| Source: | CATT |
| --- | --- |
| Title: | Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI |
| Agenda Item: | 9.10.1 |
| Document for: | Discussion and Decision |

- Introduction

In RAN#95e meeting, a Rel-18 WI on Multi-carrier enhancement was approved as follows.

1. Specify a solution for multi-cell PUSCH/PDSCH scheduling (one PDSCH/PUSCH per cell) with a single DCI [RAN1] - Identify the maximum number of cells that can be scheduled simultaneously - Consider both intra-band and inter-band CA operation - Consider both FR1 and FR2 - The single DCI shall be optimized for 3 or more cells for the multi-cell PUSCH/PDSCH scheduling 2. Study and if necessary specify following enhancements for multi-carrier UL operation [RAN1, RAN2, RAN4] - UL Tx switching schemes across up to 3 or 4 bands with restriction of up to 2 Tx simultaneous transmission for FR1 UEs, including mechanisms to enable more configured UL bands than its simultaneous transmission capability and to support dynamic Tx carrier switching across the configured bands for both single TAG and multiple TAGs configurations (RAN1, RAN4) - UE capability and RRC configuration related signalling (RAN2) - Note: strive for RAN1/2 design agnostic with the number of bands, i.e., common design between 3 and 4 bands - Note: no additional TAG is introduced for UL transmission on a carrier without corresponding DL carrier - Note: this objective does not target to extend the SUL framework to support more than 1 SUL for 1 NUL - Switching time and other RF aspects, and RRM requirements for above UL Tx switching schemes across up to 3 or 4 bands (RAN4) - Note: Prioritize UL Tx switching across up to 3 bands is to be addressed first and then that for up to 4 bands can also be addressed

This contribution discusses multi-cell PUSCH/PDSCH scheduling with a single DCI.

- Discussion

  - The maximum number of cells scheduled by a single DCI

For enabling multi-cell PUSCH/PDSCH scheduling with a single DCI, the scheduling DCI should be re-designed to support scheduling multiple PUSCHs/PDSCHs which is transmitted in different serving cells. As the number of the cells that can be scheduled by a single DCI increases, the payload of the DCI will increase as well. Considering the trade-off between control overhead reduction and the payload of DCI, it is necessary to limit the maximum number of cells that can be scheduled by a DCI.

Table 1 the payload size of DCI that can schedule 3CC or 4 CC

| Bits Field | Maximum of bitwidth | 3CC |  | 4 CC |  |
| --- | --- | --- | --- | --- | --- |
|  |  | status | Bits | Bits | status |
| Identifier for DCI format | 1 | Common | 1 | Common | 1 |
| Carrier indicator | 3 | Common | 3 | Common | 3 |
| BWP indicator | 2 | Common | 2 | Common | 2 |
| FDRA | 16 | Independent | 48 | Independent | 64 |
| TDRA | 4 | Common | 4 | Common | 4 |
| VRB-to-PRB | 1 | Common | 1 | Common | 1 |
| PRB bundling size | 1 | Common | 1 | Common | 1 |
| Rate matching indicator | 2 | Common | 2 | Common | 2 |
| ZP CSI-RS trigger | 2 | Common | 2 | Common | 2 |
| MCS of TB1 | 5 | Independent | 15 | Independent | 20 |
| NDI of TB1 | 1 | Independent | 3 | Independent | 4 |
| RV of TB1 | 2 | Independent | 6 | Independent | 8 |
| HARQ process ID | 4 | Common | 4 | Common | 4 |
| DAI | 6 | Common | 6 | Common | 6 |
| TPC | 2 | Common | 2 | Common | 2 |
| PRI | 3 | Common | 3 | Common | 3 |
| K1 indicator | 3 | Common | 3 | Common | 3 |
| Antenna port | 6 | Common | 6 | Common | 6 |
| TCI indication | 3 | Common | 3 | Common | 3 |
| SRS request | 3 | Common | 3 | Common | 3 |
| DMRS | 1 | Common | 1 | Common | 1 |
| CRC | 24 | Common | 24 | Common | 24 |
| Total | 95 |  | 143 |  | 167 |

Assuming the cases listed in the Table 1, the FDRA field , MCS field, NDI field and RV field are independent for each TB, and other fields except for those fields are common for all TBs of different scheduled cells. The maximum payload of DCI will be 143 bits when the number of cells scheduled by a single DCI is 3, and the maximum payload of DCI will be 167 bits when the number of cells scheduled by a single DCI is 4. To ensure the coverage performance of DCI, the maximum number of cells that can be scheduled simultaneously should be no more than 4 Cells with each TB scheduled per cell.

Proposal 1: The maximum number of cells that can be scheduled by a single should be no more than 4 Cells with each TB scheduled per cell.

The maximum number of cells that can be scheduled by a single DCI is defined as M that depends on UE capability reporting, and the actual number of scheduled cells by a single DCI is defined as N, where the value of N is no more than the value of M. Another issue is that whether the actual number of the cells scheduled by the DCI can be dynamically changed or not. There are two options on the number of scheduled cells by a DCI as follows.

- Option-1: The number of the cells scheduled by a DCI is fixed to N; the scheduled cells are configured by higher layer.

- Option-2: The number of the cells scheduled by a DCI can change dynamically from 1 to M, the combination of scheduled cells is indicated by DCI, e.g. carrier indicator field.

For option-1, the number of the scheduled cells is M. The scheduled cells are configured by higher layer and the fields of multi-cell scheduled DCI are determined by those configured cells. The benefit of option-1 is that the payload of DCI is relative stable. For option-2, the number of the scheduled cells can change from 1 to M, and the scheduled cells are dynamically indicated by the DCI. Compared with option-1, option-2 can bring the more schedule flexibility since the scheduled cells can be indicated according to the traffic requirements.

Proposal 2: There are two options on the actual number of scheduled cells by a DCI as follows.

- Option-1: is fixed to N, the scheduled cells are configured by higher layer.

- Option-2: can dynamically change from 1 to M, the combination of scheduled cells is indicated by DCI, e.g. carrier indicator field.

  - DCI for multi-cell PUSCH/PDSCH scheduling

In Rel-17, multiple PDSCHs/PUSCHs in single cell scheduled by DCI format 1_1/0_1 was introduce for control overhead saving, with the shorter symbols for 480kHz SCS and 960kHz SCS. The maximum number of PDSCHs/PUSCHs scheduled by a single DCI is up to 8. For a DCI that can schedule multiple PDSCHs/PUSCHs, some of fields in DCI format 1_1/0_1 in Rel-17 are specified as follows.

- TDRA: It applies commonly for multiple PDSCHs/PUSCHs. The bitwidth of this field is determined by the configured TDRA table, and the configured TDRA table is extended such that each row indicate up 8 multiple PDSCHs/PUSCHs. Each PDSCH/PUSCH has a separate SLIV and mapping type.

- MCS: It applies commonly for multiple PDSCHs/PUSCHs based on the channel state. The bitwidth of this field is 5 bit.

- NDI: Each bit of NDI applies independently for each PDSCH/PUSCH. The bitwidth of this field depend on the number of the scheduled PDSCH/PUSCH.

- RV: Each bit of RV applies independent for each PDSCH/PUSCH. The bitwidth of this field depend on the number of the scheduled PDSCH/PUSCH.

- HARQ process ID: The HARQ process ID of the first scheduled PDSCH/PUSCH is included in the DCI, while the remaining PDSCHs use incremental HARQ process ID. The bitwidth of this field is 4 or 5 bits.

For supporting multi-cells PDSCHs/PUSCHs scheduled by a single DCI, the DCI design scheduling multiple PDSCHs/PUSCHs in Rel-17 can be reused as base line, and RAN1 can discuss which bits field can be modified , which can decrease the complexity of DCI design. For example, for the field of TDRA, the configured TDAR table can be extended to indicate multi-cell  PDSCHs/PUSCHs. For the fields of NDI and RV, the same scheme can be reused directly for multi-cell PDSCHs/PUSCHs scheduling DCI.

Proposal 3: For supporting multi-cells PDSCHs/PUSCHs scheduled by a single DCI, the design of the DCI scheduling multiple PDSCHs/PUSCHs in Rel-17 can be reused as baseline, and RAN1 can discuss which bits field should be modified.

In Rel-15/Rel-16/Rel-17, DCI format 1_1/0_1 is used for the scheduling of one or multiple PDSCH in one cell, and each configurable field is determined by the higher layer parameters of the scheduled cell. The main difference between the DCI that can schedule multi-cells PDSCH/PUSCH and legacy DCI format1_1/0_1 is that the number of the scheduled cells is extended from one to multiple. Thus, each configurable field may correspond to multiple higher layer parameters from multiple scheduled cells. Whether to introduce a new DCI format for the DCI that can schedule multi-cells PDSCH/PUSCH need to be discussed until it is clear how to determine each configured field of the DCI. In our view, if one reference cell can be specified for multiple scheduled cells, it is not necessary to define a new DCI format for multi-cells scheduling, the DCI format 1_1/0_1 can be reused.

Proposal 4: Whether to introduce a new DCI format for the DCI that can schedule multi-cells PDSCH/PUSCH need to be discussed until it is clear how to determine each configured field of the DCI.

In order to ensure blind decode for UE to a reasonable level, the different sizes of DCI format configured to monitor should be no more than 4 cells that include up to 3 sizes of DCI formats with CRC scrambled by C-RNTI. With consideration of UE implementation complexity, the ‘3+1’budget of DCI format size should be maintained. On the premise that no new requirement of blind detection is introduced for multi-cells scheduling, the ‘3+1’budget of DCI format size should be maintained. For example, if the total number of different DCI size configured to UE is more than 4 after including the DCI for multi-cells scheduling, the DCI size alignment procedure will be applied and the DCI can be regarded as one of the ‘C-RNTI’ or ‘others RNTI’ .

Proposal 5: On the premise that no new requirement of blind detection is introduced for multi-cells scheduling, the ‘3+1’budget of DCI format size should be maintained.

  - The SCS configuration among multi-cells

In Rel-15/Rel-16, when the UE is scheduled to receive PDSCH in scheduled cell by a DCI, K0 is used to indicate the slot offset between PDSCH and PDCCH, and K0 is based on the SCS configuration of scheduled cell. The K1 is used to indicate the slot offset between PDSCH and PUCCH, and K1 is based on the SCS configuration of UL cell. For multi-cells scheduling, the UE will be scheduled to receive PDSCHs in multiple cells by a single DCI, and feedback HARQ-ACK information in a UL cell. The SCS configuration among multi-cells can be divided in the following cases as show in the Table 2.

Table 2：SCS configuration among multi-cells

|  | scheduled cell-1 | other scheduled cells | scheduling cell | UL cell |
| --- | --- | --- | --- | --- |
| Case 1 | SCS-1 | SCS-1 | SCS-1 | SCS-1 |
| Case 2 | SCS-1 | SCS-1 | SCS-2 | SCS-3 |
| Case 3 | SCS-1 | SCS-2 | SCS-3 | SCS-4 |

For the case 1, the SCS configurations among all the cells including the multiple scheduled cells, scheduling cell and UL cell are the same. The value of K0 and K1 can be used directly without scaling. For the case 2, the SCS configurations among multiple scheduled cells are the same, and different from the SCS configuration of scheduling cell and UL cell. The legacy scheme of K0 and K1on determination of the offset between PDSCH and PDCCH, PDSCH and PUCCH can be reused. For the case 3, the SCS configurations among multiple scheduled cells are different. In this case, for the scheduled cell, the slot offset derived from SCS configuration based on the value of K0 and K1 will different from another scheduled cell with different SCS configuration, which will cause different PUCCH slot and PDSCH slot.

Observation 1：The SCS configuration among multi-cells can be divided in the following cases:

- Case-1: The SCS configurations among all the cells including the multiple scheduled cells, scheduling cell and UL cell are the same.

- Case-2: The SCS configurations among multiple scheduled cells are the same, and different from the SCS configuration of scheduling cell and UL cell.

- Case-3: The SCS configurations among multiple scheduled cells are different.

In our view. at lease the cases-1 and case-2 that the SCS configurations among multiple scheduled cells are the same should be supported firstly for the multi-cell PUSCH/PDSCH scheduling. And the legacy mechanism on determination of the slot offset between PDSCH and PDCCH, PDSCH and PUCCH can be reused directly. For the case 3 that the SCS configurations among multiple scheduled cells are the different, it should be further studied.

Proposal 6: At least the case that the same SCS configuration among multiple scheduled cells should be supported firstly. Whether to support different SCS configuration among multiple scheduled cells can be further studied.

Conclusion

This contribution discussed the PDCCH enhancements for the multi-cell PUSCH/PDSCH scheduling with a single DCI. We have the following proposals,

Proposal 1: The maximum number of cells that can be scheduled by a single should be no more than 4 Cells with each TB scheduled per cell.

Proposal 2: There are two options on the actual number of scheduled cells by a DCI as follows.

- Option-1: is fixed to N, the scheduled cells are configured by higher layer.

- Option-2: can dynamically change from 1 to M, the combination of scheduled cells is indicated by DCI, e.g. carrier indicator field.

Proposal 3: For supporting multi-cells PDSCHs/PUSCHs scheduled by a single DCI, the design of the DCI scheduling multiple PDSCHs/PUSCHs in Rel-17 can be reused as baseline, and RAN1 can discuss which bits field should be modified.

Proposal 4: Whether to introduce a new DCI format for the DCI that can schedule multi-cells PDSCH/PUSCH need to be discussed until it is clear how to determine each configured field of the DCI.

Proposal 5: On the premise that no new requirement of blind detection is introduced for multi-cells scheduling, the ‘3+1’budget of DCI format size should be maintained.

Observation 1：The SCS configuration among multi-cells can be divided in the following cases:

- Case-1: The SCS configurations among all the cells including the multiple scheduled cells, scheduling cell and UL cell are the same.

- Case-2: The SCS configurations among multiple scheduled cells are the same, and different from the SCS configuration of scheduling cell and UL cell.

- Case-3: The SCS configurations among multiple scheduled cells are different.

Proposal 6: At least the case that the same SCS configuration among multiple scheduled cells should be supported firstly. Whether to support different SCS configuration among multiple scheduled cells can be further studied.

Reference

RP-220834, Revised WID on Multi-carrier enhancements, NTT DOCOMO,INC, RAN#95-e meeting