3GPP TSG RAN WG1 #109-e                                                                               R1-2203450

e-Meeting, May 9th – 20th, 2022

Source:	CATT

Title:	Discussion on AI/ML framework for air interface

Agenda Item:	9.2.1

Document for:	Discussion and Decision

Introduction

Artificial Intelligence (AI) and Machine Learning (ML) are powerful in solving non-linear issues, and become one of the most popular research directions around the world. For wireless communication, AI/ML also attracts strong interests from academic circle, and already shows its capability in improving performance in many aspects. 3GPP also finished an RAN3-led AI/ML study in Rel-17, in which AI models are applied for better data collections in several typical use cases, including network energy saving, load balancing, and mobility optimization [1].

In Rel-18, a study item on AI/ML in RAN1 was approved [2], to facilitate the support of AI/ML in physical layer other than implementation-based approaches. The objective of the SID includes:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In this contribution, we investigate the general aspects of AI/ML approaches for air interface, including the general AI/ML framework, collaboration levels, dataset, and life cycle management. We also provide our views on the common evaluation methodology and KPIs for different use cases.

Discussion

General AI/ML framework

Functional framework

The framework of AI/ML application in air interface should embody how AI model is trained, deployed and interactive with modules for wireless communication. Thus the framework shall at least include function blocks for:  data collection (for AI model training and inference, respectively), model training, model management, AI model and actor. Figure 1 illustrates how these blocks are connected with each other.

Figure 1 Functional framework of AI/ML.

The function for each block is briefly introduced as follows:

- Model management

Model management block is responsible for AI model deployment, model monitoring, model update, etc. It can also determine whether AI/ML-based approach should be disabled and fallback to traditional (non-AI based) mechanism. As observed in Figure 1, model management block can send the request of data collection to data collection block (#1), the request of model generation to model training block, and deploy/update the AI model. The full functions of model management block should be carefully considered, since it is also the core of life cycle management.

- Data collection

Two data collection blocks are considered. One is used to collect data for model training before an AI model is deployed, while the other one is used to collect data as the input of a deployed AI model. Both of them include data pre-processing function, e.g. normalization. In reality, these two blocks may be deployed at the same or different nodes.

- Model training

The model training block is responsible in model training, validation and testing. Academic model training strategies may be different for different use cases, including supervised learning, unsupervised learning, reinforcement learning, federal learning, transfer learning and more. Considering the difficulty and performance, companies can start with supervised learning, but other learning strategies are not precluded. At least for study purpose, the ideal label can be acquired based on simulated data set, which is to be discussed in Section 2.4. Whether simulated data set can be used for model training in real-world deployment is for further investigation. Once model training is completed, the new AI model will be uploaded to model management block for further deployment/update.

- AI Model

AI model is for AI/ML-based inference according to the input from data collector (#2). The output will be delivered to the actor block. Other than inferring, AI model block may also prepare feedback to model management block to help smart model management. This may require some capability of ‘intermediate evaluation’ based on the output of AI model, which is to be discussed in Section 2.3.

- Actor

Actor receives the output from AI model and triggers or performs the corresponding actions. At least two kinds of output can be expected. One is data feedback to data collector(s), which can be part of the input data for AI model or helps future training. The other one is feedback to model management blocks, i.e. the result of ‘final evaluation’, which is to be discussed in Section 2.3 too.

Based on the analysis above, we have the following proposal:

Proposal 1: Consider an AL/ML framework at least includes function blocks for: data collection, model training, model management, AI model inference and actor.

Reference AI models

In principle, 3GPP is not aiming at specifying the detailed structure of AI model. This is not only to leave enough flexibility on realization, but also respect the fact that new/better AI models are emerging rapidly. However, for qualitative study on the benefit of AI/ML-based approaches, we can focus on a narrow set of AI models as the starting point to save the effort and speed up the study. Such narrow set of AI models can be called as ‘reference AI model(s)’. With a common reference AI model, it is easier to define parameters/hyper parameters for cross check and calibration.

Different reference AI models may be defined for different use cases. When determining the reference AI model, some key aspects should be considered, including maturity, complexity, and matching degree with wireless communication system. We observe that neutral networks like deep neutral network (DNN), convolutional neutral network (CNN), recurrent neutral network (RNN) and their variants (e.g. CsiNet) have been widely studied by academic circle. Most of the academic results shows promising potential of DNN/CNN/RNN in air interface. Hence, it seems suitable to consider DNN/CNN/RNN and their variants as the reference AI models.

Proposal 2: At least for the purpose of cross check and calibration, consider a common narrow set of AI models as reference AI model(s) as the starting point.

- DNN/CNN/RNN and their variants can be considered.

Collaboration between UE and gNB

As required by the SID, several collaboration levels between UE and gNB should be identified. According to the interaction degree between UE and gNB for AI inference and/or training, four collaboration levels for inference can be considered as Collaboration Level#0, Level#1, Level#2 and Level#3. Table 1 summarizes the definition of each level and provides an example for each of them.

Table 1 Collaboration levels for inference.

| Levels | Definition | Example |  |
| --- | --- | --- | --- |
| Level#0 | - Implementation of AI model at one node is transparent to the other node. - Inference and training are conducted all by oneself. - No collaboration is required. |  | The gNB resumes the channel using an AI model and traditional CSI feedback. |
| Level#1 | - Inference is conducted at either UE or gNB side. - Non-AI-related assistance signal/information is transmitted to help AI model inference. |  | The UE estimates the best DL beam from a large beam/CSI-RS set, based on measuring a small set of CSI-RS configured by gNB. |
| Level#2 | - Inference is conducted at either UE or gNB side. - AI-related assistance signal/information is transmitted to adjust/update/configure the AI model. |  | The gNB informs the UE the parameters/hyper parameters of the AI model for certain purpose. |
| Level#3 | - AI inference is jointly conducted at both UE and gNB side to achieve certain purpose. - AI-related assistance signal/information is transmitted to align the AI models. |  | The UE compresses the CSI information based on AI model#1, while the gNB resumes the channel based on AI model #2 and the output of AI model#1. |

In Table 1, it can be seen that Collaboration Level#0 is purely implementation based, and no spec impact is expected. Collaboration Level#1 may have spec impact on defining the interface (input and output) of the AI model based on the existing signal/information. Collaboration Level#2 is more aggressive, since the parameters/hyper parameters of AI model needs to be specified. In Collaboration Level#3, the AI models in UE and gNB need to be aligned/synchronized with each other. The output of the AI model from one side is may not have clear physical significance, but will become the input of the AI model in the other side.

Proposal 3: Consider defining four collaboration levels, i.e.

- Level#0: No collaboration. AI model in one side is transparent to the other side.

- Level#1: Inference is conducted at one side with exchanging non-AI-related assistance signal information.

- Level#2: Inference is conducted at one side with exchanging AI-related assistance signal information.

- Level#3: Inference is jointly conducted at both sides with exchanging AI-related assistance signal information for AI model alignment/synchronization.

Common aspects of different use cases

Common evaluation methodology

The evaluation methodologies of the initial set of use cases, i.e. CSI feedback, beam management and positioning, are unsurprisingly different. For instance, the use case of CSI feedback may focus on channel compression and spectrum efficiency, while the use case of positioning may concern the positioning accuracy under different LOS/NLOS (e.g. InF-DH) assumption. In our companion papers [3][4][5], several specific evaluation methodologies for different use cases/sub use cases are discussed in details.

Besides, there may be common spirit of evaluation methodology among all use cases. When AI/ML-based approach is applied, some conventional functions of the wireless network will be replaced or enhanced. And hence, output of AI model can be achieved and evaluated from functional point of view. We can denote such evaluation as ‘intermediate evaluation’.  For example, intermediate evaluation may include: compression ratio for CSI feedback, probability of selecting the best beam in beam management, correct rate of NLOS/LOS identification in positioning. Although in some specific (sub) use cases, there may be no intermediate result, e.g. the AI model infers the UE position directly via conventional PRS.

Observation 1: Intermediate evaluation/comparison may be achievable based on the output of AI model and legacy module.

The output of AI model will be subsequently utilized in signal processing procedure to acquire the final results. Once the signal processing procedure is finished, the final results are mostly evaluated by common KPIs of system performance, e.g. BLER, spectrum efficiency, throughput, or CDF of positioning accuracy. Such evaluation is typically based on wireless communication terminology, which can be denoted as ‘final evaluation’. Figure 2 shows the relationship between intermediate evaluation and final evaluation.

Figure 2 Intermediate evaluation and Final evaluation.

For study purpose, both intermediate evaluation and final evaluation are considerable. First of all, evaluation and comparison based on final results are basically from view of the performance of wireless communication system, which implies the necessity of final evaluation. Secondly, when applicable, the intermediate results from AI model and legacy module can be evaluated and compared to show the advantage of AI model in a functional level.

Proposal 4: For AI/ML evaluation, both intermediate evaluation and final evaluation are considerable.

- Intermediate evaluation is based on the direct output of AI model, when applicable.

- Final evaluation is from view of performance of wireless communication system.

Common KPI for AI/ML-based approach

Conventional KPIs for wireless communication system focus on the transmission efficiency/accuracy. For example, it is expected that the BLER or throughput/spectrum efficiency will be evaluated in the use case of CSI feedback, or the CDF of positioning error will be drawn in the use case of positioning. Besides the KPIs mentioned above, some AI-specific KPIs should be taken into consideration when applying AI/ML-based approaches in NR.

- Size of AI model

With a specific structure, the performance of an AI model heavily depends on the scale of AI model. A larger size of AI model is generally more powerful in addressing complex-non-linear issues, at the cost of at least buffering size. Some typical AI models in academic circle have more than 10 thousand parameters, leading to a size larger than million bytes. This may be a sensitive issue, especially when AI model is deployed at UE side.

- Computation complexity & latency

AI model inference is powered by computation capability. For example, it is widely assumed that such computation consumes the resource of XPU (e.g. NPU or GPU). In general, the required computation capability depends on the size of AI model. Higher computation capability is required by stricter latency requirement, and vice versa. Such computation complexity should be carefully planned, since it not only brings computing burden, but also power consumption challenge on battery.

- Generalization ability

Unlike conventional approaches, AI/ML-based approaches rely on data of training. Training data is collected from some specific scenarios, and thus an AI model becomes more or less ‘scenario-based’. For an AI model, generalization ability is constantly concerned. It is still unclear whether an AI model can be trained in one scenario and deployed in another scenario with acceptable performance loss in wireless communication system. Also, from view of implementation, it has critical impact on the periodicity of model training/update.

- Overhead of exchanging AI-specific signaling

For purely implementation-based AI approaches, there is no overhead for exchanging AI-specific signaling. But in case the AI model inference or AI model training needs collaboration between UE and gNB, the overhead of exchanging AI-specific signaling may be non-negligible. The signaling may include not only communication-specific control information, but also parameters/hyper parameters for AI model synchronization or update.

Note that the above AI-specific KPIs are common for the AI/ML-based approaches in different use cases. A common understanding on these KPIs is beneficial for leveraging the future study.

Proposal 5: Consider the following common KPIs for AI/ML-based approaches:

- Size of AI model,

- Computation complexity & latency,

- Generalization ability,

- Overhead of exchanging AI-specific signalling.

Dataset

For AI/ML-based approaches, the dataset is indubitably important. At least for study purpose, a common dataset of wireless communication is fundamental for calibration across different companies. The construction of dataset should be carefully designed. The following two aspects should be taken into consideration.

- Completeness

The completeness of the dataset is responsible for the performance of AI/ML-based approaches. To improve the generalization ability and avoid overfitting, the dataset used for training should cover different situations/assumptions as much as possible. As the amount of required data may be huge, it is difficult to construct a complete dataset by field data only. Moreover, if the data of ‘corner case’ plays an important role in AI training, it is unstable to totally rely on field data, since this case may not be observed/collected during the data collection. On the other hand, once the training dataset covers most of the concerned case, further increasing the data volume does not help much in improving the performance. This brings a trade-off between dataset size and AI model performance. To handle such trade-off in a more flexible way, it is preferred that the generation of dataset can be more controllable.

- Availability

In wireless communication system, various data is generated constantly. As an example, the channel state information and positioning information of the UE are achievable at the serving cell. However, even from view of gNB, the effort in data collection, processing, storage and transfer can be excessively huge. Also some data may be hard to acquire, e.g. precise location of the UE. Additionally, privacy of the data needs to be protected. Even if the privacy issue is technically solvable (e.g. by data washing), it is still questionable if collecting and exchanging UE data internationally is always legal in all regions.

With the above analysis, we think the existing 3GPP models are more suitable to construct the dataset for the study of AI/ML-based approaches. One significant advantage is that it can easily satisfy any required amount of data, as long as the dropping cases are sufficiently conducted. Another attractive merit is that there is no risk of privacy or legality issue. Comparing to field data, although the 3GPP data are generated by simulation, the models in TR 38.901 and TR 38.857 are based on statistic materials, which already reflects the real-world scenarios to some degree.

Hence, at the first stage of the study, 3GPP statistic models in TR 38.901 and TR 38.857 should be used to construct the dataset. This facilitates the study progress and future calibration.

Proposal 6: At least for the purpose of study and calibration, 3GPP statistic models (TR 38.901 and TR 38.857) are used to construct the dataset for AI/ML-based approaches at first.

It is expected to achieve some consensus based on the simulated data at the first stage (e.g. methodology, attainable gain, complexity). Meanwhile, the difference between field data and simulate data still exists. To confirm the effectiveness of AI/ML-based approaches in real world, using field data is the most direct choice. Hence, field data can be additionally considered in a later phase. For cross check, we should consider how to achieve open, stable field datasets for different use cases.

Proposal 7: Field data for different use cases can be additionally considered in a later phase of the study.

Specifically, during the study, companies should reach the common understanding on the partitioning of the dataset. For example, the proportion between the training data, validation data and testing data should be determined as part of the evaluation/calibration work. Empirically, when the scale of dataset is not huge (e.g. less than 10 thousand samples), a proportion of 6:2:2 is considerable. When the scale of dataset grows, the ratio of training data should be increased, and the ratio of validation data and testing data will be decreased subsequently.

Proposal 8: Study the proper partitioning of the dataset, e.g. proportion between training data, validation data and testing data.

Life cycle management

Before deploying an AI model, the model should be trained, validated and tested. Generally, from view of the instantaneity, two kinds of training strategies have been applied, i.e. online training and offline training. Online training is quite ‘dynamic’, which leads to fast iteration of parameters in the AI model. In an extreme case, the parameters of the AI model can be adjusted by each new training data/label. It is often used in application level to chase the variable user’s habits, or hot news feed. In some cases, e.g. lack of training label of a specific user, online training may be the only choice. Compared to online training, offline training seems to be ‘semi-static’. The training is at least batch-level, and allows more time for validation/test and determining whether the model needs update or not. The interaction between model training, data collection and model deployment is less frequent than online training.

At the first phase of this study, we can assume offline training as the starting point. This is due to the fact that the scenario of AI/ML-based approach is unlikely to change vastly within a concerned duration. Online training or fine tuning in some use cases can be further considered. But we should aware that the cost of computation and information exchange will be increased for online training. Whether this is a suitable trade-off need to be carefully considered.

Proposal 9: For AI application in air interface, use offline training as the starting point.

Once an AI model is deployed, the model management block shall periodically monitor the validity of the AI model. If the deployed AI model is considered as invalid, different ways can be considered. For example, the AI model may be updated to a new version trained by the newly collected data. For another example, fallback to non-AI/ML-based approaches may be another choice.

Observation 2: When a deployed AI model becomes invalid, it may be updated to a new version trained by newly collected data, or may be disabled as a fallback option.

On air interface, how to judge the validity of AI model is still unclear. Intuitively, the feedback from AI model block and/or actor block should be taken into consideration. The criterion for validity may be different from one use case to another one. Possibly, such criterion may be specified, or just implementation-based.

Proposal 10: Study the criterion for the validity of AI model.

Other aspects

Contrary to deterministic approaches, AI/ML-based approaches are all computational and data-driven, and thus lack of theoretical guarantee on the best or worst performance. This lays the following two questions:

- How to characterize the UE capability on support of AI/ML-based approaches.

- How to guarantee the robustness of UE/network when AI/ML-based approaches are applied.

We’ve analyzed these two questions in our companion paper [6]. In short, for the first question, it is considerable to define UE capabilities based on quantized computation and/or buffering capability. For the second question, efficient enabling/fallback between AI/ML-based approaches and legacy mechanism is a safe choice to guarantee the robustness.

We have the following proposals.

Proposal 11: For support of AI/ML, consider defining several levels of UE capabilities based on quantized computation and/or buffering capability.

Proposal 12: Efficient enabling/fallback between AI/ML-based approaches and legacy mechanisms should be considered.

Conclusion

In this contribution, we provide our views on general AI/ML framework for air interface. The observations and proposals are summarized as follows:

Observation 1: Intermediate evaluation/comparison may be achievable based on the output of AI model and legacy module.

Observation 2: When a deployed AI model becomes invalid, it may be updated to a new version trained by newly collected data, or may be disabled as a fallback option.

Proposal 1: Consider an AL/ML framework at least includes function blocks for: data collection, model training, model management, AI model inference and actor.

Proposal 2: At least for the purpose of cross check and calibration, consider a common narrow set of AI models as reference AI model(s) as the starting point.

- DNN/CNN/RNN and their variants can be considered.

Proposal 3: Consider defining four collaboration levels, i.e.

- Level#0: No collaboration. AI model in one side is transparent to the other side.

- Level#1: Inference is conducted at one side with exchanging non-AI-related assistance signal information.

- Level#2: Inference is conducted at one side with exchanging AI-related assistance signal information.

- Level#3: Inference is jointly conducted at both sides with exchanging AI-related assistance signal information for AI model alignment/synchronization.

Proposal 4: For AI/ML evaluation, both intermediate evaluation and final evaluation are considerable.

- Intermediate evaluation is based on the direct output of AI model, when applicable.

- Final evaluation is from view of performance of wireless communication system.

Proposal 5: Consider the following common KPIs for AI/ML-based approaches:

- Size of AI model,

- Computation complexity & latency,

- Generalization ability,

- Overhead of exchanging AI-specific signalling.

Proposal 6: At least for the purpose of study and calibration, 3GPP statistic models (TR 38.901 and TR 38.857) are used to construct the dataset for AI/ML-based approaches at first.

Proposal 7: Field data for different use cases can be additionally considered in a later phase of the study.

Proposal 8: Study the proper partitioning of the dataset, e.g. proportion between training data, validation data and testing data.

Proposal 9: For AI application in air interface, use offline training as the starting point.

Proposal 10: Study the criterion for the validity of AI model.

Proposal 11: For support of AI/ML, consider defining several levels of UE capabilities based on quantized computation and/or buffering capability.

Proposal 12: Efficient enabling/fallback between AI/ML-based approaches and legacy mechanisms should be considered.

References

- TR 37.817, Study on enhancement for Data Collection for NR and EN-DC, v 17.0.0.

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#94-e, e-Meeting, December 6th – 17rd, 2021.

- R1-2203451, Discussion on evaluation on AI/ML for CSI feedback, CATT, RAN1#109-e, e-Meeting, May 9th – 20th.

- R1-2203453, Discussion on evaluation on AI/ML for beam management, CATT, RAN1#109-e, e-Meeting, May 9th - 20th.

- R1-2203455, Discussion on evaluation on AI/ML for positioning, CATT, RAN1#109-e, e-Meeting, May 9th - 20th.

- R1-2203457, Views on UE capability of AI/ML for air interface, CATT, RAN1#109-e, e-Meeting, May 9th - 20th.