3GPP TSG RAN WG1 #109-e			R1-2203451

e-Meeting, May 9th – 20th, 2022

Source: 	CATT

Title:	Discussion on evaluation on AI/ML for CSI feedback

Agenda Item:	9.2.2.1

Document for:	Discussion and Decision

- Introduction

In RAN #94-e meeting, AI/ML for NR air-interface was agreed and the following objective was approved in the SID [1]:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In this contribution, evaluation methodology and KPIs for AI/ML based CSI feedback enhancement are discussed, and initial performance evaluation results are provided.

- Discussion

  - Evaluation Methodology

According to the technique effect and collaboration levels between network and UE, there are 3 types of sub use cases for AI/ML based CSI feedback enhancement:

- Case 1: AI/ML based CSI compression/decompression at both sides

- Case 2: AI/ML based CSI feedback acquisition at UE side only

- Case 3: AI/ML based channel prediction at one side or both sides

The details of each sub use case can be found in [2]. Since there is lots of sub use cases, evaluating all of them incurs considerable work, it would be better to down select the sub use cases first, and only evaluate the selected sub use cases.

Proposal 1:

- Down select sub use cases for AI/ML based CSI feedback first, and evaluate the selected sub use cases.

Achieving obvious performance gain with acceptable complexity increasing of devices is a basic precondition of specifying a solution in 3GPP. Therefore for AI/ML based CSI feedback, at least system level simulation for selected sub use case is needed.

Various types of AI models for CSI feedback are proposed in academia. Different AI models are of different sizes, different complexities, and achieving different performance. Identifying which type of AI models can be used for CSI feedback, and which parameters for the AI models should be used require lots of evaluations. It is known that system level simulation is time consuming, and the performance of the simulation results would be affected by lots of factors besides AI/ML model(s). It is not a good way to evaluate large amount of AI models through system level evaluation directly.

When an AI/ML based CSI feedback approach is applied, conventional CSI acquisition module(s) at one side or both sides of the wireless network are replaced or enhanced by AI model(s). In order to reduce evaluation workload and identify candidate AI models more efficiently, evaluations for the replaced/enhanced modules can be considered. Such evaluations can be denoted as “intermediate evaluation”. For intermediate evaluation, the KPI for AI model and legacy CSI feedback module can be compared. Then preliminary AI model selection can be done with the evaluation. Note that when selecting AI models, besides performance evaluation, complexity and size of AI models should also be considered. When candidate AI models are available, the system level performance of the selected AI models can be evaluated, by incorporating the AI model to the system level platform. We can denote such system level evaluation as “Final evaluation”.

Proposal 2:

- For the evaluation on AI/ML based CSI feedback, the following two phases are considered:

  - Phase 1: Intermediate evaluation for AI model selection, with performance, model size and complexity of AI model considered;

  - Phase 2: Final evaluation for system level performance verification.

  - Baseline of the evaluation

The evaluation for AI/ML based CSI feedback is to verify the overhead reduction and/or accuracy improvement of AI based approaches. Since DL Type II codebook is a high precision CSI feedback, it can be used as the baseline of the evaluation.

Proposal 3:

- On the evaluations for AI/ML based CSI feedback, CSI feedback with Rel-16/Rel-17 DL Type II codebook can be considered as baseline.

  - Scenarios and simulation assumption

The scenarios in TR 38.901 can be considered as a baseline for the evaluation of AI/ML based CSI feedback. However, it seems evaluating all of the scenarios is not necessary. AI based approach are data driven. For AI/ML based CSI feedback, the more complex the channel environment is, the more features the AI model needs to learn. It is expected that if an AI model works well in the scenario with a complex channel environment, it also can work well in a scenario with simpler channel environment. That doesn’t mean that the same parameters for the AI model are used in different scenarios. It is possible that the AI model shall be trained separately for different scenarios. In order to test AI model’s generalization ability, evaluations for multiple scenarios is necessary. Choosing several typical scenarios for the evaluation can be considered.

Proposal 4:

- Several typical scenarios in TR 38.901 are considered for the evaluations for AI/ML based CSI feedback.

There is a separate use case for beam management addressing the main issues of FR2. For CSI feedback enhancement, it is expected to focus on FR1 as usual.

Proposal 5:

- For the evaluations for AI/ML based CSI feedback, only FR 1 is considered.

  - KPIs

In our companion contribution [3], the following four common AI-specific KPIs are provided:

- Size of AI model

- Computation complexity & latency

- Generalization ability

- Overhead of exchanging AI-specific signalling

These common KPIs should be considered for AI/ML based CSI feedback. Note that, when evaluating generalization ability, the factors that shall be considered may be different for different sub use cases. How to evaluate generalization abilities should be carefully studied. For instance, the performance of AI/ML based CSI feedback compression is related to frequency selectivity of the channels. When evaluating the generalization ability for AI/ML based CSI feedback compression, scenarios with different frequency selectivity shall be considered. The performance of AI/ML based channel prediction in time domain is related to time selectivity of the channels. When evaluating the generalization ability for AI/ML based channel prediction in time domain, scenarios with different time selectivity shall be considered.

Proposal 6:

- How to evaluate generalization abilities for AI/ML based CSI feedback should be studied,

  - Different factors may be considered for different use cases.

Besides the above common AI-specific KPIs, use case specific KPIs for performance also should be considered for AI/ML based CSI feedback.

For system level evaluation, SE/throughput is an important KPI to reflect the performance of solutions for AI/ML based CSI feedback. In the evaluation for use cases targeting overhead reduction, the trade-off between SE/throughput and overhead of CSI shall be considered.

For intermedia evaluation, since the AI model is not deployed in the system, some intermedia KPIs can be considered. As the purpose of intermediate evaluation is to identify candidate AI models with overhead reduction and/or CSI acquisition accuracy improvement, KPIs related to overhead and/or accuracy can be considered. Such KPIs can be analyzed case-by-case. For example, for the sub use cases for PMI compression, cosine similarity can be used as a KPI. Take the sub use case shown in Fig. 1 as an example. For this sub use case, traditional PMI acquisition module is replaced by an auto encoder at UE side, and an auto decoder is deployed at gNB to reconstruct PMI. The accuracy of PMI compression can be evaluated by estimating cosine similarity of reconstructed PMI of auto decoder and target PMI (e.g. eigenvector of channel). Since one of the purposes of the sub use case is CSI feedback overhead reduction, trade-off between performance and overhead reduction shall be considered. For the sub use cases for full channel compression/channel prediction, NMMSE/MSE of recovered channel and target channel can be considered as a KPI.

Figure 1 AI based PMI compression and decompression

Proposal 7:

- For AI/ML based CSI feedback, the following KPIs are considered:

  - Size of AI model

  - Computation complexity & latency

  - Generalization ability

  - Overhead of exchanging AI-specific signalling

  - SE/throughput

  - Cosine similarity/MMSE/MSE.

Proposal 8:

- For AI/ML based CSI feedback, trade-off between overhead and performance is considered.

  - Simulation Results

In this section, our initial simulation results on AI based PMI compression feedback are provided. The simulation assumptions are shown in Appendix. In our simulation, transformer based AI model is used in both auto encoder and auto decoder, and fully-connected layers are used for both up/down-sampling and linear transformation.

The simulation results are shown in Fig. 2, wherein the horizontal axis is the overhead of CSI feedback for PMI, and the vertical axis is the cosine similarity calculated with the following formula:

ρ=bHvb2v2

where v is channel eigenvector, b is the selected PMI by the traditional PMI determination module for traditional codebook based approach or the recovered PMI by auto decoder for AI based approach.

It can be seen from the results that under the same cosine similarity, compared with traditional codebook based CSI feedback, 40 ~ 150 bits can be saved by AI based PMI compression feedback.

Figure 2 Comparison of AI based approach and codebook based approach for CSI feedback

- Conclusions

In this contribution, we provide our views on evaluation and KPIs for AI/ML based CSI feedback. We have the following proposals:

Proposal 1:

- Down select sub use cases for AI/ML based CSI feedback first, and only simulate the selected sub use cases.

Proposal 2:

- For the evaluation on AI/ML based CSI feedback, the following two phases are considered:

  - Phase 1: Intermediate evaluation for AI model selection, with performance, model size and complexity of AI model considered;

  - Phase 2: Final evaluation for system level performance verification.

Proposal 3:

- On the evaluations for AI/ML based CSI feedback, CSI feedback with Rel-16/Rel-17 DL Type II codebook can be considered as baseline.

Proposal 4:

- Several typical scenarios in TR 38.901 are considered for the evaluations for AI/ML based CSI feedback.

Proposal 5:

- For the evaluations for AI/ML based CSI feedback, only FR 1 is considered.

Proposal 6:

- How to evaluate generalization abilities for AI/ML based CSI feedback should be studied,

  - Different factors may be considered for different use cases.

Proposal 7:

- For AI/ML based CSI feedback, the following KPIs are considered:

  - Size of AI model

  - Computation complexity & latency

  - Generalization ability

  - Overhead of exchanging AI-specific signalling

  - SE/throughput

  - Cosine similarity/MMSE/MSE.

Proposal 8:

- For AI/ML based CSI feedback, trade-off between overhead and performance is considered.

- References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#94-e, e-Meeting, December 6th – 17rd, 2021.

- R1-2203450, Discussion on AI/ML framework for air interface, CATT, RAN1#109-e, e-Meeting, May 9th – 20th.

- R1-2203452, Discussion on other aspects on AI/ML for CSI feedback, CATT, RAN1#109-e, e-Meeting, May 9th – 20th.

- Appendix

Table 1 Simulation assumption

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 4 GHz |
| Bandwidth | 10MHz |
| Subcarrier Spacing | 15kHz |
| Channel Model | Same as 3D-UMa in TR36.873 |
| BS antenna element radiation pattern | According to TR36.873 |
| UE antenna element radiation pattern | Omnidirectional |
| ISD | 200m |
| BS Antenna Configuration | (M,N,P,Mg,Ng) = (8,8,2,1,1) (dH,dV) = (0.5, 0.8)λ |
| UE Configuration | (M,N,P,Mg,Ng) = (1,1,2,1,1) (dH,dV) = (0.5, 0.5)λ |
| UE distribution | 20% Outdoor in cars3): 30km/h, 80% Indoor in houses: 3km/h 10 users per TRP |
| CSI feedback | Periodic: 5ms Scheduling delay: 4ms PRG: 4PRBs |