3GPP TSG RAN WG1 #109-e                                     R1-2203453

e-Meeting, May 9th – 20th, 2022

Source: 	CATT

Title:	Discussion on evaluation on AI/ML for beam management

Agenda Item:	9.2.3.1

Document for:	Discussion and Decision

- Introduction

In RAN #94 e-meeting, AI/ML/ML for NR Air Interface was agreed and the following objective was approved in the SID [1]:

Study the 3GPP framework for AI/ML/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI/ML model(s) for calibration - AI/ML model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML/ML scheme, as well as generalization capability should be considered. - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI/ML Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML/ML based performance enhancements and ensuring that UE and gNB with AI/ML/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML/ML processing capabilities definition Note 1: specific AI/ML/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In this contribution, evaluation methodology and KPIs for AI/ML/ML based beam management enhancement are discussed.

- Discussion

  - Methodology

The evaluation methodology common to all sub use cases is discussed in our companion paper [2]. When some traditional function of the wireless network is replaced by AI/ML-based module, the output of the AI/ML module can be evaluated from functional point of view. We denote such evaluation as ‘intermediate evaluation’. The output of AI/ML module will be utilized in signal processing procedure to acquire the final results, which can be denoted as ‘final evaluation’.

For AI/ML-based beam management, three sub use cases are considered in our companion paper [3], i.e. narrow beam prediction based on wide beam measurement, beam prediction based on partial beam measurement and beam prediction in time domain. For all these cases, it is expected that AI/ML-based approach can replace the conventional beam management strategy, and thus largely reduce the resource overhead at the cost of minor performance loss. Therefore, AI/ML based beam management can be evaluated based on two stages: Firstly, replace the convention functional module in the system with AI/ML modules, and evaluates the output results of AI/ML model, i.e. evaluates the selection of beam (pair), or the overhead reduction. This refers to the ‘intermediate evaluation’ mentioned above. Secondly, the output of AI/ML model is applied in the system-level platform, and then the system-level performance is evaluated as the ‘final evaluation’.

Proposal 1: For AI/ML based beam management, the following evaluation can be considered:

- Intermediate evaluation: Evaluate the result of beam selection from the AI model, or the overhead reduction due to AI/ML-based approach;

- Final evaluation: Evaluate the system-level transmission performance based on the output of AI model.

  - Baseline of the evaluation

Usually, conventional beam management exhaustively sweeps all the transmitting and receiving beam pairs, which is an optimal beam selecting scheme. The main purpose of AI/ML-based beam management is to verifying the overhead reduction of AI/ML-based approaches with similar performance. Conventional beam management strategy, i.e. exhaustive beam sweeping, can be used as the baseline of the evaluation.

Proposal 2: On the simulations for AI/ML-based beam management, conventional beam management strategy (i.e. exhaustive beam sweeping) can be considered as baseline.

  - Scenarios and simulation assumption

The scenarios in TR 38.901 can be considered as a reference for the evaluation of AI/ML-based beam management. For example, UMa or indoor can be considered as the simulated scenario. The number of gNB antennas can be set to 128, while the number of UE antennas can be 32. However, there is no need to exhaustively simulate all possible scenarios to verify the effectiveness of AI/ML model. It is expected that AI/ML has a degree of generalization ability, or can be fine-tuned/updated to improve the performance in different scenarios. Hence, it should be enough to only consider a few typical scenarios.

Proposal 3: A few typical scenarios in TR 38.901 can be considered for the evaluations for AI/ML-based beam management.

Besides, for beam management, we should pay more attention to FR2 than FR1. This is due to the fact that the number of beam pairs in FR2 is much larger than in FR1, which leads to higher demand of overhead reduction for measurement. Additionally, AI/ML-based beam prediction in time domain seems attractive to address the blockage issue in FR2, by which the gNB and UE can prepare an unblocked beam pair in advance. Therefore, we suggest focusing on FR2 for evaluation of AI/ML-based beam management.

Proposal 4: For the evaluations for AI/ML-based CSI feedback, focus on FR2.

  - KPI

The AI-specific KPIs are discussed in our companion paper [2]. These include the following four common AI specific KPIs：

- Size of AI model

- Computation complexity & latency

- Generalization ability

- Overhead of exchanging AI-specific signaling

In addition, the conventional KPIs of AI/ML-based beam management should also be considered. For AI/ML-based beam management, there are also two kinds of KPIs, i.e. “intermediate KPI” and “final KPI”. The “intermediate KPI” may include: the probability of selecting the optimal beam (pair) by the output of AI model, or the average power loss between the selected beam (pair) and the optimal beam (pair), or the ratio of overhead reduction. The “final KPI” can be the system-level SE/throughput, or the SE/throughput loss compared to that of beam-sweeping approach.

Proposal 5: For AI/ML-based beam management, in addition to AI-specific KPIs, the following KPIs can be considered:

- Intermediate KPI: Probability of identifying the best beam (pair), average power loss between the selected beam (pair) and the optimal beam (pair), and the ratio of overhead reduction;

- Final KPI: System-level SE/throughput or the SE/throughput loss compared to that of beam-sweeping approach.

- Conclusions

In this contribution, we provided our views on AI/ML-based beam management. We have the following and proposals:

Proposal 1: For AI/ML based beam management, the following evaluation can be considered:

- Intermediate evaluation: Evaluate the result of beam selection from the AI model, or the overhead reduction due to AI/ML-based approach;

- Final evaluation: Evaluate the system-level transmission performance based on the output of AI model.

Proposal 2: On the simulations for AI/ML-based beam management, conventional beam management strategy (i.e. exhaustive beam sweeping) can be considered as baseline.

Proposal 3: A few typical scenarios in TR 38.901 can be considered for the evaluations for AI/ML-based beam management.

Proposal 4: For the evaluations for AI/ML-based CSI feedback, focus on FR2.

Proposal 5: For AI/ML-based beam management, in addition to AI-specific KPIs, the following KPIs can be considered:

- Intermediate KPI: Probability of identifying the best beam (pair), average power loss between the selected beam (pair) and the optimal beam (pair), and the ratio of overhead reduction;

- Final KPI: System-level SE/throughput or the SE/throughput loss compared to that of beam-sweeping approach.

- References

- RP-213599, New SI: Study on Artificial Intelligence (AI/ML)/Machine Learning (ML) for NR Air Interface, RAN #94-e.

- R1-2203450, Discussion on AI/ML framework for air interface, CATT, RAN1#109-e, e-Meeting, May 9th – 20th.

- R1-2203454, Discussion on other aspects on AI/ML for beam management, CATT, RAN1#109-e, e-Meeting, May 9th – 20th.