3GPP TSG RAN WG1 #109-e                                     R1-2203454

e-Meeting, May 9th – 20th, 2022

Source: 	CATT

Title:	Discussion on other aspects on AI/ML for beam management

Agenda Item:	9.2.3.2

Document for:	Discussion and Decision

- Introduction

In RAN #94 e-meeting, AI/ML for NR Air Interface was agreed and the following objectives were approved in the SID [1]:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In this document, sub use cases and possible spec impacts for AI/ML-based beam management are analyzed.

- Discussion

  - Motivation

Conventional beam management usually exhaustively sweeps all the transmitting and receiving beam pairs. It relies on measurement on all possible beam pairs and selecting the best ones, which may be an optimal beam training scheme. In 5G NR, beam alignment between gNB and UE is crucial for operation at mmWave band. Three procedures are defined in NR to achieve the beam alignment.

•	Joint Tx/Rx beam sweeping (P-1): Both gNB and UE perform beam sweeping. Specifically, each candidate Tx beam is transmitted N times by one side to allow the other side to test up to N different Rx beams. The N transmission instances of a given Tx beam can be enabled on time domain over N slots, or across N different OFDM symbols.

•	Tx beam sweeping (P-2): gNB performs Tx beam sweeping, while UE fixes its Rx beam.

•	Rx beam sweeping (P-3): UE performs Rx beam sweeping, while gNB fixes Tx beam.

Take P-1 procedure as an example. A gNB capable of transmitting M analog beams can configure up to M reference signals for beam measurement. Each reference signal is beamformed with its associated analog beam pointing to a particular direction. These M reference signals are presumably transmitted in different time instants to allow gNB to tune the phase shifter for each Tx beam. The analog beam applied to each reference signal may also be held fixed for a certain number of time units, thus providing an opportunity for the UE to test different RX beams for a given TX beam. In this manner, up to MN beam pairs shall be measured to find out one or more recommended beam pairs. That is, up to MN reference signals shall be transmitted to establish the beam alignment between gNB and UE. With the increasing number of beams, it becomes challenging because of the high overhead and computational complexity.

It is desired to reduce the measurement overhead by AI/ML-based approach. In most cases, measuring all beam pairs is unnecessary, as some beam pairs are highly correlated. AI/ML model can be trained to learn the spatial relationship among the beams and then figure out the best beam pairs based on partial measurements. Furthermore, the measurements results are also correlated in time domain. AI/ML model can be employed to predict the best beam(s) in the future by utilizing time domain relationship and historical beam measurement information.

  - Sub use cases

Three possible sub use cases are further elaborated in the following.

- Sub use case 1: Narrow beam prediction based on wide beam measurement

Wide beams can cover the whole beam space area with a relative smaller number of beams than narrow beams. There is a certain spatial correlation between wide beams and narrow beams. If the information of narrow beams can be obtained from wide beam measurement, the overhead of measurements for narrow beams can be significantly reduced or even avoided. From AI/ML point of view, beam selection problem can be treated as an image reconstruction problem. The information (e.g. RSRP of the corresponding reference signal) of wide beams and narrow beams are viewed as low-resolution and high-resolution beam domain images, respectively. In the field of image processing, the basic principle of super-resolution is to reconstruct the image based on the correlation information of adjacent pixels, by extracting high-frequency information. Therefore, an AI model can be trained to obtain the corresponding high-resolution image with an input of low-resolution image.

Similarly, an AI/ML model can be trained to find the optimal narrow beam by the wide beam measurement information. The principle is illustrated in Figure 1 and Figure 2. In Figure 2, the input of AI/ML model is the low resolution image representing the wide beams information matrix, i.e., the measured values of widebeam#1, 2 and 3 in Figure 1. The output is a high-resolution image representing the narrow beam information matrix, for example, the best narrow beam index or quality information of all narrow beams in Figure 1.

Figure 1  Beam configuration for narrow beam prediction based on wide beam measurement.

Figure 2  The structure of narrow beam prediction based on wide beam measurement.

Specifically, AI/ML model training and inference can be conducted at one sides of UE or network, as shown in Figure 3 and Figure 4. In Figure 3, the AI/ML model is deployed at UE side. With the input of measured wide beams, the information of the narrow beams can be inferred by the AI/ML model. The output of the AI/ML model, i.e. inferred narrow beam information, can be feedback to gNB.

Figure 3  AI/ML model of narrow beam prediction based on wide beam at UE.

In Figure 4, the AI/ML model is deployed at gNB side. The wide beam information reported by UE is used as the input of AI/ML model. Narrow beam information can be derived from the output of the AI/ML model.

Figure 4  AI/ML model of narrow beam prediction based on wide beam at gNB.

- Sub use case 2: Beam prediction based on partial beam measurement

AI/ML model can be used for obtaining the information of all narrow beams by measurement of partial beams, due to the spatial relevance between them. Similar to sub use case 1, partial beams and all beams can be regarded as low-resolution and high-resolution beams. Once the partial beams information is obtained, the information of all beams can be reconstructed by a well-trained AI/ML model.

The principle is illustrated in Figure 5 and Figure 6. The input of AI model in Figure 6 is the low-resolution image representing part of the beam information matrix, i.e., the measured values of narrowbeams#3, #5 and #7 in Figure 5. Through the AI/ML model, the output is the high-resolution image representing the information matrix of all narrow beams, for example, the best narrow beam index of all narrow beams or inferred information of all narrow beams (from #1 to #9) in Figure 5.

Figure 5  Beam configuration of beam prediction based on partial beam measurement.

Figure 6  The structure of beam prediction based on partial beam measurement.

Concretely, AI/ML model training and inference can be conducted at one sides of UE or network, as shown in Figure 7 and Figure 8. In Figure 7, the AI/ML model is deployed at UE side. With the input of measured partial beams, the information of all narrow beams can be inferred by the AI/ML model. The inferred information of all narrow beams can be feedback to gNB.

Figure 7  AI/ML model of all beam prediction based on partial beam measurement at UE.

In Figure 8, the AI/ML model is deployed at gNB side. Information of partial beam reported by UE is used as the input of AI/ML model. Information of all beams can be derived from the output of the AI/ML model.

Figure 8  AI/ML model of all beam prediction based on partial beam measurement at gNB.

- Sub use case 3: Beam prediction in time domain

AI/ML model can be utilized to predict the beam information of a time instant based on the previous beam information. This is because AI/ML model can extract the temporal correlation of beams in different time instants. The historical time-domain information can provide additional information to improve the prediction accuracy.

The structure is illustrated in Figure 9. An AI/ML model utilizes the historical beam information at time {t0,t1,t2,…, tN} to predict the beam information at time {tN+1,…,tN+X}.

Figure 9  The structure of beam prediction in time domain model.

For this sub use case, AI/ML model training and inference can be conducted at one sides of UE or network, as shown in Figure 10 and Figure 11. In Figure 10, the AI/ML model is deployed at UE side. In Figure 11, the AI/ML model is deployed at gNB side.

Figure 10  AI/ML model of beam prediction based on historical beam information at UE.

Figure 11  AI/ML model of beam prediction based on historical beam information at gNB.

In summary, we propose to consider the above three sub use cases for Rel-18 AI/ML-based beam management.

Proposal 1: The following sub use cases can be considered in Rel-18 AI/ML-based beam management:

- Narrow beam prediction based on wide beam measurement.

- All beam prediction based on partial beam measurement.t

- Beam prediction in time domain.

  - Spec impacts

Specification impact on AI/ML-based approach highly depends on the collaboration level [2]. In general, we have the following consideration regarding to AI/ML-based beam management.

For AI model management, signaling and procedure of AI model training, updating and fallback may need to be specified, except for the case of no collaboration between UE and the network.

For AI model synchronization and beam reporting:

- In case of no collaboration between UE and the network, no spec impact is expected.

- In case AI/ML model is deployed in at least one side (especially at UE side) and known by the other side, spec impact may include:

  - Interface of AI model, i.e. relationship between measured RS and reported information;

  - New procedure for RS measurement and reporting;

  - Signaling and procedure on exchanging non-AI-related assistance information;

  - Signaling and procedure on exchanging AI-related assistance information;

Potential spec impacts for AI/ML based beam management are summarized as follows:

Proposal 2: The following spec impact of AI/ML based beam management can be considered:

- Signaling/procedure of AI model training/updating/fallback;

- Interface of AI model, i.e. relationship between measured RS and reported information;

- New procedure for RS measurement and reporting;

- Signaling/procedure design on exchanging AI-related/non-AI-related assistance information.

- Conclusions

In this contribution, we provided our views on sub use cases for AI/ML based beam management. We have the following proposals:

Proposal 1: The following sub use cases can be considered in Rel-18 AI/ML-based beam management:

- Narrow beam prediction based on wide beam measurement.

- All beam prediction based on partial beam measurement.t

- Beam prediction in time domain.

Proposal 2: The following spec impact of AI/ML based beam management can be considered:

- Signaling/procedure of AI model training/updating/fallback;

- Interface of AI model, i.e. relationship between measured RS and reported information;

- New procedure for RS measurement and reporting;

- Signaling/procedure design on exchanging AI-related/non-AI-related assistance information.

- References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #94-e.

- R1-2203450, Discussion on AI/ML framework for air interface, CATT, RAN1#109-e, e-meeting, May 9th – 20th.