3GPP TSG RAN WG1 #109-e	                                     R1-2203455

e-Meeting, May 9th – 20th, 2022

Source: 	CATT

Title:	Discussion on evaluation on AI/ML for positioning

Agenda Item:	9.2.4.1

Document for:	Discussion and Decision

- Introduction

In RAN #94 e-meeting, AI/ML for NR Air Interface was agreed and the following objectives were approved in the SID [1]:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In this document, we share our views on evaluation on AI/ML-based positioning.

- Discussion

  - Methodology

The evaluation methodology common to all sub use cases is discussed in our companion paper [2]. When some traditional function of the wireless network is replaced by AI/ML-based module, the output of the AI/ML module can be evaluated from functional point of view. We denote such evaluation as ‘intermediate evaluation’. The output of AI/ML module will be utilized in signal processing procedure to acquire the final results, which can be denoted as ‘final evaluation’.

Positioning use case focuses on positioning accuracy enhancement for different scenarios including heavy NLOS conditions. In our companion paper [3], AI/ML-based intermediate measurement estimation, AI/ML-based LOS/NLOS identification and end-to-end positioning based on AI/ML are proposed to be considered in Rel-18 discussion.

- For AI/ML-based intermediate measurement estimation, traditional algorithms can be replaced by AI/ML module for ToA/AoA/AoD estimation. The AI/ML-based ToA/AoA/AoD estimation can be evaluated by its output, i.e., ‘intermediate evaluation’. The AI/ML-abased ToA/AoA/AoD estimation is also used to acquire the final UE position, i.e., ‘final evaluation’.

- For AI/ML-based LOS/NLOS identification, AI/ML module is used to replace these typical solutions to identify LOS/NLOS, and the AI/ML-based LOS/NLOS identification can be evaluated by its output, i.e., ‘intermediate evaluation’. The identified LOS/NLOS is also utilized to acquire the final UE position, i.e., ‘final evaluation’.

- For end-to-end positioning based on AI/ML, AI/ML module is used to directly infer UE position without intermediate ToA/AoA/AoD estimation, and thus AI/ML module is evaluated by the final positioning accuracy and this evaluation is an AI/ML-based ‘final evaluation’.

Based on the analysis above, we have the following proposal:

Proposal 1: For AI/ML-based positioning, the evaluation methodology is as follows:

- For AI/ML-based intermediate measurement estimation, consider evaluating the result of ToA/AoA/AoD estimation as intermediate evaluation, and evaluating the positioning accuracy as final evaluation.

- For AI/ML-based LOS/NLOS identification, consider evaluating the LOS/NLOS identification as intermediate evaluation, and evaluating the positioning accuracy as final evaluation.

- For end-to-end positioning based on AI/ML, consider evaluating the positioning accuracy as final evaluation.

In Rel-16, various positioning technologies were specified to support regulatory as well as commercial use cases, and the positioning requirements include high accuracy, high availability and low latency. In Rel-17, the target horizontal positioning requirements were < 1 meter (for 90% of UEs) for commercial scenario and < 0.2 meter (for 90% of UEs) for industrial internet of things (IIoT) scenario. To fulfill the requirement of higher accuracy, lower latency, higher reliability in IIoT scenario, some positioning enhancements have been introduced in Rel-17. In IIoT scenario, NLOS and multipath phenomena are universal due to the blockage of obstacles. The amount of LOS paths from TRPs to a given UE is mostly less than 4, which seriously degrades the positioning performance. In addition, when the network synchronization errors between TRPs are considered, the positioning accuracy will be further degraded. Under these circumstances, the traditional positioning schemes are difficult to meet the target positioning requirements. Meanwhile, AI/ML-based approaches are good at solving non-linear issues and seems promising in this scenario. Thus, in the study of AI/ML-based positioning, IIoT scenario should at least be included.

In Rel-17, IIoT scenarios such as InF-HH, InF-SH, InF-SL, InF-DH and InF-DL were defined and studied. Specifically, for InF-DH and InF-DL, the number of LOS paths is nearly to 0 due to the heavy blockage of obstacles. Thus, these heavy NLOS scenarios, e.g. InF-DH and InF-DL, should be considered firstly in the AI/ML-based positioning evaluation.

Proposal 2: For AI/ML-based positioning evaluation in Rel-18, the scenarios of InF-DH and InF-DL should be considered.

Note that several positioning methods were supported in Rel-16, and some positioning enhancements have been carried out in Rel-17. When comparing the performance of AI/ML-based positioning, the baseline should be some existing positioning technologies in Rel-16/Rel-17, such as DL-TDOA, UL-TDOA, UL-AOA, etc.

Proposal 3: For AI/ML-based positioning evaluation, the baseline for comparing AI/ML model performance is positioning methods specified in Rel-16/Rel-17.

  - Dataset

AI/ML approaches are data-driven and dataset is essential to the performance of AI/ML model. General aspects related to dataset are discussed in our companion paper [2]. Completeness and availability are important to construct dataset. For positioning, in order to explore the ability of AI/ML-based approaches in evaluation, the dataset for training, validating and testing can be constructed based on 3GPP statistic models in TR 38.857. Field data can be considered to further verify the effectiveness of AI/ML in real world. The applied dataset constructed by field data needs to be stable and available, and allows cross checking among different companies. One possible way is that different companies provide some field data to construct an open field data set.

Proposal 4: For AI/ML-based positioning, 3GPP statistic models in TR 38.857 are used to construct the dataset in the first stage, and field data is considered in the next stage.

Reasonable proportion between the training data, validation data and testing data is the basis for verifying the performance of AI/ML model, and the partitioning of dataset is discussed in [2]. In addition, the construction of dataset will also impact evaluation results. When constructing dataset for subsequent AI/ML evaluation, the following aspects should be considered.

- Dataset without non-ideal factors

The amount of data in the dataset will affect the final positioning accuracy. When the positioning performance is poor in some scenarios, it is difficult to confirm whether the positioning error is caused by the insufficient data or non-ideal factors such as synchronization errors between TRPs. Therefore, it is preferred to construct a dataset without non-ideal factors to access the ability of AI/ML positioning when the data is sufficient.

- Dataset with non-ideal factors

In current IIoT scenario, non-ideal factors will deteriorate the positioning accuracy, e.g. heavy NLOS, UE Rx/Tx and/or gNB Rx/Tx timing delays, synchronization errors between TRPs and so on. In order to access whether the AI/ML-based positioning has excellent ability to overcome non-ideal factors to improve positioning accuracy, the dataset with non-ideal factors should be considered.

- Dataset containing data from same or different TRPs

Generally, if different TRPs can use the same AI/ML model, or the AI/ML model trained by the dataset of one TRP can achieve good performance for another TRP, it shows that the AI/ML model has good generalization performance. In order to verify the generalization ability, we can compare the performance of the following AI models in single/multi-TRP scenario: (1) an AI model trained by a dataset constructed by data from only one TRP, and (2) an AI model trained by dataset constructed by mixing data from multiple TRPs.

- Dataset containing data from same or different scenarios

The IIoT scenarios of 3GPP include InF-HH, InF-SH, InF-SL, InF-DH and InF-DL. When applying an AI/ML model with poor generalization ability in different IIoT scenarios, there may be significant performance gap. But if an AI/ML model can be applied to different simulation scenarios, e.g. InF-DL and InF-DH, with acceptable positioning accuracy, it shows that the AI/ML model has good generalization ability. Considering that the dataset has deep impact to the AI/ML performance, we can consider constructing the dataset with data from same or different scenarios, to better understand the generalization ability of an AI/ML model.

Proposal 5: For evaluating AI/ML-based positioning, the construction of dataset should consider the data with or without non-ideal factors, data from same/different TRP(s) and data from same/different scenario(s).

  - KPI

The AI-specific KPIs are discussed in our companion paper [2], e.g. size of AI model, computation complexity & latency, generalization ability and overhead of exchanging AI-specific signaling. In addition, the conventional KPIs for AI/ML-based positioning should be taken into consideration. For AI/ML models with different output types, the KPIs are different. For AI/ML-based intermediate measurement estimation, the KPIs are intermediate KPI and final KPI, and the intermediate KPI is the accuracy of estimated measurement results (e.g. error of ToA/AoA/AoD) and the final KPIs are the positioning accuracy (e.g. positioning error at 90% CDF). For AI/ML-based LOS/NLOS identification, the intermediate KPI is correct rate of identified LOS/NLOS and the final KPI are positioning accuracy. For end-to-end positioning based on AI/ML, the KPI is positioning accuracy.

Proposal 6: For AI/ML-based positioning, in addition to AI-specific KPIs, the following KPIs are considered:

- The accuracy of intermediate measurement results (e.g. error of ToA/AoA/AoD) if AI/ML-based intermediate measurement estimation is applied.

- The correct rate of LOS/NLOS identification if AI/ML-based LOS/NLOS identification is applied.

- The final positioning accuracy (e.g. positioning error at 90% CDF).

- Conclusions

In this contribution, we provided our views on AI/ML for positioning. We have the following observations and proposals:

Proposal 1: For positioning, the evaluation methodology is as follows:

- For AI/ML-based intermediate measurement estimation, consider evaluating the result of ToA/AoA/AoD estimation as intermediate evaluation, and evaluating the positioning accuracy as final evaluation.

- For AI/ML-based LOS/NLOS identification, consider evaluating the LOS/NLOS identification as intermediate evaluation, and evaluating the positioning accuracy as final evaluation.

- For end-to-end positioning based on AI/ML, consider evaluating the positioning accuracy as final evaluation.

Proposal 2: For AI/ML-based positioning evaluation in Rel-18, the scenarios of InF-DH and InF-DL should be considered.

Proposal 3: For AI/ML-based positioning evaluation, the baseline for comparing AI/ML model performance is positioning methods specified in Rel-16/Rel-17.

Proposal 4: For AI/ML-based positioning, 3GPP statistic models in TR 38.857 are used to construct the dataset in the first stage, and field data is considered in the next stage.

Proposal 5: For evaluating AI/ML-based positioning, the construction of dataset should consider the data with or without non-ideal factors, data from same/different TRP(s) and data from same/different scenario(s).

Proposal 6: For AI/ML-based positioning, in addition to AI-specific KPIs, the following KPIs are considered:

- The accuracy of intermediate measurement results (e.g. error of ToA/AoA/AoD) if AI/ML-based intermediate measurement estimation is applied.

- The correct rate of LOS/NLOS identification if AI/ML-based LOS/NLOS identification is applied.

- The final positioning accuracy (e.g. positioning error at 90% CDF).

- References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#94-e, e-Meeting, December 6th – 17rd, 2021.

- R1-2003450, Discussion AI/ML framework for air interface, CATT, RAN1#109-e, e-Meeting, May 9th - 20th.

- R1-2003456, Discussion on other aspects on AI/ML for positioning, CATT, RAN1#109-e, e-Meeting, May 9th - 20th.