3GPP TSG RAN WG1 #109-e	                                     R1-2203456

e-Meeting, May 9th – 20th, 2022

Source: 	CATT

Title:	Discussion on other aspects on AI/ML for positioning

Agenda Item:	9.2.4.2

Document for:	Discussion and Decision

- Introduction

In RAN#94 e-meeting, AI/ML for NR Air Interface was agreed the following objectives were approved in the SID [1]:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In this document, we share our views on the sub use cases for AI/ML-based positioning enhancement.

- Discussion

  - Motivation

In Rel-16, various positioning technologies were specified to support regulatory as well as commercial use cases. To address the higher accuracy positioning requirements from new applications and industry verticals, NR Positioning in Rel-17 evaluated and specified enhancements and solutions to meet the following exemplary performance targets [2]:

(a) For general commercial use cases (e.g., TS 22.261): - sub-meter level position accuracy (< 1 m) (b) For IIoT Use Cases (e.g., 22.804): - position accuracy < 0.2 m The target latency requirement is < 100 ms; for some IIoT use cases, latency in the order of 10 ms is desired.

In Rel-17, some positioning enhancements have been introduced, e.g. UE Rx/Tx and/or gNB Rx/Tx timing delays mitigation, multipath/NLOS mitigation, UL-AoA and DL-AoD positioning accuracy improvement and so on. However, in current IIoT scenario, it is still worth thinking about overcoming the following non-ideal factors to further improve the positioning accuracy:

- Heavy NLOS

- UE Rx/Tx and/or gNB Rx/Tx timing delays

- Synchronization errors between multiple TRPs

- Low Rice factor

In Rel-18, in order to better overcome these non-ideal factors, positioning accuracy enhancement based on AI/ML is identified as one use case in the initial use cases set. AI/ML-based positioning is beneficial to improve position accuracy and even predict UE position in future. As a new positioning technology, AI/ML-based positioning has data-driven characteristics. When AI/ML-based approach is applied in positioning, the non-ideal factors mentioned above may be included in the corresponding training data set and test data set. If the non-ideal factors are captured in the training set, AI/ML model would be able to learn how to overcome those non-ideal factors to improve positioning accuracy. By learning the internal relationship between the training data set with non-ideal factors and the corresponding labels, the AI/ML model can overcome the influence of non-ideal factors on the test data set, so as to improve the position accuracy.

Observation 1: In IIoT scenario, AI/ML-based positioning has promising ability to overcome non-ideal factors to improve positioning accuracy.

  - Sub use cases

For traditional positioning methods, delay and angle estimation is widely used to support UE-based and UE/BS-assisted positioning. AI/ML module can be used to replace an intermedium function for ToA/AoA/AoD estimation. It can also be used to realize end-to-end positioning without intermedium ToA/AoA/AoD estimation. In addition, AI/ML-based LOS/NLOS identification, positioning accuracy improvement based on historical information could also be considered for positioning enhancements.

- Sub use case 1: AI/ML is used to replace an intermedium function for positioning

Among the currently supported positioning methods, DL-TDOA, UL-TDOA and Multi-RTT perform position calculation based on time domain measurement, and DL-AoD and UL-AoA perform position calculation based on spatial domain measurement. The time and spatial measurements is an intermedium quantity for positioning. Then the AI/ML model can be used to replace traditional algorithms to estimate time and spatial measurements such as ToA/AoA/AoD, and the traditional methods are used to perform UE position calculation based on estimated ToA/AoA/AoD, as shown in Figure 1. The AI/ML model may be deployed at UE, gNB or LMF side.

Figure 1: AI/ML-based function for ToA/AoA/AoD estimation

Another possible solution is to use AI/ML model to perform UE position calculation, and the input of the AI/ML model is ToA/AoA/AoD estimated by traditional methods. However, the non-ideal factors inevitably deteriorate the accuracy of ToA/AoA/AoD estimation by traditional methods. By learning the internal relationship between the ToA/AoA/AoD estimated by traditional methods and the corresponding UE positions, the negative impact from non-ideal factors on positioning accuracy may also be reduced.

- Sub use case2: AI/ML-based LOS/NLOS identification

Additional delay of NLOS may affect the determination of first path and sharply reduce the ToA estimation accuracy, thereby deteriorating the final positioning accuracy. In Rel-17 positioning enhancement, several typical NLOS and multipath identification solutions were proposed, e.g. LOS/NLOS identification information can be defined as a function of Rice factor in the time domain, the variance of CFR in the frequency domain, or the combination of the above two parameters. The LOS/NLOS identification information can help identify and select LOS links between TRPs and UE for mitigating the influence of NLOS. Soft values with [0, 0.1, …, 0.9, 1] and hard values with [0, 1] were introduced for LOS/NLOS indicator reporting from UE/TRP. AI/ML can be used to replace these typical solutions to identify LOS/NLOS. The input of AI/ML model is channel information like CIR or CFR, as shown in Figure 2. For LOS/NLOS indicator reporting with soft values, the output of AI/ML model is the probability of LOS or NLOS, e.g., the AI/ML model is a neural network and the output is the probability of LOS. For LOS/NLOS indicator reporting with hard values, and the output of AI/ML model is the categorization of LOS or NLOS, which is a typical classification problem. The identified LOS or NLOS can be used to support reporting of LOS/NLOS indicators and assist in the final position calculation.

Figure 2: AI/ML-based LOS/NLOS identification

- Sub use case 3: End-to-end positioning based on AI/ML

AI/ML model is used to realize end-to-end positioning without intermedium ToA/AoA/AoD estimation, and the input of AI/ML model can be channel information such as channel impulse response (CIR) and channel frequency response in frequency domain (CFR), and the output of AI/ML model is the UE position. As shown in Figure 3, CIR is assumed as the input of AI/ML model, and the AI/ML model can be deployed at UE or LMF side.

Figure 3: End-to-end positioning based on AI/ML

- Sub use case 4: AI/ML-based positioning using historical time-domain information

For positioning, the locations of UE in time domain are highly correlated and continuous, and the historical time-domain information provides additional information to improve the positioning accuracy or predict the UE position in future. In Figure 4 a), historical CIR at {t0, t1, t2, …, tN-1} in additional to CIR at tN is utilized to improve the positioning accuracy at tN. In Figure 4 b), historical CIR at {t0, t1, t2, …, tN-1, tN} is used to predict the UE position at {tN+1, …, tN+X}, which is conducive to the PRS/SRS overhead reduction and the positioning latency reduction. In addition, the prediction of UE position provides some useful information to the application layers, such as predicting the possibility of crash in future and sending out alarm prompts, and some services/devices can be intelligently turned on/off according to the prediction of UE position in smart home scenario.

- Positioning accuracy improvement

- Prediction of UE position

Figure 4: AI/ML-based positioning using historical time-domain information

For the above sub use cases, it is preferred to focus on ToA/AoA/AoD estimation, AI/ML-based LOS/NLOS identification and end-to-end positioning based on AI/ML in Rel-18 research firstly. And AI/ML-based positioning using historical time-domain information can be taken as possible enhancements for the next stage of research.

Proposal 1: Consider the following sub use cases in Rel-18 AI/ML-based positioning:

- AI/ML-based intermediate measurement estimation, e.g. ToA/AoA/AoD estimation.

- AI/ML-based LOS/NLOS identification.

- End-to-end positioning based on AI/ML.

  - Collaboration level of sub use cases

In SID, the following collaboration levels are identified.

- Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation.

According to the interaction degree between UE and gNB for AI inference and/or training, four collaboration levels can be considered as Collaboration Level#0, Level#1, Level#2 and Level#3 in [3]. For the proposed sub use case1, sub use case 2 and sub use case 3, the AI/ML model related training, inference and assisted information exchange corresponding to the above four Collaboration Levels are analyzed in Table 1.

Table 1: Analysis on collaboration levels of proposed sub use cases

| Collaboration levels | Sub use case 1: AI/ML-based ToA/AoA/AoD estimation | Sub use case 2: AI/ML-based LOS/NLOS identification | Sub use case 3: End-to-end positioning based on AI/ML |
| --- | --- | --- | --- |
| Level#0 | AI/ML related inference is conducted at UE/gNB side and transparent to other sides. |  |  |
| Level#1 | AI/ML related training and inference are conducted at UE/gNB/LMF side, additional antenna information, gNB position coordinates, and UE capability may be needed. | AI/ML related training and inference are conducted at UE/gNB/LMF side, additional antenna information, gNB position coordinates/ indications and UE capability may be needed. | AI/ML related training and inference are conducted at UE /LMF side, additional antenna information, gNB position coordinates, and UE capability may be needed. |
| Level#2 | AI/ML model is trained at gNB/LMF side and provided to UE for inference, the type of AI/ML model, hyper-parameters and specific parameters may be exchanged. | AI/ML model is trained at gNB/LMF side and provided to UE for inference, the type of AI/ML model, hyper-parameters and specific parameters may be exchanged. | AI/ML model is trained at LMF side and provided to UE for inference, the type of AI/ML model, hyper-parameters and specific parameters may be exchanged. |
| Level#3 | AI/ML model is trained at one side, and AI/ML inference is jointly conducted at different sides. |  |  |

Proposal 2: In Rel-18 AI/ML-based positioning, the following collaboration levels between UE and network can be considered:

- Level#0: No collaboration. AI model at one side is transparent to the other side.

- Level#1: Inference is conducted at one side with exchanging non-AI-related assistance signal information.

- Level#2: Inference is conducted at one side with exchanging AI-related assistance signal information.

- Level#3: Inference is jointly conducted at both sides with exchanging AI-related assistance signal information for AI model alignment/synchronization.

  - Spec impacts

As the discussion in section 2.3, the collaboration level corresponding to level#1 level#2 and level#3 will impact the specification since the exchange of assistance data and signalling. The collaboration level is related to the specific sub use cases, so the sub use cases and corresponding collaboration levels should be discussed and agreed first, and then the spec impacts are further identified.

Proposal 3: The specification impacts can be further considered after the sub use cases and the corresponding collaboration level are clear.

- Conclusions

In this contribution, we provided our views on AI/ML for positioning. We have the following observations and proposals:

Observation 1: In IIoT scenario, AI/ML-based positioning has promising ability to overcome non-ideal factors to improve positioning accuracy.

Proposal 1: Consider the following sub use cases in Rel-18 AI/ML-based positioning:

- AI/ML-based intermediate measurement estimation, e.g. ToA/AoA/AoD estimation.

- AI/ML-based LOS/NLOS identification.

- End-to-end positioning based on AI/ML.

Proposal 2: In Rel-18 AI/ML-based positioning, the following collaboration levels between UE and network can be considered:

- Level#0: No collaboration. AI model at one side is transparent to the other side.

- Level#1: Inference is conducted at one side with exchanging non-AI-related assistance signal information.

- Level#2: Inference is conducted at one side with exchanging AI-related assistance signal information.

- Level#3: Inference is jointly conducted at both sides with exchanging AI-related assistance signal information for AI model alignment/synchronization.

Proposal 3: The specification impacts can be further considered after the sub use cases and the corresponding collaboration level are clear.

- References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #94-e.

- RP-202094, Revised SID: Study on NR positioning Enhancements, RAN #89-e.

- R1-2203450, Discussion on AI/ML framework for air interface, RAN1#109-e.