3GPP TSG RAN WG1 #109-e                                                                                R1-2203457

e-Meeting, May 9th – 20th, 2022

Source:	CATT

Title:	Views on UE capability of AI/ML for air interface

Agenda Item:	9.2.5

Document for:	Discussion and Decision

Introduction

Artificial Intelligence (AI) and Machine Learning (ML) are powerful in solving non-linear issues, and become one of the most popular research directions around the world. For wireless communication, AI/ML also attracts strong interests from academic circle, and already shows its capability in improving performance in many aspects. 3GPP also finished a RAN3-led AI/ML study in Rel-17, in which AI models are applied for better data collections in several typical use cases, including network energy saving, load balancing, and mobility optimization [1].

In Rel-18, a study item on AI/ML in RAN1 was approved, to facilitate the support of AI/ML in physical layer other than implementation-based approaches. In the study, the objectives focus on finalizing the sub use case, characterizing the terminology and general framework, evaluating the AI/ML-based approach with aligned methodology, and analyze the potential spec impacts. The detailed description can be found in [2]. In this contribution, we additionally provide our views on the UE capability of supporting AI/ML for air interface.

Discussion

Capability definition for AI-specific approaches

Nowadays, the prosperity of AI/ML-based approaches should be credited to the development of computing power.  On one hand, the size of AI model largely impacts the capability of fitting and solving non-linear issues. An AI model with larger size is generally more capable to tackle complicated issues. On the other hand, larger size of AI model requires higher computation capability for inference within a required latency. This is quite sensitive in wireless communication system, in which the channel condition is fast varying and the processing delay must be satisfied. As a result, it is often assumed that XPU (e.g. NPU or GPU) is implemented to facilitate the application of AI/ML-based approach.

As discussed in [3], when applied in air interface, AI model may be deployed at UE side or network side or both sides. In general, deploying AI model at network side is simpler and promising. It can release the UE burden and ease the co-scheduling of AI/ML-based UEs and non-AI/ML-based UEs of gNB. However, deploying AI model at UE side is still possible, especially in the use case of CSI feedback. Thus additional requirement on storage and computation is directly raised for UE, which should be carefully studied.

Assume that different AI models are deployed at UE for different use case/purpose. The key issue is that, all these AI models share the storage and computing power of UE hardware. This makes AI/ML-based approaches a little ‘exclusive’: with limited buffering and computation capability, if one AI model is deployed, another AI model may not be able to deployed. Of course, if the UE is very capable, it may support multiple AI/ML-based approaches at the same time.

All in all, it is beneficial to consider different levels of capabilities on supporting AI/ML-based approaches. These AI-specific levels may consider the following dimensions:

- Storage/buffering size

The capability of storage/buffering size may impact: (1) the number of AI models that can be supported/configured to the UE, and (2) the size of each AI model that can be supported/configured.

- Computation power

The capability of computation power may impact: (1) the number of simultaneous active AI models, and (2) the inferring/training latency subject to a specific size of AI model.

Note that, the above levels should be associated with quantized reference capabilities. For example, several levels of storage may be defined by a few thresholds of buffering size in a unit of Mbytes. For another example, different computation levels may be defined by different ranges in a unit of FLOPS or TOPS. AI-specific capabilities can be defined by one or both dimensions above.

Proposal 1: For support of AI/ML, consider defining several levels of UE capabilities based on quantized computation and/or buffering capability.

In addition, it is also possible that AI-specific capabilities will also be related to the supported collaboration level between UE and network. This can be further studied based on the common understanding of collaboration levels.

Resilience of AI/ML-based approaches

AI/ML-based approaches are computational methods. The performance of AI/ML-based approach highly depends on not only the AI model structure but also the training dataset. From study point of view, it is reasonable to generate the training dataset (including validating dataset and testing dataset) by statistic air interface models, i.e. channel model in 3GPP TR 38.901 and TR 38.857. However, the environment of real world channel is more complicated than simulated one. Even if the AI model is tested by a few field data, whether its validity always holds in the real world is still unclear.

For example, overfitting is one concerned issues for AI/ML-based approach. Empirically, overfitting may happen when the AI model size is unnecessarily huge while the training data set is too small. But if the AI model is too small, it may not be capable to address a complicated issue. This raises a question on how to ensure the robustness of the performance of UE and network when AI/ML-based approaches are applied.

One possible way is to set up online training/tuning mechanism, which allows network or UE to update the AI model. However, updating AI model may require longer time than legacy RRC update, since the new AI model should be sufficiently trained, validated and tested before being deployed. Therefore, efficient enabling/fallback between AI/ML-based approach and existing deterministic approach should be considered.

Figure 1 Enabling/fallback between AI/ML-based approach and legacy approach.

Proposal 2: Efficient enabling/fallback between AI/ML-based approaches and legacy mechanisms should be considered.

Trade-off  between generalization and performance

In academic world, the effectiveness of AI/ML-based approaches in wireless communication has already been proven. For a specific purpose in a concerned scenario, AI/ML-based approach often outperforms the existing deterministic approach. Meanwhile, it is noticed that in academic studies, training data and testing data are often achieved in the same or similar way, e.g. constructed by simulation with independent and identically distributed assumption. However, when this condition is not met, the performance may be degraded. One example is AI/ML-based positioning in the factory. An AI model trained by a data set collected from one factory may not be very outstanding in another factory.

Still, it is desired to improve the average performance of AI model in different scenarios, i.e. generalization. This is a practical demand from implementation point of view. Hence, when evaluating AI/ML-based approaches, the trade-off between generalization and performance should be carefully studied.

Observation 1: When evaluating the effectiveness of AI/ML-based approaches, the trade-off between generalization and performance should be carefully studied.

Conclusion

In this contribution, we provide our additional views on support of AI/ML for air interface. The observation and proposals are summarized as follows:

Observation 1: When evaluating the effectiveness of AI/ML-based approaches, the trade-off between generalization and performance should be carefully studied.

Proposal 1: For support of AI/ML, consider defining several levels of UE capabilities based on quantized computation and/or buffering capability.

Proposal 2: Efficient enabling/fallback between AI/ML-based approaches and legacy mechanisms should be considered.

References

- TR 37.817, Study on enhancement for Data Collection for NR and EN-DC, v 17.0.0.

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#94-e, e-Meeting, December 6th – 17rd, 2021.

- R1-2203450, Discussion on AI/ML framework for air interface, CATT, RAN1#109-e, e-Meeting, May 9th – 20th.