3GPP TSG RAN WG1 #109-e		                                                      R1-2203459

e-Meeting, May 9th – 20th, 2022

Source:	CATT

Title:	Discussion on subband non-overlapping full duplex

Agenda Item:	9.3.2

Document for:	Discussion and Decision

Introduction

For Rel-18 NR duplex evolution, the following objectives were given in SID [1].

- Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

In this contribution, we give some initial considerations for subband non-overlapping full duplex in Rel-18 NR duplex evolution.

Discussion

Subband non-overlapping Full Duplex (SBFD) allows simultaneous existence of downlink and uplink on non-overlapping subbands at gNB side within a conventional TDD band while half-duplex operation is assumed at UE side. Consequently, new interference scenarios compared with semi-static/dynamic/flexible TDD are introduced, including self-interference (SI), inter-subband CLI and potentially intra-subband CLI, which may impact the feasibility and benefit of SBFD.

Although DL and UL transmissions are separated on different subbands, there would be strong self-interference due to large difference between transmission and reception power at gNB side. We provided preliminary analysis on the gap between the received power of self-interference and the existing BS in-band blocking requirement for FR1 in Table 1.

Table 1: Gap between the received SI power and the existing BS in-band blocking requirement (FR1)

| Parameter | Value |  |
| --- | --- | --- |
|  | Indoor | Macro |
| BS Tx power (dBm) | 24 | 49 |
| BS Tx antenna gain (dBi) | 0 | 8 |
| BS Rx antenna gain (dBi) | 0 | 8 |
| Rx self-interference power (dBm) | 24 | 65 |
| BS in-band blocking requirement (dBm) | -35 | -43 |
| Gap (dB) | 59 | 108 |

It is observed that self-interference would lead to BS in-band blocking without self-interference mitigation. Therefore, solutions including spatial isolation and analog domain interference mitigation are needed to avoid BS in-band blocking. In addition, digital domain interference mitigation can be applied to further reduce the leakage of inter-subband interference as shown in Figure 1. RAN4 should study and inform RAN1 the feasibility, capability and impact of self-interference mitigation solutions.

Figure 1: self-interference mitigation

For UL, in addition to self-interference, there are at least inter-band inter-sector and inter-gNB CLI as shown in Figure 2. For inter-gNB CLI, the propagation loss helps to reduce the received interference power. For inter-sector CLI, the interference power depends on the antenna isolation between different sectors and potential analog domain and digital domain interference mitigation solutions.

Figure 2: inter-sector/inter-gNB CLI

Proposal 1: RAN4 study and inform RAN1 the feasibility, capability and impact of spatial isolation and analog/digital domain interference mitigation for self-interference, inter-sector CLI and inter-gNB CLI.

For DL, even if assuming the subband configurations across different cells are the same, the UE-to-UE inter-subband CLI may lead to UE in-band blocking as shown in Figure 3 based on the preliminary analysis in Table 2 assuming existing UE in-band blocking requirement for FR1.

Figure 3: UE-to-UE inter-subband CLI

Table 2: UE Rx interference power vs. in-band blocking requirement (FR1)

| Parameter | Value |
| --- | --- |
| UE Tx power (dBm) | 23 |
| UE Tx/Rx antenna gain (dB) | 0 |
| Path loss (dB) [TR38.802] | 54 (3m) |
| Rx interference power (dBm) | -31 |
| UE in-band blocking requirement (dBm) | -56 |

It is not clear whether UE RF improvements are feasible to mitigate the UE-to-UE inter-subband CLI. In addition, RAN1 should discuss whether UE-to-UE CLI handling schemes introduced in Rel-16 are applicable/sufficient for UE-to-UE inter-subband CLI handling.

Proposal 2: RAN4 study and inform RAN1 the feasibility to mitigate the UE-to-UE inter-subband CLI based on UE RF improvements.

Proposal 3: RAN1 discuss whether Rel-16 UE-to-UE CLI handling schemes are applicable/sufficient for UE-to-UE inter-subband CLI handling.

We think a legacy NR UE should be able to be served by a cell operating SBFD. In addition, the UE capability for supporting SBFD, if defined, is expected to be optional. Therefore not all Rel-19 and later release UEs are capable of supporting SBFD. Those UEs should be able to be served by a cell operating SBFD as well.

Proposal 4: A cell operating subband non-overlapping full duplex (SBFD) should be able to serve legacy UEs and new UEs without SBFD capability if defined.

In general, we think there are two possible modes for UE operating in a SBFD cell, i.e. UE transparent mode and UE non-transparent mode as follows.

- UE transparent mode

In this mode, UE does not need to know whether a cell is operating SBFD and all UEs can operate in this mode. Legacy NR schemes for UEs to determine a transmission direction can be reused, e.g., RRC configuration, SFI indication or DCI indication. As illustrated in the left figure in Figure 4, gNB can configure the symbols with both downlink and uplink transmissions as flexible symbols and configure/schedule UE to transmit or receive on these symbols.

- UE non-transparent mode

In this mode, SBFD operation is not transparent to SBFD capable UEs. For other UEs including legacy UE and UEs without SBFD capability, UE should still operate in transparent mode. Then it is possible that a SBFD UE can transmit or receive on a respective DL or UL symbol as shown in the right figure in Figure 4. But for other non-SBFD UEs, the UL subband cannot be used. Note that gNB can also configure the symbols with both downlink and uplink transmissions as flexible symbols in UE non-transparent mode.

Figure 4: UE transparent mode vs. UE non-transparent mode

It is our view that although UE transparent mode only is feasible, there are some restrictions on gNB configuration/scheduling as shown in Figure 5, including:

- Configuration of both DL reception and UL transmission on the same semi-static flexible symbol(s) are not allowed.

- UL transmissions on SSB symbol(s) are not supported.

- Configuration of DL reception/UL transmission in UL/DL subband is not allowed unless SFI or dynamic scheduling is used to overwrite the configured reception/transmission.

- Other restriction on configurations of PUCCH, CSI-RS, etc.

- Potential restriction on PDSCH scheduling due to UL subband.

In addition, for UE transparent mode, no UE RF improvements e.g. TX filtering can be utilized to minimize inter-subband interference.

Figure 5: restriction of UE transparent mode

Observation 1: There are some restrictions to support SBFD only in UE transparent mode, such as restriction on gNB configuration/scheduling and/or UE RF improvements etc.

Therefore, we propose to study whether/how to support UE non-transparent mode for SBFD operation. To be more specific, RAN1 should discuss and decide whether to support UE non-transparent mode based on the benefit of UE non-transparent mode compared with UE transparent mode. And if supported, RAN1 should also discuss how to support UE non-transparent mode.

Proposal 5: Study whether/how to support SBFD in UE non-transparent mode, including:

- Potential benefit compared with UE transparent mode

- Configuration/indication to the SBFD UE

- Potential enhancements for transmission direction determination for SBFD UE

- Potential enhancements for physical channel/signal transmission/reception for SBFD UEs

Conclusion

In this contribution, we discuss subband non-overlapping full duplex and give the following observation and proposals.

Observation 1: There are some restrictions to support SBFD only in UE transparent mode, such as restriction on gNB configuration/scheduling and/or UE RF improvements etc.

Proposal 1: RAN4 study and inform RAN1 the feasibility, capability and impact of spatial isolation and analog/digital domain interference mitigation for self-interference, inter-sector CLI and inter-gNB CLI.

Proposal 2: RAN4 study and inform RAN1 the feasibility to mitigate the UE-to-UE inter-subband CLI based on UE RF improvements.

Proposal 3: RAN1 discuss whether Rel-16 UE-to-UE CLI handling schemes are applicable/sufficient for UE-to-UE inter-subband CLI handling.

Proposal 4: A cell operating subband non-overlapping full duplex (SBFD) should be able to serve legacy UEs and new UEs without SBFD capability if defined.

Proposal 5: Study whether/how to support SBFD in UE non-transparent mode, including:

- Potential benefit compared with UE transparent mode

- Configuration/indication to the SBFD UE

- Potential enhancements for transmission direction determination for SBFD UE

- Potential enhancements for physical channel/signal transmission/reception for SBFD UEs

Reference

- RP-220633, Revised SID: Study on evolution of NR duplex operation, CMCC, RAN #95-e.